3GPP RAN WG2 Meeting #131	R2-2505907

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.4.3

Source:	InterDigital, Inc.

Title:	Remaining issues on RRM measurement relaxation and offloading

Document for:	Discussion

# 1. Introduction

In RAN2#130 following agreement were made [1].

The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

In this contribution, we discuss RRM relaxation of UE MR (Main Radio) with offloading condition from MR to LR (Low power wake-up Receiver).

# 2. Discussion

Offloading and relaxed RRM measurement

In RAN4#112bis, RAN4 discussed RRM relaxation and serving cell measurement offloading (e.g., fully offloading and relaxed measurement) and the following agreements were made [2]

Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON.

Offloading measurement

RAN2 agreed that when an offloading condition is satisfied, serving cell measurement is performed by LR (fully offloaded from MR) and serving cell measurement via MR is not required. In this case, it is desired to deactivate MR for additional power saving. However, when the serving cell measurement is offloaded from MR to LR, there may be no serving cell measurement via MR.

Hence, the UE is not able to determine whether the legacy Rel-16 relaxation condition for neighboring cell measurements is satisfied due to absence of the serving cell measurement via MR (e.g., offloading case). To handle this issue, measured value (e.g., LP-SS) of the serving cell measurement via LR can be used to determine whether to satisfy the conditions of relaxed measurement for neighboring cell measurement (while the offloading condition is satisfied).

Proposal 1: For offloading case, RAN2 agrees the current working assumption for neighboring cell measurement relaxation with LR measurement (WA: FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available).

Proposal 2: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R19 criteria) can be determined by serving cell measurement results via LR.

Relaxed measurement

RAN2 discussed the partially offloading case (e.g., Case #3/Relaxed case, defined by RAN4) and made agreements. But, it is still FFS on exit condition whether to include MR measurement. In our view, we prefer a unifed exit condtion with LR-based measurement. If we consider a separate exit condtion for partial offloading case with MR measurement, it may increase complexity to combine two different measurement results, e.g., LR measurement and relaxed MR measurement.

Morevoer, there is another FFS that if the entry condition for serving relaxation is the same as neighbour cell RRM measurement relaxation. In our view, each condition is designed with different scenario and criterion, For example, the entry condition is specified in Rel-19 LP-WUS for serving cell relaxation and neigboring cell measurement relaxation is specified in Rel-16 (e.g., ‘UE not at cell edge’ or ‘UE with low mobility’). Hence, there is no reason to configure as the same condition due to different objectives.

For Rel-19 offloading and relaxed criteria, UE with low mobility condition can be supported with reusing legacy Rel-16 defined criterion. The low mobility UE can perform offloading or relaxed RRM measurement according to low mobility conditions which are configured by network.

Proposal 3: For relaxed case, support the exit condition based on LR measurement.

Proposal 4: Do not support merging criteria (e.g., thresholds) for Rel-19 entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation.

Proposal 5: Support low mobility criteria both offloading and relaxed RRC mesasurement.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: For offloading case, RAN2 agrees the current working assumption for neighboring cell measurement relaxation with LR measurement (WA: FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available).

Proposal 2: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R19 criteria) can be determined by serving cell measurement results via LR.

Proposal 3: For relaxed case, support the exit condition based on LR measurement.

Proposal 4: Do not support merging criteria (e.g., thresholds) for Rel-19 entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation.

Proposal 5: Support low mobility criteria both offloading and relaxed RRC mesasurement.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #130.

[2] Chairman’s notes, 3GPP TSG RAN WG4 #112bis.