3GPP TSG-RAN WG2 Meeting #131	R2-2505912

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.1.2.2

Source:	Nokia, Qualcomm, Ericsson, T-Mobile

Title:	On Unifying and Simplifying ASN.1 and Procedures for AI/ML

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses the unification and simplification of data structures and procedures related to the configuration of inference, applicability determination based on sets of inference-related parameters, and the provision of candidates for UE-side data collection. The first aspect to be discussed is the relationship between the configuration for inference, and inference-related parameters and candidates for UE-side data collection. Then, we discuss how to write a universal procedure, applicable to all RRC-configured use cases, for handling the configuration and reporting of applicability based on inference-related parameters.

While the only RRC-configured use case supported in Release 19 is beam management (BM), Release 20 will introduce CSI prediction, CSI compression, and mobility. Ideally, we will not need to redefine behaviours to implement CSI prediction and CSI compression and introducing AI/ML mobility should reuse as much of the work we have already done as is possible.

In this paper, we propose to unify data structures based on the configuration IE used for a use case and to unify procedures for providing data collection configuration candidates for UE-side data collection and reporting of applicability based on inference-related parameters.

# 2	Discussion

## 2.1	Relationship between inference, inference-related parameters and candidates for UE-side data collection

Inference, applicability determination and UE-side data collection are inherently related to one another and the configuration of each uses the same collection of parameters. Therefore, we would like to avoid the replication of parameters across the three functions by reusing the inference configuration container for sets of inference-related parameters and for the provision of options for UE-side data collection both for specification efficiency and to reduce the significant chance for error, e.g., inconsistent field descriptions in copied parameters, omission of a parameter which is to be copied from one IE to another, or the redefinition of a parameter with a different data format.

Observation 1: Configurations for inference, sets of inference-related parameters and UE-side data collection options are inherently related, and all of the parameters necessary for inference-related parameters and UE-side data collection options are wholly contained inside the configuration IE for inference, e.g., CSI-ReportConfig.

The topic of which container to use has been discussed previously. For the provision of sets of inference-related parameters, RAN2#129 [1] reached the following agreement to “reconsider CSI-ReportConfig for option B, for example if the list of inference related parameters is fully contained within existing CSI-ReportConfig,” where “Option B” refers to applicability determination based on sets of inference-related parameters.

RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list).

Potential aspects to consider if RAN2 revisit:

-    To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig.

-    to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

During RAN1#121 [2] the following was agreed regarding the sets of inference-related parameters, the parameters of which have been underlined in the agreement below.

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

- associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

- TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present

Lastly, the parameter list provided after RAN1#121, contributed in R1-2503243 [3], shown in Table 2.1-1, indicates that all of these parameters will be present inside of CSI-ReportConfig. An abbreviated version of the spreadsheet, containing only the relevant fields has been reproduced below for reference. The leftmost column repeats the field names from the RAN2#120 agreement copied above to check for presence.

Table 2.1-1: Parameters selected as inference-related parameters for beam management compared to the higher layers parameters list from RAN2#121 [3]

| RAN1#121 Parameter | Parameters List after RAN1#121 |  |  |
| --- | --- | --- | --- |
| Inference-Related Parameter | Sub-feature group | RAN2 Parent IE | Parameter name in the spec |
| associatedIDforSetA-r19 | BM-UE side | CSI-ReportConfig | associatedIdforSetA-r19 |
| resourcesForSetA-r19 | BM-UE side | CSI-ReportConfig | resourcesForSetA-r19 |
| resourcesForChannelMeasurement | Legacy | CSI-ReportConfig | resourcesForChannelMeasurement |
| associatedIDforSetB-r19 | BM-UE side | CSI-ReportConfig | associatedIdforSetB-r19 |
| reportQuantity-r19 | BM-UE side | CSI-ReportConfig | reportQuantity-r19 |
| reportConfigType | Legacy | CSI-ReportConfig | reportConfigType |
| nrofreportedpredictedrs-r19 | BM-UE side | CSI-ReportConfig | nrofreportedpredictedrs-r19 |
| TimeGap-r19 | BM-UE side | CSI-ReportConfig | TimeGap-r19 |
| nroftimeinstance-r19 | BM-UE side | CSI-ReportConfig | nroftimeinstance-r19 |

Observation 2: RAN2#129 agreed to reconsider the use of CSI-ReportConfig for option B if all of the inference-related parameters are contained in CSI-ReportConfig.

Observation 3: RAN1#121 agreed to the following list of parameters to be reused for signaling sets of inference-related parameters: associatedIDforSetA-r19, resorucesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, and nroftimeinstance-r19, whose presence in CSI-ReportConfig has been confirmed in the parameters list provided after RAN1#121.

We recognize that the presence of extension markers and irrelevant optional parameters in some large IEs such as CSI-ReportConfig introduces some overhead, but in our analysis that overhead was limited to an additional 8 bytes in the case of CSI-ReportConfig: 2 bytes of optional parameters and 6 bytes of extension markers. We do not envision that the UE will be provided so many sets of inference-related parameters and UE-side data collection options such that the 8 bytes of overhead would accumulate to a level comparable to the maximum size of an RRC message, which is 9000 bytes.

Observation 4: The overhead incurred by reusing CSI-ReportConfig for signaling sets of inference-related parameters and UE-side data collection options is 8 bytes.

Proposa1 l: Use CSI-ReportConfig as the IE to configure the UE with sets of inference-related parameters, each CSI-ReportConfig representing one set of inference-related parameters.

Next, we will address the candidates for UE-side data collection. During RAN2#130 [4], the following agreement was reached regarding candidate data collection configurations.

For beam management, candidate data collection configuration includes at least:

- CSI-ResourceConfigId of Set A

- CSi-ResourceConfigId of Set B

- One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements

FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

Observation 5: During RAN2#130, it was agreed that the following parameters are included in candidate data collection configurations: CSI-ResourceConfigId of Set A, CSI-ResourceConfigId of Set B, and one or two associated IDs, e.g., associatedIdforSetB-r19 and associatedIdforSetB-r19.

All of the parameters agreed in RAN2#130 are also present in the list of parameters provided in Table 2.1-1, which means that all of the parameters for candidate data collection configurations are present in CSI-ReportConfig.

Observation 6: Every parameter of UE-side candidate data collection configurations exists in CSI-ReportConfig.

Therefore, using the same reasoning as was used to reuse CSI-ReportConfig as the configuration IE for sets of inference-related parameters, we can propose to use CSI-ReportConfig as the container to configure the UE with candidate data collection configurations.

Proposal 2: Use CSI-ReportConfig as the IE to configure the UE with UE-side candidate data collection configurations, each CSI-ReportConfig representing one UE-side candidate data collection configuration.

## 2.2	Universal procedure for configuring and reporting applicability, including sets of inference-related parameters

During RAN2#130 [4], the following agreements were reached pertaining to applicability reporting. The agreements suggest unifying the behavior for Option A – applicability determination based on full inference configuration and Option B – applicability determination based on a set of inference-related parameters for the following functions.

- Enabling and disabling applicability reporting through UAI

- Reporting applicability in RRCReconfigurationComplete and UAI

- Configuring Option A and Option B simultaneously in the same RRCReconfiguration message

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

- (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

- (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

Taking the identified functions, we will analyse the impact to the specification.

- Enabling and disabling applicability reporting through UAI – Only a single flag will be used in OtherConfig, applying to both Option A and Option B.

- Reporting applicability in RRCReconfigurationComplete and UAI – A single applicability report IE will be defined, supporting reporting on Option A and Option B across all use cases.

- Configuration Option A and Option B simultaneously in the same RRCReconfiguration message – An Option A configuration, such as a CSI-ReportConfig would be provided in CSI-MeasConfig->csi-ReportConfigToAddModList and an Option B configuration, reusing the CSI-ReportConfig IE would be provided in a list of sets of inference-related parameters in OtherConfig-v19->applicabilityReportConfig-r19 in the same RRCReconfiguration message.

First, we will discuss how to unify the configuration of sets of inference-related parameters regardless of use case.

The applicabliityReportConfig-r19, shown below, will include two main fields: one field enabling or disabling applicability reporting, and a second including configurations of inference-related parameters, which can be provided for supported use cases. We show the addition of the list of sets of inference-related parameters.

ApplicabilityReportConfig-r19 ::=                   SEQUENCE {

applicabilityReportingEnabled   ENUMERATED {enabled, disabled},

inferenceRelatedSetList-r19     SEQUENCE (SIZE(1..maxNrofInferenceRelatedSets)) OF InferenceRelatedSet-r19 OPTIONAL

}

Using a high-level container for sets of inference-related parameters, i.e., InferenceRelatedSet-r19, a single procedure can be defined to report applicability based on inference-related parameters for all current and future use cases and would reuse the same set of IDs identifying sets of inference-related parameters.

Observation 7: Using a high-level container for sets of inference-related parameters, i.e., InferenceRelatedSet-r19, a single procedure can be defined to report applicability based on inference-related parameters for all current and future use cases and would reuse the same set of IDs identifying sets of inference-related parameters.

The InferenceRelatedSet-r19, shown below, includes an inferenceRelatedSetId-r19, which will be referred to in the eventual applicability report. The inferenceRelatedSetConfig-r19 is a CHOICE wherein different configuration types can be provided by the gNB. In this example, we only have CSI-ReportConfig since the only RRC-based Rel-19 use cases are configured with CSI-ReportConfig. In the future, there are two potential options for extensibility: include extension markers in inferenceRelatedSetConfig-r19 or include extension markers to add new lists in the future, e.g., inferenceRelatedSetConfig-r20.

InferenceRelatedSet-r19 ::= SEQUENCE {

inferenceRelatedSetId-r19     InferenceRelatedSetId-r19,

inferenceRelatedSetConfig-r19   CHOICE {

csi-ReportConfig                CSI-ReportConfig

} OPTIONAL,

-- Optional so that we do not have to write exceptions when we add new types.

-- The field description can indicate that only one version of the IE can be used.

...

}

InferenceRelatedSetId-r19 ::= INTEGER (0..maxNrofInferenceRelatedSets-1-r19)

Proposal 3: The CSI-ReportConfig configured in inferenceRelatedSetConfig-r19 is not to be passed to lower layers for configuration. That is, there shall be no RAN1 impact due to the reuse of CSI-ReportConfig.

Proposal 4: Define a new IE called InferenceRelatedSet-r19, which is configured with an identifier, inferenceRelatedSetId-r19 for use as a reference in the applicability report and a configuration containing a set of inference-related parameters, inferenceRelatedSetConfig-r19, which will minimally include CSI-ReportConfig as one of the choices.

The next issue to address is that the CSI-ReportConfig includes many fields, mandatory and optional, which are not relevant to the determination of applicability based on sets of inference-related parameters. To solve this problem, we propose to use field descriptions indicating which fields are relevant. An example is shown below for the case of beam management case 1 (BM-Case1).

inferenceRelatedSetConfig
If the field is set to CSI-ReportConfig: if reportQuantity is set to p-CRI-RSRP, then only the following fields are to be used: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19.

Proposal 5: The field description for inferenceRelatedSetConfig-r19 will be used to specify which fields are relevant for the applicability determination using sets of inference-related parameters based on the type of configuration.

Finally, the procedure for reporting the applicability of sets of inference-related parameters would then be generic to all use cases since all sets of inference-related parameters would use the same type of ID, i.e., inferenceRelatedSetId-r19, and because all sets of inference-related parameters are subject to the applicability determination procedure.

Applicability is reported via the ApplicabilityReportConfigIdList-r19 IE defined in the draft running RRC CR and is shown below. To accommodate the reporting of applicability of sets of inference-related parameters, inferenceRelatedSetId was added to ApplicabilityReportConfigId-r19 below.

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..maxNrofServingCellsFFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19                  ServCellIndex,

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19     OPTIONAL,

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19          CHOICE {

csi-ReportConfigId                         CSI-ReportConfigId,
       inferenceRelatedSetId                      InferenceRelatedSetId,

spare7                                     NULL,

spare6                                     NULL,

spare5                                     NULL,

spare4                                     NULL,

spare3                                     NULL,

spare2                                     NULL,

spare1                                     NULL

}

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                                      OPTIONAL,

...

}

Proposal 6: Add inferenceRelatedSetId as a choice in applicabilityReportConfigId-r19 under ApplicabilityReportConfigIdList-r19 to support the reporting of the applicability of sets of inference-related parameters.

There ApplicabilityReport-r19 contains a list called applicabilityReportConfigIdList-r19, which is a sequence of IEs with the same name, ApplicabilityReportConfigIdList-r19. The ApplicabilityReport-r19 includes the serving cell index, applicabilityCellId-r19, but we think this could be moved to the IE containing the applicabilityReportConfigId-r19 and applicabilityStatus-r19 as the savings of reporting the ServCellIndex once instead of per report is minimal.

Proposal 7: Move applicabilityCellId-r19 to the ApplicabilityReportConfigIdList-r19.

Then, the IE ApplicabilityReport-r19 can be a simple sequence of ApplicabilityReportConfigIdList IEs. Furthermore, ApplicabilityReportConfigIdList-r19 could be renamed to ApplicabilityInformation-r19 for clarity. The Final ASN.1 prototype would read as follows.

ApplicabilityReport-r19 ::= SEQUENCE (SIZE (1..maxNrofApplicabilityInformation)) OF
        ApplicabilityInformation-r19

ApplicabilityInformation-r19 ::=    SEQUENCE {

applicabilityCellId-r19                  ServCellIndex,

applicabilityReportConfigId-r19          CHOICE {

csi-ReportConfigId                         CSI-ReportConfigId,
       inferenceRelatedSetId                      InferenceRelatedSetId,

spare6                                     NULL,

spare5                                     NULL,

spare4                                     NULL,

spare3                                     NULL,

spare2                                     NULL,

spare1                                     NULL

}

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                                      OPTIONAL,

...

}

Figure 2.2-1: Simplified ASN.1 structure for applicability reporting

Proposal 8: RAN2 to discuss adopting the ASN.1 definition of Figure 2.2-1 to simplify the reporting of applicability, reporting the applicability of configurations pertaining to any cell in the same flat list.

The following procedure could be used to generically report the applicability of sets of inference-related parameters. Note that it is FFS whether the UE can request to release the configuration of a set of inference-related parameters as this could cause unnecessary overhead if the sets of inference-related parameters do not use an AddMod/Release.

Observation 8: It might not be useful to allow a UE to request the release of a set of inference-related parameters and could cause unnecessary overhead if the sets of inference-related parameters are not configured with an AddMod / Release.

1> if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to the applicability determination procedure according to 5.7.4.2:

2> include ApplicabilityReport in this UEAssistanceInformationMessage;

2> if inferenceRelatedSetList is present in ApplicabilityReportConfig:

3> for each inferenceRelatedSetId associated to a InferenceRelatedSet:

4> include an entry in ApplicabilityReport in the UEAssistanceInformation message, and set the content as follows:

5> set applicabilityCellId-r19 to the serving cell index of the cell;

5> set applicabilityReportConfigId to the corresponding inferenceRelatedSetId;

5> set applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

5> (FFS) if the applicabilityStatus is set to inapplicable:

6> (FFS) include releaseConfigurationPreferenceif the UE prefers to release the configuration;

Proposal 9: RAN2 to discuss adopting a generic procedure to support the reporting of the applicability of sets of inference-related parameters as exemplified in the provided procedural text prototype.

## 2.3	Universal procedure for configuring and reporting UE-side data collection preferences

A similar approach as was described for implementing the applicability reporting based on sets of inference-related parameters can be used to implement the provision of candidates for UE-side data collection and the reporting of preference for one or more candidates.

The IE DataCollectionPreferenceConfig-r19 has yet to be defined since it is still under discussion. However, given our arguments from Section 2.1 and the relevant RAN1 and RAN2 agreements, we think that the following ASN.1 definition could be used as a starting point, mimicking the approach used for sets of inference-related parameters.

The DataCollectionPreferenceConfig-r19 will contain a field enabling or disabling the procedure for UE to provide UE-side data collection options and a list of data collection options, i.e., dataCollectionCandidateConfigList-r19.

DataCollectionPreferenceConfig-r19 ::=                   SEQUENCE {

<other configuration fields>

dataCollectionCandidateConfigList-r19    SEQUENCE (SIZE (1..maxNrofDataCollectionCandidateConfig-r19)) OF DataCollectionCandidateConfig-r19

}

As with the reporting of applicability of sets inference-related parameters, the UE-side data collection preferences would also be identified by an ID, i.e., dataCollectionCandidateConfigType-r19, which could be configured with the appropriate container depending on the use case. In the example below, CSI-ReportConfig is shown as on option, and additional options could be added by including extension markers in the dataCollectionCandidateConfigType-r19 or by including extension markers in DataCollectionCandidateConfig-r19 to enable the addition of new dataCollectionCandidateConfigTypes in the future.

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionCandidateConfigId-r19   DataCollectionCandidateConfigId,

dataCollectionCandidateConfigType-r19  CHOICE {

csi-ReportConfig                         CSI-ReportConfig

} OPTIONAL, 
  -- Optional so that we do not have to write exceptions when we add new types.

-- The field description can indicate that only one version of the IE can be used.

...

}

DataCollectionCandidateConfigId ::= INTEGER (0..maxNrofDataCollectionCandidateConfigs-1-r19)

As with InferenceRelatedSet-r19, a field description would be provided for the field dataCollectionCandidateConfigType-r19 to describe which fields of a particular configuration type, e.g., CSI-ReportConfig are relevant for the UE to evaluate a candidate UE-side data collection configuration.

Observation 9: As with InferenceRelatedSet-r19, a field description would be provided for the field dataCollectionCandidateConfigType-r19 to describe which fields of a particular configuration type, e.g., CSI-ReportConfig are relevant for the UE to evaluate a candidate UE-side data collection configuration.

Proposal 10: Adopt the structure of DataCollectionCanddiateConfig-r19 defined in the provided ASN.1 example, which includes a dataCollectionCandidateConfigId and a dataCollectionCandidateConfigType, which is a choice between different configuration types which could be used to indicate preference for UE-side data collection.

The running CR for 38.331 defined an extension to UEAssistanceInformation containing dataCollectionPreference-r19, which is currently defined as shown below.

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStartStop-r19                      ENUMERATED {start, stop}                   OPTIONAL,

dataCollectionPreferredConfiguration-r19         FFS                                        OPTIONAL,

...

}

The DataCollectionPreference-r19 includes the fields dataCollectionStartStop-r19 to initiate or terminate a preference for a candidate for UE-side data collection and dataCollectionPreferredConfiguration-r19. Whether or not it is eventually agreed to support more than one candidate data collection preference, we can still discuss the contents of dataCollectionPreferredConfiguration-r19. Given our preferred definition for DataCollectionCandidateConfig, we think that the dataCollectionPreferredConfiguration-r19 should point to a dataCollectionCandidateConfigId. The modified ASN.1 shown below implements this change.

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStartStop-r19                        ENUMERATED {start, stop}                   OPTIONAL,

dataCollectionPreferredConfigurationId-r19         DataCollectionCandidateConfigId                                        OPTIONAL,

...

}

If this ASN.1 is agreed, the procedure for configuring the UE with candidate UE-side data collection configurations and reporting them could be written in a similarly generic way to that described in Section 2.2 for reporting the applicability of sets of inference-related parameters.

Proposal 11: The UE will report its preference for a candidate for UE-side data collection by sending a UEAssistanceInformation message. FFS how to signal, e.g., 1) UE indicates list of preferred configuration(s), wherein the presence indicates “start”, and the absence indicates “stop” 2) UE indicates a start list and a stop list, each containing a list of zero or more data collection candidate configurations or 3) UE indicates a list of data collection candidate configurations, each with a corresponding “start” or “stop” indication.

## 2.4 Summary

In summary, we have proposed to use CSI-ReportConfig to configure the UE with sets of inference-related parameters for applicability reporting and to configure the UE with candidates for UE-side data collection. To simplify the specification and future extensions to new configuration types for applicability reporting and UE-side data collection, an outer-level IE has been proposed for each, such that a single type of ID would be used to report the applicability of a set of inference-related parameters and another single type of ID would be used to report preferred candidates for UE-side data collection. Where possible, the data structures have been flattened for simplicity.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: Configurations for inference, sets of inference-related parameters and UE-side data collection options are inherently related, and all of the parameters necessary for inference-related parameters and UE-side data collection options are wholly contained inside the configuration IE for inference, e.g., CSI-ReportConfig.

Observation 2: RAN2#129 agreed to reconsider the use of CSI-ReportConfig for option B if all of the inference-related parameters are contained in CSI-ReportConfig.

Observation 3: RAN1#121 agreed to the following list of parameters to be reused for signaling sets of inference-related parameters: associatedIDforSetA-r19, resorucesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, and nroftimeinstance-r19, whose presence in CSI-ReportConfig has been confirmed in the parameters list provided after RAN1#121.

Observation 4: The overhead incurred by reusing CSI-ReportConfig for signaling sets of inference-related parameters and UE-side data collection options is 8 bytes.

Observation 5: During RAN2#130, it was agreed that the following parameters are included in candidate data collection configurations: CSI-ResourceConfigId of Set A, CSI-ResourceConfigId of Set B, and one or two associated IDs, e.g., associatedIdforSetB-r19 and associatedIdforSetB-r19.

Observation 6: Every parameter of UE-side candidate data collection configurations exists in CSI-ReportConfig.

Observation 7: Using a high-level container for sets of inference-related parameters, i.e., InferenceRelatedSet-r19, a single procedure can be defined to report applicability based on inference-related parameters for all current and future use cases and would reuse the same set of IDs identifying sets of inference-related parameters.

Observation 8: It might not be useful to allow a UE to request the release of a set of inference-related parameters and could cause unnecessary overhead if the sets of inference-related parameters are not configured with an AddMod / Release.

Observation 9: As with InferenceRelatedSet-r19, a field description would be provided for the field dataCollectionCandidateConfigType-r19 to describe which fields of a particular configuration type, e.g., CSI-ReportConfig are relevant for the UE to evaluate a candidate UE-side data collection configuration.

And proposed the following:

Proposa1 l: Use CSI-ReportConfig as the IE to configure the UE with sets of inference-related parameters, each CSI-ReportConfig representing one set of inference-related parameters.

Proposal 2: Use CSI-ReportConfig as the IE to configure the UE with UE-side candidate data collection configurations, each CSI-ReportConfig representing one UE-side candidate data collection configuration.

Proposal 3: The CSI-ReportConfig configured in inferenceRelatedSetConfig-r19 is not to be passed to lower layers for configuration. That is, there shall be no RAN1 impact due to the reuse of CSI-ReportConfig.

Proposal 4: Define a new IE called InferenceRelatedSet-r19, which is configured with an identifier, inferenceRelatedSetId-r19 for use as a reference in the applicability report and a configuration containing a set of inference-related parameters, inferenceRelatedSetConfig-r19, which will minimally include CSI-ReportConfig as one of the choices.

Proposal 5: The field description for inferenceRelatedSetConfig-r19 will be used to specify which fields are relevant for the applicability determination using sets of inference-related parameters based on the type of configuration.

Proposal 6: Add inferenceRelatedSetId as a choice in applicabilityReportConfigId-r19 under ApplicabilityReportConfigIdList-r19 to support the reporting of the applicability of sets of inference-related parameters.

Proposal 7: Move applicabilityCellId-r19 to the ApplicabilityReportConfigIdList-r19.

Proposal 8: RAN2 to discuss adopting the ASN.1 definition of Figure 2.2-1 to simplify the reporting of applicability, reporting the applicability of configurations pertaining to any cell in the same flat list.

Proposal 9: RAN2 to discuss adopting a generic procedure to support the reporting of the applicability of sets of inference-related parameters as exemplified in the provided procedural text prototype.

Proposal 10: Adopt the structure of DataCollectionCanddiateConfig-r19 defined in the provided ASN.1 example, which includes a dataCollectionCandidateConfigId and a dataCollectionCandidateConfigType, which is a choice between different configuration types which could be used to indicate preference for UE-side data collection.

Proposal 11: The UE will report its preference for a candidate for UE-side data collection by sending a UEAssistanceInformation message. FFS how to signal, e.g., 1) UE indicates list of preferred configuration(s), wherein the presence indicates “start”, and the absence indicates “stop” 2) UE indicates a start list and a stop list, each containing a list of zero or more data collection candidate configurations or 3) UE indicates a list of data collection candidate configurations, each with a corresponding “start” or “stop” indication.

# References

[1] R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, ETSI MCC

[2] R1-250xxxx, Draft Report of 3GPP TSG RAN WG1 #121 v0.2.0 (St Julian’s, Malta, May 19th – 23rd, 2025)

[3] R2-2503314, LS on Rel-19 higher layers parameters list Post RAN1#120bis

[4] R2-250xxxx, Draft Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta, ETSI MCC