3GPP TSG-RAN WG2 #131	R2-2505915

India, Bangalore, August 25th – 29rd, 2025

Agenda item:	   8.3.2.3

Source:		Lenovo

Title:                             UE side data collection for AI Mobility

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI/ML for mobility SI will focus on mobility enhancement in RRC_CONNECTED mode over the air interface by following the existing mobility framework. In this paper, we discuss essential data collection aspects for UE-sided model, considering RRM measurement prediction and measurement event prediction use cases.

# Discussion

For training UE-side models, requires UE to measure and log relevant data which can be transferred to the training entity (e.g., UE-side server). Configuration from the network for data collection is necessary to enable the UE to measure and log the data for training. As highlighted in the AI mobility SI description, potential mobility-specific enhancements should be based on the R19 AI/ML-air interface WID general framework. Thus, the general principles of data collection agreed in AI/ML-air interface WI can be extended to AI for Mobility: RRM measurement and event prediction use cases.

Different options for data collection, considering different first termination entities for data transfer, were discussed in AI/ML for air interface WI, and respective requirements for data collection and data transfer procedure have been defined. Currently, SA2 is studying the feasibility of data transfer via the core network, and further RAN2 progress is dependent on the conclusion of SA2 study.

- Agreements - R19 AI/ML-air interface

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

As agreed in the last meeting, the above agreements made for R19-AI/ML air interface, related to data collection configuration and initiation of data collection, are taken as a baseline for AI Mobility use cases. As the UE may need to collect data for certain configurations, such as set A/set B configuration, it should be possible for UE to request a preferred data collection configuration/set of parameters to the network.

Proposal 1: Network can configure UE to request preferred data collection configurations or a set of parameters for model training. FFS if UE can request based on associated ID(s).

The following agreements were made in the scope of network side data collection for AI for mobility in RAN2 #130. Most of these agreements are applicable to network side data collection as well to UE side data collection.

Agreements (AI for Mobility – network side data collection)

For the collection of data for the training of a network-sided model for RRM measurement prediction:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

- L3 cell level measurements,

- L3 beam level measurements,

- L1-filtered beam level measurements (if sub-case 1 and 3 is supported)

- Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback)

- Time info (if as agreed by AI/ML and/or if needed)

- The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase.   Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

- The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case.

- The availability indication can be triggered due to:

- full buffer being reached (if configured), (FFS if buffer is per use case)

- buffer threshold being reached (if configured), or (FFS if buffer is per use case)

- low power (if configured)

- Upon sending the availability indication, UE indicates:

- Data is available

- Reason for the triggering of the availability indication (full buffer. Threshold)

- Low power indication

Proposal 2: RAN2 extends the following agreements to UE side data collection:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

  - L3 cell level measurements,

  - L3 beam level measurements,

  - L1-filtered beam level measurements (if sub-case 1 and 3 is supported)

  - Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback)

  - Time info (if as agreed by AI/ML and/or if needed)

- The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase.   Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

How logged data for UE-side model is handled must be treated differently compared to the network-side data collection. As the training entity in this case is not the gNB, the data transfer is directed towards the core network directly, based on option 2 from R19-AI/ML air interface. Although further progress is dependent on SA2 conclusions, general aspects related to data collection configuration and data logging can be discussed. For network-side data collection, UE triggers data availability indication for logged data to the network based on buffer being full, buffer threshold being fulfilled or low power state, whereas whether a data availability indication or similar indication is required for UE-side data collection depends on the potential solution for data transfer procedure. However, it is important that the logged data for network-side data collection and logged data for UE-side data collection are maintained separately at the UE. The data availability indication and buffer status/buffer threshold for network side model should not include data logged for UE-side model. As per RAN2 #130 AIML-air interface agreements, the data is collected on per data logging configuration basis and UE indicates data logging configuration ID to the network. UE should be able to differentiate the configuration for network-side data collection and UE-side data collection, such that it can manage the storage of respective data accordingly.

Proposal 3: RAN2 clarifies whether the UE needs to distinguish between data collection configurations for UE-side data collection and network-side data collection.

Proposal 4: Data logged for UE-side data collection is stored and managed separately from data logged for network-side data collection.

Proposal 5: Data is collected on per data collection configuration basis, and UE indicates an identifier related to logged data (or data collection configuration).

# Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: Network can configure UE to request preferred data collection configurations or set of parameters required for model training. FFS if UE can request based on associated ID(s).

Proposal 2: RAN2 extends the following agreements to UE side data collection:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

  - L3 cell level measurements,

  - L3 beam level measurements,

  - L1-filtered beam level measurements (if sub-case 1 and 3 is supported)

  - Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback)

  - Time info (if as agreed by AI/ML and/or if needed)

- The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase.   Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

Proposal 3: RAN2 clarifies whether the UE needs to distinguish between data collection configurations for UE-side data collection and network-side data collection.

Proposal 4: Data logged for UE-side data collection is stored and managed separately from data logged for network-side data collection.

Proposal 5: Data is collected on per data collection configuration basis, and UE indicates an identifier related to logged data (or data collection configuration).

# References

[1] RP-250971, TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

[3] TS 38.300 running CR for AI for air interface