3GPP TSG RAN WG2 Meeting #131	R2-2505956

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.8.3

Source:	CMCC

Title:	Remaining issues on uplink capacity/throughput enhancement for NR NTN

Document for:	Discussion, Discussion

Introduction

Regarding the detail OCC configuration, the latest RRC running CR [2] provides detail parameters configuration for CG Type 1, DG and CG Type 2 based on the Rel-19 higher layers parameters list LS from RAN1 [3], which seems acceptable and further RAN2 discussion is not needed .

Then in this contribution, we would like to provide some discussion on the remaining issues on uplink capacity/throughput enhancement for NR NTN.

Discussion

UE capability

In RAN1 #119 meeting, the UE capability aspects is confirmed as follows:

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Considering that this meeting is the lase meeting for R19 WI, therefore, for RAN2, we could just confirm the agreement, and introduce new UE capability in TS 38.306 to support OCC to keep alignment. Define one UE capability with OCC length 2 and other one capability with OCC length 4 which means also support OCC length 2 simultaneously. Then based on the WID scope, Uplink Capacity/Cell Throughput Enhancement for  NTN is limited to FR1, therefore we provided a corresponding TP in annex.

Proposal 1: Introduce 2 new UE capabilities in TS 38.306 to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory. A TP is provided in annex.

OCC for RACH-less HO

In last meeting, whether apply OCC for RACH-less HO is raised. From our perspective, R19 is coming to an end, and any unnecessary enhancements should not be considered especially in cases where it might have some impacts on other working groups. Therefore, we could only support OCC for RACH-less HO without the need for any further enhancement, and whether and how to apply is up to implementation.

Proposal 2: OCC for RACH-less HO could only be supported without the need for any further enhancement.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on RAN2 aspects about uplink capacity/throughput enhancement for NR NTN and have the following proposals:

Proposal 1: Introduce 2 new UE capabilities in TS 38.306 to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory. A TP is provided in annex.

Proposal 2: OCC for RACH-less HO could only be supported without the need for any further enhancement.

Reference

- R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM

- 38331 Running CR - Introduction of NTN enh phase 3_v15_clean

- R2-2505000_R1-2503242 LS on Rel-19 higher layers parameters list Post RAN1#121

Annex

- TP for TS 38.306

  - 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| nonTerrestrialNetwork-r17 Indicates whether the UE supports NR NTN access. If the UE indicates this capability the UE shall support the following NTN essential features, e.g., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | UE | No | No | No |
| ntn-occ-Length2 Indicates the UE supports OCC length 2. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR1 only |
| ntn-occ-Length4 Indicates the UE supports both OCC length 2 and OCC length 4. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR1 only |