3GPP TSG-RAN WG2 Meeting #131	R2-2505996

Bengaluru, India, August 25th – 29rd, 2025

Source:        ZTE Corporation

Title:           Clarification on the MUSIM Gap

Agenda item:   7.0.2.12

Document for:  Discussion and Decision

- Introduction

In the last meeting, a reply LS was sent from RAN4 on the MUSIM gap and Measurement Gap collision issue [1]. The main information is copied as follows:

RAN4 deems that MUSIM requirements are applied based on the following conditions: - The requested MUSIM gaps is configured with the priority level. - The configured MUSIM gaps’ priorities retain the same relative priorities among MUSIM gaps as requested by the UE. RAN4 requirements apply provided that MUSIM gaps are configured with the priority level, otherwise, no RAN4 requirements are defined. Consequently, RAN4 would like to clarify and outline the cases for which collision handling requirements in TS38.133 are defined as shown in Table 1 below. Table 1: Cases of collision handling requirements from RAN4 prospective. MUSIM gap; Measurement Gap; Requirements. With Priority; With Priority; Based on Priority rule. With Priority; Without priority; Based on MGRP. Without priority; With Priority; No RAN4 requirements. Without priority; Without priority; No RAN4 requirements.

In this paper we would like to share our views on this Reply LS.

- Discussion

Based on the Table 1 in reply LS, it’s quite clear on how to solve different MUSIM Gap and measurement Gap collision issue. Considering that the description on the collision is specified in RAN4 spec, there is no need to further specify them in RAN2 spec.

Proposal 1: No further RAN2 specification impact is required for resolving MUSIM gap and measurement gap collisions, based on the reply LS [1].

Furthermore, from the LS, we can see that the RAN4 assume that the network would always configure the priority and retain the same relative priorities among MUSIM gaps requested by the UE.

RAN4 deems that MUSIM requirements are applied based on the following conditions: - The requested MUSIM gaps is configured with the priority level. - The configured MUSIM gaps’ priorities retain the same relative priorities among MUSIM gaps as requested by the UE.

However, as clarified in the previous meeting, the network may not configure the priority even the UE indicated the preferred MUSIM priority. According to the LS, for this case, there is no RAN4 requirements.

Proposal 2: RAN2 to confirm that when network does not configure the priority even the UE has indicated the preferred MUSIM priority, UE behaviour is not specified.

In the current spec, there is a note to clarify the case that the network does not configure the relative priorities among MUSIM gaps as indicated by the UE as follows:

5.3.5.9a MUSIM gap configuration /**************omit the unrelated part**********************************************************************/ NOTE: If network does not configure the relative priorities among MUSIM gaps as indicated by the UE, UE behaviour is not specified.

However, it’s not clear whether it has included the case that the network may not configure the priority even the UE indicated the preferred MUSIM priority.

Proposal 3: RAN2 to confirm whether the NOTE below has included the case in the Proposal 2.

NOTE: If network does not configure the relative priorities among MUSIM gaps as indicated by the UE, UE behaviour is not specified.

Proposal 3a: If not included, the NOTE should be further clarified, e.g.

NOTE: If network does not configure the relative priorities among MUSIM gaps (including the case that the network does not configure the priority for the MUSIM Gaps) as indicated by the UE, UE behaviour is not specified.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: No further RAN2 specification impact is required for resolving MUSIM gap and measurement gap collisions, based on the reply LS [1].

Proposal 2: RAN2 to confirm that when network does not configure the priority even the UE has indicated the preferred MUSIM priority, UE behaviour is not specified.

Proposal 3: RAN2 to confirm whether the NOTE below has included the case in the Proposal 2.

NOTE: If network does not configure the relative priorities among MUSIM gaps as indicated by the UE, UE behaviour is not specified.

Proposal 3a: If not included, the NOTE should be further clarified, e.g.

NOTE: If network does not configure the relative priorities among MUSIM gaps (including the case that the network does not configure the priority for the MUSIM Gaps) as indicated by the UE, UE behaviour is not specified.

- Reference

- R4-2508447	 LS to RAN2 on MUSIM gap and Measurement gap   ZTE