3GPP TSG-RAN WG2 Meeting #131                                    R2-2506004

Bangalore, India Aug 25th – 29th, 2025

Agenda Item	: 8.2.3 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Contention resolution boundary

In RAN2#130 meeting, the contention resolution boundary was discussed, and the following text is captured in the running CR because the contention resolution boundary has not been concluded yet.

- 5.3.1.3 Reception of Random ID Response message Once the Access Random ID message is transmitted, the device shall monitor for Random ID Response message until it has received [FFS one or k] Access Trigger message or one A-IoT Paging message (i.e., the device does not process the Random ID Response message after that).

For the contention resolution boundary, the following options were discussed.

- Option B – the contention resolution boundary is the reception of either the next R2D trigger message or the subsequent paging message (K is one)

- Option C – the contention resolution boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

The motivation for introducing the contention resolution boundary is to handle case where different devices select the same random number to be transmitted on different access occasions before scheduling the Msg2. Since the random number is randomly selected by the device, the reader does not predict whether the different devices transmit the same random number across the different access occasions.

For example, after reception of the paging message, device A transmits the Msg1 containing the random number = 10. Then, after reception of the access trigger message, device B transmits the Msg1 containing the random number = 10. In this case, the reader cannot schedule the Msg2 containing the random number = 10 after receiving the Msg1 from the device B because the transmission of the random number in Msg1 has collided between device A and device B.

To prevent the above case as much as possible, the reader needs to transmit the Msg2 for the device A before receiving the same random number from the device B. In other words, the smart reader implementation is that the reader transmits the Msg2 before transmitting the next access trigger message.

Observation 1. The smart reader implementation is that the reader transmits the Msg2 before transmitting the next access trigger message.

Some companies said that Option C has a benefit in terms of the Msg2 scheduling flexibility from the reader side. This is because the reader can respond to Msg2 even after transmission of additional K-th access trigger messages. However, Option C would increase the probability of the problem explained above if a large K value is used. In addition, considering that the device can receive Msg2 targeted to other devices whose Msg1 was transmitted on other access occasions, additional device energy consumption is caused by unnecessarily processing the Msg2 for other devices.

Observation 2. Increasing contention resolution boundary would cause additional device energy consumption.

Considering that the device should efficiently use its energy due to limited battery capacity, we think that Option B is more reasonable. Thus, we propose that the contention resolution boundary should be the reception of either the next R2D trigger message or the subsequent paging message, i.e., K = 1

Proposal 1. The contention resolution boundary should be the reception of either the next R2D trigger message or the subsequent paging message, i.e., K value=1.

If proposal 1 is not agreeable due to the restriction of the Msg2 scheduling flexibility on the reader side, the reasonable K value should be discussed. In our view, the reasonable K value may vary based on the number of access occasions and number of targeted devices. If the number of access occasions is much greater than the number of targeted devices, the larger K value can be used to give more Msg2 scheduling flexibility. Otherwise, if the number of access occasions is slightly greater than or smaller than the number of targeted devices, the small K value is sufficient.

Considering that the number of access occasions and the number of targeted devices can be changed for each paging round, the fixed K value would not be beneficial for the Msg2 scheduling flexibility, and it would be better to determine the K value based on the reader’s decision.

Observation 3. If proposal 1 is not agreeable, the K value should be determined by the reader’s decision, i.e., the K value should not be fixed.

Then, RAN2 need to discuss how to indicate the K value determined by the reader. In our view, the K value would not need to be dynamically changed within a paging round because the K value may be determined by the number of access occasions and number of targeted devices. Thus, the reader should indicate the K value in the paging message.

Proposal 2. If the proposal 1 is not agreed, the K value should be contained in the paging message.

  - The frequency information in Msg2

In RAN2#129 meeting, it was agreed that the multiple random numbers can be multiplexed in Msg2. Since multiple A-IOT devices can transmit the Msg1 containing random number in different frequency domain, the reader transmits Msg2 containing the multiple random numbers received in Msg1s.

Since multiple random numbers can be multiplexed in Msg2, collision between random number can occur. For example, the device A and the device B could transmit Msg1s containing same random number on different access occasions located in different frequencies before reception of the next access trigger message. (hereinafter referred to as the collision problem)

To avoid above collision problem, in RAN2#130 meeting, whether to include the frequency index in Msg2 for each random number was discussed. Companies agree that it is feasible to use the frequency information to reduce the collision problem, however, it is FFS on whether to include the frequency information in Msg2 or not.

In our view, since the collision problem can occur regardless of whether a larger K value or smaller K value is used for the contention resolution boundary, including frequency information would be helpful in reducing the collision problem. Therefore, we think that the frequency information should be contained in Msg2.

Proposal 3. The frequency information should be contained in Msg2

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation 1. The smart reader implementation is that the reader transmits the Msg2 before transmitting the next access trigger message.

Observation 2. Increasing contention resolution boundary would cause additional device energy consumption.

Proposal 1. The contention resolution boundary should be the reception of either the next R2D trigger message or the subsequent paging message, i.e., K value=1.

Observation 3. If proposal 1 is not agreeable, the K value should be determined by the reader’s decision, i.e., the K value should not be fixed.

Proposal 2. If the proposal 1 is not agreed, the K value should be contained in the paging message.

Proposal 3. The frequency information should be contained in Msg2