3GPP TSG-RAN2 Meeting #131	R2-2506012

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2111 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Power Ramping for early TA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#127,change 1 and change 2 from R2-2406349 are agreed during the AT meeting offline discussion. The second change on power ramping for early TA was included in the coversheet of the raporteur CR R2-2407587 as change 3 (given below) and the CR got agreed in the meeting: Reason for change: 3) From R2-2406349 According to 5.1.1, the power ramping counter is to be reset at early RACH if the cell is different from the last cell where the UE has performed the last preamble transmission, but in NR-DC this may be done by the other MAC entity, in which case the power ramping counter should not be reset. Summary of change: 3) In 5.1.1, change "the UE" to "the MAC entity Consequences if not approved: 3) In NR-DC, early RACH may fail due to no power ramping But the actual change was missed in the procedural text in the CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the change 3 which was missed in the procedural text from agreed CR R2-2407587. i.e. include the below missing change: 3) In 5.1.1, change "the UE" to "the MAC entity Impact analysis Impacted functionality: LTM Impacted 5G architecture options: NR-DC Inter-operability: - If the network is implemented according to the CR and the UE is not, then in NR-DC, early RACH may fail due to no power ramping. - If the UE is implemented according to the CR and the network is not, then there is no issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In NR-DC, early RACH may fail due to no power ramping. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell or an LTM candidate cell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

NOTE 2:	If there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one and not initialized again.

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types;

-	prach-ConfigurationPeriodScaling-IAB: the scaling factor defined in TS 38.211 [8] and applicable to IAB-MTs, extending the periodicity of the PRACH occasions baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationFrameOffset-IAB: the frame offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs frame defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationSOffset-IAB: the subframe/slot offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs subframe or slot defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	msgA-PRACH-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type;

-	preambleReceivedTargetPower: initial Random Access Preamble power for 4-step RA type;

-	msgA-PreambleReceivedTargetPower: initial Random Access Preamble power for 2-step RA type;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP;

-	rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b);

-	rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

-	FeatureCombination: feature or a combination of features associated with a set of Random Access resources;

-	featurePriorities: priorities for features, such as (e)RedCap, Slicing, etc. (see clause 5.1.1d);

-	msgA-TransMax: The maximum number of MSGA transmissions when both 4-step and 2-step RA type Random Access Resources are configured;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	recoverySearchSpaceId: the search space identity for monitoring the response of the beam failure recovery request;

-	powerRampingStep: the power-ramping factor;

-	msgA-PreamblePowerRampingStep: the power ramping factor for MSGA preamble;

-	powerRampingStepHighPriority: the power-ramping factor in case of prioritized Random Access procedure;

-	scalingFactorBI: a scaling factor for prioritized Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	msgA-SSB-SharedRO-MaskIndex: Indicates the subset of 4-step RA type PRACH occasions shared with 2-step RA type PRACH occasions for each SSB. If 2-step RA type PRACH occasions are shared with 4-step RA type PRACH occasions and msgA-SSB-SharedRO-MaskIndex is not configured, then all 4-step RA type PRACH occasions are available for 2-step RA type (see clause 7.4);

-	ssb-SharedRO-MaskIndex: defines PRACH occasions, on which preambles are allocated for a feature or a combination of features, associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	preambleTransMax-Msg1-Repetition: the maximum number of Random Access Preamble transmissions with a given Msg1 repetition number before switching to Msg1 repetition with the next available higher Msg1 repetition number;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 4-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	msgA-CB-PreamblesPerSSB-PerSharedRO: defines the number of contention-based Random Access Preambles for 2-step RA type mapped to each SSB when the PRACH occasions are shared between 2-step and 4-step RA types;

-	msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 2-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	numberOfPreamblesPerSSB-ForThisPartition: defines the number of consecutive preambles for a feature or a combination of features mapped to each SSB;

-	msgA-PUSCH-ResourceGroupA: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group A;

-	msgA-PUSCH-ResourceGroupB: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group B;

-	msgA-PUSCH-Resource-Index: identifies the index of the PUSCH resource used for MSGA in case of contention-free Random Access with 2-step RA type;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in groupBconfigured Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

-	if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type.

-	Amongst the contention-based Random Access Preambles for 2-step RA type associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in GroupB-ConfiguredTwoStepRA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 3:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured for 4-step RA type:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles for 4-step RA type;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in groupBconfigured;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in groupBconfigured.

-	if Random Access Preambles group B is configured for 2-step RA type:

-	msgA-DeltaPreamble: ∆MsgA_PUSCH in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in GroupB-ConfiguredTwoStepRA;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in GroupB-ConfiguredTwoStepRA;

-	ra-MsgA-SizeGroupA: the threshold to determine the groups of Random Access Preambles for 2-step RA type.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only);

-	msgB-ResponseWindow: the time window to monitor RA response(s) for 2-step RA type (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5], and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI;

-	RA_TYPE;

-	POWER_OFFSET_2STEP_RA;

-	MSGA_PREAMBLE_POWER_RAMPING_STEP.

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	flush the MSGA buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	if the Random Access procedure is initiated on a Serving Cell; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the MAC entity performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission:

2>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	set POWER_OFFSET_2STEP_RA to 0 dB;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

NOTE 4:	Void.

1>	perform the BWP operation as specified in clause 5.15, except when the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell;

1>	select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

1>	if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000; or

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC; or

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure; or

1>	if the contention-free Random Access Resources have been explicitly provided in the LTM Cell Switch Command MAC CE:

2>	set the RA_TYPE to 4-stepRA.

1>	else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources within the selected set of Random Access resources (as specified in clause 5.1.1b) and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or

1>	if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources within the selected set of Random Access resources according to clause 5.1.1b; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

END of CHANGE