3GPP TSG-RAN WG2 Meeting #131	R2-2506014

Bengaluru, India, August, 25th-29rd, 2025

Agenda Item:	8.8.2

Source: 	CSCN, ZTE Corporation, Sanechips, Huawei, HiSilicon, CATT

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion

Introduction

In RAN2#130, some agreements on downlink coverage enhancement have been achieved as follows [1]:

Agreements: 1. the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) 2. To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) 3. We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported 4. No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) Working Assumption: 1. We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: Alt 1: UE provide the closest N reference locations/neighbor cells to network. Alt 2: UE reports the selected SMTCs from configured SMTC set to network. We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In this contribution, we further discuss SMTC selection enhancements for idle/inactive mode and connected mode in NR NTN based on the agreements [1] and offline discussions [2] from the RAN2#130 meeting.

Discussion

SMTC enhancements in RRC_IDLE/Inactive

During the last meeting, RAN2 agreed to introduce a location-based SMTC selection procedure, where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. They will further discuss whether to support an SSB-index based SMTC selection.

The location-based SMTC selection procedure requires UEs to support GNSS and to enable GNSS measurement in idle mode, which increases the power consumption and complexity of UE. Meanwhile, this solution is highly dependent on the availability of GNSS. Considering the GNSS information may be temporarily unavailable, UEs may fail to determine its distance to the neighbour cell.

Observation 1: The location-based SMTC selection is highly dependent on the availability of GNSS.

We introduce an SSB-index based SMTC selection procedure where each SMTC will be associated with an SSB index list of the serving cell. Even if the UE may be at the edge of a beam or the overlapping area of beams, the UE always selects one beam to camp on, depending on the UE implementation as legacy. Once the beam that UE camps on in the serving cell is determined, the UE can use this index to select the SMTCs simultaneously. While the UE moves, it can select new SMTCs corresponding to the new SSB index it camps on. Companies may also be concerned that the mapping between SSB index and SMTC may change frequently. From our point of view, first, NW operator has the responsibility to avoid frequent changes in network topology and connection relationships between the serving cell and neighbour cell(s). Otherwise, it will lead to frequent changes in the neighboring cell(s), and the NW needs to update the neighbour cell information frequently with SIB update. Second, during the time of satellite coverage (i.e. within t-Service), the relationships between the serving cell and neighboring cells are predictable and stable, which means the NW can configure a consistent mapping between SSB index and SMTC for the UEs within the serving cell. Compared to the location-based SMTC selection, SSB-index based SMTC selection is more direct and does not rely on the availability of GNSS. The benefit of SSB-based solution is that it can minimize the impact on UE.

Due to the fact that both one single beam per cell and multiple beams per cell can be considered in network deployment, we need to analyze whether SSB-index based SMTC selection can work in both deployments. For one single beam per cell, network can configure no more than 4 SMTCs based on implementation. However, for multiple beams per cell, SSB-index based SMTC selection can be applied for UE measurements.

Observation 2: Once the UE camps on a beam, it can select the SMTCs based on the association between the SMTCs and the index of the SSB on which it camps in the serving cell.

To support SMTC enhancements on DL CE, UE can select the SMTCs based on the association between the SMTCs and the index of the SSB on which the UE camps in the serving cell. Even when considering the maximum number of SSB index in FR2, at most 64 additional bits are required for indicating the SSB index for each SMTC.

Observation 3: For SSB-index based SMTC selection, a list of SSBs containing multiple SSBs configured for each SMTC is feasible for SSB-index based solution.

Proposal 1: RAN2 should introduce an SSB-based solution that uses the selected beam which UE camps on to map neighbour cell SMTCs for SMTC selection enhancement in RRC_Idle/Inactive mode.

SMTC enhancements in RRC_CONNECTED

During the last meeting, RAN2 achieved the working assumption to introduce a mechanism to assist the NW to configure the SMTCs in connected mode and there are two options:

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

RAN2 will further select from them or not confirm the working assumption.

From our point of view, gNB can configure "area list" (e.g. referenceLocation and radius of neighbour cells) to indicate each neighbour cell coverage in OtherConfig through RRCReconfiguration. The UE determines the top-N nearest neighbour cells and reports the ID list of the configured area to gNB via UEAssistanceInformation. The network performs refined configurations for SMTC and gap based on this information. If the UE’s current area changes from the last report, it triggers a subsequent report of new located area list change via UAI.

Proposal 2: RAN2 should introduce a mechanism to assist the NW to configure SMTCs based on UAI in RRC_CONNECTED mode.

Conclusions

Based on above analyses, observations and proposals are summarized as follows:

Observation 1: The location-based SMTC selection is highly dependent on the availability of GNSS.

Observation 2: Once the UE camps on a beam, it can select the SMTCs based on the association between the SMTCs and the index of the SSB on which it camps in the serving cell.

Observation 3: For SSB-index based SMTC selection, a list of SSBs containing multiple SSBs configured for each SMTC is feasible for SSB-index based solution.

Proposal 1: RAN2 should introduce an SSB-based solution that uses the selected beam which UE camps on to map neighbour cell SMTCs for SMTC selection enhancement in RRC_Idle/Inactive mode.

Proposal 2: RAN2 should introduce a mechanism to assist the NW to configure SMTCs based on UAI in RRC_CONNECTED mode.

Reference

- Report of 3GPP TSG RAN WG2 meeting #130

- R2-2504775, Report of [AT130][304][R19 NR NTN] DL CE (Xiaomi)