3GPP TSG-RAN-WG2 Meeting #131	R2-2506020

Bengaluru, India, 25th– 29th Aug, 2025

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	Remaining issues and solutions on C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#130 meeting, RAN2 reached the following agreements [1]:

Agreements: - Do not pursue fast forwarding. - Continue during this meeting to discuss including the RRC state in relay discovery, including: - Spec impact to capture it in the RRC container - Implications for (re)selection procedure - Continue discussion of whether/how to extend timers. - There is no remote UE ID in SIB request or SIB delivery, i.e. RemoteUEInformationSidelink and UuMessageTransferSidelink. - In-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire SIB broadcast by the last relay UE’s serving cell via Uu interface directly for it’s own concerned SIB. - For the SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly. - For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly. - Below b), c), and e) are applied also in IDLE/INACTIVE states for intermediate relay UE without additional specification change. - The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: - b) Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) - c) Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) - e) Upon receiving updated SIB1 from the network (as in Rel17) - For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if IDLE/INACTIVE intermediate relay UE obtain the requested SIB via Uu interface or unsolicited SIB1 forwarding and CAN respond SIB request of child UE, adopt legacy condition(“if the UE has SIB request information to provide”) to determine SIB request in parent link is needed or not. Applicability of this legacy language to the multihop case to be documented in RRC. - Legacy condition “if the UE has SIB request information to provide” is enough, no additional specification change is needed to support “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu” (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay. - Do not implement reflective bearer mapping in the running CR(s) now; interested companies are asked to converge a single detailed TP for final go/no-go decision next meeting. - Extend the T300, T301 and T319 timers for multi-hop U2N relay operation. The UE and gNB multiply the signalled timer value by the hop count. - Working assumption: - The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container. As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed. No SA2 spec impact is expected. - Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information. - The intermediate relay UE can have its own traffic acting as a remote UE simultaneously. Running CRs will be checked to make sure this functionality is supported. - Clarify in the normative text that the UE can be a relay and remote UE simultaneously (to be determined case by case where it needs to be documented).

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - C-plane procedure for multi-hop relaying

Paging Delivery

RAN2 agreed that an intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. And it is FFS whether it also releases paging information for the child UEs and how the child UEs receive the paging message when the intermediate relay UE is in RRC_CONNECTED. Here, current paging delivery is supported by following mechanism [2]:

- Method 1: The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space.

- Method 2: The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).

If the paging information for a child UE is released at a parent relay UE upon an intermediate relay UE enters RRC_CONNECTED state, gNB must send paging message for a remote UE by dedicated signalling. In Rel-17/18, it is NW implementation which method is used for paging delivery. RAN2 should keep this principle in Rel-19. Furthermore, RAN2 agreed to support paging monitoring on Uu by an intermediate relay UE. To maximise the benefits of this mechanism, an intermediate relay UE should keep monitoring PO for a grandchild UE if a child UE enters in RRC_CONNECTED.

- When intermediate relay UE enters RRC_CONNECTED, paging information at a parent relay UE is not released.

If proposal 1 is agreed, RAN2 should further discuss how to release “a remote UE’s paging information” at a parent relay UE. In the current spec, to release the information at L2 U2N relay UE, a L2 U2N remote UE sends RemoteUEInformationSidelink including sl-PagingInfo-RemoteUE-r17 which its value is set to release. Then, the relay UE releases paging information for the remote UE. This paging information should be associated with PC5-RRC connection since the relay UE should not release other paging information associated with PC5-RRC connections other than PC5-RRC connection for the remote UE.

- Paging information at R17/18 U2N relay UE is associated with a PC5-RRC connection.

If we reuse this IE for multi-hop relay, a relay UE releases all paging information associated with a child UE upon the relay UE receives the release message. To release appropriate paging information upon receiving the message, some information indicating which paging information should be released, e.g. UE ID. However, a UE releases local ID when UE leaves to RRC_IDLE or RRC_INACTIVE. Thus, local ID is not applicable. Also, a UE changes L2 ID upon related timer expires. So, L2 ID is not applicable too. So, it may be better to use paging UE ID for indicating paging information to be released.

- Paging information release list should be included in RemoteUEInformaionSidelink.

In RAN2#129bis meeting, RAN2 agreed that:

- Support the child UE relying on the parent relay UE for paging monitoring.

- Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

Thus, we need to consider how to avoid duplicated paging deliveries. If we consider simply based on the existing system, it is natural that an intermediate relay UE checks whether the received paging record from a parent relay UE is already received/delivered before the intermediate relay UE transfers the paging record to a child UE. To determine whether the paging record is already delivered to a child UE, paging UE ID included in the paging record can be used. It means that an intermediate relay UE will forward a received paging record only if a paging record containing the corresponding paging UE ID has not already been forwarded to the child UE.

- An intermediate relay UE will forward a received paging record only if a paging record containing the corresponding paging UE ID has not already been forwarded to the child UE.

SIB Delivery

And it should be discussed how to treat duplicated SI forwarding, as with paging. For paging delivery, RAN2 agreed to consider how to avoid duplicated paging delivery. From radio resource perspective, duplicated SI forwarding should be avoided too. Thus, an intermediate relay UE should check whether the received SIB is updated to avoid duplicated SIB forwarding.

- An intermediate relay UE should check whether the received SIB is updated to avoid duplicated SIB forwarding and forward the SIB if it is updated.

SRAP configuration

SRAP layer supports the routing function, i.e., determination of egress RLC channel/link, determination of UE/BEARER ID field and data transfer. For routing function, gNB configures relay UE(s) and remote UE with local UE ID and mapping rule between E2E RB and RLC channel. Then, the relay UE(s) and remote UE determines the route of the packet based on the SRAP header of the packet. There are two types of SRAP header, one is for U2N SRAP data PDU and another one is for U2U SRAP data PDU. RAN2 already agreed that single local UE ID (L2 U2N relay) format is used. And the value of BEARER ID refers appropriate bearer identity. It means that, in case of U2N relay, the value refers srb-Identity or drb-Identity (minus 1). In case of U2U relay, the value refers SL-SRB identity or slrb-PC5-ConfigIndex, the dedicated values should be used for SL-SRB 0/1/2/3 and other values are used for SL-DRBs. The reason why dedicated values are used for SL-SRBs in U2U relay is that both-side hop is PC5-RLC channel. In the multi-hop relay case, the situation at an intermediate relay UE is same with U2U relay. Therefore, the value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2.

- The value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2 as with U2U relay.

QoS handling

The latency requirement is important when we consider the QoS handling. Especially, some use cases, e.g., public safety, wearable and V2X, needs very low E2E latency. Therefore, optimization should be done to achieve low latency.

- QoS related optimization should be considered since very low E2E latency is required for some use cases.

When we consider QoS related optimization, it is important to know achievable QoS at each hop. Here, gNB can guarantee at least one hop (i.e., last relay hop) QoS if gNB allocate sidelink and/or uplink grant for the last relay UE using resource allocation mode 1. Thus, we propose that the principles of resource allocation for U2N relay operation follow the legacy mechanism.

- The principles of resource allocation follow the legacy mechanism, i.e., last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

- gNB can guarantee at least one hop (i.e., last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

In U2U relay, U2U relay UE split the E2E QoS for each hop, and then each UE configures each hop configuration to guarantee the split QoS. Because U2U relay UE knows quality of both hops and can configure appropriate values. In multi-hop relay, there is no special UE like a U2U relay UE. However, if gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality. Thus, we propose that the gNB sets the appropriate (split) QoS to be met for each hop and UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

- If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

- UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Sidelink UE Information reporting

In Rel-17, remote/relay UE sends SUI to gNB for resource acquisition including UE-type information. And in Rel-18, U2U specific UE-type is introduced. In multi-hop case, the last relay UE should use the ue-Type as relayUE, and the remote UE should set the ue-Type as remoteUE. For the intermediate relay UE, RAN2 should discuss whether the new UE type is specified, or legacy UE type (remoteUE or relayUE) is reused. In our understanding, a CONNECTED intermediate relay UE is always a remote UE for receiving/sending RRC messages from/to gNB. But the intermediate relay UE may or may not have its own UP data. Therefore, new UE type should be introduced for an intermediate relay since it has two roles.

- New UE type in SUI is introduced for an intermediate relay UE.

And it should be discussed which information should be included if the SUI is sent from an intermediate relay UE. In Rel-18, Relay UE and Remote UE has specific report in sl-TxResourceReqL2U2NRelay in sl-TxResourceReqListCommRelay. One option is that an intermediate relay UE sets the fields in one or more entries as a relay UE and sets the fields in one entry as a remote UE. In that case, some information might be duplicated from NW perspective, e.g., sl-TxInterestedFreqListL2U2N, sl-TypeTxSyncListL2U2N, sl-PagingIdentityRemoteUE (it might also be reported by the (parent) last relay UE) and sl-CapabilityInformationSidelink (in case of 3-hop relay, a first relay UE sends information of a remote UE and an intermediate relay UE, an intermediate relay UE sends information of a first relay UE and a last relay UE, and a last relay UE sends information of an intermediate relay UE). Thus, to avoid such duplicated information reporting, intermediate relay UE specific information reporting is helpful, i.e. intermediate relay UE doesn’t indicate capability of a parent UE in SUI. The following table is summary of above discussion.

Table 1. ue-Type specific field settings in sl-TxResourceReqL2U2N-Relay within SUI

| IE | field | Rel-18 U2N relay UE | Rel-18 U2N Remote UE | Intermediate Relay UE |
| --- | --- | --- | --- | --- |
| sl-TxResourceReqL2U2N-Relay | SL-DestinationIdentity | x |  | x (for a child UE) |
|  | sl-TxInterestedFreqListL2U2N | x | x | x |
|  | sl-TypeTxSyncListL2U2N | x | x | x (current sync types for each frequency) |
|  | sl-LocalID-Request | x |  | x (for a child UE) |
|  | sl-PagingIdentityRemoteUE | x |  | ? (for a child UE) |
|  | sl-CapabilityInformationSidelink | x | x | x (for a child UE) |
| ue-Type |  | relayUE | remoteUE | (newType) |

- Intermediate relay UE specific information reporting is introduced in SUI to avoid duplicated information reporting by multiple relay UEs.

Resource allocation mode

Regarding to the resource allocation mode, RAN2 agreed that both of mode 1 and 2 can be supported at least relay UEs. And it is FFS whether a remote UE support both modes. If this agreement means that intermediate relay UE can perform resource allocation mode 1, it is natural that mode 1 and 2 can be supported by remote UE. However, in Rel-17/18, remote UE cannot perform mode1 RA regardless whether the UE is in coverage or not. Here, we remind Rel-17 discussion on resource allocation mode 1 for a remote UE[3]. According to the reports, the reasons not to support RA mode 1 for remote UE are below [4]:

- ACK/NACK from gNB has to be disabled for CG type 1. In this case, CG type 1 is similar to Mode 2 operation without gNB’s involvement.

- In Rel-16, Mode 1 RA was mainly discussed in RAN1. Considering we may have RAN1 impacts (e.g., disable ACK/NACK), RAN2 can’t make decision without involving RAN1

- Type 1 CG was initially introduced in Rel-15 mainly for URLLC, it is obvious that URLLC is hard to be supported for relay scenarios

These reasons are still valid for the current SL and SL relay, and the situation with RAN1 has not changed significantly. Thus, this principle should be applied to the multi-hop relay. In other words, resource allocation mode 1 is only supported by a last relay UE.

- In Rel-17, RAN2 decided not to support resource allocation mode 1 for remote UE because HARQ using gNB is disabled, RAN1 hasn’t discussed mode 1 for remote UE, and CG type 1 is introduced for URLLC.

- RAN2 confirms that resource allocation mode 1 can only be supported by a last relay UE. Remote UE and intermediate relay UE only support resource allocation mode 2 regardless it is IC or OOC.

(RRC-13) Timer extension

In RAN2#130 meeting, RAN2 made some agreements on timers:

- Do not pursue fast forwarding.

- Continue discussion of whether/how to extend timers.

- Extend the T300, T301 and T319 timers for multi-hop U2N relay operation.  The UE and gNB multiply the signalled timer value by the hop count.

We consider here whether the extended timer in the CR is enough or too long. In the CR, the timer for a single hop relay multiplied by a number of hops is defined as the extended timer. As a representative example, verify whether extended T300 has sufficient value.

Here, legacy t300-RemoteUE is configured by considering the delays of Uu-hop and PC5-hop. In Uu-hop, the dominant factor in connection delay is the RRC procedure required to set up the Uu RLC channel for SRB0. Because if the last relay is in idle, RRCSetup, SecurityModeCommand, SidelinkUEInformation and RRCReconfiguration are needed. In multi-hop relay scenario, RAN2 already agreed that PC5 RLC channel for SRB0 at an intermediate relay UE is configured by dedicated RRCReconfiguration. Therefore, if all relays are in idle, these procedures are needed for relay UEs. And these procedures are carried out via multi-hop and the delays increase.

If we count the steps required to forward the SRB0/1 message for each hop and roughly estimate the time it takes for the RRCSetup message to reach the remote UE, we get the following:

Figure 1. RRC Setup time in multi-hop scenarios

We calculated and plotted the normalized setup time as a function of the number of hops.

Case 1[Time for setup (PC5-connected)]: PC5 connections are established between all relays and all relay UEs are in RRC_IDLE. [diamond]

Case 2[Time for setup (PC5-unconnected)]: PC5 connections are not established between all relays and all relay UEs are in RRC_IDLE. [square]

Case 3[Time for setup (RRC-connected)]: First relay UE is in RRC_CONNECTED (all UEs are in connected). [X]

We also plotted the value of T300-remoteUE multiplied by the number of hops "in triangles." The vertical axis shows the estimated setup time normalized by the setup time for a single hop, and the horizontal axis shows the number of hops from the remote UE. This estimate also considers ProSe layer signaling to be equivalent to L2 relay level delays. As such, actual times may take longer than this estimate.

For this analysis, we counted the number of times messages are physically sent and then we normalize it by dividing it by the number of physical message transmissions in single hop relay. Then, we calculated the number of times messages are physically sent, by this formula: 8n(n+1)/2+2(n+1) [Case 1]. Here, n is hop-count (i.e. a number of relay UEs), the first term means the number of times RRC messages are physically sent to reconfigure a first relay UE, and the second term means that the number of times a RRC setup/setup request message for a remote UE are physically sent. For case 2, 4(n-1) is added to the above formula for considering unicast link establishment. Lastly, for case 3, it can be calculated the number of times messages are physically sent by this formula: 3n(n+1)/2+2(n+1). And the comparison with the timer on the Y axis assumes that the time required for single-hop setup is equivalent to that of the legacy T300-Remote.

Based on this analysis, even if it is the best case (case 3), T300 for Remote UE introduced in the current CR will expires. Furthermore, in the worst case (case 2), the RRC setup procedure of the remote UE may take about twice as long as the timer value. Therefore, RAN2 needs to estimate the time required for RRC Setup and determines the timer calculation formula at the next meeting.

- (RRC-13) According to the current CR, time for RRC setup increases exponentially with the number of hops.

- (RRC-13) Multiplying the existing timer by the number of hops alone cannot absorb connection delays caused by multi-hops.

- (RRC-13) For T300/301/319 extension, RAN2 estimates the time required for RRC Setup etc. and determines the timer calculation formula at the next meeting.

Others

In Rel-18 multi-path scenario, RAN2 agreed to assume that a multi-path relay UE is not in IDLE/INACTIVE state. Since the multi-path remote UE is always in CONNECTED, the MP relay UE should be in CONNECTED. In case of multi-hop relay, a last relay UE and intermediate relay UEs should not be in IDLE/INACTIVE while a remote UE is in CONNECTED. In MAC specification, there is a mechanism for autonomous entering to IDLE state i.e., dataInactivityTimer. The dataInactivityTimer is not applicable STCH but there is no restriction to configure it with sidelink UE. Here, we suggest discussing whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

- Discuss whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

In Rel-18, during the RRC connection re-establishment procedure, if a UE is capable of a remote UE, it could either cell selection or relay selection or both perform them while a U2N relay UE could perform only cell selection but not relay selection.

In the latest running CR, the editor tried to reuse the word “capable of” to determine UE behavior. However, in multi-hop scenario, according to SA2 decision, the authorization for intermediate relay UE and last relay UE in multi hop could be separated, which means a relay UE could be either capable of both or just one.

So, it is unclear if a UE capable of intermediate relay UE means the UE is only capable of intermediate relay UE or the UE could be capable of last relay as the same time.

Furthermore, when a relay UE initiating an RRC re-establishment procedure, cell selection and relay selection will result different behavior depending on what the UE is capable of. If it is capable of intermediate relay UE and also last relay UE, either cell selection or relay selection could be done. The UE could still act as a relay UE no matter cell or relay is selected. If it is only capable of intermediate relay UE, if a cell is selected, it will act as a normal UE but not a relay, the child UEs connecting to it have to be released to perform relay reselection. RAN2 has not yet discussed the expecting behavior here.

From our view, there should be a common understanding on the UE behavior to perform relay selection or cell selection considering the UE’s role in the multi-hop.

- Discuss UE behavior in RRC re-establishment procedure to perform relay selection or cell selection considering the UE’s role in the multi-hop.

Conclusion

In this paper, we made following observations and proposals:

- When intermediate relay UE enters RRC_CONNECTED, paging information at a parent relay UE is not released.

- Paging information at R17/18 U2N relay UE is associated with a PC5-RRC connection.

- paging information release list should be included in RemoteUEInformaionSidelink.

- An intermediate relay UE will forward a received paging record only if a paging record containing the corresponding paging UE ID has not already been forwarded to the child UE.

- An intermediate relay UE should check whether the received SIB is updated to avoid duplicated SIB forwarding and forward the SIB if it is updated.

- The value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2 as with U2U relay.

- QoS related optimization should be considered since very low E2E latency is required for some use cases.

- The principles of resource allocation follow the legacy mechanism, i.e., last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

- gNB can guarantee at least one hop (i.e., last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

- If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

- UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

- New UE type in SUI is introduced for an intermediate relay UE.

- Intermediate relay UE specific information reporting is introduced in SUI to avoid duplicated information reporting by multiple relay UEs.

- In Rel-17, RAN2 decided not to support resource allocation mode 1 for remote UE because HARQ using gNB is disabled, RAN1 hasn’t discussed mode 1 for remote UE, and CG type 1 is introduced for URLLC.

- RAN2 confirms that resource allocation mode 1 can only be supported by a last relay UE. Remote UE and intermediate relay UE only support resource allocation mode 2 regardless it is IC or OOC.

- (RRC-13) According to the current CR, time for RRC setup increases exponentially with the number of hops.

- (RRC-13) Multiplying the existing timer by the number of hops alone cannot absorb connection delays caused by multi-hops.

- (RRC-13) For T300/301/319 extension, RAN2 estimates the time required for RRC Setup etc. and determines the timer calculation formula at the next meeting.

- Discuss whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

- Discuss UE behavior in RRC re-establishment procedure to perform relay selection or cell selection considering the UE’s role in the multi-hop.

Reference

- R2-250xxxx, Report from session on positioning and sidelink relay

- TS 38.300

- R2-2201970, Report of 3GPP TSG RAN WG2 meeting #116-e, Online

- R2-2111273, [Offline-604][Relay] Summary of Agenda item 8.7.2.4: QoS (Qualcomm)