3GPP TSG-RAN WG2 Meeting #131	R2-2506034

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.12.3

Source: 	ASUSTeK

Title:  	Discussion on RRC impacts for MIMO phase 5

Document for:	Discussion and Decision

- Introduction

In email discussion [Post130][218][MIMO_Ph5]. Some open issues were discussed including some configuration issues regarding UEI Beam Reporting. In this document, we continue discussing RRC impacts for UEI Beam Reporting mode-B.

- Discussion

Configuration for modeB PUSCH resource

RAN1 provides three parameters to indicate the index of the Configured Grant configurations, which is currently used in RRC running CR indicating Type-1 CG resource for mode-B PUSCH transmission:

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							   ServCellIndex

}

}

},

CSI-ReportUE-IBM field descriptions

pusch-ResourceOfModeB Indicates Type-1 CG PUSCH resource for the UE-initiated beam reporting in mode-B.

However, from RAN2 point of view, a single parameter ConfiguredGrantConfigIndexMAC-r16 should be sufficient as it can indicate the index of the CG configurations within the MAC entity.

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

In addition, since it was agreed and specified in RAN1 spec that PUSCH resource for mode-B should be configured in a same CC as the corresponding CSI-ReportConfig, indicating Serving Cell index for the CG resource would be redundant:

RAN1#118 agreement: Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC. (38.213) After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet […]. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

Observation 1:  Since Type 1 CG-PUSCH for UEI reporting mode-B has been restricted to be on the same Serving Cell where the corresponding CSI-ReportConfig is configured, introducing servCellIndex as a RRC parameter for the Type 1 CG resource is redundant and may cause unnecessary conflict from network’s point of view.

Therefore, we propose to use a single MAC entity-specific parameter ConfiguredGrantConfigIndexMAC-r16 to indicate the index of the CG configurations for mode-B UEI beam reporting.

Proposal 1:  Use a single parameter ConfiguredGrantConfigIndexMAC to indicate Type-1 CG PUSCH resource for the PUSCH transmission in mode-B UE-initiated beam reporting.

HARQ processes for 2nd Step in ConfiguredGrantConfig for mode B

RAN2 has confirmed that the CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH), meaning that HARQ operation should not be used for transmitting the beam report. Therefore, configuring nrofHARQ-Processes for such CG is unnecessary. However, since this field is currently mandatory, it may cause confusion for the network regarding how to set the value. The best way for the UE is to ignore this field to avoid occupying HARQ processes in MAC layer.

Observation 2: Since nrofHARQ-Processes is mandatory for a CG used in UE-initiated beam reporting Mode-B, it may cause confusion for the network regarding how to set the value. Additionally, the UE may unnecessarily occupy HARQ processes in the MAC layer.

Proposal 2:  The network can set any value to nrofHARQ-Processes, and the UE shall ignore this field if the nrofHARQ-Processes is configured within ConfiguredGrantConfig for UE-initiated beam reporting Mode-B.

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

...

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix). The network sets the value of this field to 1 when cg-LTM-Configuration is configured.

- Conclusion

In this document, we have the following proposals for UEI Beam Reporting Mode-B.

Observation 1:  Since Type 1 CG-PUSCH for UEI reporting mode-B has been restricted to be on the same Serving Cell where the corresponding CSI-ReportConfig is configured, introducing servCellIndex as a RRC parameter for the Type 1 CG resource is redundant and may cause unnecessary conflict from network’s point of view.

Observation 2: Since nrofHARQ-Processes is mandatory for a CG used in UE-initiated beam reporting Mode-B, it may cause confusion for the network regarding how to set the value. Additionally, the UE may unnecessarily occupy HARQ processes in the MAC layer.

Proposal 1:  Use a single parameter ConfiguredGrantConfigIndexMAC to indicate Type-1 CG PUSCH resource for the PUSCH transmission in mode-B UE-initiated beam reporting.

Proposal 2:  The network can set any value to nrofHARQ-Processes, and the UE shall ignore this field if the nrofHARQ-Processes is configured within ConfiguredGrantConfig for UE-initiated beam reporting Mode-B.

Reference

Text proposal for proposal 1 (based on running RRC CR for MIMO ph5)

CSI-ReportUE-IBR-r19             ::=     SEQUENCE {

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19          ConfiguredGrantConfigIndexMAC                                                                                                                                       OPTIONAL,    -- Need R

},

Text proposal for proposal 2

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

...

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix). The network sets the value of this field to 1 when cg-LTM-Configuration is configured. The UE shall ignore this field if it’s configured within ConfiguredGrantConfig for UE-initiated beam reporting in mode-B.