3GPP TSG RAN WG2 #131				R2-2506036

Bangalore, India, Aug. 25th – 29th, 2025

Agenda Item:	8.13.2

Source:		ASUSTeK

Title:				Discussion on remaining issues on notification message

Document for: Discussion and Decision

- Introduction

In RAN2#130 meeting [1], the following agreements related to the notification message were made:

Agreements: Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism. FFS new cause values. In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED.

Regarding the notification behaviour when the intermediate relay UE is in RRC_CONNECTED, whether NotificationMessageSidelink is always sent to its child UE is still FFS. In this contribution, we further discuss this issue.

- Discussion

2.1  Notification behaviour when intermediate relay UE is in RRC_CONNECTED

In multi-hop U2N relay, an intermediate relay UE in RRC_CONNECTED initiates the RRC connection re-establishment procedure upon reception of NotificationMessageSidelink from its parent relay UE or upon detecting sidelink radio link failure between itself and its parent relay UE. During the RRC connection re-establishment procedure, the intermediate relay UE performs cell selection and relay selection to find a suitable cell or a suitable relay.

The intermediate relay UE, following legacy behaviour, also sends NotificationMessageSidelink to notify its child UE(s). However, whether it is always necessary for the intermediate UE in RRC_CONNECTED sending NotificationMessageSidelink to its child UE could be discussed.

Possibly, the new serving cell of the intermediate relay UE may be the same as the previous one (either the intermediate relay UE selects the current serving cell as a suitable cell or selects another relay as a suitable relay UE which is also served by the current serving cell of the intermediate relay UE). Since the serving cell of the intermediate relay UE is not changed, it implies that the serving cell of the child UE(s) is also not changed. As a result, it seems unnecessary for the intermediate relay UE to send NotificationMessageSidelink to its child UE(s), since the existing radio bearer (RB) configurations, SRAP configurations (e.g., local ID) and/or PC5 Relay RLC channel configurations could be reused for the child UE(s).

Proposal 1: The intermediate relay UE does not send NoficationMessageSidelink to the child UE(s) if it is still served by the same cell based on cell/relay selection during the RRC connection re-establishment procedure.

On the other hand, the new serving cell of the intermediate relay UE may be different from the previous one (either the intermediate relay UE selects a different cell as a suitable cell or selects another relay as a suitable relay UE which is served by a different cell). Since the serving cell of the intermediate relay UE is changed, it implies that the serving cell of the child UE(s) needs to change. As a result, it is necessary for the intermediate relay UE to send NotificationMessageSidelink to its child UE(s), since the existing radio bearer (RB) configurations, SRAP configurations (e.g., local ID) and/or PC5 Relay RLC channel configurations could not be reused for the child UE(s). Upon receiving NotificationMessageSidelink, the child UE should initiate its own RRC connection re-establishment procedure toward another relay or cell.

Proposal 2: The intermediate relay UE sends NoficationMessageSidelink to the child UE(s) if the serving cell of the intermediate relay UE changes based on cell/relay selection during the RRC connection re-establishment procedure.

In case the intermediate relay UE fails to select a suitable cell or a suitable relay in the cell selection / relay selection during the RRC connection re-establishment procedure, the RRC connection re-establishment procedure will fail. As a result, it is necessary for the intermediate relay UE to send NotificationMessageSidelink to its child UE(s), since the existing radio bearer (RB) configurations, SRAP configurations (e.g., local ID) and/or PC5 Relay RLC channel configurations could not be reused for the child UE(s). Upon receiving NotificationMessageSidelink, the child UE should initiate its own RRC connection re-establishment procedure toward another relay or cell.

Proposal 3: The intermediate relay UE sends NoficationMessageSidelink to the child UE(s) if no suitable cell/relay can be selected during the RRC connection re-establishment procedure.

- Conclusion

In this contribution, we propose:

Proposal 1: The intermediate relay UE does not send NoficationMessageSidelink to the child UE(s) if it is still served by the same cell based on cell/relay selection during the RRC connection re-establishment procedure.

Proposal 2: The intermediate relay UE sends NoficationMessageSidelink to the child UE(s) if the serving cell of the intermediate relay UE changes based on cell/relay selection during the RRC connection re-establishment procedure.

Proposal 3: The intermediate relay UE sends NoficationMessageSidelink to the child UE(s) if no suitable cell/relay can be selected during the RRC connection re-establishment procedure.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta