3GPP TSG-RAN WG2 Meeting #131	R2-2506061

Bengaluru, India, Aug. 25th – 29th , 2025

Agenda Item:	8.2.2

Source:	HONOR

Title: 	Discussion on A-IoT paging

Document for:	Discussion and Decision

1	Introduction

The majority issues were discussed in the open issues lists [1], in this contribution, we focus on the left issues.

2	Discussion

Fill bits

In the current running CR of TS38.391, the padding bit of R2D message is descripted as following:

Fill Bits: This field is of variable size and is optional present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the device shall ignore the values of this field

There is a R bit in the paging message to indicate future extension. If the R bit indicates the future extension, the future parameters should be after the last part of Rel-19 fields, which is naturally not the fill bits. Due to the fill bits are not always present, the purpose of future extension is not suitable. Therefore, the fill bit is only used for padding not for future extension.

It is suggested the modification as following:

Fill Bits: This field is of variable size and is optional present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the device shall ignore the values of this field

Proposal 1: Introducing fill bits in the paging message to pad only for byte alignment.

MAC PDU format for paging

The current running CR of TS38.391 has captured the D2R scheduling information, which includes resource-related information and scheduling information. The Time Resource Indication field indicates the number of time domain resources for access occasions and is only present in the A-IoT Paging message indicating CBRA. The D2R TBS field, which is used for scheduling the D2R TB, is absent in the A-IoT Paging message indicating CBRA. Up to the device implementation, it is possible that the device reads the information bit by bit. In this case, the device needs to first acquire the AT field and then determine which information is included in the D2R scheduling information. Therefore, for the paging message, at least the AT field for access type is present before the D2R scheduling information.

Proposal 2: The AT filed is present before the D2R scheduling information in the paging message.

D2R scheduling information

In the running CR discussion, the companies proposed a new method to indicate the bit duration and frequency resource allocation. For example, they suggested reducing the total number of bits used for this indication, such as using only 9 bits to jointly signal both parameters. And the issue 4-6 has been captured as an open issue and as following:

Issue 4-6: Whether/how to reduce the size of Frequency Resource Indication in D2R Scheduling Info field - In the current version of CR, Frequency Resource Indication is defined as an 8-bit bitmap based on RAN1 agreement in RAN1 LS in R1-2504915. Some companies propose to reduce the size of the indication. There are two directions raised: 1. If bit duration is set to values other than 266.67μs, shorter bitmap can be used; 2. If the scheduling is for only one resource, the 8-bit bitmap can be replaced with a 3-bit frequency index. Considering the 8-bit bitmap as captured in current running CR work well, this signaling reduction is an optimization, which can be discussed only if time allows.

For this issue, the bit duration is not only used to determine the set of R values but also the other purpose, e.g. interval between consecutive midambles. Therefore, the two parameters should be indicated separately rather than jointly. Meanwhile, using variable lengths for both the bit duration and frequency resource indication would increase device implementation complexity. Therefore, it is recommended using fixed bit for the bit duration and frequency resource indication.

Proposal 3: (issue 4-6) Using an 8-bit bitmap to indicate frequency resource indication and 3 bits to indicate bit duration.

3	Conclusions

Proposal 1: Introducing fill bits in the paging message to pad only for byte alignment.

Proposal 2: The AT filed is present before the D2R scheduling information in the paging message.

Proposal 3: (issue 4-6) Using an 8-bit bitmap to indicate frequency resource indication and 3 bits to indicate bit duration.

4	References

[1]	A-IoT MAC open issues