3GPP TSG-RAN WG2 Meeting #131	R2-2506063

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.5.3

Source:	HONOR

Title: 	Discussion on on-demand SIB1

Document for:	Discussion and decision

- Introduction

In this contribution, we mainly analyse the issues of the running CR regarding OD-SIB1 acquisition scenarios and NES cell camping restrictions.

- Issues of 38.300 CR for NES

2.1 NES cell camping restrictions.

In the CR of 38.300 Section 15.4.2.5, the description of NES cell camping restrictions is as follows[1]:

Section 15.4.2.5 Camping Restrictions

If a cell provides on-demand SIB1, the cell can allow the access of UEs supporting OD-SIB1 but prevent the access of UEs not supporting OD-SIB1 based on no SIB1 indication in MIB as described in clause 7.3.1.

However, in the Section 7.3.1, we did not find any description about “prevent the access of UEs not supporting OD-SIB1” and " no SIB1 indication in MIB ". Regarding whether legacy UEs are allowed to camp on NES cells, RAN2 reached the following conclusions:

Agreement[2]

Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

Agreement[3]

There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

Agreement[4]

When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

Based on our analysis, we believe that the situation where legacy UEs receive broadcasted SIB1 in NES cells does not actually occur. The value of K_SSB in the MIB of an NES cell always falls within the NCD-SSB range, i.e. K_SSB > 23 for FR1 or K_SSB > 11 for FR2. When legacy UEs reselect to an NES cell and acquire the MIB, regardless of whether the current SSB is actually associated with SIB1, they will first treat the current SSB as an NCD-SSB according to the K_SSB value in the MIB. Subsequently, they will try to search for a CD-SSB based on the information in PDCCH-ConfigSIB1:

- If a CD-SSB is found, they will continue to acquire the MIB information in the CD-SSB, confirm the SIB1 reception parameters in PDCCH-ConfigSIB1, and ultimately acquire SIB1.

- If no CD-SSB is found, the UE will reselect the cell after a period of time due to the inability to acquire SIB1.

It is crucial to note that NES cells do not transmit CD-SSBs at all. Therefore, when a legacy UE initially detects an SSB and interprets it as an NCD-SSB, it will attempt to search for a nearby CD-SSB but will invariably fail to find one, thereby being unable to acquire SIB1. Therefore, we believe that regardless of whether the SIB1 of the NES cell is being broadcast, the legacy UE cannot acquire SIB1 and camp on the NES cell. The K_SSB in the MIB is sufficient to prevent legacy UEs from accessing the NES cell.

Observation 1: Regardless of whether the SIB1 of the NES cell is being broadcast, legacy UEs cannot acquire the SIB1 and camp on the NES cell. No specification impact is foreseen.

Observation 2: The barring mechanisms to prevent legacy UEs from accessing cells providing on-demand SIB1 is the K_SSB in the MIB.

Proposal 1: It is recommended that the description of TS 38.300 Section 15.4.2.5 Camping Restriction be revised to “…based on the K_SSB in MIB.”

2.2 Scenarios for initiating OD-SIB1 request

The Section 15.4.2.x2 in the CR of 38.300 provides an overview of OD-SIB1[1].

Section 15.4.2.x2 On-demand SIB1

To facilitate reducing gNB downlink transmissions, the gNB can provide SIB1 on-demand, i.e., upon receiving an OD-SIB1 request from a UE supporting OD-SIB1. OD-SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, where MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. OD-SIB1 request configurations of one or more cells which support OD-SIB1 are included in SIBxx, which can be broadcasted in any cell, including cell’s own OD-SIB1 request configuration. UE may request SIB1 based on the OD-SIB1 request configuration from SIBxx in order to determine the suitability of a cell during and after cell reselection.

According to the description in the highlight part, it is understood that acquiring SIB1 during cell reselection is to determine the suitability of the cell. As stated in 38.300, cell reselection identifies the cell that the UE should camp on[5]. However, after cell reselection, the UE should camp normally on the NES cell as shown in the figure of TS 38.304 below. At Step 2, the UE only requests SIB1 to confirm the broadcast status of the OSI, unless the next cell reselection process is triggered to determine the suitability of the cell.

Observation 3: After cell reselection, the NES UEs should camp normally on the NES cell and do not need to acquire SIB1 to determine this cell suitability again.

Observation 4: When the NES UEs camp on the NES cell, they need to acquire SIB1 to confirm the broadcast status of the OSI in SI change scenario.

Figure 5.2.2-1: RRC_IDLE and RRC_INACTIVE Cell Selection and Reselection[5]

Proposal 2: It is suggested to supplement the purpose of initiating OD-SIB1 procedure, for example, changing the highlighted description to “…in order to determine the suitability of a cell during cell reselection and to determine broadcast status of OSI when camping on the cell supporting OD-SIB1.”

- Open Issue of 38.331 CR for NES

3.1 （Issue 2）SI change procedure in NES Cell

As for the UE behavior after receiving SI change notification, RAN2 has reached an agreement in 126 meeting[6]：

Agreement

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Upon reviewing the discussion minutes, we identified that the topic leading to the agreement is in the RAN 1 Case 3 scenario: If Cell A can provide the SIB1 of NES cell, then after the UE camps on NES cell, if the SIB1 of NES cell is updated, does it need to return to Cell A to request the SIB1 of NES cell? The following is the record of the 126th meeting report[6]:

2. Case 3 in RAN1 (P1: R2-2404226: Samsung)

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

- Postponed.

[Fraunhofer]: Ok with the proposal. In addition, we need to consider how to reduce the signalling overhead. [Nokia, Ericsson]: Case 3 requires quite close coordination between two cells in order to guarantee the latest SIB1. [Ericsson]: Once the UE gets SIB1 from cell A and camps on NES cell, if the UE requires OD-SIB1 again for the camped NES cell, will the UE back to cell A and request it again? [Samsung]: Once camped on the NES cell, the SIB1 will be maintained by SIB change notification, so the scenario Ericsson mentioned will not happen.

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

In Case 2, we understand that this conclusion merely indicates that after camping on the NES cell, the UE acquires the updated SIB1 from the NES cell without returning to Cell A. It does not imply that the NES cell will broadcast SIB1 after the system message changes, nor that the NES cell will switch between transmitting NCD-SSB and transmitting CD-SSB.

In legacy spec, the network broadcasts the updated SIB1 in the next modification period (MP) after notifying UEs about changes in system information (SI). If no UEs are present in the NES cell during this broadcast, it results in wasted resources and energy. Therefore, to better align with the goal of NES, the network will not actively broadcast the updated SIB1 after sending the SI change indication. Instead, the UE should trigger WUS transmission to request SIB1 after receiving SI change indication.

Proposal 3: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

Based on the proposal 3, we believe that after receiving SI change notification, the NES UE camping on the NES cell will continue to initiate OD-SIB1 according to the parameters in WUS configuration to acquire the updated SIB1. But there is a problem with this situation, if CORESET#0 in the WUS configuration changes, resulting in a change in SIBx and triggering the SI change procedure, the UE camped on the NES cell may not be able to successfully receive the updated SIB1 using the stored expired CORESET#0. In this scenario, we consider the following possible solutions:

- Alt1: After a period of continuous SIB1 acquisition failure, the UE performs cell reselection.

- Alt2: Regardless of the K_SSB value in the MIB of NES cells, the PDCCH-ConfigSIB1 in MIB always provides CORESET#0 and SearchSpaceZero for receiving SIB1, consistent with the CORESET#0 and SearchSpaceZero values in the WUS configuration.

- Alt3: It is up to the UE’s implementation, e.g. when the SIB1 reception parameters in the WUS configuration change, the network broadcasts SIBx according to the SIBx scheduling information in SIB1. After a period of continuous SIB1 acquisition failure, the UE actively receives SIBx and then re-acquires the updated SIB1.

Proposal 4: RAN2 to discuss how the NES UE camped on the NES cell acquires SIB1 when the CORESET#0 in the WUS configuration of the NES cell changes, triggering SI change procedure in the cell. As following example options can be considered. Other existing options are not excluded.

- Alt. 1: After a period of continuous SIB1 acquisition failure, the UE performs cell reselection.

- Alt. 2: Regardless of the K_SSB value in the MIB of NES cells, the PDCCH-ConfigSIB1 in MIB always provides CORESET#0 and SearchSpaceZero for receiving SIB1, consistent with the CORESET#0 and SearchSpaceZero values in the WUS configuration.

- Alt. 3: It is up to the UE’s implementation, e.g. when the SIB1 reception parameters in the WUS configuration change, the network broadcasts SIBx according to the SIBx scheduling information in SIB1. After a period of continuous SIB1 acquisition failure, the UE actively receives SIBx and then re-acquires the updated SIB1.

- Conclusions

In this contribution, we discussed several detailed on-demand SIB1 issues, and made the following proposals:

Observation 1: Regardless of whether the SIB1 of the NES cell is being broadcast, legacy UEs cannot acquire the SIB1 and camp on the NES cell. No specification impact is foreseen.

Observation 2: The barring mechanisms to prevent legacy UEs from accessing cells providing on-demand SIB1 is the K_SSB in the MIB.

Proposal 1: It is recommended that the description of TS 38.300 Section 15.4.2.5 Camping Restriction be revised to “…based on the K_SSB in MIB”.

Observation 3: After cell reselection, the NES UEs should camp normally on the NES cell and do not need to acquire SIB1 to determine this cell suitability again.

Observation 4: When the NES UEs camp on the NES cell, they need to acquire SIB1 to confirm the broadcast status of the OSI in SI change scenario.

Proposal 2: It is suggested to supplement the purpose of initiating OD-SIB1 procedure, for example, changing the highlighted description to “…in order to determine the suitability of a cell during cell reselection and to determine broadcast status of OSI when camping on the cell supporting OD-SIB1.”

Proposal 3: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

Proposal 4: RAN2 to discuss how the NES UE camped on the NES cell acquires SIB1 when the CORESET#0 in the WUS configuration of the NES cell changes, triggering SI change procedure in the cell. As following example options can be considered. Other existing options are not excluded.

- Alt. 1: After a period of continuous SIB1 acquisition failure, the UE performs cell reselection.

- Alt. 2: Regardless of the K_SSB value in the MIB of NES cells, the PDCCH-ConfigSIB1 in MIB always provides CORESET#0 and SearchSpaceZero for receiving SIB1, consistent with the CORESET#0 and SearchSpaceZero values in the WUS configuration.

- Alt. 3: It is up to the UE’s implementation, e.g. when the SIB1 reception parameters in the WUS configuration change, the network broadcasts SIBx according to the SIBx scheduling information in SIB1. After a period of continuous SIB1 acquisition failure, the UE actively receives SIBx and then re-acquires the updated SIB1.

- References

- R2-25xxxxx 38300-i50 NES enh CR v05 rm.docx

- R2-2409502 RAN2#127bis Meeting Report MCC report

- Draft_RAN2_129_Meeting_Report_v3.docx

- Draft_RAN2_129bis_Meeting_Report_v2.docx

- TS 38.300 V18.3.0, NR; NR and NG-RAN Overall Description, 2024-09

- R2-2406202 Report of 3GPP TSG RAN WG2 meeting #126