3GPP RAN WG2 Meeting #131 	R2-2506065

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on L1 measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

During the after-meeting email discussion for the MAC open issues, issues related to L1 measurement are identified as to be discussed by company contributions:

| MAC-8 | FFS the coexistence between event triggered report and mTRP, e.g. how UE determine the current beam or candidate beam in this case? | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |
| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell | Issue Type: Essentialv How to address it: based on companies’ contribution |

In this contribution, we further discuss the remaining issues about L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

2.1 (MAC-8) coexistence with mTRP

mTRP is one of the most important features introduced to improve the coverage of the NW and mobility management is one of the most important features introduced to guarantee the service continuity. Thus, it is beneficial to support the coexistence of the mTRP and L1 MR.

Proposal 1: (MAC-8) RAN2 to support the coexistence of the mTRP and L1 MR.

To support the coexistence of the mTRP and L1 MR, there are two important issues to be solved:

- the event evaluation with multiple current beams.

- the report content with multiple current beams.

Even though the current beam is not the beam belongs to the serving cell, the NW could communicate with the UE with this beam. Thus, there is no need to treat the current beams differently. To have more up-to-date information about the candidate beams as well as the current beams, it is important to trigger the report as soon as one of the current beams is satisfied. Based on our understanding, as long as the best current beam meets the corresponding entry condition, all the current beams are not good enough to continue the serving. Thus, the UE could just evaluate the best beam whichever the beam is during the TTT. Thus, we propose:

Proposal 2: (MAC-8) For LTM3/5, if the best beam of the current beams met the trigger condition during TTT, then the report could be triggered when the corresponding TTT expires. If all the current beams don’t meet the trigger condition, the TTT stops.

The current beam would be reported without index since there is only one current beam when the mTRP and L1 MR are not configured simultaneously. However, if there is coexistence of the mTRP and L1 MR, the index of the reported current beam should be also included to identified beam for the measurement result.

Proposal 3: (MAC-8) The RSRP of the best current beam upon the TTT expiring is included in the MR MAC CE if this is mTRP.

Proposal 4: (MAC-8) The index of best current beam is also included in the MR MAC CE if this is mTRP.

2.2 (MAC-25) Handling of the activated SP CSI-RS for target cell after LTM cell switch

After the cell switch, the target cell becomes the serving cell of the UE. The L1 measurement for this cell is not only used for the cell switch determination, but also the monitoring the quality of the serving channel and assisting the configuration related to the channel quality. Thus, there is no need to continue the measurement of SP CSI-RS for target cell, UE could follow the CSI-RS measurement configuration of the serving cell which could be already delivered within the RRCReconfiguration container in LTM-candidate from this cell and applied upon the CSC.

Proposal 5: (MAC-25) UE deactivates SP CSI-RS resource of the target cell after LTM cell switch.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: (MAC-8) RAN2 to support the coexistence of the mTRP and L1 MR.

Proposal 2: (MAC-8) For LTM3/5, if the best beam of the current beams met the trigger condition during TTT, then the report could be triggered when the corresponding TTT expires. If all the current beams don’t meet the trigger condition, the TTT stops.

Proposal 3: (MAC-8) The RSRP of the best current beam upon the TTT expiring is included in the MR MAC CE if this is mTRP.

Proposal 4: (MAC-8) The index of best current beam is also included in the MR MAC CE if this is mTRP.

Proposal 5: (MAC-25) UE deactivates SP CSI-RS resource of the target cell after LTM cell switch.

4	References