3GPP TSG-RAN WG2 Meeting #131							R2-2506083

Bengaluru, India, 25 - 29 Aug, 2025

Title: 	Discussion on SONMDT for others

Source: 	Huawei, HiSilicon

Agenda item:	8.10.3

Document for:	Discussion/Decision

- Introduction

For running 38.331 CR for R19 SONMDT, open issues have been discussed in [1]. For now, NSAG logging issue is still open and the rapporteur's summary is shown as below:

Open issue RRC-3: NSAG logging on slice-based reselection failure In the meeting RAN2#130 RAN2 discussed logging the NSAG information when the UE fails to find a suitable cell upon cell selection/reselection based on slice priorities. In the end RAN2 agreed to log the highest ranked NSAG information potentially in a report. - We aim to specify that: If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG. FFS the need of cell or frequency info. However, RAN2 didn’t converge on how to associate such information in a SON/MDT report. The main problem with this scenario is that the UE could not find a suitable cell in the frequency at which it searches for suitable cells based on the provided NSAG priorities. Therefore, in order to efficiently investigate this issue rapporteur’s proposal is that companies provide their input on how to fix this issue by providing a text proposal to the next meeting. [Open issue RRC-3] concerning logging highest ranked NSAG info associated with cell, freq etc.) companies are invited to submit their contribution addressing this issue with a text proposal reflecting their solution.

In this paper, we provide our analysis on this issue.

- Discussion

Firstly, we think it should be straightforward to capture the following UE behavior into RAN2 specs:

If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG.

Currently, the UE checks logging MDT configuration in idle/inactive state, and then performs measurement logging, i.e. adding one LogMeasInfo-r16. For the above UE behavior, the possible RAN2 impacts are:

(1) add one flag into LogMeasInfo-r16, i.e. NSAG-ID-r17 for the highest ranked NSAG information

(2) update the procedural text by adding this flag

For the FFS the need of cell or frequency info, we are unclear about the need of such information as the NW should be able to retrieve corresponding information based on legacy logged MDT data and the above flag.

Proposal: (RRC-3) It is sufficient to reply on the following UE behavior (from the previous RAN2#130 agreement):

If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG.

And then the possible RAN2 impacts are:

(1) add one flag into LogMeasInfo-r16, i.e. NSAG-ID-r17 for the highest ranked NSAG information

(2) update the procedural text by adding this flag

- Conclusion

In this paper, we provide our analysis on Open issue RRC-3. It is proposed:

Proposal: (RRC-3) It is sufficient to reply on the following UE behavior (from the previous RAN2#130 agreement):

If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG.

And then the possible RAN2 impacts are:

(1) add one flag into LogMeasInfo-r16, i.e. NSAG-ID-r17 for the highest ranked NSAG information

(2) update the procedural text by adding this flag

- References

[Post130][605][SONMDT] Running NR RRC CR (Ericsson)