3GPP TSG-RAN WG2 Meeting #131	R2-2506130

Bangalore, India  Aug 25th – 29th , 2025

Source:				ETRI

Title:					Remaining issues of XR rate control

Agenda Item:		8.7.6 XR rate control

Document for:		Discussion and Decision

1. Introduction

During the discussion of XR rate controls at the RAN2#130 meeting, agreements on UL Rate Control MAC CE and UE behavior have been made as the followings:

Agreements on XR rate control: - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - We rule out explicit signalling of PDU session ID + QFI - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping. - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - No optimization is needed to address DC case for XR rate control MAC CE - We capture in stage-2 that XR rate indication is for application and not for MAC entity. - RAN2 will not specify DL rate control and DL rate control query

During the e-mail discussion on MAC running CR, the open issues related to the XR rate control was identified as the followings:

Open issues on XR rate control: [MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE. [MAC-4] Format of the UL Rate Control MAC CE. [MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

In this contribution, we provide discussions on the open issues for the XR rate control.

2. Discussion

UL Rate Control MAC CE

[MAC-3]	Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

During the last meeting, there was a discussion regarding the identifier to be included in the Uplink (UL) Rate Control MAC CE, and it was agreed to down-select between an explicit and an implicit method. The explicit method identifies a QoS flow using a combination of the DRB ID and QFI, which are configured during the RRC setup procedure. With a field size of 5 bits for the DRB ID and 6 bits for the QFI, these fields together would occupy 2 octets within the UL Rate Control MAC CE. Using the DRB ID instead of the Logical Channel ID (LCH ID) is appropriate, as the recommended bit rate is information that is ultimately conveyed to higher layers.

To reduce this size, an implicit method is being considered. This approach would involve allocating a shorter, 1-octet identifier during RRC configuration that maps to the DRB ID + QFI combination. However, while this method can reduce the identifier size in the MAC CE, it requires an additional mapping process to be performed between the implicit identifier and the DRB ID + QFI.

This contribution proposes the adoption of the explicit method, consisting of the DRB ID + QFI identifiers. We propose designing the UL Rate Control MAC CE in a way that overcomes the size constraints by optimizing the overall structure and minimizing the overhead of these fields.

Proposal 1) A combination of DRB ID and QFI is to be used for the explicit identification of a targeted QoS flow in the new UL rate control MAC CE.

[MAC-4]	Format of the UL Rate Control MAC CE.

QoS flows are mapped to Data Radio Bearers (DRBs). Multiple QoS flows can be mapped to a single DRB, or a single QoS flow can be mapped across multiple DRBs. The identifiers for these mappings are composed of a DRB ID and a QFI. This structure, however, can lead to redundancy, especially in the fields of the Uplink (UL) rate control Medium Access Control (MAC) Control Element (CE).

Observation 1) An UL rate control MAC CE that includes multiple identifiers, each composed of a DRB ID and a QFI, often contains redundant fields.

For instance, consider two identifiers: DRB ID (10) + QFI (5) and DRB ID (10) + QFI (8). When both are included in a UL rate control MAC CE, the DRB ID (10) is a common field and is unnecessarily duplicated. A different scenario shows redundancy in the QFI, such as with DRB ID (10) + QFI (8) and DRB ID (13) + QFI (8), where QFI (8) is the common and duplicated field.

To solve this problem of field duplication and minimize the size of the MAC CE, a more efficient identification scheme is used. The UL rate control MAC CE is configured to omit the common identifier field and indicate this omission. This is achieved by adding two new fields: a DI (DRB ID Included) field and a QI (QFI Included) field.

The DI field indicates whether the DRB ID field is explicitly included or if the previous DRB ID should be used as a common value. If the DI field is set, the DRB ID is included. If it's not set, the previous DRB ID is implicitly reused. The QI field applies the same logic to the QFI field. The inclusion of these fields results in a more compact and flexible structure, as shown in the following examples (assuming a value of '1' for inclusion):

- When DI = 1 and QI = 1, both the DRB ID and QFI fields are present.

- When DI = 1 and QI = 0, only the DRB ID field is present.

- When DI = 0 and QI = 1, only the QFI field is present.

This method effectively reduces redundancy, allowing for more efficient signaling and a smaller UL rate control MAC CE.

To demonstrate the efficiency of this method, let's configure three recommended bit rates within a single UL Rate Control MAC CE. The first recommended bit rate corresponds to the identifier DRB ID 10 and QFI 5. Since this is the initial entry, both the DI and QI fields are set to '1', indicating that both the DRB ID and QFI fields must be explicitly included in the MAC CE. For the second recommended bit rate, the DRB ID remains the same at 10, while the QFI changes to 8. To avoid duplicating the DRB ID, the DI field is set to '0', signaling that the DRB ID from the previous entry should be reused. The QI field is set to '1', as a new QFI (8) is being introduced. Finally, the third recommended bit rate involves a new DRB ID of 13 but reuses the QFI of 8 from the previous entry. In this case, the DI field is set to '1' to include the new DRB ID (13), while the QI field is set to '0', indicating that the QFI (8) is common and can be reused.

This clever use of DI and QI fields significantly reduces the overall size of the MAC CE by transmitting only the necessary, non-redundant fields for each subsequent entry.

Proposal 2) To efficiently represent DRB ID and QFI fields, the UL Rate Control MAC CE includes DI and QI fields that define their format:

- The DI and QI fields indicate whether the respective DRB ID or QFI field is explicitly included, or if the DRB ID or QFI from the previous entry should be reused as a common value.

3. Conclusion

In the discussion we made the following observations:

Observation 1) An UL rate control MAC CE that includes multiple identifiers, each composed of a DRB ID and a QFI, often contains redundant fields.

We propose the followings:

Proposal 1) A combination of DRB ID and QFI is to be used for the explicit identification of a targeted QoS flow in the new UL rate control MAC CE.

Proposal 2) To efficiently represent DRB ID and QFI fields, the UL Rate Control MAC CE includes DI and QI fields that define their format:

- The DI and QI fields indicate whether the respective DRB ID or QFI field is explicitly included, or if the DRB ID or QFI from the previous entry should be reused as a common value.

4. References