3GPP TSG-RAN WG2 Meeting #131													     R2-2506186

Bangalore, India, Aug 25th-29th 2025

Agenda item:	8.1.3

Source:	NTT DOCOMO, Apple, vivo, Xiaomi, OPPO

Title:	User consent for NW-side data collection

Document for:	Discussion and Decision

1	Introduction

RAN2 agreed to support NW-side data collection for AI/ML (in both AI/ML PHY and recently also in AI/ML Mobility). We believe that this feature has important privacy implications, that require user consent as we explain in this contribution.

2   	Discussion

RAN2 have agreed to support network-side data collection for AI/ML PHY using a variant of immediate MDT, as per (for example) the stage-2 running CR [3]: “For NW-side model, model training, data collection can be initiated by OAM (via immediate MDT) or by gNB.”.

Furthermore, most recently RAN2#129bis also agreed to support a similar mechanism for AI/ML Mobility:

Agreements

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

One important issue we need to discuss in this context, which applies to both AI/ML PHY and Mobility, is user privacy.

There are two, somewhat different but related privacy issues that need to be addressed:

- Collection of measurements, and

- Storage of measurements (both “raw” logged measurements and measurements “embedded” in an AI/ML model).

Measurements, collected over time for a UE, is obviously sensitive and private information. Here it is important to note that we are not talking about a single measurement, but rather a time-series measurements which include not only the current location of a user but also their mobility patterns. The meta data stored alongside the measurements is also likely to include timing information and location (even if location is not reported by a UE, the network can have a fairly good estimate of it through e.g. E-CID). In the AI/ML PHY feature, this information can be combined with beam-level information to provide even more precise UE location estimate.

We expect counter arguments along the lines that the data “can” be anonymized and is “likely” to contain time-series which are limited in time. While of course anonymization is always welcome, this is not something that 3GPP can standardize, as it is a network implementation. With regards to the length of time-series collected (and stored, see more on this below), there are two issues:

- First off, it is up to the network implementation to decide the length of a time-series to store – this is not something that standards can limit as we never specify network algorithms

- Secondly, even if a long time-series with UE mobility is broken up into smaller “pieces”, this does not guarantee privacy as such disparate pieces can be correlated to reconstruct a long enough UE mobility pattern

Observation 1: measurements collected for NW-sided AI/ML training is private and sensitive information.

An additional and separate (from collection) privacy related issue is storing the information. Here the concern is not just privacy (which is important enough in and out of itself), but also regulation. The issue is that in order to train AI/ML algorithms, especially deep NN algorithms, large data sets must be used. In the study companies used data sets of around 100K measurements and in reality, such data sets are likely to be much larger. Large data sets imply retaining sensitive information for long periods of time (to collect the data, to train the model and to update the model over time).

Observation 2: NW-sided AI/ML model training requires collection of large data sets; such large data sets must be stored for long periods of time to: collect enough data, train a model, refine the model over time and train other models.

In addition to storing the logged time-series measurements over long periods of time for training, the resulting trained model would actually contain (at least partially) some sensitive user location patterns information – potentially forever. There is a significant body of research on the topic of Reconstructing Training Data from Trained Neural Networks [1][2], which show there are significant likelihood that at least some private data may “leak” from trained models.

Observation 3: research shows that a trained model itself contains the mobility data, which potentially can be reconstructed – and therefore the mobility data is potentially stored forever in the model itself.

As mentioned above, this is a privacy issue, but additionally it is also a regulatory issue. For instance, General Data Protection Regulation (GDPR) in Europe which contains many elaborate requirements about how user sensitive data, collected and stored over time, is regulated. Other jurisdictions may have similar regulations.

Observation 4: GDPR regulates how user private information can be collected and stored over time.

All these considerations, user privacy and regulatory, lead to the following conclusion: NW-sided data collection requires user consent.

Proposal 1: NW-sided data collection for AI/ML model training requires user consent.

In terms of standards impacts, since we agreed to rely on a variant of MDT for data collection, it makes sense  to re-use the same user consent framework as the one used for MDT.

Proposal 2: re-use the existing user consent framework (defined for MDT) for NW-side data collection.

3	Observations

Observation 1: measurements collected for NW-sided AI/ML training is private and sensitive information.

Observation 2: NW-sided AI/ML model training requires collection of large data sets; such large data sets must be stored for long periods of time to: collect enough data, train a model, refine the model over time and train other models.

Observation 3: research shows that a trained model itself contains the mobility data, which potentially can be reconstructed – and therefore the mobility data is potentially stored forever in the model itself.

Observation 4: GDPR regulates how user private information can be collected and stored over time.

4	Proposals

Proposal 1: NW-sided data collection for AI/ML model training requires user consent.

Proposal 2: re-use the existing user consent framework (defined for MDT) for NW-side data collection.

5	References

[1]		Reconstructing Training Data from Diverse ML Models by Ensemble Inversion, Qian Wang, Daniel Kurz

[2]		Reconstructing Training Data from Trained Neural Networks, Niv Haim, Gal Vardi, Gilad Yehudai, Ohad Shamir, Michal Irani

[3]		R2-2501806, Introduction of AI/ML for NR Air interface feature in TS38.300, vivo