3GPP TSG-RAN WG2 Meeting #131bis		R2-2506847

Prague, Czech Republic, Oct. 13th-17th

Source:	China Telecom

Title:	Discussion on remaining MAC issues

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the remaining MAC issues.

- Discussion

Issue 1 Discussion on UE behavior if the TAT for PUSCH is expired while the TAT for PUCCH is running

According to Rapporteur summary, UE behavior can be the following options:

- Option-1: release the associated PUCCH;

- Option-2a: not release PUCCH and transmit the Report Indication on PUCCH;

- Option 2b: not release PUCCH and not transmit the Report Indication on PUCCH.

It is noted that the pucch-Resource is described as follows in the current RRC specification. It can be seen that the PUCCH-resource configured here is used for the UE initiated CSI reporting indicator for both mode-A and mode-B UE initiated CSI reporting. Therefore, it is preferred to release the associated PUCCH in this scenario for it can improve resource efficiency. Regarding the view of the proponents of option 2a/2b that releasing PUCCH resources would do harm to the efficient utilization of PUCCH resource to transmit HARQ-ACK, we think that prior the introduction of this pucch-Resource, other mechanisms already existed to ensure HARQ-ACK. Transmission. Thus, releasing the PUCCH resource in this scenario will not have a significant impact on the transmission efficiency of HARQ-ACK. On the other hand, if the description of pucch-Resource in current 38.331 CR does not restrict the purpose of PUCCH-resource, then using this PUCCH resource for HARQ-ACK or other purposes constitutes an additional benefit brought by the configuration of this pucch-Resource. Further consideration is needed to figure out whether retaining this PUCCH resource harms or improves overall resource efficiency.

pucch-Resource Indicates the periodic PUCCH resource for the UE initiated CSI reporting indicator for both mode-A and mode-B UE initated CSI reporting: - to request dynamically scheduled PUSCH to carry UE-initiated/event-driven beam report for mode-A; - to notify the network of a Type-1 CG PUSCH to carry UE-initiated/event-driven beam report for mode-B.

Proposal 1[Issue-1]: Prefer Option-1, i.e., release the associated PUCCH. And it is OK with both TPs.

Issue 2

For UEI reporting, PUCCH and type-1 CG PUSCH can be configured on different cells and different BWPs. We agreed the proposal that UE does not transmit a PUCCH for beam measurement report notification in Mode B if the BWP for the associated PUSCH is deactivated is agreeable.  However, MAC spec can be revised to make this behavior clearer. Two reference TPs are provided below.

TP-1: Add a Note Clause 5.15.1: … 1> if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG: 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not monitor the PDCCH on the BWP; 2> not transmit PUCCH on the BWP; 2> not report CSI for the BWP; Note: if the CSI report is a UE initiated beam measurement report in Mode B, not transmit a PUCCH for beam measurement report notification on a different BWP.

TP-2: For each activated Serving Cell configured with a BWP, the MAC entity shall: 1> if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG: 2> transmit on UL-SCH on the BWP; 2> transmit on RACH on the BWP, if PRACH occasions are configured; 2> monitor the PDCCH on the BWP; 2> transmit PUCCH on the BWP, if configured; 2> report CSI for the BWP; 2> transmit SRS on the BWP, if configured; 2> receive DL-SCH on the BWP; 2> (re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2; 2> if lbt-FailureRecoveryConfig is configured: 3> stop the lbt-FailureDetectionTimer, if running; 3> set LBT_COUNTER to 0; 3> monitor LBT failure indications from lower layers as specified in clause 5.21.2. 2> if Mode-B PUSCH for UEI reporting is configured on another BWP which is deactivated: 3> not transmit the PUCCH configured for UEI beam measurement report notification in Mode-B on the BWP, if configured.

Proposal 2[Issue-2]: It is preferred to make it clearer on MAC spec that UE does not transmit a PUCCH for beam measurement report notification in Mode B if the BWP for the associated PUSCH is deactivated. Two TPs are provided for reference.

TP-1: Add a Note Clause 5.15.1: … 1> if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG: 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not monitor the PDCCH on the BWP; 2> not transmit PUCCH on the BWP; 2> not report CSI for the BWP; Note: if the CSI report is a UE initiated beam measurement report in Mode B, not transmit a PUCCH for beam measurement report notification on a different BWP.

TP-2: For each activated Serving Cell configured with a BWP, the MAC entity shall: 1> if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG: 2> transmit on UL-SCH on the BWP; 2> transmit on RACH on the BWP, if PRACH occasions are configured; 2> monitor the PDCCH on the BWP; 2> transmit PUCCH on the BWP, if configured; 2> report CSI for the BWP; 2> transmit SRS on the BWP, if configured; 2> receive DL-SCH on the BWP; 2> (re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2; 2> if lbt-FailureRecoveryConfig is configured: 3> stop the lbt-FailureDetectionTimer, if running; 3> set LBT_COUNTER to 0; 3> monitor LBT failure indications from lower layers as specified in clause 5.21.2. 2> if Mode-B PUSCH for UEI reporting is configured on another BWP which is deactivated: 3> not transmit the PUCCH configured for UEI beam measurement report notification in Mode-B on the BWP, if configured.

Issue 10

Proposal 8 in R2-2507497 discussed if a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, when does UE stop staying in DRX active time. And two options are provided:

- Option 1: until the next PUCCH resource for the UEIRI.

- Option 2: Introduce a maximum PDCCH monitoring time period

We prefer Option 1. Since the PUCCH resource for UEIRI is periodic, if the PDCCH for the prior UEIRI is not received, the UE can send a new UEIRI in the next available PUCCH resource, if needed, to make UE stay in DRX active time. If not, UE can stop DRX active time when the next PUCCH resource for the UEIRI is available without extra configuration. In contrast, Option 2 requires a new parameter, and an improperly set monitoring window could either cause PDCCH reception failure or fail to mitigate power consumption.

Proposal 3[Issue-10]: Prefers Option 1, i.e., until the next PUCCH resource for the UEIRI.  And it is OK with the proposed TP.

- Conclusion

Based on the discussion, we have the following proposals.

Proposal 1[Issue-1]: Prefer Option-1, i.e., release the associated PUCCH. And it is OK with both TPs.

Proposal 2[Issue-2]: It is preferred to make it clearer on MAC spec that UE does not transmit a PUCCH for beam measurement report notification in Mode B if the BWP for the associated PUSCH is deactivated. Two TPs are provided for reference.

TP-1: Add a Note Clause 5.15.1: … 1> if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG: 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not monitor the PDCCH on the BWP; 2> not transmit PUCCH on the BWP; 2> not report CSI for the BWP; Note: if the CSI report is a UE initiated beam measurement report in Mode B, not transmit a PUCCH for beam measurement report notification on a different BWP.

TP-2: For each activated Serving Cell configured with a BWP, the MAC entity shall: 1> if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG: 2> transmit on UL-SCH on the BWP; 2> transmit on RACH on the BWP, if PRACH occasions are configured; 2> monitor the PDCCH on the BWP; 2> transmit PUCCH on the BWP, if configured; 2> report CSI for the BWP; 2> transmit SRS on the BWP, if configured; 2> receive DL-SCH on the BWP; 2> (re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2; 2> if lbt-FailureRecoveryConfig is configured: 3> stop the lbt-FailureDetectionTimer, if running; 3> set LBT_COUNTER to 0; 3> monitor LBT failure indications from lower layers as specified in clause 5.21.2. 2> if Mode-B PUSCH for UEI reporting is configured on another BWP which is deactivated: 3> not transmit the PUCCH configured for UEI beam measurement report notification in Mode-B on the BWP, if configured.

Proposal 3[Issue-10]: Prefers Option 1, i.e., until the next PUCCH resource for the UEIRI.  And it is OK with the proposed TP.