3GPP TSG RAN WG2 #131bis          	                                                R2-2506860

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Overview on 6G Radio Access Technology

Agenda Item:	10.2

Document for:	Discussion and Decision

- Introduction

In recent years, 5G significantly facilitates the industry modernization and enriches our lives. Along with these achievements, the growing demand from commercialization and technique evolution is now calling for a more efficient, intelligent and resilient wireless communication system, requesting extreme performance, flexibility, and scalability, as the fundamental infrastructure for the future decade.  Therefore, it is time to study a new generation of wireless communication system - 6G, rather than pursuing ‘add-on evolution’ based on 5G-A.

A new study item for 6G Radio has been approved in RAN#108 [1] and updated in RAN#109 [2]. RAN2 is requested to study radio interface protocol architecture and procedures for 6GR. In this contribution, we share our views on the outline and highlight of 6G radio interface protocol architecture and procedures, including: general considerations of 6GR design, deployment scenarios, existing and new services and UE capability framework. Details are elaborated in the following section.

- Discussion

  - General considerations of 6GR design

The user-centric network is the center of the 6G network design to target the 6G system design of improving the user experiences and meeting the QoS requirement of wireless services, minimizing the total cost of operation and maximizing the spectrum efficiency. The 6G system needs to have an intelligent system design and breakthrough in elevating the spectrum efficiency to provide the user with optimal service experience and diverse service types. The improvement of the spectrum efficiency needs to have sophisticate functions in radio resource and interference management scheme to dynamically allocate the resource with optimal link adaptation for each user. The AI/ML are the functions to enable the 6G system with emulated human intelligent and should be incorporated as the function for the radio resource and interference management with optimal link adaptations.

UE centric network is a comprehensive design, which involves many aspects across different layers, including, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling, etc. To approach the target of UE centric, the following aspects can be considered:

- Common UE ID in an area used for access procedure;

- Common configuration across cells per UE for optimal mobility.

Observation 1: The 6G network is designed as user-centric network to optimize the user experience, which involves, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling.

Some important KPIs for 6GR should be considered as following aspects:

- Energy efficiency: Energy efficiency is an important metric of sustainability, which is introduced as a new technical performance requirement in ITU-R for IMT-2030 [3]. It includes both network and device energy efficiency. Some aspects need to be considered to improve the energy efficiency including system information design, initial access design, UP and CP design.

- Spectrum efficiency: The requirements of spectrum efficiency for 6GR was agreed to support 3xIMT-2020 spectrum efficiency, which is an important KPI to improve system performance compared with 5G [3]. To improve the spectrum efficiency, more antennas need to be considered for both BS and UE. RAN2 also need to consider the efficient design for User Plane and Control Plane protocol and procedure.

- Service continuity: Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is discussed during RAN#109 and agreed as 0 ms at least for mobility between TRxPs of a single base station [3]. To achieve the 0ms mobility interruption time, some mobility technologies can be consider in 6GR, e.g., LTM.

- Latency: The latency of User Plane and Control Plane was discussed during RAN#109. 4ms for IC and 1ms for HRLLC were agreed for User Plane latency and 20ms was agreed for Control Plane latency [3]. To achieve these latency requirements, some User Plane and Control Plane functionalities and procedures should be designed to reduce the latency, e.g., ARQ/HARQ functionality.

- Security: Security is one of an important KPI for communication system. For 6GR, 5G AS security for SRB/DRB can be taken as baseline. Furthermore, MAC CE and System information security should be considered in 6G.

Observation 2: 6GR design should consider some important KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security.

The advantage of 5G design is flexible and configurable. Many options can be configured to support various use cases and scenarios. However, this will cause 5G design is very complicated. Thus, for 6GR, simplified and efficient is the most important design principle, including protocol architecture design, functionality design, procedure design and etc.

Observation 3: Compared with 5G, 6GR design should consider more simplified and efficient design.

NTN can provide services for those areas where the TN is not available or not efficient, and also for those services that can be more efficiently supported by the NTN system such as broadcasting service [4]. Per guidance of the SID [2], 6G aim at a harmonized 6G radio design for TN and NTN, including the integration. Therefore, 6GR design should consider the harmonized and integration of TN and NTN in 6G day1.

Proposal 1: From RAN2 point of view, the design target of 6GR should take into account:

- User-centric network design for optimized user experience;

- Substantial gains in term of KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security;

- Simplicity and efficiency design;

- Harmonized and integration of TN and NTN.

  - Supporting existing and new services

6GR design should consider support of existing and new services to meet the technical performance requirements for IMT-2030. Our general views on NTN, Positioning, AI, Immersive communication and Sensing are shared as following.

Non-Terrestrial Network

In NR/LTE, we supported NTN from Rel-17. However, at that stage, we focused on how to make the NTN system fit in the existing TN system, the NTN system is supported like a patch and its advantage is not fully displayed. Since 6G is starting from scratch, we expect to have a fully study in Day1 on how to integrate the TN and NTN to achieve neater design and better communication performance.

Observation 4: In NR/LTE, the NTN is supported like a patch of the existing TN system and its advantage is not fully displayed. In 6G Day1, it is expected to have a fully study on how to integrate the TN and NTN to achieve neater design and better communication performance.

In 6G, a multi-layer network deployment scenario can be considered, an example is given in the following figure. The deployment scenario comprises TN and NTN, with the NTN including LEO, MEO and GEO orbits.

Figure 1: 6G deployment scenario

For harmonized 6GR design and integration of TN and NTN, the technical aspects affected by NTN characteristics need to be studied and identified. Compare with the TN system, the NTN has characteristics of long propagation delay, high mobility and broad coverage. Therefore, it is unavoidable for us to make some NTN specific designs in 6G, including aspects of system information, mobility and measurement, TA maintenance, L2 timer handling, HARQ feedback mechanism, etc. Considering these basic issues have been studied in NR NTN/IoT NTN WI, 6G can take the NR NTN/IoT NTN designs as baseline. Apart from the NR/LTE specified features, to further improve the performance of 6G, further enhancements and new services are worth studied, e.g., enhancements on the mobility between TN and NTN with different orbits, NTN positioning, etc. It is proposed that:

Proposal 2: The deployment scenario of 6G comprises TN and NTN, with the NTN including LEO, MEO and GEO orbits. For harmonized 6GR design and integration of TN and NTN, study and identify the technical aspects affected by NTN characteristics.

Positioning

Based on the description in clause 5.4.2 in TR 22.870[6], the 6G system is assumed to support most of the existing 5GS service requirements, including the location service.

- 5.4.2	Support of legacy services

The 6G system shall be able to support the following services:

-	Location and positioning services, ref TS 22.261 [14], TS 22.071 [61],

6G networks must maintain backward compatibility with essential 5G positioning use cases, ensuring service continuity for E911 and regulatory emergency call positioning from Day-1. Retaining the 5G positioning accuracy definitions (e.g., sub-meter level for critical services) minimizes deployment risks and shortens time-to-market.

Observation 5: 6G shall be able to support most of existing 5G services, including positioning services.

There are multiple essential/regulatory services included in work task#1.4 which has been discussed by SA2 for 6G as follows:

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

SA2 will design architecture, functions and procedures to support location service, as well as takes how to support location service exposure, e.g. to 6GC NF, AF into consideration.

Observation 6: SA2 is discussing the 6G architecture, functions and procedures to support location service in 6G.

The position information may be requested by and reported to a client (e.g., an application) associated with the UE, or by a client within or attached to the core network. The Standard UE Positioning Methods has been defined clearly in TS 38.305.

Observation 7: The RAN is responsible for determining the UE positioning methods to be supported within its functional scope.

If RAT-Dependent positioning is supported in 6G, RAN1 should finalize reference signal designs and physical layer procedure for RAT-Dependent positioning methods, such as DL-TDOA, etc.

Observation 8: The design and evaluation of positioning reference signals (PRS and SRS) are up to RAN1.

Proposal 3: In order to support location service in 6G, RAN2 further study positioning-related functions, architecture, and procedures, with collaboration from SA2 and RAN3 as needed.

AI

It is expected that AI/ML will play an important role in 6G for tackling fundamental challenges and achieving improvements in mobility robustness, energy efficiency, and other critical KPIs. Study on AI/ML for 6G can comprise two aspects: framework and use case. Toward 6G AI/ML use cases study, we think the use cases can be grouped into two categories: specified AI/ML use cases in 5G-A, and new use cases that can be in 6G.

- 5G-A use cases

5G-A high-layer use cases include RRM measurement prediction, and measurement event prediction. Each of the use case has shown performance gains over the existing non-AI/ML baselines in simulations.

In 6G, these use cases can be further studied considering the evolvement with new design in 6G.

Observation 9:  RRM measurement prediction and measurement event prediction introduced in 5G-A shows benefits on overhead reduction and handover performance which are also essential use cases in 6G.

- New use cases

The LTM (L1/L2 Triggered Mobility) mechanism is a cell change process triggered by L1/L2 commands, designed to reduce delays, overhead, and interruption time caused by L2 reset behaviors, L3 measurements, and signaling procedures. Since AI/ML based L3 cell level RRM measurement prediction can enhance mobility performance or reduce measurement overhead for different use cases based on simulation result, it is hopeful for LTM prediction to show remarkable gain on mobility performance and overhead reduction. Simulation on AI/ML based LTM could be considered for the evaluation of performance.

Observation 10: For 6G high-layer new use cases, AI/ML based LTM brings significant improvement on mobility performance and lower measurement overhead.

The Conditional Handover is a mobility procedure where the UE can perform handover according to the conditional configurations from the network. Firstly, the source base station provides the UE with configuration information of one or more candidate cells along with execution conditions. Upon receiving the pre-configured cell information, the UE does not immediately initiate the handover process. Instead, it performs RRM (Radio Resource Management) measurements to evaluate the candidate cells. When at least one candidate cell meets the predefined execution conditions based on RRM measurement results, the UE selects a suitable candidate cell and triggers the handover process to access the target cell. Since the AI/ML based L3 cell level RRM measurement prediction can improve mobility performance, the conditional mobility scenario can be assumed to have benefits since the UE can perform CHO based on the predicted result without TTT delay.

Observation 11: For 6G high-layer new use cases, AI/ML based CHO can also be used to improve the handover performance.

Besides the use cases of LTM prediction and CHO prediction, other AI/ML use cases such as TA prediction can also be considered. In order to obtain the UL synchronization, the UE has to perform RACH procedure which involves preamble sending and RAR reception before PUSCH transmission, which brings additional latency especially for HO and initial access procedure. With AI/ML predicted TA, UE can perform access to NW without RACH procedure which reduces the transmission latency and further brings improvement on system performance.

Observation 12: For 6G high-layer new use cases, TA prediction is useful for latency reduction especially for handover and initial access procedure.

Proposal 4: For 6G high-layer air-interface AI/ML use cases, the following use cases can be considered RRM measurement prediction, measurement event prediction, AI/ML based LTM, AI/ML based CHO, TA predication, etc.

Immersive communication

Immersive communication extends the enhanced Mobile Broadband (eMBB) and includes the typical use cases of communication for immersive XR, remote multi-sensory telepresence, and holographic communications. Supporting mixed traffic of video, audio, and other environment data in a time-synchronized manner is an integral part of immersive communications, including also stand-alone support of voice [5].

In the current 6G SA1 TR [6], multiple use cases for immersive communication have been identified, such as Immersive Gaming [9.2] [9.9], Seamless Immersive Reality in Education [9.5], Collaborative services in multi-site immersive program viewing [9.6], and Holographic telepresence in healthcare [9.8], etc. There are three main aspects could be considered I RAN2: 1) Service continuity and improved user experience; 2) Adaptive QoS management, including dynamic QoS adaptation performance requirements, QoS policy adjustments based on network conditions, and QoS adjustments at different granularities; and 3) Synchronization requirements: Synchronizing multi-modal data streams of different media for each participant, as well as the data streams for coordinated participants.

In 5G, XR has been introduced since Rel-18 in RAN2. The technologies introduced in 5G XR can be taken as the basis for the immersive communication, especially for immersive XR. Compared to the early Releases of eMBB, the notable enhancements of 5G XR are XR Awareness and related technologies, such as the issues of PDU set. From 5G XR, the service characteristics controlled by RAN are no longer limited to static information per QoS flow, but also extended to burst-level and PDU set-level information. The core network informs RAN not only semi-static control plane information such as periodicity and traffic jitter, but also the dynamic user plane information like EoDB and PSI. Due to the introduction of XR Awareness, 5GS can enhance the transmission data rate, reduce transmission latency, and improve system capacity for XR services efficiently.

5G XR has made valuable explorations in service awareness. 6G immersive communication has the similar and extended requirements on service awareness so as to guarantee the requirements of service continuity and improved user experience. For example, immersive XR inherits the packets formats of data burst, PDU sets and other service characteristics such as variable packet size. Accurate perception of service and packets characteristics by RAN facilitates rational Uu configurations and optimal data scheduling. Therefore, 6G Day1 should support Service-Aware QoS management and scheduling.

Observation 13: Service Awareness is beneficial for addressing the data characteristics and service requirements of immersive communication, making it suitable as the information foundation for service transmission and scheduling.

The immersive communication has the characteristic of adaptive QoS. For example, user experience is related to the content complexity; simpler content requires less bitrate to achieve the same UX level as a more complex content. In the context of video transmission, simple content may refer to video frames that have low temporal variation (i.e. mostly static) or spatial variation (i.e. mostly plain). Complex content, on the other hand, refers to videos that are highly dynamic and rich in spatial information. For a given user, the content complexity is dynamic (varies with time), and for the network serving multiple users, the complexity could vary across users in the network also. From a network’s perspective, inability to set the bit rate appropriately may lead to suboptimal network resource allocation. [6]

In 5G, RAN nodes attend some QoS management such as alternative QoS for GBR and RAN feedback for adaptation of Burst Arrival Time and Periodicity. In 6G, RAN may consider tighter cooperation with SA2 for timely QoS management and parameters adjustment.

Observation 14: RAN2 should consider the impacts of adaptive QoS introduced by immersive communication.

Multi-sensory telepresence and holographic communications further raise the requirements for multi-modal transmission especially for the time-synchronized data transmission. While multi-modality was discussed in 5G XR, 5GS lacked native support, even for single-UE multi-modality. RAN could not accurately determine synchronization delay requirements related to packet sequencing. Although the synchronized multi-modal data streams could be guaranteed roughly at the application layer, RAN-level solution is essential for fine-grained management, so as to maximize system capacity for immersive communication. In 6G, multimodality support combined with other potential issues (e.g., FEC mechanisms) needs to be discussed in SA2 and RAN from the scratch.

Observation 15: Multi-modality and other potential issues (e.g., FEC mechanisms) should be discussed in SA2 and RAN from the scratch.

Proposal 5: For immersive communication, the following aspects can be considered: service awareness, adaptive QoS, multi-modality, etc.

Sensing

In 5G-A, TRP mono-static sensing for UAV use case is supported. In RAN#109, it is agreed that 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. For the sensing modes only involving TRP, there is no RAN2 impact. For the sensing modes involving UE, UE acts as a sensing signal transmitter and/or receiver. Thus sensing signal related configuration needs to be provided to the UE so that the UE can perform sensing. In addition, the UE acting as a sensing signal receiver needs to provide sensing data to the network. Thus a UE sensing data collection mechanism needs to be studied.

Proposal 6: In order to support the sensing modes involving UE, RAN2 should study procedures of UE sensing signal configuration and UE sensing data collection.

  - UE capability framework

In LTE UE capability framework, UE category approach was adopted to indicate the supported peak data rate.

5G adopted a flexible UE capability framework to accommodate diverse features and use cases. In 5G, all UE capabilities are explicit reported in UECapabilityInformation message. The size of UECapabilityInformation message grows exponentially due to complex framework and continuously increasing band combinations. In Rel-16, RAN2 initiated RACS WI to investigate efficient signaling approach for UE Capability information. Then UE capability ID and RRC segmentation were introduced for the excessive message size of UE capability. Neither approach modifies the fundamental framework of the UECapabilityInformation message.

Observation 16: Excessive message size is the critical challenge for 5G UE capability information.

For 6G UE capability framework design, simplification should be a key principle to address the signaling challenges observed in 5G.

In 6G day1, RAN2 should revisit the UE capability framework. Two feasible alternatives are proposed:

- Alt 1: 5G-like UE capability framework: Capabilities are explicitly signalled, with message structure enhancements to reduce overhead. E.g. multiple band combinations share a set of capability value;

- Alt 2: LTE-like category-based UE capability framework: RAN2 defines multiple UE categories, each representing a set of mandatory capabilities and optional features. Additionally, RAN discusses diverse device types based on SI[2] objectives, where UE categories may correlate with device types.

Regardless of the adopted framework, RRC compression can further reduce UE capability signaling overhead.

Proposal 7: Simplification is a key principle of UE capability framework in 6G. RAN2 evaluates potential UE capability frameworks from the following alternatives：

- Alt 1: 5G-like UE capability framework;

- Alt 2: LTE-like category based UE capability framework.

The UE capability signaling overhead can be further reduced through RRC compression.

- Conclusion

In this contribution, we provide our general views on 6G Radio Access Technology including below observations and proposals:

Observation 1: The 6G network is designed as user-centric network to optimize the user experience, which involves, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling.

Observation 2: 6GR design should consider some important KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security.

Observation 3: Compared with 5G, 6GR design should consider more simplified and efficient design.

Observation 4: In NR/LTE, the NTN is supported like a patch of the existing TN system and its advantage is not fully displayed. In 6G Day1, it is expected to have a fully study on how to integrate the TN and NTN to achieve neater design and better communication performance.

Observation 5: 6G shall be able to support most of existing 5G services, including positioning services.

Observation 6: SA2 is discussing the 6G architecture, functions and procedures to support location service in 6G.

Observation 7: The RAN is responsible for determining the UE positioning methods to be supported within its functional scope.

Observation 8: The design and evaluation of positioning reference signals (PRS and SRS) are up to RAN1.

Observation 9:  RRM measurement prediction and measurement event prediction introduced in 5G-A shows benefits on overhead reduction and handover performance which are also essential use cases in 6G.

Observation 10: For 6G high-layer new use cases, AI/ML based LTM brings significant improvement on mobility performance and lower measurement overhead.

Observation 11: For 6G high-layer new use cases, AI/ML based CHO can also be used to improve the handover performance.

Observation 12: For 6G high-layer new use cases, TA prediction is useful for latency reduction especially for handover and initial access procedure.

Observation 13: Service Awareness is beneficial for addressing the data characteristics and service requirements of immersive communication, making it suitable as the information foundation for service transmission and scheduling.

Observation 14: RAN2 should consider the impacts of adaptive QoS introduced by immersive communication.

Observation 15: Multi-modality and other potential issues (e.g., FEC mechanisms) should be discussed in SA2 and RAN from the scratch.

Observation 16: Excessive message size is the critical challenge for 5G UE capability information.

Proposal 1: From RAN2 point of view, the design target of 6GR should take into account:

- User-centric network design for optimized user experience;

- Substantial gains in term of KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security;

- Simplicity and efficiency design;

- Harmonized and integration of TN and NTN.

Proposal 2: The deployment scenario of 6G comprises TN and NTN, with the NTN including LEO, MEO and GEO orbits. For harmonized 6GR design and integration of TN and NTN, study and identify the technical aspects affected by NTN characteristics.

Proposal 3: In order to support location service in 6G, RAN2 further study positioning-related functions, architecture, and procedures, with collaboration from SA2 and RAN3 as needed.

Proposal 4: For 6G high-layer air-interface AI/ML use cases, the following use cases can be considered RRM measurement prediction, measurement event prediction, AI/ML based LTM, AI/ML based CHO, TA predication, etc.

Proposal 5: For immersive communication, the following aspects can be considered: service awareness, adaptive QoS, multi-modality, etc.

Proposal 6: In order to support the sensing modes involving UE, RAN2 should study procedures of UE sensing signal configuration and UE sensing data collection.

Proposal 7: Simplification is a key principle of UE capability framework in 6G. RAN2 evaluates potential UE capability frameworks from the following alternatives：

- Alt 1: 5G-like UE capability framework;

- Alt 2: LTE-like category based UE capability framework.

The UE capability signaling overhead can be further reduced through RRC compression.

- References

- RP-251881, Study on 6G Radio.

- RP-252912, Revised SID: Study on 6G Radio.

- RP-252882, 3GPP TS 38.914 v0.2.0, Study on 6G Scenarios and Requirements.

- 3GPP TS 38.913 v18.0.0, Study on Scenarios and Requirements for Next Generation Access Technologies.

- Recommendation ITU-R M.2160-0 Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU, 2023.11.

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements, V0.4.0.