3GPP TSG-RAN2 Meeting #131bis			                                                          R2-2506937

Prague, Czech Republic, 13th-17th  October

Agenda item:	10.3.3

Title:	Security Requirements in 6GR

Source:	CMCC

Document for:	Discussion

# 1	Introduction

RAN Plenary #108 agreed the RAN Working Group SID “Study on 6G Radio” in [1]RP-251881, including an objective related to RAN security aspects:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This paper is to provide our observation and view on security requirements in 6G.

# 2	Discussion

2.1	Security Protection of MAC CE

In 5G, multiple MAC CEs are introduced for different purposes, such as LTM cell switch, SCell activation/deactivation, PDCP duplication and TCI state activation/deactivation, etc. The ciphering and integrity protection related AS security functions are performed in  PDCP layer in 5G. And there’s no discussion on MAC CE’s security until the introduction of inter-CU LTM. For inter-CU LTM, NCC (Next Hop Chaining Counter parameter) value, which is associated with NH(Next hop), used for security key derivation in the next hop handover is indicated in LTM Cell Switch MAC CE.  In legacy L3 HO, NCC value is delivered by RRC signaling, in other words, it is encrypted and under integrity protection. Therefore, during the discussion of inter-CU LTM, both RAN2 and SA3 discussed whether and how to deliver NCC value with security protection concurrently. Due to the limited time and RAN2’s decision to use MAC CE for NCC value delivery, SA3 decided not to pursue a security protection way for MAC CE in 5G, also, SA3 declared the potential risk of increased attack scenarios and probability of handover failure with unprotected MAC CE in the replied LS [2].

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

Observation 1: There’s no security protection mechanism on 5G MAC CE, including LTM Cell Switch Command.

For 6G, based on companies’ RAN plenary contributions, LTM is possible to be inherited in 6G, due to its preponderances on cell switch latency and signaling overhead. Since 6GR is to meet the growing requirements of different services, achieve a more efficient and safer network, we think LTM Cell Switch Command MAC CE should be considered with security protection. Without security protection, the UE may switch to a wrong target cell or access to the right target cell with wrong parameters like wrong NCC value, as shown in Figure 1, both of the two cases may result in LTM cell switch failure and RRC re-establishment, increasing UE’s cell switch latency and reducing mobility success probability.

Figure 1 LTM failure due to tampered LTM CSC

What’s more, the MAC CE is expected to take a more important role in 6G, with more information delivered by MAC CE. If MAC CE security protection can be supported from 6G day1, it may help to facilitate the potential discussion in different 6G topics. From our point of view, like LTM cell switch command, 6G MAC CEs supporting services with high reliability requirements or privacy/security  requirements should be protected. In other words, not all MAC CEs need to be protected.

Observation 2: Supporting security protection on MAC CE from 6G day1 could facilitate to the smoother discussion across various 6G topics.

As mentioned above, current security related function are all included in PDCP layer, if security protection on MAC CE is supported in 6G, which layer to perform such function should be discussed in RAN2. Two potential solutions can be considered:

Option 1.  All security functions are performed in the same layer, i.e., PDCP;

Option 2.  Different layers perform security protection on different data/signaling, i.e., PDCP is responsible for data and RRC signaling security protection, while security protection on MAC CE is performed in MAC.

Option 1 enables the centralized design of security function within the UE, which simplifies the protocol design, but it may introduce extra interaction between layers and extra time delay, especially in case there’s intermedium layer (RLC) if current protocol design is inherited in 6G. Besides, for CU-DU splitting architecture, additional F1 signaling and latency should be considered. What’s more, since MAC CE is a specific signaling for UE, how to handle it during the inter-layer transmission to avoid the confusion with other data/signaling, may need great effort of RAN2.

As to option 2, though it seems a duplicated function design, it may enable different security algorithms in different layers. And from UE perspective, it’s a clearer way to distinguish MAC CE and other data from upper layers.

Observation 3: With security protection function centralized in one layer like PDCP, it may introduce extra inter-layer interactions  and extra time delay, and it may results in confusion at the UE between MAC CE and other data.

What’s more, currently, security protection function in PDCP layer includes ciphering and integrity protection, when it comes to security protection function in MAC layer, whether both functions are needed, or new security functions should be introduced, whether they all should be applied to a specific MAC CE type, need further discussion in both RAN2 and SA3.

2.2	Security Protection of System Information

System information broadcast by RAN nodes is not protected in 5G&5G-A. A cell periodically broadcasts synchronization signals and SI. These broadcasted messages are intended for all UEs which are camping on a cell. In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services.

The TR33.809[2] in R18 have addressed key-issues and solutions that will potentially enhance 5G system's resistance to false base stations even further. It also includes security protection for system information, key-issue #2, related to RAN2.

5.2 Key Issue #2: Security protection of system information 5.2.1 Key issue details Broadcasting system information (SI) is one of the functions of the RRC protocol, defined in TS 38.331 [2]. A cell periodically broadcasts synchronization signals and SI. These broadcasted messages are intended for all UEs which are camping on a cell. In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services. The system information includes information, among others, like cell (re-)selection parameters, neighbouring cell information, frequency priority, blocklisted cell, common channel configuration information, NAS common information, and public warning system (PWS) messages. In general, the system information is applicable for UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED. UE in IDLE mode performs PLMN selection, monitor paging, performs cell selection, cell re-selection, and applies access control before making an access attempt. In future releases, other services such as MBMS, proximity services, etc. are also likely to be supported by UEs in IDLE mode. This key issue is about investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). Since SI messages are broadcast messages meant for all UEs, it is not apparent that an integrity and replay protection is strictly necessary. Nevertheless, in general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages. SA3 has studied the PWS security in TR 33.969 [3], in which the security mechanisms used to protect the Warning notifications message from Public Warning System against false base station can be reference for the study in the present document. There are some distinct challenges when studying the broadcast messages protection as below: a) Key management. It is because of heterogenous trust-boundaries, and diverse regulations (or requirements) per countries (or regions); b) Time synchronization. It is because of difficulty to achieve fairly acceptable time synchronization between one gNB and other gNBs, and between UEs and gNBs; c) Signaling complexity. It is because of restrictive signaling expected from UEs in RRC_IDLE. Nevertheless, it is only prudent if the 5G system could be enabled (i.e., support) to achieve protection of SI messages in general. NOTE 1: This key issue is concerned with the "over-the-air" interface. Therefore, integrity protection of SI "within-the-network" is not in the scope of this key issue. 5.2.2 Security Threats Lack of protection of SI could potentially have following impacts in some cases: - DoS attack on UE - Rogue services 5.2.3 Potential Requirements 5G system should provide a means to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a cell.

Among them, the security of system information mainly involves the following threats:

#1 DoS attack on UE: attempts to hinder the UEs' access to the network.

#2 Rogue services: attempts to deliver unauthorized or unsolicited services (e.g., SMS and calls) to the UEs.

As companies were unable to reach an agreement on the various solutions and did not clarify whether the threat of system information protection in 5G was serious or not, it was concluded in TR 33.809 as follow:

7.2 Conclusions on Key Issue #2 This key issue is not concluded.

Observation 4: Although SA3 studied the security design of system information during the SI stage, the final NR specification does not capture the security protection of system information.

Nevertheless, we can summarize its potential threats and impacts on the system information design of RAN2.

Threats besides DoS and Rogue service attack：

—Public safety threats in ETWS and CMAS related SIBs.

—Service attack of MBMS, NTN and any other service-related SIBs.

Impacts：

—SIB Overhead: Signature/integrity protection/hash fields need to be transmited along with system information, and the corresponding overhead may have a significant impact on the design of system information

—System information that is protected and does not need to be protected: Whether all SI information or some part need to be protected and how to indicate partially protected system information.

—Design after recognized false system information: For example, UE may need leave the current cell and select a new cell when the signature verification fails.

Figure 2 False Base Station Threats of System Information

Look forward to 6G, system information still has broadcast characteristics, so the risk of replay attacks may still exist. This requirement has already been captured in SA1 SID “Study on 6G Use Cases and Service Requirements” , see TR 22.870 V0.3.1 [3]

5.5.3 Use case on UE selecting a base station after assessing its legitimacy 5.5.3.1 Description The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS is a major security flaw in the existing communication system. There is an important opportunity to address this concern in the next generation wireless systems. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system. By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, False Base Station attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure.

SA3 is in the process of  6G SID: Way forward for 6G Study Item on Security for the 6G System. From our prespective, the key issue is about whether the system information in 6G network face the above security threats of or or what new potential problems exist. SA3 needs to first determine whether there is an agreement on this issue to avoid ineffective solution discussions. Then, SA3 can investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is or other threats. We can wait for their progress and then consider whether it is necessary to enhance the protection of system information and further impacts of 6G system information design.

Observation 5: A key question to SA3 regarding system information security is whether the system information in 6G network is genuinely faces security threats or what are new potential risks.

In the just-ended RAN plenary meeting, an LS is sent to SA3 to reflect RAN’s security requirements to SA3 as following [4]:

1. Overall Description: As already reported at SA#108, RAN Plenary #108 agreed the RAN Working Group SID “Study on 6G Radio” in RP-251881. This SID includes an objective related to RAN security aspects as below. Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signaling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] Following further discussions during RAN#109, RAN Plenary believes that the security requirements for the access stratum can significantly influence the overall system design. Therefore, it is crucial to establish when these security requirements will be aligned between RAN and SA and particularly between SA3 and RAN2. TSG RAN believes that June 2026 would be an appropriate date. RAN invites SA to consider adding a June 2026 check point (in SA3’s SID on 6G security S3-252973) for the alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design. 2. Actions: To SA: ACTION: RAN invites SA to consider adding a June 2026 check point (in SA3’s SID on 6G security S3-252973) for the alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design.

Based on the discussion above, we propose RAN2 to include the security protection on MAC CE and system information  as requirements for the following alignment between SA3 and RAN2.

Proposal 1: RAN2 is proposed to incorporate the security protection for MAC CEs and System Information as a requirement for subsequent alignment between SA3 and RAN2, and send LS to SA3 to clarify RAN2’s requirements. The detail solution should be determined by SA3.

Proposal 2: The MAC layer should be responsible for security protection operation on MAC CE:

- Not all types of MAC CEs will require security protection;

- Detailed security functions to be supported at the MAC layer, such as ciphering, integrity protection and other potential functions should be further discussed in RAN2 and SA3.

# 3	Conclusion

In this contribution, the following observations and proposal is provided.

Observations:

Observation 1: There’s no security protection mechanism on 5G MAC CE, including LTM Cell Switch Command.

Observation 2: Supporting security protection on MAC CE from 6G day1 could facilitate to the smoother discussion across various 6G topics.

Observation 3: With security protection function centralized in one layer like PDCP, it may introduce extra inter-layer interactions  and extra time delay, and it may results in confusion at the UE between MAC CE and other data.

Observation 4: Although SA3 studied the security design of system information during the SI stage, the final NR specification does not capture the security protection of system information.

Observation 5: A key question to SA3 regarding system information security is whether the system information in 6G network is genuinely faces security threats or what are new potential risks.

Proposals:

Proposal 1: RAN2 is proposed to incorporate the security protection for MAC CEs and System Information as a requirement for subsequent alignment between SA3 and RAN2, and send LS to SA3 to clarify RAN2’s requirements. The detail solution should be determined by SA3.

Proposal 2: The MAC layer should be responsible for security protection operation on MAC CE:

- Not all types of MAC CEs will require security protection;

- Detailed security functions to be supported at the MAC layer, such as ciphering, integrity protection and other potential functions should be further discussed in RAN2 and SA3.

# 4	References

[1] RP-251881, “New SID: Study on 6G Radio;

[2] TR 33.809 V18.1.0;

[3] TR 22.870 V0.2.1;

[4] RP-252891   LS on Early Alignment on Access Stratum security aspects.

# Annex: Draft LS

3GPP TSG-RAN2 Meeting #131bis			                                                      RP-25xxxxxx

Prague, Czech Republic, 13th-17th  October

Title:	LS on security requirements in 6G

Response to:

Release:	Rel-20

Work Item:	FS_6G-REQ

Source:	CMCC[To be RAN2]

To:	SA3

Cc:

#### Contact Person:	Li Chai

#### E-mail Address:	chaili@chinamobile.com

1. Overall Description:

RAN2 noted that SA3 would initiate discussions on the SID 6G Study Item on Security for the 6G System. To prevent misalignment between SA3 and RAN2 on security requirements related to AS protocol during SA3 study on the 6G security issues, this LS provides potential security requirements from RAN2’s perspective for SA3’s consideration:

- System Information (SI): Although the TR33.809 in R18 studied the potential schemes to protect system information against false base stations attacks, system information protection mechanism finally has not been specified in 5G. Looking ahead to 6G, system information will still be in broadcast manner, so the risk of replay attacks may still persist.

- Security protection on MAC CE: During the 5G LTM discussions, both RAN2 and SA3 identified the potential security risks due to NCC value delivery via MAC CE. However, due to the  time constraints, no protection mechanism for MAC CE  was specified in SA3. Given that LTM is expected to be one of the important mobility mechanisms  in 6G, it’s necessary to specify security protection solution on LTM Cell Switch Command MAC CE. And it would be highly appreciated if a common solution could be designed to protect multiple types of MAC CEs at the MAC layer, to support various services in 6G, especially  those  with high reliability requirements or stringent privacy/security requirements.

2. Actions

To SA3 group

ACTION: 	RAN2 respectfully asks SA3 to take the above RAN2 requirements into account. To progress the 6G study,RAN2 expects to receive the feedback from SA3 until RAN2#133(09 - 13 Feb, 2026).

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #132	17 - 21 November   2025	            Dallas, US

TSG-RAN2 Meeting #133	09 - 13 February     2026	            Stor-Göteborg, SE