3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507085

Prague, Czech Oct 13th – 17th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Comparison of solutions for voice call support over NB-IoT NTN

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

In RAN#108, a Rel-20 new WID [1, RP-251867] for IoT over NTN Phase 4 enhancements has been agreed. It has been further revised in RAN#109[2, RP-252473]. The main objective of this WID is to support IMS voice call over GSO case of NB-IoT over NTN, as following:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

For supporting voice over NB-IoT NTN, SA2 has already had much study and several solutions have been proposed, which can be roughly categorized into two types: one based on Control Plane CIoT EPS Optimization (CP solution), and the other based on User Plane CIoT EPS Optimization (UP solution). Till now, no conclusion has been achieved in SA2 yet regarding the preference for the solution and their analysis/comparison is still on-going. SA4 has also conducted research on topic “Ultra Low Bit rate Speech Codecs (ULBC)” for a period of time. At the RAN2#131 meeting, RAN2 received a LS from SA4 [3, S4-251584] on the RAN simulation assumptions for ULBC but did not response it. After RAN2#131 meeting, two more LSs were received from SA2 in [4, S2-2507578] [5, S2-2507636].

As the first step, based on all the previous progress in other working groups, RAN2 needs to down-select the baseline solution for supporting IMS-based voice service for NB-IoT over GSO. In this paper, we will discuss the relevant issues and give our observations and proposals.

- Discussion

  - General aspects

IMS (IP Multimedia Subsystem)-based voice is an upper-layer application that relies on underlying transport and signaling protocols (e.g., SIP, RTP, IP). It’s typically a service with relatively fixed packet size and packet interval, and requires strict Quality of Service (QoS).

NB-IoT is primarily designed for low-power, low-data-rate, and massive machine-type communication (mMTC) IoT applications. These applications typically involve small amounts of data, infrequent transmissions, long battery life requirements (years), and deep indoor or underground coverage — but do not require low latency, high bandwidth, or real-time communication, like voice.

Shortly to say, IMS-based voice service is not directly supported in legacy NB-IoT. Using NB-IoT to transmit voice service involves many aspects that require further research.

    - Main data transmission schemes in legacy NB-IoT/NB-IoT over NTN

Legacy NB-IoT supports two basic data transmission schemes, Control Plane CIoT EPS Optimization (CP solution) and User Plane CIoT EPS Optimization (UP solution):

- In CP solution, small data packets are transmitted directly via NAS (Non-Access Stratum) messages over the signaling radio bearers (SRBs), without establishing a dedicated data radio bearer (DRB). The CP solution is well suited for intermittent small data transmissions that have short transmission duration but relatively long intervals between transmissions, with benefits of reducing connection setup delay and signaling overhead.

- UP solution uses a traditional user data path, where a DRB is established and data is transmitted over the user plane via the S1-U interface between eNB and S-GW. It supports larger data volumes and higher throughput compared to CP solution. UP solution is more suitable for applications requiring frequent or larger data transfers.

The general procedures for CP solution and UP solution schemes can be found in Annex B.1.

Legacy NB-IoT also supports Non-IP Data Delivery (NIDD) (see some references in Annex B.2). The architecture for CIoT capable of supporting Non-IP Data Delivery is cited as below:

(TS 23.401 V19.3.0) Figure L.2-1: Optimised EPS architecture option for CIoT - Non-roaming architecture

Generally, there is no differentiation between the different data types (i.e. IP, non-IP or SMS) in the AS layer. In other word, Encapsulating different types of data into NAS PDU or DRB makes no difference to the transmission processing at the AS layer or over the air interface. Meanwhile, the core network has the following two methods to deliver Non-IP data to the SCS/AS (Services Capability Server/Application Server):

- Transmission using the SCEF (Service Capability Exposure Function): The core network establishes a Non-IP PDN connection towards the selected SCEF. The APN used for SCEF-based transmission is a FQDN (Fully Qualified Domain Name) that resolves to the hostname of the SCEF or the IP address of the SCEF. This solution only supports integration with the CP solution.

- Transmission using a Point-to-Point (PtP) SGi tunnel: A point-to-point tunneling technique, typically implemented using UDP/IP encapsulation, is used. During PDN connection establishment, the P-GW determines, based on pre-configuration, the point-to-point tunneling technique to be used over SGi for transport between the P-GW and the Application Server (AS). The P-GW performs the IP related operations (e.g. allocates IP address for the PDN connection), but the IP address or IP prefix is not provided to the UE. For uplink Non-IP data, the P-GW forwards the received data to the AS over the SGi PtP tunnel using UDP/IP encapsulation. For downlink Non-IP data, the AS sends the data using UDP/IP encapsulation with the IP address of the UE and the 3GPP defined UDP port for "Non-IP" data. The P-GW decapsulates the received data (i.e. removes the UDP/IP headers) and forwards the data to S-GW on the GTP-U tunnel identified by the IP address of the UE (i.e. PDN connection) for delivery to the UE.

Observation 1a: NB-IoT is primarily designed for low-power, low-data-rate, and massive machine-type communication (mMTC) IoT applications. Legacy NB-IoT supports two basic data transmission schemes, e.g., CP solution and UP solution. Also Legacy NB-IoT has Non-IP data transmission capability.

Per our understanding, the Non-IP data delivery capability in legacy NB-IoT primarily serves ultra-lightweight industrial IoT UEs. By avoiding IP and UDP protocol overhead (saving at least 20 bytes IP protocol header and 8 bytes UDP protocol header) at least in the Uu interface transmission, it enables efficient end-to-end transmission of small raw binary/hex data packets, which may be only one or two bytes in size. This helps reduce power consumption on the UE side and improves transmission efficiency for some industrial applications, such as smart metering, environmental monitoring, and industrial alarm systems, especially in kind of private network environments where IP-based addressing may be not required. Some example high-layer protocols used in industrial scenarios that are compatible with Non-IP Data are LwM2M, MQTT etc. So we assume in legacy NB-IoT, Non-IP data delivery via the SCEF and also combined with CP solution may be the mainly used scheme for supporting some special industrial applications.

However, please note, in the SA2 TR for supporting IMS voice for NB-IoT NTN, it gives a general guideline that only architecture option with data via SGi will be considered in the study, if Non-IP data delivery scheme is used.

For services like IMS voice that inherently rely on the IP protocol stack, both the UE and the core network still need to support the IP protocol stack. Therefore, even it’s possible to apply Non-IP data delivery for IMS voice transmission, the IP protocol layer is only removed over the air interface and the IP protocol layer encapsulation must still be restored at the UE and the CN side (e.g., P-GW). So only the architecture option with data via SGi may be feasible.

Observation 1b: Per our understanding, Non-IP data delivery capability in legacy NB-IoT primarily serves ultra-lightweight industrial IoT devices. The architecture option with data delivery via the SCEF may be commonly used. However, for IMS voice support in NB-IoT, both the UE and the core network still need to support the IP protocol stack. So as guided by SA2 report, only architecture option with data via SGi can be considered where the IP protocol layer encapsulation is only removed over the air interface and must still be restored at the UE and the CN side (e.g., P-GW).

    - QoS capabilities for supporting voice service in NB-IoT NTN

According to the IMS-related specs, for IMS-based voice service, generally two service data flows (SDFs) need to be handled: one for IMS signaling and the other for IMS voice packets. They have different QoS and scheduling requirements, e.g., reliable delivery is important to SIP signaling and meanwhile, the IMS voice packet flow is a real-time flow with stringent delay requirements.

Observation 2a: In traditional IMS-based voice service, two service data flows (SDFs) need to be handled: one for IMS signaling and the other for IMS voice packets. They generally have different QoS and scheduling requirements, e.g., reliable delivery is important to SIP signaling and meanwhile, the IMS voice packet flow is a real-time flow with stringent delay requirements.

Generally, end-to-end bearers with QoS profile enable differentiated handling of services with different QoS requirements. According to the brief description of different bearer types in section Annex B.3, it can be roughly summarized as below for the QoS implementation when supporting IMS-based voice service:

- A PDN Connection is an end-to-end logical connection between the UE and an external PDN (Packet Data Network) identified by an APN (e.g., Internet or IMS), established via a specific PGW. The APN (Access Point Name) is a logical identifier used to determine the external packet data network (e.g., Internet, IMS, or a private enterprise network) the UE wants to access.

- To support IMS-based voice service, a PDN connection to the IMS APN needs to be established. Considering the UE may also need to access the internet, so another APN, e.g., APN=Internet, may be also expected to access. Each PDN Connection is associated with one APN and typically results in the UE being assigned an IP address (IPv4, IPv6, or both). Each PDN Connection has a PDN type and the PDN type typically is IP type. For Non-IP data delivery, generally a default APN for PDN Type Non-IP would be assumed. Shortly to say, there is one-to-one mapping between an APN and a PDN connection.

- The IP-CAN Bearer (realized as the EPS Bearer in 4G) provides the end-to-end logical connection with a defined QoS. There may be one or more IP-CAN Bearers/EPS Bearers within a PDN connection. The GBR/non-GBR classification is an attribute of the EPS Bearer, indicating whether bandwidth is guaranteed.

- SDFs are the actual application data flows that are mapped to EPS Bearers based on QoS and policy rules.

- The E-RAB is the radio access network implementation of the EPS Bearer. There generally is a one-to-one mapping between an EPS Bearer and an E-RAB, ensuring that the end-to-end QoS is enforced over the wireless interface.

However, according to TS 23.401, some QoS support capabilities (bearer related) which are commonly used in LTE/5G networks to support IMS voice service are not supported by legacy NB-IoT:

- Dedicated bearers are not supported over NB-IoT. The PDN GW uses the RAT Type to ensure that no dedicated bearers are active when the UE is accessing over NB-IoT. In the case of inter-RAT mobility from WB-EUTRA to NB-IoT, the UE and MME indicate local deactivation of non-default EPS bearers at TAU as specified in TS 24.301 [46].

- GBR bearers are not supported by NB-IoT. The PDN GW uses the RAT Type to ensure that GBR bearers are not active when the UE is using NB-IoT.

The above restrictions are also mentioned in several solutions in the SA2 study report [TR 23.700-19]. Per our understanding, to support dedicated bearer and GBR bearer are very basic features for supporting voice service with QoS guarantees. So in the follow discussion, we assume those restrictions on bearers should be removed to facilitate IMS voice service support over NB-IoT NTN.

Observation 2b: End-to-end bearers with QoS profile enable differentiated handling of services with different QoS requirements. However, some QoS support capabilities (e.g., dedicated bearer and GBR bearer) which are commonly used in LTE/5G networks to support IMS voice service are not supported by legacy NB-IoT. We understand to support dedicated bearer and GBR bearer are very basic features for supporting voice service with QoS guarantees. So it’s assumed those restrictions on bearers need to be firstly removed from SA2 specs.

Moreover, for end-to-end IMS service transmission, it generally needs a PDN connection to the “ims” APN, with PDN type IP. The PDN connection can contain two bearers for SIP signaling and voice data, respectively. However, some solutions in SA2 study suggest the use of PDN connection with PDN type Non-IP. With the following observation, we think such suggestion introduces unnecessary complexity and not needed.

Observation 2c: Considering that even if Non-IP data delivery scheme can be considered, only architecture option with data via SGi is possible where the IP protocol layer encapsulation anyway is involved, we understand for supporting IMS voice, UE and network only need to setup PDN connection with PDN type IP, e.g., no need of PDN connection with PDN type Non-IP.

    - Traditional protocol stack for SIP signaling and voice data packets

In LTE/5G, IMS voice service is traditionally carried over user-plane data transmission channels. SIP (Session Initiation Protocol) signaling are carried over IP networks using transport protocols such as UDP or TCP. Meanwhile, voice data packets are encapsulated with RTP, UDP (or TCP), and IP headers sequentially to ensure timing synchronization, lightweight transport-layer communication, reliable delivery and network-layer addressing and routing.

In the SA2 study, several solutions are proposed (see some citations in Annex B.6) with diverse protocol stack encapsulations. Among them, Solution#2 seems closest to the traditional implementation of IMS-based voice services. The protocol stack for IMS voice control signaling and voice data mentioned in Solution #2 are cited as below:

TR 23.700-19 V1.0.0, Solution#2: …………….The PDN connection is based on IP type, therefore, the protocol stack could be as follows: Figure 6.2.2-3: Protocol stack for IMS signalling Figure 6.2.2-4: Protocol stack for IMS voice data

The multi-protocol layer encapsulation of voice packets generally bring the following protocol header overhead:

- RTP Header: 12 bytes

- UDP Header: 8 bytes

- IP Header: 20 bytes for IPv4 or 40 bytes for IPv6

So the total overhead is about: 12 (RTP) + 8 (UDP) + 20 (IPv4) = 40 bytes or 12 (RTP) + 8 (UDP) + 40 (IPv6) = 60 bytes

As voice packets are typically small in payload size (e.g., 160 bytes for 20 ms of G.711 audio, or smaller from ultra low bit rate speech codecs), after encapsulation with protocol headers, the total packet size becomes significantly larger—often resulting in a high header-to-payload ratio.

Therefore, the ROHC protocol is also conventionally required to compress the headers of voice packets, reducing header-to-payload ratio, as well as the signaling load transmitted between the UE and the network.

Observation 3: In legacy LTE/5G, IMS voice service is traditionally carried over user-plane data transmission channels. SIP signaling are carried over IP networks using transport protocols such as UDP or TCP. Meanwhile, voice data packets are encapsulated with RTP, UDP (or TCP), and IP protocols sequentially. And the ROHC protocol is also conventionally required to compress the protocol headers overhead of voice data packets.

    - SIP signaling size

In section Annex B.4, a basic IMS procedure and its step descriptions are cited. And some functional explanations for the main SIP signaling messages are also mentioned. In TR 23.700-19, the SA2 study report also provides an IMS procedure with resource pre-reservation.

Furthermore, the size of the main SIP signaling messages and the corresponding transmission delay are summarized, cited in below:

TR 23.700-19 V1.0.0 A.2 Example message size and corresponding transmission delay for SIP messages in IMS …………………………. Table A.2-1 shows the example SIP signalling size and corresponding transmission delay: Table A.2-1: Example SIP message size and corresponding transmission delay in 1kbps Sequence; Message Name; Message Size (B) (proportional); Transmission Delay (s) in 1kbps. 1; INVITE; 2,564 (+52); 20.93. 2; 100 Trying; 445 (+52); 3.98. 3; 183 Progressing; 1,705 (+52); 14.06. 4; PRACK; 1,054 (+52); 8.85. 5; 200 OK (PRACK); 726 (+52); 6.22. 6; UPDATE; 1,929 (+52); 15.85. 7; 200 OK (UPDATE); 1,494 (+52); 12.37. 8; 180 Ringing; 983 (+52); 8.28. 9; 200 OK (INVITE); 1,365 (+52); 11.34. 10; ACK; 840 (+52); 7.14. Table A.2-2 shows the example EPS signalling size and corresponding transmission delay during the call setup: Table A.2-2: Example EPS message size and corresponding transmission delay in 1kbps Sequence; Message Name; Direction; Message Size (B) (proportional); Transmission Delay (s) in 1kbps. a.1; RRCConnectionReconfiguration; DL; 92 (+5); 0.78. a.2; RRCConnectionReconfigurationComplete; UL; 2 (+5); 0.06. ………………………

From the above citations, we have the following observation:

Observation 4: SIP signaling involved in IMS-based voice call are typically large, e.g., at least several hundred bytes and significantly larger than the air-interface signaling used for connection control.

It’s also heard that other working groups, such as SA2, are currently discussing solutions for SIP procedure optimization under the “Key Issue#2: IMS enhancement for GEO NB-IoT NTN access”. However, we understand the characteristics of the SIP protocol cause that even simplified signaling may remain relatively large.

    - Estimated voice data packet size

For IMS service, voice data packets also exhibit characteristics different from those of regular data packets for internet browsing, e.g., small packet size, regular and periodic transmission.

From RAN2 perspective, we think it’s needed to understand the potential size of voice data packets in NB-IoT NTN, as this may be related to header compression schemes and resources scheduling in the air interface. However, SA4 has not decided yet the potential new Ultra Low Bit rate codec scheme(s) which may be essential for supporting the voice service in NB-IoT over GSO, so we roughly estimate the possible packet size based on the target rate range mentioned in the WID description in below:

RP-252473 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4 ………………………………………… SA1 has studied the use case of IMS voice call using GSO satellite access, and the results are documented in TR 22.887. Normative service requirements and KPIs on IMS voice call using GSO satellite access is captured in TS 22.261, as shown in Table 1 below. Table 1: Performance requirements for satellite access Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL) (note 1); Area traffic capacity (UL) (note 1); Overall user density; Activity factor; UE speed; UE type; Others. IMS voice call using GEO (note 7); [1-3] kbit/s; [1-3] kbit/s; N/A; N/A; N/A; N/A; TBD; Handheld; Call set up time (note 8) ≤30 s (note 9). Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57];. SA has approved a new SA4-led SID on Ultra Low Bitrate Speech Codec in SP-250378. According to the agreed contribution in SA4 in S4-251123, the PHY bite rate is set to 1~5kbps and codec bite rate is set to 0.8~4.8kbps. SA also approved a new SA2-led SID on IMS voice call over GSO via NB-IoT-NTN in SP-250400. As WT#1 of the SID, both CP and UP based solutions are being studied and the SI is be concluded by TSG#110 (Dec. 2025), including the down-selection between CP and UP solutions. To enable end-to-end support of IMS voice call over NB-IoT-NTN via GSO, RAN specification work is required. This work item aims to specify the RAN related enhancements to support IMS voice call over NB-IoT-NTN via GSO.

The size of the voice data packet to be transmitted is generally influenced by the voice codec bit rate and the sampling interval length, or called frame duration.

In general, low codec bit rate can represent a segment of speech signal with less data, thereby reducing the bandwidth required for transmitting voice packets. This allows more concurrent calls to be supported within the same bandwidth, thus increasing system capacity. Additionally, low-rate voice signals can have stronger noise resistance, enabling longer coverage range. Therefore, low codec bit rate is suitable for transmission scenarios like NB-IoT over GSO.

For a certain codec bit rate, the frame duration has various impacts:

- Generally, a shorter frame duration will result in lower voice latency, making it more suitable for real-time interaction service (e.g., voice calls). But it’s noted that, when the frame duration is short, it leads to a higher frequency of encoding/decoding per unit time, increasing the computational complexity on the UE. A shorter frame duration also generates smaller packets, and more packets during a certain time period. After adding packet headers, e.g., the protocol headers from RTP/IP/UDP, the smaller packets will result in a higher protocol overhead ratio.

- In contrast, a longer frame duration can mitigates the issues in shorter frame duration, e.g., high computational complexity and high protocol overhead ratio. A longer frame duration can leverage more context information, achieving higher compression ratios. Moreover, a longer frame duration would result in more bits per packet, and further result in larger packet size and less packets during a certain time period. So fewer transmitted packets might be helpful for reducing congestion. However, under a longer frame duration, the impact of losing a single packet is relatively greater. Also a longer frame duration will increases end-to-end delay.

Based on above comparison, we understand a longer frame duration is more suitable to NB-IoT over GSO.

The frame duration for traditional voice service is typically 20ms. Currently it is observed that in SA4 discussion, higher frame duration are used, such as 80ms, 120ms, 160ms, 240 ms etc. We select some examples within this range for subsequent estimations.

As mentioned in the WID description, as target, the codec bite rate is set to 0.8~4.8kbps. Based on this information, we roughly estimate example voice packet sizes (based on several different codec bitrates and frame durations (as examples)) to be supported by NB-IoT over GSO as in the following table).

For the convenience of subsequent discussions, we also provide here the transmission duration of a certain voice data packet under certain assumptions on system transmission data rate. Considering that in GEO scenarios, regular Internet browsing services may reach transmission rates around 10kbps (and may reach tens of kbps if HARQ feedback is disabled), we do not choose the extremely conservative transmission data rate, e.g., 1kbps, but instead use 2kbps

Table 1.

| Codec bitrates | Frame Duration (Sampling Interval) | Number of voice frames per second | Size of a voice packet (=Codec bitrates * Frame Duration ) | Transmission duration of a voce packet with 2kbps transmission data rate |
| --- | --- | --- | --- | --- |
| 0.8 kbps | 80ms | 12.5 frames/s | 76 bits | 38ms |
|  | 160ms | 6.25 frames/s | 152 bits | 76ms |
| 2.5 kbps | 80ms | 12.5 frames/s | 200 bits | 100ms |
|  | 160ms | 6.25 frames/s | 400 bits | 200ms |
| 3.4 kbps | 80ms | 12.5 frames/s | 272 bits | 136ms |
|  | 160ms | 6.25 frames/s | 544 bits | 272ms |
| 4.8 kbps | 80ms | 12.5 frames/s | 384 bits | 192ms |
|  | 160ms | 6.25 frames/s | 768 bits | 384ms |

Based on the above estimation, we have the following observation:

Observation 5: For IMS voice service, the size of the voice data packet (voice frame) to be transmitted is primarily influenced by the voice codec bit rate and the frame duration. With the target codec bite rate set to 0.8~4.8kbps in WID description, the estimated example sizes of voice data packet in NB-IoT over GSO are given. It can be seen that the range of size of voice frame is about 10 bytes to 100 bytes, with the example frame duration of 80ms or 160ms.

  - Comparison between CP and UP solutions

Based on the above observations on the characteristics of IMS voice service and their QoS-guaranteed transmission, we compare in the following sections whether CP solution-based or UP solution-based schemes are more suitable for voice service support in NB-IoT NTN.

Moreover, in previous meeting, RAN2 has received the first LS from SA4 where two questions were asked to RAN2:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In the following sections, we give some comparisons on the functions, protocol header overhead etc., between CP solution-based scheme and UP-based scheme. Based on the discussion, we give draft RAN2 feedback to SA4 in the Annex A.1.

Furthermore, also based on the comparisons in the following sections, we give draft RAN2 feedback to another LS from SA2 in Annex A.2.

    - Brief classification on SA2 solutions

In SA2 study, till now, 11 solutions have been proposed for "Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC". From the RAN2 and AS layer perspective, we provide the following brief summary and classification of SA2 solutions, focusing on how they utilize radio bearers and which transmission solution (CP solution or UP solution) they are based on:

| Class 1 | UP: 2 DRBs 2 EPS bearers: - 1 Default EPS bearer - 1 Dedicated EPS bearer | Solution#2: IMS signalling is transmitted via the default EPS bearer while IMS voice data is transmitted via a dedicated EPS bearer Solution#3: IMS Signalling (e.g. SIP INVITE ... etc.) over default EPS bearer using IP while IMS voice traffic as non-IP (NIDD) traffic over dedicated EPS bearer.;…The PDN Type for this IMS PDN is IP Type. Solution#4: To establish separate Data Radio Bearers (DRBs) for signalling and voice media packets;…to use User Plane Non-IP Data Delivery (UP NIDD) for both call control signalling and voice media;…It is also recommended to configure separate APNs for signalling and media traffic. |
| --- | --- | --- |
| Class 2 | UP: 1 DRB 1 EPS bearer: - 1 Default EPS bearer | Solution#1: Default EPS bearer for both IMS signalling and voice data. |
| Class 3 | CP: 2 SRBs 2 EPS bearer: - 1 Default EPS bearer - 1 Dedicated EPS bearer | Solution#5: The voice packet flow requires a separate SRB from the SRB used for IMS and NAS signalling. Whether a separate SRB is needed for the IMS signalling flow depends on RAN…. This architecture includes a new SRB dedicated to transporting the voice packet flow (SRBx)…The SRBx dedicated to the transport of the voice SDF to eliminate both the RRC and the NAS layer overhead. The NAS layer overhead discussed in this clause does not include the NAS security overhead (only 1B MAC and NAS SN) Solution#6: The UE maps to NAS PDU carrying the IMS signalling to the SRBx. The UE maps to UL traffic of IMS voice packet to the SRBy…When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer. The MME encrypts and integrity protects Downlink IMS voice data. The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message. The eNB maps the DL traffic of IMS voice packet to the SRBy. |
| Class 4 | CP: 1 SRBs 1 EPS bearer: - 1 Default EPS bearer | Solution #7: The default EPS bearer is used for both IMS signalling and voice data transmission where an SRB is used for IMS signalling and can be switched to S1-U data transfer for voice data transmission |
| Class 5 | CP+UP: 1 SRB+1 DRB 2 EPS bearer: - 1 Default EPS bearer - 1 Dedicated EPS bearer | Solution #8: Combined CP/UP bearers in a PDN connection (IMS PDN session), with CP for default bearer and UP for dedicated bearer Solution #9: The UE use the Control Plan Service Request to send SIP INVITE message with indicating active flag when initiating voice call......Then the rest of IMS SIP signalling and voice are transferred via user plane default and dedicated bearers (GBR bearer) as existing procedure. Solution #10: Two PDN connections are used for IMS voice, where one without DRB is used for SIP message exchange and the other with DRB is used for voice media. These two PDN connections are linked together by using (newly introduced) PDN connectivity pair ID. Solution #11: This solution is intended to be used together with Solution #10. For UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets |

    - Functional weakness of CP solution for supporting voice service

Although traditional IMS voice services are primarily transmitted using user-plane bearers (UP solution-based schemes), some solutions proposed in SA2 suggest using control-plane bearers (CP solution-based schemes) for transmitting both SIP signaling and voice data or at least for transmitting SIP signaling.

In NB-IoT, three SRBs are supported: SRB0, SRB1, and SRB1bis, which are used to transmit RRC messages and NAS messages. Only SRB1/SRB1bis are used for transmitting higher layer service data which is encapsulated in NAS PDU:

- SRB0: Carries RRC messages over the CCCH, used for RRC connection establishment, RRC connection resume, or RRC connection re-establishment.

- SRB1: Carries RRC messages and NAS messages over the DCCH after access layer security activation.

- SRB1bis: Carries RRC messages and NAS messages over the DCCH before access layer security activation. SRB1bis is only used in NB-IoT systems and is not supported in LTE systems.

For UE using CP solution, only SRB1bis can be used for transmitting SIP signaling and voice data. Correspondingly, only one EPS bearer in CN can be used for this higher layer signaling and data delivery. That means only one end-to-end QoS flow can be provided, so the QoS differentiation for SIP signaling and voice data transmission cannot be achieved. In other word, if it’s expected to separately transmit SIP signaling and voice data on two different control-plane bearers, at least a new SRB (similar as SRBx mentioned in Solution#5) needs to be introduced for providing another end-to-end QoS flow.

Observation 6a: There is only one QoS flow for CP solution-based scheme in the current specification. So the QoS differentiation for SIP signaling and voice data transmission cannot be achieved. If it’s expected to separately transmit SIP signaling and voice data, at least a new SRB needs to be introduced for providing another end-to-end QoS flow.

SRB is the abbreviation for "Signalling Radio Bearers". SRBs are used to carry essential RRC and NAS signaling messages, so there are high requirement on the reliability and in-sequence delivery for the SRBs transmission. So the RLC configuration for both SRB1 and SRB1bis are RLC AM mode, as below:

| SRB | Parameter name | value |
| --- | --- | --- |
| SRB0(CCCH) | RLC configuration | TM |
|  | logicalChannelIdentity | 0 |
| SRB1 | RLC configuration | AM |
|  | logicalChannelIdentity | 1 |
| SRB1bis | RLC configuration | AM |
|  | logicalChannelIdentity | 3 |

As mentioned in section 1.1.2 QoS aspects for IMS voice, reliable delivery is important to SIP signaling and meanwhile, the IMS voice packet flow is a real-time flow with stringent delay requirements. So generally, it’s suitable to use RLC AM mode to transmit SIP signaling while to use RLC UM mode to transmit voice data packet. However, if only SRB1bis is used to transmit both SIP signaling and voice data, only RLC AM mode can be applied. Then the transmission efficiency of voice data packet would also be impacted. One solution is to introduce another new SRB with RLC UM mode, however, such a design would violate the principle of designing SRBs to ensure high reliability.

Observation 6b: In legacy NB-IoT, the RLC configuration for both SRB1 and SRB1bis are RLC AM mode as there are high requirement on the reliability and in-sequence delivery for the signaling transmission. In order to facilitate transmission of voice data packet via SRB with kind of low latency, one solution is to introduce another new SRB with RLC UM mode. However, such a design appears to violate the principle of designing SRBs, e.g., to ensure high reliability.

For NB-IoT UE using CP solution, the higher layer service data is encapsulated in NAS PDU and then encapsulated in RRC signaling for transmission. However, for non-EDT procedure, the RRC messages primarily used for encapsulating UL/DL NAS PDUs, e.g., DLInformationTransfer-NB and ULInformationTransfer-NB are also defined to use RLC-AM mode. Therefore, in addition to the need of introducing a new SRB operating in RLC-AM mode, it is also necessary to introduce RRC signaling messages operating in UM mode, for efficiently transmitting voice data packets encapsulated within NAS PDUs.

Observation 6c: For CP solution, the RRC messages mainly used for encapsulating UL/DL NAS PDUs, e.g., DLInformationTransfer-NB and ULInformationTransfer-NB are also defined to use RLC AM mode. So in order to support voice data transmission via SRB, new RRC message with RLC UM mode also needs to be introduced.

For supporting IMS voice service, RRC reconfiguration procedure is generally necessary, for example, it’s needed for the case that voice service preempts the on-going data transmission. However, RRC reconfiguration is NOT supported for CP solution in current spec. so if to support transmitting IMS voice service via SRB, RRC reconfiguration procedure needs to be retook for CP solution.

Observation 6d: RRC reconfiguration is NOT supported for CP solution in current spec, but necessary for the case that voice preempt the on-going data transmission. So if to support transmitting IMS voice service via SRB, RRC reconfiguration should be retook for CP solution.

As indicated in NAS layer spec, if CP solution is supported for PDN connections of IP PDN type, ROHC for IP header compression may need to be performed between the UE and the MME. It reuses the ROHC profiles defined in 3GPP TS 36.323 (PDCP spec) which are mainly used by UP solution. Moreover, the ROHC profiles need to be included in the Header compression configuration in the related EPS session management messages.

Although the ROHC profile for IP/UDP/RTR header compression has already been supported in ROHC configuration in PDCP spec which are applicable to both UP solution and CP solution, as legacy NB-IoT does not support services using RTP protocol, for CP solution, the IP/UDP/RTR profile has not yet been added to the Header compression configuration in the NAS layer spec. Similarly, for the UP solution, the IP/UDP/RTR profile has not been added to the PDCP dedicated configuration either.

Observation 6e: For CP solution, RoHC compression can be performed between the UE and the MME. It reuses the ROHC profiles defined in TS 36.323 (PDCP spec) which are mainly used by UP solution. Moreover, the ROHC profiles need to be included in the Header compression configuration in the related EPS session management messages. However, as legacy NB-IoT does not support services using RTP protocol, for CP solution, the IP/UDP/RTP profile has not yet been added to the Header compression configuration in the NAS layer spec. This needs to be enhanced if CP solution is allowed to transmit IMS voice service. Similar issue also exists for UP solution, e.g., the IP/UDP/RTR profile needs to be added to the PDCP dedicated configuration.

The above functional weakness of CP solution for supporting voice service are summarized in below Table, as comparison, for each issue, the corresponding schemes in UP solution are also mentioned. According to the comparison between the CP and UP solutions, it can be seen that the CP solution has difficulty supporting the required QoS guarantee for IMS voice service. In other words, it would require significant standard modifications if CP solution is used to deliver voice service in RAN.

|  | ## CP solution | ## UP solution |
| --- | --- | --- |
| QoS parameters | There is only one QoS flow for CP solution in the current specification. At least another QoS flow should be introduced for CP solution to support voice. | Multiple QoS flows can already be supported for UP in the current specification. No change is necessary. |
| RB | There is only one SRB with RLC AM mode in NB-IoT, it cannot provide QoS differentiation between SIP signalling and voice data transmission. New SRB with RLC UM mode should be supported. | Two DRBs with configurable RLC mode can already be supported for UP. No change is necessary. |
| RRC signalling | Only one RLC AM mode DL RRC signalling(DLInformationTransfer-NB) and one RLC AM mode UL RRC signalling(ULInformationTransfer-NB) can be used for data transmission in CP. New RRC DL/UL signalling with RLC UM mode should be introduced for CP solution. | User plane data encapsulation does not go through the RRC layer, so this issue does not exist for the UP solution. |
| RRC reconfiguration | Not supported, but necessary for the case that voice preempt the on-going data transmission. RRC reconfiguration should be retook for CP. | RRC reconfiguration is already supported for UP solution. No change is necessary. |
| RoHC | IP/UDP/RTP RoHC profile (e.g., profile 0x0001 and 0x0101) should be introduced for Header compression configuration for CP solution by SA2. | IP/UDP/RTP RoHC profile (e.g., profile 0x0001 and 0x0101) should be introduced for PDCP dedicated configuration for UP solution by RAN2. |

Observation 7: It can be seen that solutions using control-plane bearers have certain weaknesses that make them unsuitable for voice transmission. The main reason is, CP solution-based scheme has very simple RB design and cannot provide sufficient QoS guarantee required by IMS voice service.

    - Higher protocol header overhead of CP solution compared to UP solution

The first question in SA4 LS is whether a packet overhead of 1 byte of MAC header is realistic. According to the TS 36.321 spec, the two types of minimal MAC subheader are defined as follows:

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader with 7-bits and 15-bits L field Figure 6.1.2-2: R/F2/E/LCID MAC subheader

Considering that “L” field is necessary for indicating the length of the following MAC SDU that encapsulates the voice packet, and the “R” field has already be used for QCDR (Downlink Channel Quality Report) function as follows, it is impossible to define a 1 byte MAC subheader for voice packet. So the 2 bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

- R: Except when this field is used for short DCQR, reserved bit, set to "0". For short DCQR, the mapping of R field to short DCQR value is described in table 6.2.1-5.

Moreover, there has already been extensive discussion in SA2 regarding the variation in packet size during voice transmission, so we cannot assume that voice packets are of fixed length, hence exclude the need for a length indicator. Even if the packet size remains constant during stable state of RoHC protocol, it is impractical from a scheduling perspective to perfectly align the TBS exactly with the upper-layer service packets. Therefore, a length indicator for the MAC layer SDU is still really necessary.

Observation 8a: The “R” field in MAC subheader has already be used for DCQR function and the “L” field is necessary for indicating the length of voice MAC SDU. Furthermore, in most cases, it is impractical from a scheduling perspective to perfectly align the TBS exactly with the upper-layer service packets, so a length indicator for the MAC layer SDU is still necessary. Therefore, to define only 1 byte MAC subheader for voice packet is not realistic. 2 bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

Another questions is about evaluation and comparison on protocol overhead for potential solution combinations.

Based on the current specifications, we list the minimum protocol header size that will be required for the different options listed in SA4 LS and transmitted over Uu as follows:

Table 2. Minimum header size of different protocol layers

| Transmission solutions Minimum header size for each layer Protocol layers |  | ## CP(CIoT EPS Control Plane optimization) | ## UP(CIoT EPS User Plane optimization) |
| --- | --- | --- | --- |
| AS layer | ## RRC | ## UL: 1 bytes (total 8 bits for UL DCCH MessageType and others, aligned to 1B) ## DL: 2 bytes (total 10bits for DL DCCH MessageType and others, aligned to 2B) ## Taking 1 bytes as minimum value. | ## 0 byte |
|  | ## PDCP | ## 0 byte | ## 1 byte |
|  | ## RLC | ## UM: 1 byte ## AM: 2 bytes | ## UM: 1 byte ## AM: 2 bytes |
|  | ## MAC | ## 2 bytes (see analysis in section 2.1) | ## 2 bytes |
| NAS layer |  | ## 7 bytes (with NAS security) ## (note: CT1 still work in R19 to reduce NAS layer overhead) | ## 0 byte |
| Higher layers for Non-IP (NIDD) based option | ## IP | ## 0 byte | ## 0 byte |
|  | ## UDP | ## 0 byte | ## 0 byte |
|  | ## RTP | ## 12 bytes | ## 12 bytes |
| Higher layers for IP based option without RoHC | ## IP | ## 20 bytes | ## 20 bytes |
|  | ## UDP | ## 8 bytes | ## 8 bytes |
|  | ## RTP | ## 12 bytes | ## 12 bytes |
| Higher layers for IP based option with RoHC |  | ## 1 bytes (e.g., UO-0 format) or 2 bytes (e.g., UO-1-TS format) | ## 1 bytes (e.g., UO-0 format) or ## 2 bytes (e.g., UO-1-TS format) |

Please note:

- For MAC layer protocol overhead, see above Observation 8a;

- For RRC layer protocol overhead, it mainly comes from the message type definition for DL-DCCH-Message-NB and UL-DCCH-Message-NB messages respectively and the presence indication of optional fields. The final resulting number of bits also needs to be Byte-aligned.

- For overhead after RoHC compression, we assume U-mode is used. Some details for RoHC can be found in section 1.2.5.

For several important scheme combinations, the final length of protocol header for voice data are listed in below table:

| Transmission solutions Minimum header size for voice data Higher layers protocols | ## CP solution | ## UP solution |
| --- | --- | --- |
| ## IP+RoHC | ## 12B (4B from AS layer+7B from NAS layer + 1B from NAS RoHC for RTP/UDP/IP) | ## 5B (4B from AS layer+1B from PDCP RoHC of RTP/UDP/IP) |
| ## Non-IP | ## 23B (4B from AS layer+7B from NAS layer +12B from RTP) | ## 16B (4B from AS layer+12B from RTP) |

Based on above comparison tables, we have the following observation:

Observation 8b: Based on the current specifications and under the same other conditions:

- The minimum protocol header for the UP solution-based scheme is 5 bytes, which is also the smallest protocol header among all solution combinations.

- The header size of CP solution is at least 7bytes (7B come from NAS) more than that of UP solution;

- The header size of Non-IP based scheme is at least 10bytes (12B RTP - 2B RoHC) or even 11bytes (12B RTP - 1B RoHC) more than that of the IP based scheme with RoHC.

    - Infeasibility of transmitting SIP signaling over SRB

For some solutions in SA2 TR 23.700-19, it is suggested to use or jointly use CP solution for supporting IMS voice service. A further suggestion is to transmit SIP signaling over SRBs, e.g., SRB1bis. However, considering that SRB1bis is already used to carry control plane signaling for the UEs (e.g., Msg4 and subsequent messages, as well as NAS messages), also taking into account the above Observation 4, e.g., SIP signaling involved in IMS-based voice call are typically large, using SRB1bis for transmitting a certain UE's SIP signaling may significantly impact the transmission of overall control plane messages and increase connection establishment latency for the UE.

In other word, the large SIP signaling is more suitable to be transmitted as service data, e.g., via DRB.

Observation 9: SIP signaling involved in IMS-based voice procedure are typically large, so to transmit SIP signaling over SRB, e.g., SRB1bis, for a certain UE, may significantly impact the transmission of overall control plane messages and increase connection establishment latency for this UE.

    - Feasibility of ROHC for UP solution for supporting voice service

The ROHC protocol has already been supported for NB-IoT. For UP solution, ROHC is typically implemented between the UE and the eNB as part of the PDCP layer, where header compression is performed before data is transmitted over the radio interface. For CP solution, ROHC is typically implemented between the UE and the MME as part of the NAS layer. ROHC is a key enabling technology for NB-IoT, making IP-based communication feasible and efficient for massive-scale, low-power IoT deployments by minimizing protocol overhead and optimizing radio resource utilization.

The basic ROHC operation modes (U-mode, O-mode and R-mode) and packet format used for each mode are summarized in Annex B.5. Among them, R-mode is most complicated, has the strictest requirements and less used, while the U-mode is well suited for simplified systems such as NB-IoT.

In SA2 discussions, some companies argued that the UP solution-based scheme requires the use of the RoHC protocol, which in NB-IoT over GEO scenarios is prone to unstable operation (e.g., it might be easy to back to initial IR state after consecutive packets loss) [see company’s contribution S2-2506371 and also a previous one S2-2504678]. Therefore, they prefer the combination of CP solution-based scheme with Non-IP data delivery, where the original header of the voice packet only includes a (specialized) RTP protocol header, eliminating the need for ROHC compression while keeping header overhead within an acceptable range.

However, we firstly understand that the main issue in NB-IoT over GSO is the large RTT, but not the packet loss, based on the reasonable assumption that there is stable signal conditions in an open environment, along with sufficient repetitions are used to compensate for the link budget. Therefore, we assume it is unlikely that multiple consecutive packet losses would occur to cause ROHC to easily back to its initial state.

Observation 10a: We think packet loss is not easily encountered in NB-IoT over GEO scenario, with reasonable assumption that there is stable signal conditions in an open environment, along with sufficient repetitions are used to compensate for the link budget. So it is unlikely for ROHC to back to its initial state due to consecutive packet losses.

Secondly, in the following, we provide some analysis to illustrate that even if occasional packet loss occurs, ROHC reversion will rarely happen:

- Packet discard issue in IR state: referred to the observation in section 1.1.5, with 2kbps transmission rate for voice in NB-IoT over GEO, transmission duration for a full header packet with total about 500bits size (e.g., 200bits voice data+40bytes RTP/UDP/IP header) is about 250ms. During this period, if Frame Duration is not so small, only 1 or 2 full header packet may be generated and possibly be discard. This is small issue and easy to be addressed.

- Packet loss issue in FO/SO states: RoHC for NB-IoT over GEO can mainly work in U-mode which has numerous design features—periodic refresh, W-LSB, CRC checks and error detection etc.—collectively form an extremely robust fault-tolerant system. These mechanisms enable ROHC to maximize time spent in the efficient FO or SO states in field deployments. Shortly to say, with the "repair mechanisms" or "mitigation techniques" of U-mode or O-mode, compression state fall back will seldom happened. The following is a brief description of some mechanisms to enhance robustness of ROHC.

  - Periodic Refresh: In U modes, the compressor periodically sends IR or IR-DYN headers. This actively "refreshes" the context in the absence of feedback, preventing context invalidation due to long idle periods or accumulated errors.

  - CRC Checks and Error Detection: In U modes, all compressed headers include a CRC (3-bit, 7-bit, or 8-bit). This allows the decompressor to detect the vast majority of header damage caused by channel bit errors. Damaged packets can be discarded without corrupting the context (preventing "damage propagation"). This protects context integrity and reduces invalid state transitions caused by erroneous packets.

Observation 10b: The "repair mechanisms" of ROHC, such as periodic refresh, CRC checks and error detection, would enable ROHC to perform "self-repair" when packet loss or bit errors occur, and maximize time spent in the efficient FO or SO states. This also makes the ROHC mechanism to be applicable in challenging environments like NB-IoT over satellite (high latency, certain error rate).

Last, there is concern that, Due to ROHC state transitions or SID frames during voice call, the SPS scheduling method or dynamic scheduling based on maximum TBS, commonly used for voice data packets, may suffer from over- or under-scheduling issues. We give the following observation for this concern:

Observation 10c: Seldom RoHC state transition enables the transmission of packets with compressed header and of consistent size for most of the time, which is beneficial for efficient resource scheduling. Moreover, a bit longer Frame Duration can also be helpful to reduce the number of SID frames, resulting in more consistent voice data packet.

According to above observations 10a~10c, we ensure the feasibility of ROHC scheme and think UP+IP+RoHC is the most suitable option for supporting IMS voice in NB-IoT over GSO.

    - Concerns for voice support over Non-IP data delivery scheme

Unlike supporting normal data service in legacy NB-IoT, where Non-IP data delivery is mainly end-to-end and used in private network environments to carry very simple industrial application data, supporting IMS voice requires enabling service routing across wide coverage areas. So if without UDP/IP protocol header encapsulation, it not only would cause incompatibility with existing network nodes, such as firewalls, NAT (Network Address Translation), load balancers, and QoS systems as they inherently rely on UDP/TCP/IP protocols at the network layer, but also cause some issues as application conflicts caused by lack of port mechanism.

Therefore, again, if it supports to transmit voice data via Non-IP data delivery scheme, UDP/IP protocol headers cannot be removed end-to-end and can only be omitted over the air interface. These protocol headers still need to be reconstructed at the core network and the UE, for end-to-end data delivery. This introduces additional complexity.

Observation 11a: For supporting wide range voice service in NB-IoT over GSO, UDP/IP protocol cannot be end-to-end removal as it may cause incompatibility with existing network nodes. To reconstruct UDP/IP protocol header at the CN and the UE would introduce additional complexity.

Regarding RTP protocol header, most SA2 solutions agree that it is still required end-to-end inclusion, but some solutions suggest applying a simplified version of the RTP protocol (e.g., only include timestamp information). However, based on our understanding, anyway RTP Fixed Header Fields may need to be included. RTP header extension (such RTP header extension may be able to be defined in our 3GPP scope, e.g., in SA4) can only be done by making use of the extension (X) bit in the existing RTP Fixed Header Fields. So we are highly doubt whether it is really possible to develop a simplified/reduced RTP protocol that can achieve the purpose of reducing RTP protocol header overhead, e.g., from 12 bytes to 1 or 2 bytes.

Moreover, a new simplified/reduced RTP protocol header may also cause inability to leverage standard security mechanisms like DTLS as it depends on the UDP/IP structure.

Finally, current RTP protocol can enable multipoint communication and mixing (e.g., in an audio conference bridge). So to apply a simplified/reduced RTP protocol header, e.g., without such information as SSRC, the receiver cannot separate or manage multiple audio sources in a conference or mixed scenario. This will block the future expansion of voice application in NB-IoT over GSO.

Observation 11b: It’s unclear how to develop a simplified/reduced RTP protocol (e.g., only include timestamp information) as it can only do RTP protocol extension by making use of the extension (X) bit in the existing RTP Fixed Header Fields. Moreover, to apply simplified/reduced RTP protocol may cause inability to leverage standard security mechanisms like DTLS, and also cause inability to future functional expansion of voice application in NB-IoT over GSO, e.g., to support multipoint communication in the future.

Based on all the above observations from comparison between CP solution-based scheme and UP solution-based scheme, we give the following proposal:

Proposal 1: It’s suggest to adopt UP solution-based scheme, e.g., UP+IP+ROHC, to support IMS voice service for NB-IoT over GSO.

  - Enhancements for UP solution for supporting voice service

If the UP solution is finally selected to support IMS voice service for NB-IoT over GSO, there are still some minor enhancements that need to be considered.

    - To add RTP related ROHC profiles into PDCP configuration

For NB-IoT, if IP header compression for UE solution is supported, the ROHC profiles defined in 3GPP PDCP protocol, e.g., TS 36.323, need to be supported.

As shown below, ROHC has already been supported for UP solution in NB-IOT but we observe that some profiles which involve RTP protocol and applicable to voice service, e.g. 0x0001 and 0x0101, are missing in the ROHC configuration in PDCP configuration.

TS 36.323 Table 5.5.1.1: Supported ROHC protocols and profiles Profile Identifier; Usage:; Reference. 0x0000; No compression; RFC 5795. 0x0001; RTP/UDP/IP; RFC 3095, RFC 4815. 0x0002; UDP/IP; RFC 3095, RFC 4815. 0x0003; ESP/IP; RFC 3095, RFC 4815. 0x0004; IP; RFC 3843, RFC 4815. 0x0006; TCP/IP; RFC 6846. 0x0101; RTP/UDP/IP; RFC 5225. 0x0102; UDP/IP; RFC 5225. 0x0103; ESP/IP; RFC 5225. 0x0104; IP; RFC 5225. TS 36.331 The IE PDCP-Config-NB is used to set the configurable PDCP parameters for data radio bearers. PDCP-Config-NB information element -- ASN1START PDCP-Config-NB-r13 ::= SEQUENCE { discardTimer-r13 ENUMERATED { ms5120, ms10240, ms20480, ms40960, ms81920, infinity, spare2, spare1 } OPTIONAL, -- Cond Setup headerCompression-r13 CHOICE { notUsed NULL, rohc SEQUENCE { maxCID-r13 INTEGER (1..16383) DEFAULT 15, profiles-r13 SEQUENCE { profile0x0002 BOOLEAN, profile0x0003 BOOLEAN, profile0x0004 BOOLEAN, profile0x0006 BOOLEAN, profile0x0102 BOOLEAN, profile0x0103 BOOLEAN, profile0x0104 BOOLEAN }, ... } }, ..., [[ cipheringDisabled-r16 ENUMERATED {true} OPTIONAL -- Cond ConnectedTo5GC ]] } -- ASN1STOP PDCP-Config-NB field descriptions. ……………….. profiles The profiles used by both compressor and decompressor in both UE and E-UTRAN. The field indicates which of the ROHC profiles specified in TS 36.323 [8] are supported, i.e. value true indicates that the profile is supported. Profile 0x0000 shall always be supported when the use of ROHC is configured. If support of two ROHC profile identifiers with the same 8 LSB's is signalled, only the profile corresponding to the highest value shall be applied.. ----------------

According to above observation, the ROHC profiles 0x0001 and 0x0101 need to be included in the “headerCompression-> PDCP-Config-NB”.

- Proposal 2a: Support ROHC for voice over NB-IoT NTN and include RTP related ROHC profiles for PDCP configuration for UP solution.

    - New RRC establishment cause

Based on the following comparison between NB-IoT and LTE, it can be seen that the establishment cause for voice and emergency call is not supported in legacy NB-IOT and need to be addressed if we decide to support voice and emergency call.

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| EstablishmentCause-NB-r13 ::= ENUMERATED {mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} | EstablishmentCause ::= ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

- Proposal 2b: Introduce establishment cause for voice call in EstablishmentCause-NB-r13.

    - More than two DRBs

- In SA2 study, there is a proposal to support more than 2 DRBs for using UP solution to support voice over GSO. The main motivation is to allow concurrent normal data service and IMS voice service for NB-IoT.

- Per our understanding, the impact on RAN2 specs of adding a new DRB would not be large. The new DRB should have the same characteristics and usage as existing DRBs. However, RAN2 first needs to elaborate whether it is necessary to support such service of concurrent data and IMS voice in a simplified systems like NB-IoT.

- Proposal 2c: If requirement for supporting concurrent normal data service and IMS voice service over NB-IoT NTN can be confirmed, RAN2 can consider to support more than 2 DRBs for UP solution-based scheme.

- Conclusion

Based on the above discussion, the following observations are given for several aspects:

Data transmission schemes in legacy NB-IoT

Observation 1a: NB-IoT is primarily designed for low-power, low-data-rate, and massive machine-type communication (mMTC) IoT applications. Legacy NB-IoT supports two basic data transmission schemes, e.g., CP solution and UP solution. Also Legacy NB-IoT has Non-IP data transmission capability.

Observation 1b: Per our understanding, Non-IP data delivery capability in legacy NB-IoT primarily serves ultra-lightweight industrial IoT devices. The architecture option with data delivery via the SCEF may be commonly used. However, for IMS voice support in NB-IoT, both the UE and the core network still need to support the IP protocol stack. So as guided by SA2 report, only architecture option with data via SGi can be considered where the IP protocol layer encapsulation is only removed over the air interface and must still be restored at the UE and the CN side (e.g., P-GW).

QoS capabilities for supporting voice service in NB-IoT NTN

Observation 2a: In traditional IMS-based voice service, two service data flows (SDFs) need to be handled: one for IMS signaling and the other for IMS voice packets. They generally have different QoS and scheduling requirements, e.g., reliable delivery is important to SIP signaling and meanwhile, the IMS voice packet flow is a real-time flow with stringent delay requirements.

Observation 2b: End-to-end bearers with QoS profile enable differentiated handling of services with different QoS requirements. However, some QoS support capabilities (e.g., dedicated bearer and GBR bearer) which are commonly used in LTE/5G networks to support IMS voice service are not supported by legacy NB-IoT. We understand to support dedicated bearer and GBR bearer are very basic features for supporting voice service with QoS guarantees. So it’s assumed those restrictions on bearers need to be firstly removed from SA2 specs.

Observation 2c: Considering that even if Non-IP data delivery scheme can be considered, only architecture option with data via SGi is possible where the IP protocol layer encapsulation anyway is involved, we understand for supporting IMS voice, UE and network only need to setup PDN connection with PDN type IP, e.g., no need of PDN connection with PDN type Non-IP.

Traditional protocol stack for SIP signaling and voice data packets

Observation 3: In legacy LTE/5G, IMS voice service is traditionally carried over user-plane data transmission channels. SIP signaling are carried over IP networks using transport protocols such as UDP or TCP. Meanwhile, voice data packets are encapsulated with RTP, UDP (or TCP), and IP protocols sequentially. And the ROHC protocol is also conventionally required to compress the protocol headers overhead of voice data packets.

SIP signaling size

Observation 4: SIP signaling involved in IMS-based voice call are typically large, e.g., at least several hundred bytes and significantly larger than the air-interface signaling used for connection control.

Estimated voice data packet size

Observation 5: For IMS voice service, the size of the voice data packet (voice frame) to be transmitted is primarily influenced by the voice codec bit rate and the frame duration. With the target codec bite rate set to 0.8~4.8kbps in WID description, the estimated example sizes of voice data packet in NB-IoT over GSO are given. It can be seen that the range of size of voice frame is about 10 bytes to 100 bytes, with the example frame duration of 80ms or 160ms.

Functional weakness of CP solution for supporting voice service

Observation 6a: There is only one QoS flow for CP solution-based scheme in the current specification. So the QoS differentiation for SIP signaling and voice data transmission cannot be achieved. If it’s expected to separately transmit SIP signaling and voice data, at least a new SRB needs to be introduced for providing another end-to-end QoS flow.

Observation 6b: In legacy NB-IoT, the RLC configuration for both SRB1 and SRB1bis are RLC AM mode as there are high requirement on the reliability and in-sequence delivery for the signaling transmission. In order to facilitate transmission of voice data packet via SRB with kind of low latency, one solution is to introduce another new SRB with RLC UM mode. However, such a design appears to violate the principle of designing SRBs, e.g., to ensure high reliability.

Observation 6c: For CP solution, the RRC messages mainly used for encapsulating UL/DL NAS PDUs, e.g., DLInformationTransfer-NB and ULInformationTransfer-NB are also defined to use RLC AM mode. So in order to support voice data transmission via SRB, new RRC message with RLC UM mode also needs to be introduced.

Observation 6d: RRC reconfiguration is NOT supported for CP solution in current spec, but necessary for the case that voice preempt the on-going data transmission. So if to support transmitting IMS voice service via SRB, RRC reconfiguration should be retook for CP solution.

Observation 6e: For CP solution, RoHC compression can be performed between the UE and the MME. It reuses the ROHC profiles defined in TS 36.323 (PDCP spec) which are mainly used by UP solution. Moreover, the ROHC profiles need to be included in the Header compression configuration in the related EPS session management messages. However, as legacy NB-IoT does not support services using RTP protocol, for CP solution, the IP/UDP/RTP profile has not yet been added to the Header compression configuration in the NAS layer spec. This needs to be enhanced if CP solution is allowed to transmit IMS voice service. Similar issue also exists for UP solution, e.g., the IP/UDP/RTR profile needs to be added to the PDCP dedicated configuration.

Observation 7: It can be seen that solutions using control-plane bearers have certain weaknesses that make them unsuitable for voice transmission. The main reason is, CP solution-based scheme has very simple RB design and cannot provide sufficient QoS guarantee required by IMS voice service.

Higher protocol header overhead of CP solution compared to UP solution

Observation 8a: The “R” field in MAC subheader has already be used for DCQR function and the “L” field is necessary for indicating the length of voice MAC SDU. Furthermore, in most cases, it is impractical from a scheduling perspective to perfectly align the TBS exactly with the upper-layer service packets, so a length indicator for the MAC layer SDU is still necessary. Therefore, to define only 1 byte MAC subheader for voice packet is not realistic. 2 bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

Observation 8b: Based on the current specifications and under the same other conditions:

- The minimum protocol header for the UP solution-based scheme is 5 bytes, which is also the smallest protocol header among all solution combinations

- The header size of CP solution is at least 7bytes (7B come from NAS) more than that of UP solution;

- The header size of Non-IP based scheme is at least 10bytes (12B RTP - 2B RoHC) or even 11bytes (12B RTP - 1B RoHC) more than that of the IP based scheme with RoHC.

Infeasibility of transmitting SIP signaling over SRB

Observation 9: SIP signaling involved in IMS-based voice procedure are typically large, so to transmit SIP signaling over SRB, e.g., SRB1bis, for a certain UE, may significantly impact the transmission of overall control plane messages and increase connection establishment latency for this UE.

Feasibility of ROHC for UP solution for supporting voice service

Observation 10a: We think packet loss is not easily encountered in NB-IoT over GEO scenario, with reasonable assumption that there is stable signal conditions in an open environment, along with sufficient repetitions are used to compensate for the link budget. So it is unlikely for ROHC to back to its initial state due to consecutive packet losses.

Observation 10b: The "repair mechanisms" of ROHC, such as periodic refresh, CRC checks and error detection, would enable ROHC to perform "self-repair" when packet loss or bit errors occur, and maximize time spent in the efficient FO or SO states. This also makes the ROHC mechanism to be applicable in challenging environments like NB-IoT over satellite (high latency, certain error rate).

Observation 10c: Seldom RoHC state transition enables the transmission of packets with compressed header and of consistent size for most of the time, which is beneficial for efficient resource scheduling. Moreover, a bit longer Frame Duration can also be helpful to reduce the number of SID frames, resulting in more consistent voice data packet.

Concerns for voice support over Non-IP scheme

Observation 11a: For supporting wide range voice service in NB-IoT over GSO, UDP/IP protocol cannot be end-to-end removal as it may cause incompatibility with existing network nodes. To reconstruct UDP/IP protocol header at the CN and the UE would introduce additional complexity.

Observation 11b: It’s unclear how to develop a simplified/reduced RTP protocol (e.g., only include timestamp information) as it can only do RTP protocol extension by making use of the extension (X) bit in the existing RTP Fixed Header Fields. Moreover, to apply simplified/reduced RTP protocol may cause inability to leverage standard security mechanisms like DTLS, and also cause inability to future functional expansion of voice application in NB-IoT over GSO, e.g., to support multipoint communication in the future.

Based on the above observations, the following proposals are given:

Down-selection between CP and UP solutions

Proposal 1: It’s suggest to adopt UP solution-based scheme, e.g., UP+IP+ROHC, to support IMS voice service for NB-IoT over GSO.

Enhancements for UP solution for supporting voice service

Proposal 2a: Support ROHC for voice over NB-IoT NTN and include RTP related ROHC profiles for PDCP configuration for UP solution.

Proposal 2b: Introduce establishment cause for voice call in EstablishmentCause-NB-r13.

Proposal 2c: If requirement for supporting concurrent normal data service and IMS voice service over NB-IoT NTN can be confirmed, RAN2 can consider to support more than 2 DRBs for UP solution-based scheme.

- Reference

[1] RP-251867, New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo (moderator), RAN#108, (2025-06)

[2] RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4, vivo (rapporteur), RAN#109, (2025-09)

[3] R2-2505068_S4-251584, LS on the RAN simulation assumptions for ULBC, RAN2#131, August, 2025

[4] S2-2507578, Reply LS on the RAN simulation assumptions for ULBC, SA WG2 Meeting #170, August 2025

[5] S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, SA WG2 Meeting #170, August 2025

[6] TR 23.700-19 V1.0.0, Technical Specification Group Services and System Aspects; Study on Integration of satellite components in the 5G architecture; Phase 4 (Release 20), (2025-09)[2] TS 36.323 V18.0.0 (PDCP), Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification (Release 18), (2024-03)

[7] TS 23.401 V19.3.0 (System Architecture for EPS; Stage 2), Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access (Release 19), (2025-06)

[8] TS 24.301 V19.3.0 (NAS), Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3 (Release 19), (2025-06)

[9] TS 23.228 V19.3.0 (IMS Stage 2), Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2 (Release 19), (2025-06)

[10] TS 24.229 V19.3.0 (IMS Stage 3), Technical Specification Group Core Network and Terminals; IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3 (Release 19), (2025-06)

[11] TS 23.203 (PCC) V19.2.0, Technical Specification Group Services and System Aspects; Policy and charging control architecture (Release 19), (2025-06)

[12] TS 23.207 (End-to-End QoS) V18.0.0, Technical Specification Group Services and System Aspects; End-to-end Quality of Service (QoS) concept and architecture (Release 18), (2024-03)

[13] IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[14] IETF RFC 4571: "Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport".

[15] TS 26.522 (RTP configurations, from SA4) V19.1.0, Technical Specification Group Services and System Aspects; 5G Real-time Media Transport Protocol Configurations (Release 19), (2025-06)

[16] S4-251388 [FS_ULBC] On ULBC bit rates (Dolby Laboratories, Inc., Nokia, Novamint, EDF)

[17] TR 26.940 V0.2.0, Study on Ultra Low Bit rate Speech Codecs (Release 20), (2025-05)

Annex A: draft RAN2 response LSs to LSin from SA4/SA2

Annex A.1: Draft LS to R2-2505068_S4-251584

The following is draft reply LS to the SA4 LS (R2-2505068_S4-251584) on the RAN simulation assumptions for ULBC.

# 1 Overall description RAN2 thanks SA4 for the LS on the RAN simulation assumptions for ULBC, RAN2 can provide the following answers to the questions raised for RAN2/SA2. ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic. Answer: RAN2 discuss the related issue and made the following agreements: - A packet overhead of 1 byte of MAC subheader for voice transmission is not realistic. 2 bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission. - UP solution is more suitable to be used for supporting voice than CP solution because of its less protocol header size and less specification impacts. - IP solution with RoHC is more suitable to be used for supporting voice than Non-IP solution because of its less protocol header size, unless new header compression mechanism is introduced, which may need much evaluation and work in other working groups. The following table on minimum header size of different protocol layers is provided for reference: Transmission solutions Minimum header size for each layer ## Protocol layers; ## CP solution; ## UP solution. AS layer; ## RRC; ## UL: 1 bytes (total 8 bits for UL DCCH MessageType and others, aligned to 1B) ## DL: 2 bytes (total 10bits for DL DCCH MessageType and others, aligned to 2B) ## Taking 1 bytes as minimum value.; ## 0 byte. ## PDCP; ## 0 byte; ## 1 byte. ## RLC; ## UM: 1 byte ## AM: 2 bytes; ## UM: 1 byte ## AM: 2 bytes. ## MAC; ## 2 bytes; ## 2 bytes. NAS layer; ## 7 bytes (with NAS security) ## (note: CT1 still work in R19 to reduce NAS layer overhead); ## 0 bytes. Higher layers for Non-IP (NIDD) based option; ## IP; ## 0 byte; ## 0 byte. ## UDP; ## 0 byte; ## 0 byte. ## RTP; ## 12 bytes; ## 12 bytes. Higher layers for IP based option without RoHC; ## IP; ## 20 bytes; ## 20 bytes. ## UDP; ## 8 bytes; ## 8 bytes. ## RTP; ## 12 bytes; ## 12 bytes. Higher layers for IP based option with RoHC; ## 1 bytes (e.g., UO-0 format) or ## 2 bytes (e.g., UO-1-TS format); ## 1 bytes (e.g., UO-0 format) or ## 2 bytes (e.g., UO-1-TS format). # 2 Actions To SA4: RAN2 respectfully asks SA4 to take the above information into account in the related works.

Annex A.2: Draft LS to R2-2506747_S2-2507636

Based on the discussion in the section 1.2, we give the following draft reply LS to the SA2 LS (R2-2506747_S2-2507636) which mentioned some issues related to support of IMS voice over NB-IoT NTN connected to EPC. We focus on answering the questions directed to RAN2:

# 1 Overall description RAN2 thanks SA2 for the LS on the issues related to support of IMS voice over NB-IoT NTN connected to EPC. RAN2 can provide the following answers to the questions raised for RAN2: Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Answer to Q1: Generally, without some prior agreed assumptions, such as channel conditions, it is difficult to estimate the extent of packet loss. Anyway, we think packet loss is not easily encountered in NB-IoT over GEO scenario, with reasonable assumption that there is stable signal conditions in an open environment, along with sufficient repetitions are used to compensate for the link budget. Furthermore, we understand the ROHC compressor and decompressor have their own internal operational logic. In the 3GPP context, we cannot identify a criteria to specify how many lost packets would trigger the RoHC state fall back at the compressor when using RoHC. Finally, per our observations, the "repair mechanisms" of ROHC, such as periodic refresh, CRC checks and error detection, would enable ROHC to perform "self-repair" when packet loss or bit errors occur, and maximize time spent in the efficient FO or SO states. Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? In the context of using "Non-IP" for voice media traffic there is no header compression. To reduce the header overhead, it is necessary to understand which parts of the RTP header, e.g., sequence number, timestamp, SSRC (Synchronization Source ID), Payload Type (PT) is essential to the receiver support IMS voice over NB-IOT NTN. Answer to Q3: As the SPS is not supported for NB-IoT and may be not easy to be introduced due to possible large specification impacts, we assume dynamically scheduling still can be used. Firstly, seldom RoHC state transition enables the transmission of vice packets with compressed header and of consistent size for most of the time, which is beneficial for efficient resource scheduling. Moreover, a bit longer Frame Duration for generating voice frames can be assumed which is also helpful to reduce the number of SID frames, resulting in more consistent voice data packet during the whole voice call. For the consideration on RTP protocol in the context of using "Non-IP" for voice, we understand to apply simplified/reduced RTP protocol may cause inability to leverage standard security mechanisms like DTLS, and also cause inability to future functional expansion of voice application in NB-IoT over GSO, e.g., cannot support multipoint communication in the future. So we think most RTP header parts are still needed for implement whole RTP function and also future extension. Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? In addition to user-plane based solutions to carry voice media, several other proposed solutions under Key Issue #1 (Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC) rely on control-plane-based mechanisms, where Control Plane CIoT EPS optimisation and SRBs are used to transport SIP signaling and/or voice media. Answer to Q6: from RAN2 perspective, we think it’s feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. We also think the impact on RAN2 specs of adding a new DRB would not be large. The new DRB should have the same characteristics and usage as existing DRBs. However, RAN2 first needs to elaborate whether it is necessary to support such service of concurrent data and IMS voice in a simplified systems like NB-IoT. Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Answer to Q9: No. From RAN2 perspective, since we are unclear whether this new SRB can only be used for carrying voice media, we are unable to assess its feasibility. Generally we think RAN2 cannot define a new SRB solely for a certain specific type of service data, e.g., voice media data, as this is not the typical way of standardization but rather an abuse of the standard. As a radio bearer, once it is defined, it should be available for any suitable service, depending on the network configuration and/or the characteristics of this new radio bearer. Moreover, since the new SRB is introduced with intention to carry voice data packet (generally need of low latency), it needs to be defined with RLC UM mode. However, such a design seems to violate the principle of defining SRBs. Since it’s still a signaling radio bearer, it should be defined with RLC AM mode for ensuring transmission reliability. Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media? Answer to Q10: To define a new SRB with RLC UM mode appears to violate the design principles of SRB. # 2 Actions To SA2: RAN2 respectfully asks SA2 to take the above information into account in the related works.

Annex B: Background information

Annex B.1: Basic CP/UP signaling procedures (TS 36.300)

TS 36.300 V18.5.0 (2025-06) E-UTRA and E-UTRAN; Overall description; Stage 2 (Release 18) 7.3a.2 Control Plane CIoT EPS/5GS optimization The RRC connection established for Control Plane CIoT EPS optimisation, as defined in TS 24.301 [20], and Control Plane CIoT 5GS Optimisation, as defined in TS 24.501 [91], are characterized as below: - A UL NAS signalling message or UL NAS message carrying data can be transmitted in a UL RRC container message (see Figure 7.3a.2-1). A DL NAS signaling or DL NAS data can be transmitted in a DL RRC container message; - for NB-IoT: - RRC connection reconfiguration is not supported; - Data radio bearer (DRB) is not used; - AS security is not used; - A non-anchor carrier can be configured for all unicast transmissions during RRC connection establishment or re-establishment. - There is no differentiation between the different data types (i.e. IP, non-IP or SMS) in the AS. Figure 7.3a.2-1: The RRC connection established for Control Plane CIoT EPS/5GS Optimisations As there is no overall procedure for Control Plane CIoT EPS/5GS Optimisations, with reference to the MO-EDT procedure, the following example figure is supplemented for briefly illustration: Overall RRC Connection establish procedure for Control Plane CIoT EPS Optimisations 0. Upon connection establishment request for Mobile Originated data from the upper layers, the UE initiates the RRC connection establishment procedure and UE sends RRCConnectionRequest, receives RRCConnectionSetup and sends RRCConnectionSetupComplete (may including UL NAS message). 1. For EPS, the eNB initiates the S1-AP Initial UE message procedure to forward the NAS message and establish the S1 connection. 2. For EPS, the MME requests the S-GW to re-activate the EPS bearers for the UE. 3. For EPS, the MME sends the uplink data to the S-GW. 4. For EPS, if downlink data are available, the S-GW sends the downlink data to the MME. 5. If downlink data are received from the S-GW, the MME forwards the data to the eNB via DL NAS Transport procedure and may also indicate whether further data are expected. Otherwise, the MME may trigger Connection Establishment Indication procedure (without NAS message) and also indicate whether further data are expected. 6. If no further data are expected, the S1 connection and RRC connection are released, and the EPS bearers are deactivated. 7.3a.3 User Plane CIoT EPS/5GS optimisations The RRC connection established for User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], and User Plane CIoT 5GS Optimisation, as defined in TS 24.501 [91], are characterized as below: - A RRC connection suspend procedure is used at RRC connection release, the (ng-)eNB may request the UE to retain the UE AS context including UE capability in RRC_IDLE; - A RRC connection resume procedure is used at transition from RRC_IDLE to RRC_CONNECTED where previously stored information in the UE as well as in the (ng-)eNB is utilised to resume the RRC connection. In the message to resume, the UE provides a Resume ID (for EPS) or I-RNTI (for 5GS) to be used by the (ng-)eNB to access the stored information required to resume the RRC connection; - At suspend-resume, security is continued. Re-keying is not supported in RRC connection resume procedure. The short MAC-I is reused as the authentication token at RRC connection reestablishment procedure and RRC connection resume procedure by the UE. For EPS, the eNB provides the NCC in the RRCConnectionResume message as well. And also the UE resets the COUNT; - Multiplexing of CCCH and DTCH in the transition from RRC_IDLE to RRC CONNECTED is not supported; - For NB-IoT, a non-anchor carrier can be configured for all unicast transmissions when an RRC connection is re-established, resumed or reconfigured additionally when an RRC connection is established. <skip> Figure 7.3a.3-2: RRC Connection Resume procedure in EPS 1. At some later point in time (e.g. when the UE is being paged or when new data arrives in the uplink buffer) the UE resumes the connection by sending an RRCConnectionResumeRequest to the (ng-)eNB. The UE includes its Resume ID (for EPS) or I-RNTI (for 5GS), the establishment cause, and authentication token. The authentication token is calculated in the same way as the short MAC-I used in RRC connection re-establishment and allows the (ng-)eNB to verify the UE identity. For 5GS, the UE resumes SRB1, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection and re-establishes the AS security. 2. Provided that the Resume ID (for EPS) or I-RNTI (for 5GS) exists and the authentication token is successfully validated, the (ng-)eNB responds with an RRCConnectionResume. For EPS, the message includes the Next Hop Chaining Count (NCC) value which is required in order to re-establish the AS security. 3. For EPS, the UE resumes all SRBs and DRBs and re-establishes the AS security. For 5GS, the UE resumes all other SRBs and all DRBs. The UE is now in RRC_CONNECTED. 4. The UE responds with an RRCConnectionResumeComplete confirming that the RRC connection was resumed successfully, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the (ng-)eNB. 5. For EPS, the eNB initiates the S1-AP Context Resume procedure to notify the MME about the UE state change. For 5GS, the ng-eNB initiates the NG-AP Context Resume procedure to notify the AMF about the UE state change. 6. For EPS, the MME requests the S-GW to activate the S1-U bearers for the UE. For 5GS, the AMF requests the SMF to resume the PDU session and the SMF requests the UPF to establish the tunnel information for the UE. 7. MME/AMF Acks step 5.

Annex B.2: Non-IP Data Delivery (NIDD) related specs description

In the SA2 TR for Rel-20 IoT NTN enhancements, it gives a general guideline that only architecture options with data via SGi will be considered in the study, if Non-IP data delivery scheme is considered:

TR 23.700-19 V1.0.0, Study on Integration of satellite components in the 5G architecture; Phase 4 (Rel-20) <skip> - 4 Architectural Assumptions For IMS voice call over NB-IoT (GEO), the following architecture assumptions and principles are applied to this study: - The EPC architecture with GEO satellite access for NB-IoT, as defined in TS 23.401 [5] is used as baseline. NOTE 1: CP CIoT EPS optimization, UP CIoT EPS optimization and S1-U based NB IoT NTN solutions will be studied. NOTE 2: Solutions should take into account that CP CIoT EPS optimization data in current pre Rel-20 NB-IoT is regarded as NAS signalling and by default has higher priority than UP data. - There will be no mobility enhancements developed for the NB-IoT NTN in this study. - Enhancements to NB-IoT (GEO), EPC, and IMS for supporting voice call via GEO shall be minimized. - Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered. - The IMS reference architecture, as specified in TS 23.228 [6] for non-emergency calls is used as baseline. For emergency IMS sessions over NB-IoT (GEO), the following additional architecture assumptions and principles are applied to this study, compared to above: - The IMS reference architecture, as specified in TS 23.167 [7] for emergency calls is the baseline. - Enhancements to LCS architecture for IMS emergency sessions and other regulatory services are out of scope of this study. The following architecture assumptions are applied to the study for UE-Satellite-UE Communication: <skip>

In TS 23.401, there is description of support for Non-IP Data Delivery. Two mechanisms, e.g., SCEF based delivery and SGi based delivery, for delivering Non-IP Data to SCS/AS, are mentioned as below:

TS 23.401 V19.3.0 (2025-06), System Architecture for EPS 4.3.17.8 Support for Non-IP Data Delivery (NIDD) - 4.3.17.8.1 General The support of Non-IP data is part of the CIoT EPS Optimisations. A PDN Type "Non-IP" is used for Non-IP data. The Non-IP data delivery to SCS/AS is accomplished by one of two mechanisms: - Delivery using SCEF as defined in clause 4.3.17.8.3.2. - Delivery using a Point-to-Point (PtP) SGi tunnel as defined in clause 4.3.17.8.3.3. When the Reliable Data Service is not used, Non-IP data in-sequence delivery cannot be guaranteed and data PDUs may be lost requiring higher protocol layers to ensure guaranteed delivery when needed. The Reliable Data Service is defined in TS 23.682 [74]. NOTE: If UEs use protocols that require broadcast/multicast mechanisms (e.g. use "IP stacks" on top of PDN connections of type "Non-IP"), this may cause increased traffic and power consumption to the UE and the network. The SMS service may also be used to deliver data without use of the IP protocol. The SMS service is always supported for CIoT EPS Optimisations, i.e. can be used simultaneously with Non-IP and IP data. When only the SMS service is needed, an attach without PDN connection establishment can be used, see clause 5.3.2. Dedicated bearers are not supported for the Non-IP data. - 4.3.17.8.2 ESM Procedures The UE indicates in the ESM connection request, e.g. in Attach or PDN Connectivity Request, that a Non-IP PDN type shall be used. The subscription information has a default APN for PDN Type Non-IP, which the MME uses for the first received Non-IP connectivity request unless the UE has included an APN in the request. 4.3.17.8.3 Delivery mechanism 4.3.17.8.3.1 General At each PDN connectivity request, the MME decides which delivery mechanism (SCEF based delivery or SGi based delivery) is used for delivering the Non-IP data between RAN and AS. An indication associated with the used APN determines if SCEF based delivery or SGi based delivery shall be used. 4.3.17.8.3.2 SCEF based delivery When the MME decides to use SCEF based delivery mechanism for Non-IP data, a PDN connection is established towards the selected SCEF. Such a PDN Connection is also known as an "SCEF Connection". The APN used for SCEF based delivery is an FQDN, which either resolves to an SCEF hostname or to an SCEF IP address. The SCEF based delivery is applicable only to the Control Plane CIoT EPS Optimisation (see clause 4.10). The support of Non-IP data via the SCEF is further defined in TS 23.682 [74]. 4.3.17.8.3.3 SGi based delivery 4.3.17.8.3.3.1 General When support of Non-IP data is provided at the SGi interface, different point-to-point tunnelling techniques may be used. Point-to-point tunnelling by UDP/IP encapsulation can be used as described in clause 4.3.17.8.3.3.2 below. Other techniques as described in clause 4.3.17.8.3.3 below may be used. Support for the SGi based delivery of Non-IP data can be used by any UE. That is, it is independent of support for the User Plane CIoT EPS Optimisation and the Control Plane CIoT EPS Optimisation (see clause 4.10). The P-GW decides at PDN connection establishment based on pre-configuration which point-to-point tunnelling technique is used for the SGi based delivery between the P-GW and the AS. NOTE: The pre-configuration can be done in the P-GW per APN or based on other criterion such as SLA between operator and 3rd party application service provider, etc. 4.3.17.8.3.3.2 SGi PtP tunnelling based on UDP/IP SGi PtP tunnelling based on UDP/IP may be used to deliver Non-IP data to AS via SGi. A point-to-point tunnel is used by the P-GW towards the AS. The tunnel parameters (i.e. destination IP address and UDP port) for SGi PtP tunnelling based on UDP/IP are pre-configured on the P-GW. IP address allocation procedures for PDN connections are performed locally (e.g. without involving the UE) by the P-GW based on APN configuration and according to clause 5.3.1. Only single IP address is used (i.e. both IPv4 and IPv6 addresses are not allocated). The P-GW acts as a transparent forwarding node for the payload between the UE and the AS. For uplink Non-IP data, the P-GW forwards the received data to the AS over the SGi PtP tunnel using UDP/IP encapsulation. When the Reliable Data Service is enabled, the P-GW processes the Reliable Data Service Header. The Reliable Data Service Configuration is pre-configured on the P-GW. The Reliable Data Service Configuration is defined in TS 23.682 [74]. For downlink Non-IP data, the AS sends the data using UDP/IP encapsulation with the IP address of the UE and the 3GPP defined UDP port for "Non-IP" data. The P-GW decapsulates the received data (i.e. removes the UDP/IP headers) and forwards the data to S-GW on the GTP-U tunnel identified by the IP address of the UE (i.e. PDN connection) for delivery to the UE. When the Reliable Data Service is enabled, the P-GW adds the Reliable Data Service Header. The P-GW performs the IP related operations (e.g. allocates IP address for the PDN connection), but the IP address or IP prefix is not provided to the UE (i.e. SLAAC / Router Advertisements are not performed. DHCP or DHCPv6 are not used). In the case of IPv6 the P-GW assigns an Interface Identifier for the PDN connection. The allocated IP address or IPv6 prefix identifies the PDN connection of the UE. The P-GW shall inform the MME of the assigned IPv4 address or IPv6 prefix and Interface Identifier for a PDN Connection of a given UE. However, the UE is not informed about the assigned IPv6 prefix and Interface Identifier. NOTE: It is recommended to use IPv6 for CIoT. IPv4 based addressing is deprecated for machine type communication used over 3GPP accesses, see TS 23.221 [27]. 4.3.17.8.3.3.3 Other SGi PtP tunnelling mechanisms SGi PtP tunnelling mechanisms such as PMIPv6/GRE, L2TP, GTP-C/U, etc, may be used to deliver Non-IP data to AS via SGi. The general handling of such delivery mechanisms is as described below. A point-to-point tunnel is established by the P-GW towards the AS. Depending on the type of protocol employed on the SGi PtP tunnel, the SGi PtP tunnel setup may be done at the time of Attach or at the time of first MO datagram being sent by the CIoT UE. The P-GW selects the AS based on the P-GW configuration (e.g. per APN, or per PtP tunnel type etc.). However, IP address allocation procedures for the UE (according to clause 5.3.1) are not performed by the P-GW. NOTE: An AS can be dedicated for handling a specific Non-IP data protocol. The P-GW acts as a transparent forwarding node between the UE and the AS. For uplink Non-IP data, the P-GW forwards the received data to the AS over the established SGi PtP tunnel. For downlink Non-IP data, the AS locates the right SGi PtP tunnel for the UE (using information such as UE identifiers in the Non-IP protocol itself, etc) to forward the data. The AS sends the data to P-GW over the established SGi PtP tunnel. The P-GW in turn sends the data to S-GW on the GTP-U tunnel identified by the associated SGi PtP tunnel for delivery to the UE. <skip> Annex L (informative): Optimised EPS Architecture option for CioT <skip> L.2 Non-Roaming Architecture Figure L.2-1: Optimised EPS architecture option for CIoT - Non-roaming architecture <skip>

Also, in RAN3 S1-AP spec, it introduces a corresponding “Bearer Type” IE in the signaling of E-RAB Management procedures for supporting Non-IP data as specified in TS 23.401. For example, the “Bearer Type” IE in E-RAB SETUP REQUEST message sent from MME to eNB can be set to "non IP", then the eNB shall not perform IP header compression for the concerned E-RAB.

TS 36.413 V18.3.0 (2024-12) 8.2 E-RAB Management procedures 8.2.1 E-RAB Setup 8.2.1.1 General The purpose of the E-RAB Setup procedure is to assign resources on Uu and S1 for one or several E-RABs and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling. 8.2.1.2 Successful Operation Figure 8.2.1.2-1: E-RAB Setup procedure. Successful operation. The MME initiates the procedure by sending an E-RAB SETUP REQUEST message to the eNB. - The E-RAB SETUP REQUEST message shall contain the information required by the eNB to build the E-RAB configuration consisting of at least one E-RAB and for each E-RAB to setup include an E-RAB to be Setup Item IE. Upon reception of the E-RAB SETUP REQUEST message, and if resources are available for the requested configuration, the eNB shall execute the requested E-RAB configuration. For each E-RAB and based on the E-RAB level QoS parameters IE the eNB shall establish a Data Radio Bearer and allocate the required resources on Uu. The eNB shall pass the NAS-PDU IE and the value contained in the E-RAB ID IE received for the E-RAB for each established Data Radio Bearer to the UE. The eNB does not send the NAS PDUs associated to the failed Data radio bearers to the UE. The eNB shall allocate the required resources on S1 for the E-RABs requested to be established. If the Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB. If the SIPTO Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB. If the Bearer Type IE is included in the E-RAB SETUP REQUEST message and is set to "non IP", then the eNB shall not perform IP header compression for the concerned E-RAB. …………………...

Annex B.3: QoS support-Bearer aspects

IMS (IP Multimedia Subsystem)-based voice is a kind of IP multimedia application. As mentioned in TS 23.228, the complete solution for the support of IP multimedia applications consists of terminals, IP-Connectivity Access Networks (IP-CAN) and the specific functional elements of the IP multimedia core network (IM CN) subsystem. For supporting IMS-based voice service in NB-IoT over GSO, the EPC core network and NB-IoT radio access network act as the IP-CAN network.

Referring to other IP-CAN networks capable of supporting IMS voice service, such as the E-UTRAN radio access network or the 5GS access network, to support IMS-based voice service via NB-IoT over GSO, the following bearer concepts are to be used:

- PDN Connection

To support IMS voice services, a PDN connection to the IMS APN needs to be established. PDN Connection is the association between a PDN (Packet Data Network) represented by an APN (Access Point Name) and a UE, represented by one IPv4 address and/or one IPv6 prefix (for IP PDN Type) or by the UE Identity (for Non-IP and Ethernet PDN Types).

Generally, A PDN connection (or PDU Session in 5G) has a single PDN type: it is either an IP type (IPv4, IPv6, or IPv4v6), a Non-IP type, or in 5G, an Ethernet type. It cannot carry PDU sessions of different types simultaneously.

All bearers within a PDN connection inherit this PDN type and carry data of that specific format. In other word, there should not be multiple bearers corresponding to different PDN types (e.g., IP or Non-IP) within the same PDN connection; that is, bearers with type of “IP” and bearers with type of “Non-IP” cannot coexist in the same PDN connection. If a UE needs to perform IP data communication (e.g., internet access) and Non-IP data communication (e.g., reporting sensor data) simultaneously, the UE and the network should establish multiple independent PDN connections/PDU sessions.

- IP-CAN Bearer

The IP-CAN (IP Connectivity Access Network) bearer, that is also called EPS bearer in 4G network, is a logical end-to-end pipe that provides IP connectivity from the UE to an external packet data network (PDN) associated with a certain APN (Access Point Name), such as the IMS APN. The IP-CAN bearer is established during the PDN connection setup and is characterized by a specific QoS profile.

There are two main types of IP-CAN bearers:

- Default Bearer: Created when the UE attaches to the network, typically a non-GBR bearer used for general Internet access, or IMS signaling.

- Dedicated Bearer: Established on-demand or pre-established for specific services requiring guaranteed QoS (e.g., VoLTE), and can be either GBR or non-GBR.

For example, a PDN connection to an "internet" APN typically includes one default bearer and zero or more dedicated bearers. More specifically:

- One Default Bearer: This is always established when the PDN connection is created. It provides a baseline "best effort" QoS (usually QCI=9) for general internet traffic like web browsing and email.

- Zero or More Dedicated Bearers: These are optionally established on demand for specific applications requiring guaranteed QoS. For example:

  - One dedicated bearer for VoIP (e.g., QCI=1).

  - Another dedicated bearer for video streaming (e.g., QCI=2).

In another example, a PDN connection to an "IMS" (IP Multimedia Subsystem) APN typically includes one default bearer and one dedicated bearer, making a total of two IP-CAN bearers.

- One Default Bearer: This is established when the PDN connection is initiated. It usually has a "best effort" QoS (typically QCI=9 or QCI=5). It is primarily used for signaling traffic related to IMS services (such as SIP registration, session setup/teardown) and other non-real-time data that doesn't require strict QoS guarantees.

- One Dedicated Bearer: This is dynamically established on demand when a real-time service session starts (e.g., when a VoLTE call is initiated or an IMS video call begins). It is configured with a guaranteed QoS profile suitable for real-time media:

  - For VoLTE (Voice): Typically uses QCI=1 (with GBR) for the voice stream.

  - For ViLTE (Video): Typically uses QCI=2 (with GBR) for the video stream.

- GBR vs non-GBR Bearer

The GBR/non-GBR type bearer can be seen as a key attribute of an IP-CAN Bearer, determined during bearer setup based on service requirements and policy:

- GBR (Guaranteed Bit Rate) Bearer: The network guarantees a minimum bit rate for this bearer, essential for real-time services like voice or video calls.

- non-GBR Bearer: For providing best-effort service without bandwidth guarantees, suitable for applications like web browsing or file downloads.

- Service Data Flow (SDF)

An SDF represents an actual application-level data flow, such as a video stream, a VoLTE voice packet stream, or a web page request.

One or multiple SDFs with similar QoS requirements can be aggregated and mapped onto the same IP-CAN bearer/EPS Bearer. For example, a critical SDF (e.g., VoLTE) may require a dedicated GBR EPS Bearer to ensure its QoS. This mapping is controlled by the PCC (Policy and Charging Control) framework (refer to TS 23.203), where the PCRF (Policy and Charging Rules Function) defines rules for classifying traffic and assigning it to the appropriate bearer.

Generally, for IMS-based voice service, two service data flows (SDFs) need to be supported: one for IMS signaling and the other for IMS voice packets. The voice packet flow is a real-time flow with stringent delay requirements. The IMS signaling flow on the other hand requires high reliability and can tolerate some delay.

- E-RAB (E-UTRAN Radio Access Bearer)

The E-RAB is the realization of the EPS Bearer within the radio access network (E-UTRAN). It exists only between the UE and the eNB. Each EPS Bearer has a corresponding E-RAB.

Technically, an E-RAB comprises:

- The S1 Bearer (logical connection between eNB and Serving Gateway, S-GW).

- The Data Radio Bearer (DRB) (logical connection between UE and eNB).

The E-RAB enforces radio-specific QoS mechanisms (e.g., scheduling priority, HARQ, RLC mode) to meet the QoS requirements defined for the EPS Bearer.

Annex B.4: SIP procedure

The baseline procedure for IMS-based voice procedure in clause 5.7a.2 of TS 23.228 is cited as below:

TS 23.228 V19.3.0 IP Multimedia Subsystem (IMS); Stage 2 (Release 19) (2025-06) …………………… 5.7a.2 Procedures for the establishment of sessions without preconditions - no resource reservation required before session becomes active Figure 5.19h: End-to-end session flow procedure without preconditions - no resource reservation required before session becomes active The description for the main steps are as below: - UE#1 sends the SIP INVITE request, containing an initial SDP, to the P CSCF#1 determined via the P CSCF discovery mechanism. The initial SDP may represent one or more media for a multi-media session. It should be noted that a media offer without preconditions in general implies that the offering entity might expect to receive incoming media for any of the offered media as soon as the offer is received by the other endpoint. Therefore either an existing IP CAN bearer is assumed to be available for use or the application is implemented such that incoming media is not expected until some later point in time. ……………….. - P CSCF#1 forwards the INVITE request to S CSCF#1 along the path determined upon UE#1's most recent registration procedure. ………………. - S CSCF#1 forwards INVITE request to I CSCF#2. - I CSCF#2 performs Location Query procedure with the HSS to acquire the S CSCF address of the destination user (S CSCF#2). - I CSCF#2 forwards the INVITE request to S CSCF#2. ……………… - S CSCF#2 forwards the INVITE request to P CSCF#2 along the path determined upon UE#2's most recent registration procedure. - P CSCF#2 examines the media parameters. If P CSCF#2 finds media parameters not allowed to be used within an IMS session (based on P CSCF local policies, or if available bandwidth authorization limitation information coming from the PCRF/PCF), it rejects the session initiation attempt. NOTE 0b: Whether the P CSCF should interact with PCRF/PCF in this step is based on operator policy. - P CSCF#2 forwards the INVITE request to UE#2. 12. - 17. UE#2 may optionally generate a ringing message towards UE#1. - Depending on the bearer establishment mode selected for the IP CAN session, resource reservation shall be initiated either by the UE or by the IP CAN itself. UE#2 may reserve a dedicated IP CAN bearer for media based on the media parameters received in the SDP offer as shown in Figure 5.19h. Otherwise, the IP CAN#2 initiates the reservation of required resources after step 20 instead. NOTE 1: The sequential ordering of 18 and 19 does not indicate that these steps are necessarily performed one after the other. If step 19 is performed before step 18 is finished, UE#2 shall use an existing IP CAN bearer to send and receive media unless the application is such that a new bearer is not needed until some later point in time. If step 18 is performed successfully, media are sent and received by UE#2 on the dedicated IP CAN bearer. - UE#2 accepts the session with a 200 OK response. The 200 OK response is sent to P CSCF#2. - Based on operator policy P CSCF#2 may instruct PCRF/PCF to authorize the resources necessary for this session. 21. - 24. The 200 OK response traverses back to UE#1. - Based on operator policy P CSCF#1 may instruct the PCRF/PCF to authorize the resources necessary for this session. - P CSCF#1 forwards the 200 OK response to UE#1. 27. - 31. UE#1 acknowledges the 200 OK with an ACK, which traverses back to UE#2. - Depending on the bearer establishment mode selected for the IP CAN session, resource reservation shall be initiated either by the UE or by the IP CAN itself. UE#1 may reserve a dedicated IP CAN bearer for media based on the media parameters received in the SDP answer as shown in Figure 5.19h. Otherwise, the IP CAN#1 initiates the reservation of required resources after step 25. NOTE 2: The sequential ordering of 27 and 32 does not indicate that these steps are necessarily performed one after the other. If step 32 is performed successfully, media are sent and received by UE#1 on the reserved dedicated IP CAN bearer. UE#1 may also use an existing IP CAN bearer to send and receive media.

In TR 23.700-19, the SA2 study report also provides an IMS procedure with resource pre-reservation as below:

TR 23.700-19 V1.0.0 A.2 Example message size and corresponding transmission delay for SIP messages in IMS Figure A.2-1 shows the typical IMS mobile call processing in TN scenario: Figure A.2-1: IMS mobile call processing in TN scenario

The above procedures and the corresponding SIP (Session Initiation Protocol) signaling can be summarized as:

INVITE (SDP offer) -> 180 Ringing (SDP answer) -> [PRACK -> 200 OK for PRACK->] 200 OK for INVITE -> ACK

In an IMS voice call, the SIP INVITE message is used to initiate a session. It is sent by the calling party (UE#1) to request that the called party (UE#2) establish a voice call, carrying session details such as the caller's identity, the called number, and the media parameters (like codec and IP address) for the voice stream, etc.

The 180 Ringing and 200 OK are both SIP response messages in an IMS call setup, but they represent different stages and outcomes of the call process:

- 180 Ringing indicates the request is being processed, but it is not a final result. 180 Ringing is sent by the called user's device (UE#2) after receiving the INVITE request, when it decides to accept the call and starts ringing (playing the ring-back tone). Also, it informs the calling party (UE#1) that the call is progressing and the called party (UE#2) is being notified. Upon receiving 180 Ringing, the calling party's device (UE#1) typically plays a ring-back tone ("beep... beep...") to the caller, indicating the remote phone is ringing. 180 Ringing usually does not carry SDP (Session Description). Media negotiation typically occurs earlier in a 183 Session Progress (which is similar as 180 Ringing but with SDP description) or in the final 200 OK

  - In some cases (not in the Figure 5.19h), the calling party (UE#1) can send a PRACK to acknowledge critical provisional responses such as 180 Ringing or 183 Session Progress. This prevents call abnormalities caused by the loss of the 180 Ringing message, for example, avoiding the retransmission of the INVITE by the calling party (UE#1). However, not all 180 Ringing responses require a PRACK. Whether PRACK is used depends on the network configuration and whether the message contains the header “Require: 100rel”. In traditional IMS core networks, this mechanism is typically enabled to ensure service quality.

    - Specifically, upon receiving a 180 Ringing response, if the calling party (UE#1) detects the presence of a special header RSeq (Response Sequence Number) and a reliability indicator such as “Require: 100rel”, it recognizes that the response requires reliable delivery and acknowledgment. The calling party (UE#1) then sends a PRACK message, which includes a RAck header containing the received RSeq number. Upon receiving the PRACK, the called party (UE#2) confirms that the 180 Ringing has been reliably delivered and responds with a 200 OK to the PRACK, completing the acknowledgment process.

- 200 OK is sent by the called user's device (UE#2) after the user physically answers the call (e.g., lifts the handset or taps "Answer"). Upon receiving 200 OK, the calling party must send an ACK message in response. After that, the ring-back tone stops, and the bidirectional media stream (voice, video) is established, allowing both parties to start talking. The 200 OK must carry SDP. This SDP contains the called party's final confirmed media capabilities (codecs, IP address, port number, etc.). The caller uses this information to set up the media receive path.

- In some cases, the called party (UE#2) may reject the call or cannot answer (failure case), then the called party (UE#2) may send a final response in the 3xx, 4xx, 5xx, or 6xx range, instead of send 200 OK. The most common ones are:

  - 486 Busy Here: The called party is busy (the phone is ringing, but the user pressed "Reject").

  - 487 Request Terminated: The called party has terminated the request (e.g., hung up).

  - 408 Request Timeout: The called party's device does not respond.

  - 603 Decline: Explicitly rejected.

- 100 Trying is also a provisional response in SIP protocol, belonging to the 1xx response class. To ensure reliability, the SIP protocol requires that if a calling party sends a request (e.g., INVITE) and receives no response within a certain time (typically 500ms), it will retransmit the request. If the called party's system or network is slow to process the request and fails to return a more specific response such as 180 Ringing or 183 Session Progress within 500ms, the calling party may mistakenly assume the request was lost and continue retransmitting the INVITE. This can cause network congestion, signaling storms, and may even result in the called party receiving multiple duplicate call requests. One solution is for the called party's proxy server (e.g., P-CSCF, S-CSCF) or the called device to immediately reply with a 100 Trying upon receiving the INVITE. Upon receiving 100 Trying, the calling party knows that its INVITE request has been successfully received by the network of the called party, stops retransmitting the INVITE, and begins waiting for subsequent, more specific responses (such as 180 Ringing, 183 Session Progress, or final responses like 200 OK / 486 Busy). In summary, 100 Trying is typically generated by a proxy server within the SIP network, rather than by the final called UE, and does not contain SDP (Session Description Protocol) information.

Annex B.5: ROHC operation modes and packet formats (RFC 3095)

The characteristics of three ROHC operation modes and their compression logic are roughly summarized as below:

| Mode | Characteristics | Figure for details of each state, state transitions, and compression logic |
| --- | --- | --- |
| U-mode | In RoHC U-mode (Unidirectional Mode), there is no feedback channel from the decompressor to the compressor. U-mode relies on a cycle of highly compressed packets (UO-0, UO-1-ID, UO-1-TS) for efficiency, interspersed with periodic context refresh packets (IR, IR-DYN) for robustness. This periodic refresh compensates for the lack of feedback, ensuring the decompressor can eventually resynchronize even after packet loss, at the cost of some bandwidth. |  |
| O-mode | Unlike U-mode, O-mode utilizes a feedback channel from the decompressor (DC) to the compressor (C).The decompressor primarily uses NACK (Negative Acknowledgment). If it cannot decompress a packet (e.g., due to context damage from lost packets), it sends a NACK to the compressor. The compressor can use highly efficient, context-dependent compression formats (like UOR-2) optimistically, assuming the channel is good. When a NACK is received, it knows the context is damaged and responds by sending a refresh packet (like IR or IR-DYN) to re-synchronize. Advantages over U-mode: Context repair is triggered immediately by a NACK, rather than waiting for a periodic refresh. |  |
| R-mode | R-mode provides the highest level of reliability and control. The compressor only advances to higher compression states (using more efficient formats) after receiving an ACK confirming the decompressor has received the necessary context updates. This ensures both ends are always in sync before aggressive compression is used. R-mode offers the highest reliability and is excellent for very poor links. R-mode is typically used in scenarios where maximum reliability is paramount and the reverse link has sufficient capacity for ACKs |  |

The packet formats primarily used in these ROHC operation modes are as follows:

| Mode | Mainly used packet formats |
| --- | --- |
| Common | - Initialization and Refresh (IR) packet: is mainly used for establishing and refreshing the decompression context, which is used for all the modes. - IR packets without static information (IR-DYN): is mainly used to update the dynamic parts of the context without resending static information, which is also used for all the modes. - Type 0 packets (e.g., UO-0, or UOR-0), type 1 packets (e.g., UO-1) and type 2 packets (e.g., UOR-2) are all packets carrying a compressed packet header. - A type 0 packet carries a packet header compressed according to a fixed pattern - Type 1, 2 packets are used when this pattern is broken. |
| U-mode | IR (Initialization and Refresh): The most critical packet for establishing and refreshing the decompression context. It contains the full static (e.g., IP addresses, port numbers) and dynamic (e.g., RTP SN, TS) header information, the Context ID (CID), and the Profile. It’s sent at the start of a flow and periodically throughout the session to prevent permanent desynchronization. IR-DYN (Initialization Dynamic): It updates the dynamic parts of the context without resending static information. It includes CID, Profile, CRC, and updated dynamic fields (like RTP SN, TS, IP-ID). It’s sent when dynamic values change significantly, serving as a lighter-weight refresh than a full IR. UO-0 (Unidirectional Optimization 0): It’s a simple, highly compressed format for in-order packets. It typically contains only the CID, a small difference in the Sequence Number (SN) and CRC. It’s used for regular data packets when the context is synchronized and packets arrive sequentially. UO-1-ID (Unidirectional Optimization 1 with IP-ID): It’s a compressed format that also encodes the IP-ID field. It typically contains CID, SN (difference), IP-ID (difference), R-bit, and CRC. It’s used when the IP-ID field needs to be compressed (e.g., for fragmentation). UO-1-TS (Unidirectional Optimization 1 with Timestamp): It’s a compressed format optimized for predictable RTP timestamp increments (common in voice). It typically contains CID, SN (difference), TS (difference), R-bit, and CRC. It’s highly efficient for voice traffic, compressing headers to 2-4 bytes by leveraging the regular pattern of timestamp growth. |
| O-mode | O-mode uses a superset of U-mode packets but introduces more efficient formats that leverage the feedback channel: IR (Initialization and Refresh): It establishes the initial context or performs a full refresh when requested (e.g., after a NACK). It has full static and dynamic header information, CID, Profile. For O-mode, it’s sent in response to a NACK or during initial setup. Less frequent than in U-mode due to faster NACK-triggered recovery. IR-DYN (Initialization Dynamic): It updates dynamic fields in the context. It typically contains CID, Profile, CRC, and updated dynamic header fields. For O-mode, it’s sent to refresh dynamic information, often in response to a NACK or when dynamic values change significantly. UOR-0 (U-mode/O-mode/R-mode 0): It’s a basic compressed format for O-mode. It contains CID, SN (difference), R-bit, and CRC. It’s a simple compressed packet, similar to UO-0 but used in bidirectional modes. UOR-1-ID / UOR-1-TS (U-mode/O-mode/R-mode 1 with IP-ID/Timestamp): It’s more advanced compressed formats for O-mode, handling IP-ID or RTP Timestamp compression. It includes CID, SN (diff), IP-ID/TS (diff), R-bit, CRC. It’s efficient compression for specific scenarios, used when the context is synchronized. UOR-2 (U-mode/O-mode/R-mode 2) - It’s the most common and efficient compressed format used in O-mode, especially for real-time traffic like voice/video. It’s extremely compact (typically 2-4 bytes) and contains a compressed SN, R-bit, CRC, and often a "First Byte" field that can carry the first byte of the payload (e.g., the RTP payload type) or other critical information. - Its efficiency relies on the feedback loop. If a UOR-2 packet is lost or corrupted and breaks decompression, the DC sends a NACK, and the C immediately responds with an IR or IR-DYN to fix the context. |
| R-mode | Like O-mode, R-mode requires a feedback channel from the decompressor (DC) to the compressor (C). It uses both ACK and NACK. - ACK: The DC sends an ACK to confirm it has successfully received and processed a context-updating packet (like IR, IR-DYN, or a packet with a "larger-than-expected" SN jump). This tells the C that the context is synchronized. - NACK: The DC sends a NACK if it cannot decompress a packet (context damaged) R-mode uses the same fundamental packet formats as U-mode and O-mode, but their usage and state transitions are governed by ACKs. IR (Initialization and Refresh): It establishes the initial context. For R-mode, it’s sent initially. The compressor waits for an ACK from the DC before transitioning to a state where it can use compressed formats like UOR-2. IR-DYN (Initialization Dynamic): It updates dynamic fields in the context. For R-mode, it’s sent when dynamic fields (like RTP timestamp base) need updating. The compressor typically waits for an ACK before resuming use of the highest compression efficiency formats. UOR-0, UOR-1-ID, UOR-1-TS: They are various levels of compressed packet formats. For R-mode, it’s used based on the current compression state, which is managed by ACKs. UOR-2 (U-mode/O-mode/R-mode 2): It’s the primary, highly efficient compressed format (2-4 bytes). For R-mode, It’s used extensively, but the compressor only uses it (or uses it with full confidence) after receiving ACKs confirming the context is synchronized. A NACK will trigger an IR/IR-DYN, and the C will wait for an ACK before returning to UOR-2. |

Annex B.6: Summary on solutions from SA2 study report

In SA2 study, till now, 11 solutions are proposed to address the "Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC" as below:

TR 23.700-19 V1.0.0 (2025-09) <skip> 6.0 Mapping of Solutions to Key Issues Table 6.0-1: Mapping of Solutions to Key Issues Key Issues. Solutions; #1; #2; #3; #4; #5. #1; X; X. #2; X. #3; X. #4; X; X; X; X. #5; X. #6; X. #7; X; X. #8; X. #9; X. #10; X. #11; X. #12; X. #13; X. #14; X. #15; X. #16; X. #17; X. #18; X. #19; X. #20; X. - Solution #1: UP based IMS voice call over NB-IoT NTN via GEO satellite using a default EPS bearer for both IMS signalling and voice data - Solution #2: Supporting establishing EPS bearers for IMS voice via user plane with IP type PDN connection - Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW - Solution #4: Support of IMS call over NB-IoT NTN via NIDD and I1 protocol - Solution #5: Support of IMS voice NB-IoT NTN connected to EPC using control plane bearers and new SRBs - Solution #6: Support establishing dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO) - Solution #7: CP based IMS voice call over NB-IoT NTN via GEO satellite using an SRB for both IMS signalling and voice data - Solution #8: Combined CP/UP bearers in a PDN connection - Solution #9: Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle - Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections - Solution #11: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes --------------------

From the RAN2 and AS layer perspective, we provide a brief summary and classification of these solutions, focusing on how they utilize radio bearers and which solution (CP solution or UP solution) they are based on. Some characteristics of the solutions are cited as below:

- (Class 1) 2DRBs, IMS signalling is transmitted via the default EPS bearer while IMS voice data is transmitted via a dedicated EPS bearer (Solution#2, #3, #4)

  - UE requests to establish an IP type PDN connection with IMS APN via user plane….. The dedicated EPS bearer may be established together with the default EPS bearer. (Solution#2)

    - One optimization to reduce the call setup time is to pre-establish the dedicated bearer prior to the IMS voice call setup. (Solution#2)

    - UE support creation of the dedicated EPS bearer for voice, and optionally together with during the attach or PDN connection establishment procedure; UE may support update packet filter locally based on the SDP negotiation result; (Solution#2)

(Solution#2)

  - IMS Signalling (e.g. SIP INVITE ... etc.) over default EPS bearer using IP while IMS voice traffic as non-IP (NIDD) traffic over dedicated EPS bearer. The PGW will translate the NIDD voice packets to IP/UDP/RTP packets before forwarding them to the IMS entity in the DN, e.g. the AGW (Solution#3)

    - The PDN Type for this IMS PDN is IP Type; and a dedicated EPS bearer is configured to use NIDD to transmit voice packets. (Solution#3)

    - For the uplink voice packets to be sent over a dedicated bearer using NIDD, i.e. without IP/UDP header the existing Packet filter component type identifier (as specified in TS 24.008 [13] clause 10.5.6.12) is not able to indicate the "voice traffic" needs to be steered to the dedicated EPS bearer context, thus a new Packet filter component type identifier may be introduced, e.g. "(IMS) voice traffic type", to indicate the UE to steer NIDD voice traffic to the dedicated EPS bearer context. (Solution#3)

    - Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice traffic is FFS. Whether an existing GBR or non-GBR QCI can be used for the dedicated EPS bearer used for voice traffic is FFS. (Solution#3)

(Solution#3)

  - To establish separate Data Radio Bearers (DRBs) for signalling and voice media packets due to different transport requirements for these two types of packets. Voice packets require consistent jitter and delay handling and may benefit from prioritization or special treatment in satellite links. (Solution#4)

    - It is proposed to use User Plane Non-IP Data Delivery (UP NIDD) for both call control (e.g. call setup, Supplementary Service Control) signalling and voice media. (Solution#4)

    - UP NIDD, operating over the user plane, also enables the use of AS-level security mechanisms. Although NB-IoT does not support dedicated data radio bearers, the default bearer used by UP NIDD allows for radio-level optimization (e.g. RLC AM for signalling, RLC UM for media), which can be configured for improved reliability and latency control. (Solution#4)

    - It is proposed to use IMS Service Centralization and Continuity architecture (see clause 4.20 of TS 23.228 [6] and TS 23.292 [15]) for IMS session establishment over NB-IoT NTN GEO links. I1 protocol (as defined in TS 24.294 [14]) is designed specifically for this scenario where the bearer path is CS or the transport used for IMS call control is bandwidth constrained, but IMS service control is still required. (Solution#4)

    - I1 is already standardized and supported by the Service Centralization and Continuity Application Server (SCC-AS), and since it eliminates the need for a SIP stack on the UE, it also greatly simplifies device implementation. (Solution#4)

    - Two default DRBs are established per IMS session, one for I1 signalling and one for voice media. This separation allows media traffic to receive differentiated treatment, even in the absence of conventional QoS mechanisms. (Solution#4)

    - For transport, the User Plane uses RLC Acknowledged Mode (AM) for the signalling bearer to ensure reliable delivery, and RLC Unacknowledged Mode (UM) for the media bearer to reduce latency. Both bearers can be configured by the eNB as needed. (Solution#4)

    - At the RAN level, the DRBs established for IMS traffic over NB-IoT GEO should be treated with elevated priority, including preferential scheduling and minimized packet discard during periods of congestion. (Solution#4)

    - QoS support: One approach is to assign an existing or newly defined QCI to voice traffic over these links. …. It is also recommended to configure separate APNs for signalling and media traffic, each using its own default EPS NIDD UP bearer. (Solution#4)

- (Class 2) One DRB? (One) Default EPS bearer for both IMS signalling and voice data. but It is FFS whether or how to differentiate QoS/priority between IMS signalling and voice data (Solution#1)

  - Due to restriction of NB-IoT RAT that dedicated EPS bearer is not used and GBR bearer is not used, we assume the default EPS bearer is used for both IMS signalling and voice data. (Solution#1)

  - If the expected resource for the voice call over NB-IoT(GEO) is static and need upgrade QoS of the default EPS bearer for the voice call traffic, in order to lessen the call setup time such bearer modification procedure can be initiated before IMS call setup procedure. (Solution#1)

- (Class 3) 2 SRBs, Two service data flows (SDFs) need to be supported: one for IMS signalling and the other for IMS voice packets….. Two SDFs need to be carried over two separate signalling radio bearers (SRBs) over the air (Solution#5, #6)

  - Solution #5:

Figure 6.5.1.4-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

    - For PDN connection types, the following options are possible: (Option 1) Both SDFs are transported over the same PDN connection with PDN type IP (The IMS signalling flow uses SIP/UDP/IP stack and The voice packet flow uses RTP/UDP/IP stack); (Option 2) Both SDFs are transported over the same PDN connection with PDN type non-IP (The protocol stack for the IMS signalling flow for this case is FFS and The voice packet flow uses RTP/NIDD stack); (Option 3) IMS signalling flow and the voice packet flows use different PDN connections. Namely, the IMS signalling flow uses PDN type IP and the voice packet flow uses PDN type non-IP (RTP/NIDD stack)

    - The voice packet flow requires a separate SRB from the SRB used for IMS and NAS signalling. Whether a separate SRB is needed for the IMS signalling flow depends on RAN and is FFS.

    - The default EPS bearer of the IMS PDN connection is that of QCI-5 and is used to carry IMS signalling; An additional dedicated EPS bearer is established in combination with or directly after the default EPS bearer activation at Attach or UE requested PDN connectivity procedures to carry the voice packets. This EPS bearer is also non-GBR and possibly needs a new QCI to be defined……Otherwise, a separate dedicated EPS bearer is established after the Attach or PDN Connection establishment. Similarly, in message 8, RRC Reconfiguration is used.

    - The SRBx dedicated to the transport of the voice SDF to eliminate both the RRC and the NAS layer overhead. The NAS layer overhead discussed in this clause does not include the NAS security overhead (only 1B MAC and NAS SN). Since SRBx is dedicated to the transport of the voice SDF, no RRC messages are needed over SRBx, i.e. SRBx can carry the DedicatedInfoNAS IE (NAS PDU) directly. This way, the RRC overhead could be reduced to zero. In other words, the voice packet would be identified over the air using the logical channel ID of the SRBx at the MAC layer.

    - Since SRBx is dedicated to the transport of voice pace SDF, any NAS PDU received by the MME over SRBx contains a voice packet. Therefore, NAS layer overhead is not needed to inform the MME about the content of the NAS PDU received over SRBx. To associate the NAS PDU with SRBx, the eNB could insert an indication e.g. signalling bearer identity in the S1-AP NAS transport messages (similar to how the legacy EDT session indication IE indicates the association with an EDT session in the S1-AP Initial UE message)

Table 6.5.1.5-1: Protocol overhead for voice packet flow over the control plane

| Protocol layer | Description | Best Case | Worst case | Comments/impacts |
| --- | --- | --- | --- | --- |
| MAC | Single MAC PDU per TBS | 1B | 1B | Supported in the specs. |
| RLC | TM or UM | 0 (TM) | 1B (UM) | 0 bytes requires spec support for RLC TM/UM for SRB. |
| RRC | RRC encapsulation of the NAS PDU | 0 | 1B | For zero RRC overhead see section 6.5.1.6. (The only overhead due to RRC encapsulation over SRBx is the length indicator for the NAS PDU, which is up to one byte. The length indicator could potentially be also eliminated) |
| NAS protocol | NAS message overhead | 0B | 3B | CT WG1 work in Rel-19 reduced NAS OH to 3 bytes. For zero NAS overhead see section 6.5.1.6. |
| NAS security | MAC and NAS SN | 1B | 5B | 1B requires MAC suppression. (NAS layer security overhead consists of 4 bytes for MAC (integrity protection) and one byte for SN. However, VoLTE is currently transported without integrity protection in all commercial deployments since User Plane Integrity Protection in E-UTRAN/EPC even though it is standardized is not commercially deployed. If accordingly user plane integrity protection is not required, the NAS security overhead could be one byte) |
| IP/UDP/RTP |  | 1B | 3B-30B | For non-IP PDN type: 1B for RTP SN. For IP PDN type: 3B for RoHC. 30B during RoHC context (re-)establishment. |
| TOTAL |  | 3B | 14B-41B | Key factors affecting OH: - MAC (integrity protection) suppression. - IP vs non-IP PDN type. - For IP PDN type: RoHC OH during RoHC context (re-)establishment. - SRB identification over S1-AP (see section 6.5.1.6). - NAS PDU size pre-negotiation over SRBx (see clause 6.5.1.6). |

  - (Solution #6) To enable the IMS voice transmission via NB-IoT control plane, an IMS PDN connection containing one default bearer and one dedicated bearer (GBR bearer) is proposed to transmit the IMS voice service

(Solution #6)

    - Based on the IMS voice over NB-IoT(GEO) Support indication, the UE can initiate a PDN connection establishment for IMS service by using an APN related to the IMS service, and the request PDN type is set as IP….…. the dedicated bearer can be pre-established in combination with the default bearer establishment……. To prevent the use of the dedicated bearer before IMS voice session is established, the TFT binding to the dedicated bearer shall be set to block all packets from the UL and DL.

    - Whether new SRB is needed for IMS signalling or IMS voice data transmission is FFS and depends on RAN implementation.

- (Class 4) An SRB for both IMS signalling and voice data (Solution #7)

  - The default EPS bearer is used for both IMS signalling and voice data transmission where an SRB is used for IMS signalling and can be switched to S1-U data transfer for voice data transmission if supported. (Solution #7)

    - The use of Control plane CIoT EPS Optimisation is assumed to provide IMS voice services via the default EPS bearer. i.e. IMS signalling and IMS voice data are served via the default EPS bearer, which can be updated its non-GBR QoS characteristics if required. Such QoS update is performed before IMS call setup procedure if UE is attached and IMS registered by NB-IoT (GEO). And so the signalling and data are delivered via SRB. Moreover, Control plane CIoT EPS Optimisation can be used initially for SIP signalling for IMS call setup, later if continuous data transmission(e.g. voice call data transmission) is expected, UE may trigger switching to S1-U data transfer

    - P-CSCF requests the resource reservation for voice call traffic(i.e. GBR traffic) over NB-IoT(GEO) to PCRF, but P-GW considers as it does not allocate dedicated & GBR bearer for the voice call traffic(i.e. skip the resource reservation) or, if required, performs bearer modification procedure for the default EPS bearer to upgrade its QoS even as a non-GBR bearer……PGW can initiate bearer modification procedure for resource reservation/QoS update for non-GBR characteristics of the default EPS bearer before IMS call setup.

    - If S1-U data transfer is supported, UE triggers switching to data delivery via DRB and S1-U bearer by sending control plane service request with active flag for voice call data transmission, which can be used to differentiate priority of voice data from IMS signalling delivered via an SRB

- (Class 5) 1 SRB + 1 DRB for voice, Combined CP/UP bearers in a PDN connection (IMS PDN session), with CP for default bearer and UP for dedicated bearer (Solution #8, #9, #10, #11)

  - Solution #8:

    - This solution utilizes the benefits of both CP and UP for separate transmission of IMS signalling and voice data, by combining a CP bearer and a UP bearer in a IMS PDN connection. The solution only uses one dynamically allocated radio UP bearer for GEO call, so the remaining radio UP bearer (s) can be used for other services during the voice call, if the NB-IoT UE is assumed to support up to 2 UP radio bearers

    - The CP bearer is the default bearer and established when the IMS PDN connection is setup, by using existing Control Plane CIOT optimization procedure. The CP bearer is used for transmission of IMS signalling. The UP bearer for transmission of voice data is dedicated bearer dynamically established when the voice path needs to be setup, e.g. during the SIP negotiation between two UEs….. The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision.

Figure 6.8.2.1-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

(Solution #8)

Table 6.8.2.1-1: Example Priority level for different type of traffics

| Traffics | Bearer | Configuration | Priority Level |
| --- | --- | --- | --- |
| RRC signalling, NAS signalling, IMS signalling | SRB1(-bis) (ACK Mode) | SRB by default has higher priority than any DRB | Highest |
| IMS voice | DRB2 (not ACK Mode) | e.g. Priority Level 2 if configured with QCI 1 as specified in TS 23.203 [17], Table 6.1.7-A | Middle |
| Internet data | DRB1 (ACK Mode) | e.g. Priority Level 9 if configured with QCI 10 as specified in TS 23.203 [17], Table 6.1.7-A | Low |

    - Whether QCI=1 is sufficient or a new QCI (GBR or non-GBR) value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes different longer ptime value

  - Solution #9: Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle

    - Basically, the UE uses user plane default bearer for IMS signalling and dedicated bearer for IMS voice when the UE is in ECM-CONNECTED. But when the UE is in ECM-IDLE, the UE use the Control Plan Service Request to send SIP INVITE message with indicating active flag when initiating voice call. The MME forwards the SIP INVITE encapsulated in the ESM message to SGW via S11-U interface and establishes S1-U bearer. Then the rest of IMS SIP signalling and voice are transferred via user plane default and dedicated bearers (GBR bearer) as existing procedure.

      - 2. After Attach or PDN Connection procedure, if the UE in ECM-CONNECTED performs IMS registration procedure over the default bearer of IMS PDN Connection (user plane). If the UE in ECM-IDLE triggers UE triggered Service Request in 5.3.4.1 of 23.401 [5] to use user plane because this is not voice call initiation from the UE.

      - 3. The eNB may trigger S1 release procedure (e.g. due to user inactivity) and the RRC Connection is released.

      - 4. When the UE initiates a voice call, it sends SIP INVITE message over the Control Plane using CP CIoT EPS Optimisation to the MME. The UE includes the SIP INVITE in the ESM DATA TRANSPORT message of ESM message container in the Control Plane Service Request and indicates active flag towards the MME.

    - The UE triggering IMS voice call over CP have advantage of saving call setup time because Control Plane Service Request message can contain the INVITE message, therefore the waiting time for service request and service accept from the network can be saved. To support QoS of IMS voice data, dedicated GBR bearer is needed

  - Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections

    - Two PDN connections are used for IMS voice, where one without DRB is used for SIP message exchange and the other with DRB is used for voice media. These two PDN connections are linked together by using (newly introduced) PDN connectivity pair ID.

    - UE:

      - Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator requested".

      - In order to solve the issue of coexistence of S11-U and S1-U: Handling of the order of activation of an SGi PDN connection using S1-U and activation of an SGi PDN connection using S11-U.

  - Solution #11: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

    - This solution is intended to be used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections".

    - For UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.