3GPP TSG-RAN WG2 Meeting #131bis	R2-2507518

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Discussion on SBFD RRC open issues

Document for:		Discussion and Decision

# Introduction

After RAN2#131 meeting, ASN.1 review is held for SBFD, and RIL is collected in [1]. In this contribution, we discuss SBFD RRC open issues.

# Discussion

## [C100][C104]

The two related issues are as follows:

| RIL Id | WI | Class | Title | Delegate | File version | Status |
| --- | --- | --- | --- | --- | --- | --- |
| C100 | SBFD | 1 | Concern on ‘CFRA’ | Jianxiang Li (CATT) | V005 | ToDo |

[Description]: For BFR, if there is a beam associated to a dedicated preamble/PRACH and above the threshold, using CFRA；Otherwise, using CBRA. If there is no suitable beam, this RO type can also be used for CBRA BFR. I’m wondering if the CFRA is required in ‘Indicates the second PRACH occasions of CFRA to be used by a SBFD aware UE’.

[Proposed Change]: remove ‘of CFRA’.

| RIL Id | WI | Class | Title | Delegate | File version | Status |
| --- | --- | --- | --- | --- | --- | --- |
| C104 | SBFD | 1 | Concern on ‘for CFRA’ | Jianxiang Li (CATT) | V005 | ToDo |

[Description]: If is RACH is triggered by RACH-ConfigDedicated, it may also be CBRA if the preamble index is set to 0b000000. In this case, the ra-OccasionType is also valid. Hence, it is suggested to remove “for CFRA”.

[Proposed Change]: remove ‘for CFRA’.

The issues are related to the following ASN.1 signalling:

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                   OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                 OPTIONAL, -- Need M

[...Omitted...]

[[

ra-OccasionType-r19                 ENUMERATED {sbfd}                  OPTIONAL  -- Need S

]]

}

BeamFailureRecoveryConfig field descriptions

ra-OccasionType Indicates the second PRACH occasions of CFRA to be used by a SBFD aware UE (see TS 38.213 [13], clause 8). If absent, indicates the first PRACH occasions to be used.

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                  OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization     OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization     OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16      OPTIONAL  -- Need S

]],

[[

ra-OccasionType-r19             ENUMERATED {sbfd}     OPTIONAL  -- Need S

]]

}

RACH-ConfigDedicated field descriptions

ra-OccasionType Indicates the second PRACH occasions for CFRA to be used by a SBFD aware UE. If absent, indicates the first PRACH occasions to be used.

During email discussion, it is common understanding that the intention of having “CFRA” is to reflect RAN2 agreement “SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA”. There was alternative proposal to add “or of the fallback CBRA” to cover both CFRA and CBRA case. The concern of removing “CFRA” is that there might be ambiguity that ra-OccasionType is also applicable to CBRA when CFRA resource is not configured. It is clear from MAC specification that ra-OccasionType is not applicable to CBRA when CFRA resource is not configured, as copied below from the following MAC specification:

1> if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000 and if the RACH occasion indicator is set to 1 (as specified in TS 38.212 [9]); or

1> if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]); or

1> if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]):

2> set the RO_TYPE to 2nd-RO.

Although MAC specification is clear regarding the usage of ra-OccasionType, it is more precise to specify in RRC field description that ra-OccasionType is also appliable to the fallback of CBRA.

Proposal 1: [C100][C104] In the field description of ra-OccasionType of IE BeamFailureRecoveryConfig and RACH-ConfigDedicated, “or of the fallback CBRA” is added after “CFRA”.

## [L701]

The open issue is as follows:

| RIL Id | WI | Class | Title | Delegate | File version | Status |
| --- | --- | --- | --- | --- | --- | --- |
| L701 | SBFD | 1 | Whether to locate the rach-ConfigCommon and sbfd-RACH-DualConfig for the same feature combination in the same additionalRACH-Config. | LGE (Hanseul Hong) | V008 | ToDo |

[Description]: In the RACH partitioning framework, when the 2-step RA resources and the 4-step RA resources are configured for the same feature combination, those two IEs are located in the same additionalRACH-Config IE, in order to simplify the UE implementation to identify the RA parameters associated with the same feature combination.

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

Since the SBFD RO (second PRACH occasion) defined in sbfd-RACH-DualConfig can also be associated with the feature combination and located in additionalRACH-Config IE, it should be discussed sbfd-RACH-DualConfig should be located in the same additionalRACH-Config with the non-SBFD RO (i.e., rach-ConfigCommon) for the same feature combination, in order to simplify the UE to determine the corresponding RA parameters (e.g., for the RO type switch case) as well.

[Proposed Change]: Discuss whether network needs to configure rach-ConfigCommon and sbfd-RACH-DualConfig in the same additionalRACH-Config, when the both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination. If so, following text can be added:

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config. If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

The restriction of proving rach-ConfigCommon and msgA-ConfigCommon in the same additionalRACH-Config was introduced in CR R2-2213055, with the reason for change as below:

In the current specifications, it is possible that the 2-step RACH configuration for a specific feature combination and 4-step RACH configuration for the same feature combination are provided in different AdditionalRACH-Config(s). When this is the case, it is unclear which parameters the UE should use for 4-step RACH when falling back from 2-step RA to 4-step RA, i.e. the parameters from the same AdditionalRACH-Config or the parameters for the same feature combination configured in the different AdditionalRACH-Config.

The same principle can be applied for SBFD RO and legacy RO so that it is easier for UE to choose the corresponding RACH configuration during RO type switching.

Proposal 2: [L701] For field additionalRACH-ConfigList, add the restriction that “If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.”

# Conclusion

In this contribution, we discuss SBFD MAC open issues, and propose the following:

Proposal 1: [C100][C104] In the field description of ra-OccasionType of IE BeamFailureRecoveryConfig and RACH-ConfigDedicated, “or of the fallback CBRA” is added after “CFRA”.

Proposal 2: [L701] For field additionalRACH-ConfigList, add the restriction that “If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.”

# References

[1] R2-2507000, Huawei, HiSilicon (Rapporteur), "WI SBFD ASN.1 Review file"