3GPP TSG-RAN WG2 Meeting #132	R2-2508138

Dallas, USA, 17th –21th November, 2025

Agenda Item:	10.3.1.3

Source:	Huawei, HiSilicon

Title:	Discussion on scheduling, retransmissions and uplink scheduling information in 6GR

Document for: Discussion and Decision

1	Introduction

In last meeting, RAN2 discussed several aspects of the User Plan for 6G and made some agreements as below [1].

Agreements on UP - RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. - UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. - 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) - Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework. - Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. - Study need for scheduling enhancements to address the SR/BSR/DSR latency - Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we first give an overview of MAC functions that RAN2 can start to discuss without dependency on the RAN1's progress, and then focus discussions on the latency reduction for scheduling and retransmission based on agreements 6 and 7 above, as well as the uplink scheduling information.

2	Discussion

In NR, MAC functions as shown in Table 1 play a key role in data transmission, particularly scheduling (SR/BSR/DSR, LCP) and retransmission mechanisms. UE first needs to initiate UL resource request via SR/BSR/DSR to obtain DG. After obtaining the UL resource, UE constructs a MAC PDU by LCP and multiplexing MAC CEs and MAC SDUs. And HARQ ensures reliable transmission of the MAC PDU. While some MAC functions are strongly related to RAN1, and discussions in RAN2 need to rely on the RAN1’s progress, e.g., PHR, or BFR/BFD. To avoid meaningless discussions in RAN2 before certain conclusions are reached in RAN1, in this paper, we focus on the scheduling aspects of the MAC functionalities that do not depend on the RAN1’s progress, e.g., SR/BSR/DSR, HARQ, LCP.

Table1, MAC functions

MAC function

SR/BSR/DSR

HARQ

LCP

Multiplexing/Demultiplexing

DRX

PHR

BFR/BFD

Mapping between logical channels and transport channels

2.1	Dynamic scheduling

During the last R2 meeting, we have agreed that 6G shall support dynamic scheduling.

- Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant.

In this section, we discuss further details of dynamic grant including the scheduling information for helping the RAN to perform dynamic scheduling and how to send such scheduling information.

2.1.1	Scheduling information

In 5G, scheduling information reporting including Buffer Status Report (BSR) and Delay Status Report (DSR) is supported.

- BSR was introduced in R15 for UE to provide with buffer status information to gNB when new data arrives or data with higher priority arrives, based on which the gNB can schedule enough UL resources for UE.

- DSR was introduced in R18 for XR. UE can use DSR to provide with buffer status and remaining time information to gNB when there is delay-critical data. And enhanced DSR was introduced in R19 for UE to provide gNB with buffer status and remaining time information of both delay-critical data and non-delay critical data when there is delay-critical data.

In 6G, immersive communications have been agreed as the main traffic as the extension of the eMBB service in 5G. With the RAN1 agreement in the last meeting that the R18 XR traffic model as the starting point of IC services in 6G, reporting of both remaining time and buffer status will continue to be important for 6G.

However, it should be noted that the buffer status reported in the legacy DSR and BSR are different: In the BSR buffer status include the buffer status of a certain logical channel group; In the single-entry DSR, buffer status only includes the buffer status of the delay-critical data for a certain LCG; while in the multiple-entry DSR, buffer status includes the buffer status of the delay-critical data and non-delay critical data for a certain LCG.

Observation 1-1:	There is difference between the buffer status reporting in 5G BSR, and single/multiple-entry DSR.

In the contributions of the last meeting, companies have mentioned about the unified BSR and DSR reporting in the MAC layer. We are open to discuss whether we can seek for a more concise solution in the spec writing. But the above observation should be considered that there is indeed difference between the buffer status report in BSR and DSR. We also think that the functionalities should be discussed first and how the spec is written could be discussed during stage3 discussion.

Proposal 1:	The scheduling information the UE can report to the RAN for dynamic grant include buffer status and remaining time.

2.1.2	How to send the scheduling information

In the legacy 5G, when there is no UL-SCH resource for BSR MAC CE or DSR MAC CE, SR can be triggered for UL resources. With this basic UL resources request procedure via SR and BSR/DSR, UE can request dynamic grant (DG) from gNB. Considering that the last RAN2 meeting has agreed to support DG in 6G, it is naturally that the UL resource request for dynamic scheduling via SR+BSR/DSR procedure shall be kept in 6G.

Proposal 2:	When there is no UL-SCH resource for the UE to send the scheduling information to RAN, UE sends scheduling request to the RAN for UL-SCH resource for sending such scheduling information.

In NR, since UE only initiates the UL resource request to obtain the DG after Layer 2 of UE obtains UL data from upper layer at AS layer, the acquisition delay of DG is relatively large. In 6G, it is necessary to study scheduling enhancements to address the SR/BSR/DSR latency.

The application and the modem reside in the same device. Hence, it should be possible for the AS layer of the UE to obtain the predicted traffic arrival time in the uplink. Considering that UE can predict the arrival time of uplink data, UE can send an uplink resource request in advance before the uplink data arrives, where the uplink resource request includes information about the arrival time of the uplink data. By using this method, the network can allocate DG to the UE according to the arrival time of the uplink data, thereby reducing the delay of dynamic scheduling.

Proposal 3:	RAN2 to study the method to reduce latency of UL resource request for the dynamic scheduling, e.g., UE predicts the traffic information for upcoming UL data and send early scheduling request.

2.2	Configured grant

Configured grant type 1 has been introduced in R15 NR for uplink data transmission. Different from configured grant type 2, which requires DCI for CG activation/deactivation, CG type 1 takes effect immediately after the configuration is passed to the UE.

During the previous discussions, solutions like “contention-based configured grant” have been discussed for which the NW could configure the same resource to the different UEs and different UE competes with each other for transmission on this resource.

Observation 4-1:	In the legacy R15 NR, it is already possible to configure the same time and frequency resource for configured grant to different UEs such the same piece of CG resource is shared among multiple UEs.

The existing UL resources pre-allocation in NR configured-grant faces challenges in assigning matched resources to UEs with varying channel conditions and traffic demands from the service point of view. Over-allocation may lead to UL data transmission collisions and re-transmissions as the number of UEs increases; while under-allocation can result in poor QoS. Under both cases, energy and resources would be wasted at the UE and network sides.

Thus, to fully harvest the technical benefit of configured grant, flexible configured grant resource allocation mechanism should be studied. For example, UE should have the opportunity to feedback assistant information to BS so that cost for handling collision and imperfect CSI can be minimized. In the end, this could help hold the scalable service capability as the number of UEs increases.

Proposal 4:	Flexible configured grant resource allocation mechanism should be studied to better match the pre-allocated resources with UL service requirement.

2.3	Tight coordination between HARQ and ARQ

In NR, HARQ retransmissions and ARQ retransmissions are specified under the MAC and RLC layers, respectively, and these two mechanisms are decoupled. ARQ retransmission is employed to handle the remaining errors after HARQ retransmission. To prevent simultaneous HARQ retransmissions and ARQ retransmissions for a single RLC SDU, the triggering of RLC status reports (e.g., triggered by the expiration of t-Reassembly) occurs after the completion of HARQ retransmissions. The Tx RLC then performs ARQ retransmissions based on the RLC status report. This means that ARQ retransmissions only occur after the expiration of t-Reassembly, which could be significantly after HARQ retransmissions have been completed. Hence, this could potentially lead to a large delay in ARQ retransmissions. For this issue, last RAN2 meeting has agreed to study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.

In the following, we firstly analyze the cases where ARQ retransmission is used to ensure reliability and the corresponding reasons why HARQ retransmission cannot guarantee reliability in 5G.

2.3.1 Downlink

For downlink transmission, there are three cases where MAC PDUs are lost after HARQ retransmissions, requiring ARQ retransmission to ensure reliability, including:

- Case1: Since HARQ feedback is not 100% reliable, network may mistakenly detect NACK or DTX as an ACK and stop scheduling HARQ retransmission, leading to the loss of MAC PDUs.

- Case2: Network may actively stop scheduling HARQ retransmission after a certain number of HARQ retransmissions in some cases. For example, in cases of radio channel degradation, HARQ retransmissions necessitate lowering the MCS order. To maintain the same TB size, more time and frequency resources are then required—which may not be available.

- Case3: Since NDI field in DCI is only 1 bit, if UE misses detecting the DCI, UE may mistakenly interpret newly received data after the missed DCI as a retransmission of a previously received data before the missed DCI, as shown in Figure 1. In this case, if the previously received data before the missed DCI has been successfully decoded before (i.e., MAC PDU 1 in Figure 1), UE will return an ACK without decoding the newly received data (i.e., MAC PDU 3 in Figure 1), leading to the loss of MAC PDU 3.

Figure1, Case of MAC PDU loss due to DCI missing detection

Observation 5-1:	L2 ARQ retransmission is needed for the following cases in the downlink to handle the residue error from HARQ:

  - Case1: HARQ-ACK mis-detection

  - Case2: Network proactively stops HARQ retransmission

  - Case3: DCI miss detection

2.3.2 Uplink

For uplink transmission, there are two cases where MAC PDUs are lost after HARQ retransmissions, requiring ARQ retransmission to ensure reliability, including:

- Case(a): Network may actively stop scheduling HARQ retransmission after a certain number of HARQ retransmissions in some cases. For example, in cases of radio channel degradation, HARQ retransmissions necessitate lowering the MCS order. To maintain the same TB size, more time and frequency resources are then required—which may not be available.

- Case(b): Since network cannot distinguish between data reception failure and UL skipping when UL skipping is enabled, if UE sends UL data but network fails to detect it, network may erroneously attribute this to UL skipping and therefore does not schedule HARQ retransmission, leading to the loss of MAC PDUs.

Observation 5-2:	L2 ARQ retransmission is needed for the following cases in the uplink to handle the residue error from HARQ:

  - Case(a): Network proactively stops HARQ retransmission

  - Case(b): UE-side uplink skipping

We summarized the above analysis by the Table 2 below.

| DL/UL transmission | Cases where ARQ retransmission is used to ensure reliability in 5G (or Cases of the loss of MAC PDU after HARQ retransmission is completed in 5G) | Reasons why HARQ retransmission cannot guarantee reliability in 5G |
| --- | --- | --- |
| DL | Case1: HARQ feedback misdetection: network mistakenly detects NACK or DTX as an ACK and stops scheduling HARQ retransmission. - DTX: UE does not send HARQ feedback because UE has not received DCI for the DL transmission (i.e., DCI missing detection), or the HARQ feedback corresponding to the DL transmission is of low priority. | HARQ feedback is not 100% reliable. |
|  | Case2: network actively stops HARQ retransmission: After a certain number of HARQ retransmissions, network stops scheduling HARQ retransmission. | Radio channel degradation |
|  | Case3: DCI missing detection: if UE misses detecting the DCI, UE mistakenly interprets newly received data after the missed DCI as a retransmission of a previously received data before the missed DCI and UE returns an ACK without decoding the newly received data. | NDI field in DCI is only 1 bit. |
| UL | Case(a): same as Case2 | Radio channel degradation |
|  | Case(b): scenario for UL skipping: if UE sends UL data but network fails to detect it, network may erroneously attribute this to UL skipping and therefore does not schedule HARQ retransmission, leading to the loss of MAC PDUs. | Network cannot distinguish between data reception failure and UL skipping. |

Table2, Cases where ARQ retransmission is used to ensure reliability in 5G

2.3.3 How to solve the problem

In NR, for the above cases, since RX cannot accurately determine whether HARQ retransmission of the MAC PDU corresponding to an RLC SDU has been completed, it can only blindly wait for the t-Reassembly timer to expire to confirm the end of HARQ retransmission. Moreover, HARQ retransmission of the MAC PDU corresponding to the RLC SDU might have already ended before the t-Reassembly timer expires, leading to a large delay in ARQ retransmission of the RLC SDU. If ARQ retransmission of an RLC SDU could start immediately after the end of HARQ retransmission of the MAC PDU containing the RLC SDU, this delay could be reduced.

Observation 5-3:	If ARQ retransmission of an RLC SDU could start immediately after HARQ retransmission of the MAC PDU containing the RLC SDU is completed, the delay of ARQ retransmission of the RLC SDU can be reduced.

Therefore, a straightforward approach is to identify the loss of an RLC SDU and trigger its retransmission by identifying the loss of MAC PDU corresponding to the RLC SDU after HARQ retransmission is completed and before the t-Reassembly timer expires for the above cases. In this way, the ARQ retransmission of the RLC SDU could be triggered earlier.

Proposal 5:	To reduce the delay of ARQ retransmission, RAN2 to study how to identify the transmission or reception failure of MAC PDU under various cases to timely trigger RLC retransmissions.

3	Conclusion

We summarize the observations and proposals in this contribution as follows:

Dynamic scheduling:

Observation 1-1:	There is difference between the buffer status reporting in 5G BSR, and single/multiple-entry DSR.

Proposal 1:	The scheduling information the UE can report to the RAN for dynamic grant include buffer status and remaining time.

Proposal 2:	When there is no UL-SCH resource for the UE to send the scheduling information to RAN, UE sends scheduling request to the RAN for UL-SCH resource for sending such scheduling information.

Proposal 3:	RAN2 to study the method to reduce latency of UL resource request for the dynamic scheduling, e.g., UE predicts the traffic information for upcoming UL data and send early scheduling request.

Configured grant:

Observation 4-1:	In the legacy R15 NR, it is already possible to configure the same time and frequency resource for configured grant to different UEs such the same piece of CG resource is shared among multiple UEs.

Proposal 4:	Flexible configured grant resource allocation mechanism should be studied to better match the pre-allocated resources with UL service requirement.

Tight coordination between HARQ and ARQ:

Observation 5-1:	L2 ARQ retransmission is needed for the following cases in the downlink to handle the residue error from HARQ:

  - Case1: HARQ-ACK mis-detection

  - Case2: Network proactively stops HARQ retransmission

  - Case3: DCI miss detection

Observation 5-2:	L2 ARQ retransmission is needed for the following cases in the uplink to handle the residue error from HARQ:

  - Case(a): Network proactively stops HARQ retransmission

  - Case(b): UE-side uplink skipping

Observation 5-3:	If ARQ retransmission of an RLC SDU could start immediately after HARQ retransmission of the MAC PDU corresponding to the RLC SDU is completed, the delay of ARQ retransmission of the RLC SDU can be reduced.

Proposal 5:	To reduce the delay of ARQ retransmission, RAN2 to study how to identify the transmission or reception failure of MAC PDU under various cases to timely trigger RLC retransmissions.

4	References

[1]. R2#131bis Chair Note.