3GPP TSG-RAN WG2 Meeting #132	R2-2508212

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	9.3.2

Source:	OPPO

Title:	Discussion on functionality management for RRM measurement prediction

Document for:	Discussion, Decision

Introduction

In RAN#109 the WID of AI mobility is further updated in [1]. The working scope is limited to intra-frequency domain case A and case B, co-located inter-frequency prediction for L3 cell level measurement prediction i.e. without L3 beam level measurement prediction. And the WID [1] also clarified that spatial domain prediction is out of the scope.

The paper discusses the inference centric issues, namely applicability procedure, inference configuration, inference output reporting in order. The other aspects e.g. (de)activation, performance monitoring and data collection will be further discussed in future.

General issue

In simulation phase, choice of reference signal i.e. SSB or CSI-RS is not critical since data set is anyway generalized based on channel model. So, in general the conclusion captured in TR 38.744 can be applied for either SSB or CSI-RS. While in normative work phase RAN2 should decide which ones to be specified. Our view is that there is no essential difference from RAN2 perspective. RAN4 started discussing and planning simulation to facilitate discussion on performance requirement from last meeting. Technically simulation based on SSB can’t be applied to CSI-RS automatically. So, we can take SSB as starting point for discussion in RAN2. In later phase, RAN2 can check whether additional work for CSI-RS can fit into R20 work plan for both RAN2 and RAN4.

Proposal 1: SSB is taken as starting point as reference signal type for normative work in RAN2.

Another issue is which measurement quantity should be covered in normative work phase. During simulation phase, RSRP is the only one assessed for either L1 level or L3 level measurement result. Although RAN2 discussed how to model radio interference for RLF prediction use case, RSRQ and SINR are not touched at all. So, it is not crystal clear whether the conclusion in current TR 38.744 [2] is applicable for RSRQ/SINR or not. Note in study phase, RAN2 agreed that “RSRP is the baseline measurement quantity”.

Proposal 2: Only RSRP is specified in R20 as measurement quantity i.e. RSRQ and SINR are not specified in R20

Measurement event A1~A6 are covered in the TR 38.744 [2]. That measurement events are for SPCell change procedure, which covers both PCell change and PSCell change. It implies NR-DC is possible network architecture. Currently MN and SN can configure measurement task separately by coordination of UE measurement capability between them. For AI Mobility the solution is kind of AIML aided measurement configuration. It means the coordination on frequency carrier is already in the place. The additional coordination on inference parameters could be part of applicability procedure. Hence, we think UE can take the coordination issue into account when it judges applicability for the inference configuration from MCG and/or SCG, which is left to UE’s implementation. Thus, no additional coordination between MN and SN is needed.

Proposal 3: For AI mobility, SA (and CA) and NR-DC are supported network architecture

Proposal 4: For NR-DC, MN and SN can configure inference configuration independently based on legacy coordination on radio capability on measurement. And it is up to UE’s implementation to coordinate the inference configurations from MN and/or SN.

The whole work item should not touch any spec related to other RAT e.g. LTE. So, the discussion on MRDC involving LTE is out of scope of this WID.

Proposal 5: The network architecture i.e. (NE)EN-DC and NE-DC are not in the scope in R20.

Applicability procedure

In this paper some conclusions from Release 19 work item NR_AIML_air-Core (called AI PHY in the rest of paper) are cited for discussion. There is discrepancy on terms between two work items. Here is the relationship between them for readability:

| AI PHY | AI Mobility |
| --- | --- |
| Option A (full inference configuration) | Full inference configuration |
| Option B (inference related parameters) | Partial inference configuration |

Table 3.1

For applicability procedure, RAN2 conclusion in SI phase are captured as following in the TR[2]:

UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 in a RRCReconfiguration message. Upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF, UE follows existing behaviour defined in [2] on whether to release or keep an inference configuration.

Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message. If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it . When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported. For an inapplicable full inference configuration, UE continues to perform the inference. It is up to network implementation what to do with reported result after UE indicates the corresponding full inference configuration is inapplicable.

If a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration.

Applicability can be updated via UAI. A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not. When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.

No prohibit timer needs to be introduced for prohibiting applicability reporting.

Table 3-2

We think the text (except for yellow part) captured in Table 3-2 from TR [2] are already stage 2 level description and can be confirmed quickly.

Proposal 6: The following text in Table 3-2 from TR [2] are agreed:

6.1, UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 of TR [2] in a RRCReconfiguration message

6.2, Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

6.3, If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported.

6.4, Applicability can be updated via UAI.

6.5, No prohibit timer needs to be introduced for prohibiting applicability reporting.

The Text “When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.” Suggests that UAI is the only choice for such case. While there is conclusion on open issue RRC-44[4] from AI PHY in RAN2#131:

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

Table 3-3

It means when the applicability status of an inference configuration is changed, its applicability information can be transmitted via either UAI or RRCReconfigurationComplete message. UE report such status change via RRCReconfigurationComplete message when:

1, UE has not transferred the updated status via UAI message

2, UE receives a RRCReconfiguration message

gNB has no idea when status of an inference configuration is changed since it is within UE side. So, these two conditions are not relevant with each other. In most case UE will update the applicability status via UAI. UE will only transfer updated status via RRCReconfigurationComplete message when UE hasn’t updated via UAI while receiving a RRCReconfiguration message, which is really a corner case. Since RAN2 already agreed that “When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.” Same solution should be also applied for the case that an inference configuration becomes applicable.

Proposal 7: When applicability status is changed from applicable/inapplicable to inapplicable/applicable, UE shall report the updated applicability status via UAI message only.

RAN2 need discuss this text” Upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF, UE follows existing behaviour defined in [2] on whether to release or keep an inference configuration.” The agreements come from AI PHY. But in AI PHY inference parameters are physical layer parameters while for AI Mobility inference parameter is purely RRC layer parameters. And looking into RRC re-establishment procedure and RRC resume procedure, the treatment on physical layer parameters and purely RRC layer parameters could be different.

In current spec for measurement configuration, upon RLF (section 5.3.7.3), transit to INACTIVE (section 5.3.8.3) and IDLE state (5.3.11), only parameters related to reportType equaling condTriggerConfig is removed from measConfig while rest measurement are maintained.

Figure 3-1 legacy UE behaviour

RAN2 agreed that “Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report.” So, inference configuration can be taken roughly as measurement configuration, which has nothing to do with conditional handover at all because the WID covers only legacy L3 handover procedure. So, our understanding of the agreed yellow text is that UE should keep configured inference configurations upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF without any spec impact.

Proposal 8: Upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF, UE keep inference configuration (full and/or partial inference configuration) without spec impact.

In RAN2#131 meeting, for AI PHY RAN2 agreed that applicability report is not supported for RRCRestablishmentComplete message when discussing open issue RRC-42 [4]. Upon RLF, UE will keep essential configuration and relies on RRC reconfiguration after successful RRC re-establishment procedure. So, supporting applicability report for RRCRestablishmentComplete message means more efforts for both standardization and implementation. This reason also standards for AI mobility use case.

Proposal 9: Applicability reporting is not supported for RRCRestablishmentComplete message

In RAN2#131 meeting, RAN2 agreed that applicability report is supported for RRCResumeComplete message. When UE transit to INACTIVE state, UE intends to store majority of the AS context for a quick RRC resume procedure. That’s why to store inference configuration doesn’t add more implementation efforts. Of course, stage 3 level detail needs be specified for AI mobility use case.

Proposal 10: Applicability reporting is supported for RRCResumeComplete message

Proposal 11: Full or partial inference configuration is stored and restored in the Inactive AS context

During handover there is an issue whether gNB is allowed to configure inference configuration for UE to report applicability. AI PHY has an agreement in RAN2#128 meeting as following:

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Table 3-4

This agreement can be also applied for AI mobility in general. In the latest stage 2 CR [3], it is clarified that “UAI can be sent from the source gNB to the target gNB to exchange applicability reporting referring to the configurations from the source gNB”. In RAN2#131bis meeting further conclusion is made as following for AI PHY:

Agreements

[B206][O300] A new IE in HandoverPreparationInformation message to convey complete applicability information.

Table 3-5

If the same agreement is applied for AI Mobility, it means target gNB has a full set of applicability status. From UE perspective, UE need report applicability status for inference configurations within RRCReconfigurationComplete message. In case applicability status is changed immediately before handover and UE may not be able to transmit via UAI message and this inference configuration is not contained with handover command, UE should report via UAI after handover is completed.

Proposal 12: A new IE in HandoverPreparationInformation message is introduced to convey complete applicability information from source gNB to target gNB.

Proposal 13: During handover, the UE may receive an inference configuration related to the target gNB via RRCReconfiguration message. Then UE reports its applicability status to the target gNB via RRCReconfigurationComplete message.

Proposal 14: If applicability status of an inference configuration is changed immediately before handover procedure without reporting via UAI in time, UE should report the updated status via UAI after handover is completed.

The yellow part text needs more discussion. The text “A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not”. This agreement inherits the conclusion from AI PHY before. However, in RAN2#131bis meeting, it is reverted as RAN2 agreed:

[E042] Applicability reporting via UAI is always enabled for option A and B.

Remove flag from RRC CR and update procedural text to indicate that it is configured if option A or B are configured.  Wording offline.

Table 3-6

It means the flag in otherConfig is removed to indicate whether applicability reporting via UAI is enabled or not. For full inference configuration, when a status of inference configuration is changed, UE can’t indicate to network in time without UAI message. If the status is changed from applicable to inapplicable and if the status is not reported in time, then UE may run the model even it performs poorly since UE is not allowed to fallback

Proposal 15: The agreement i.e. “A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not” is reverted i.e. it is not specified.

For partial inference configuration gNB is expected to configure full inference configuration after receiving applicability of partial inference configuration. when the key parameter within partial inference configuration is limited, current applicability procedure can work well. But when such key parameter e.g. PW length for temporal domain case A could have a wide range, in such case the applicability procedure may not be efficient assuming gNB check PW length one by one per procedure. To avoid such blind handshake, there could be multiple directions to improve the efficiency of applicability procedure:

Direction 1: PW length range or values is provided as UE’s capability, then gNB can check detail values through applicability reporting procedure within reported range or values

Direction 2: gNB can check expected PW length value through applicability reporting procedure and UE is allowed to report applicable PW length(s) when it reports applicability status of configured PW length regardless the status is applicable or not.

We think both directions are feasible and comparable. In addition, they are not exclusive i.e. they can be implemented together to further improve efficiency.

Proposal 16: RAN2 is kindly requested to discuss how to effectively check applicability of inference configuration

Inference configuration

For inference parameters, TR [2] already captured following agreement:

Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly.

Following inference parameters can be configured to UE for inference and assessing applicability:

- PW length for intra-frequency temporal domain case A

- Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction

- Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement).

- Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available)

- Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction

- Optional associated ID

Table 4-1

For associated ID, RAN2 agreed to have it not because it is necessary but because it may be not unnecessary. Even generalization study shows that there is no serious issue. But proponents say simulation is not exhaustive and some network configurations e.g. beam pattern could matter. Because RRM sub case 1 or 3 can be applied for all scenarios to be specified, it means associated ID can be applicable for all scenarios in scope. Still because of its uncertainty, this parameter should be optional for all scenarios in scope.

Proposal 17: Associated ID can be configured as optional inference parameters for temporal domain case A/B and inter-frequency prediction

About associated ID, a bit controversial issue is the valid scope and uniquity scope. In AI PHY, this issue is settled down when discussing open issue RRC-46 [4] and here is the conclusion in RAN2#131:

1	Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

2	Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

3	We will not define areas.  The Associated ID is 24 bits.

Table 4-1

We understand the meaning of the associated ID is unique within PLMN after it is assigned to UE and it can’t be changed anymore. Considering it is 24bits long, it should be fine. In RAN2#131bis meeting, it is agreed for AI PHY as following:

	It is left to NW implementation to maintain validity of associated ID in Rel-19, e.g. NW updates or de-configures the associated ID when it is out of date.

Table 4-2

For AI mobility, same solution can be applied.

Proposal 18: Associated Id is unique within a PLMN

Proposal 19: The length of Associated Id is 24 bits. Once it is assigned to UE, the meaning is not changed i.e. recycle is not necessary

And it could be up to network’s implementation to decide for one specific associated ID whether it can be associated with one single cell or multiple cells. For AI mobility use case, even e.g. beam pattern matters, we believe same model can be at least applicable for cells within same frequency layer i.e. associated ID should be associated with multiple cells. On the other hand, single cell is one case of multiple cells i.e. we can leave it to network’s configuration. The valid scope of associated ID should be aligned between data collection and at least inference operation. For example, if a model is trained based on data from N cells, then technically this model can only work in those N cells. When UE moves out those N cells, network should stop data collection and/or release the relevant inference configurations. Since so far model for AI mobility only works in RRC-CONNECTED so cell level tracking is feasible based on mobility scheme itself.

Proposal 20: Associated Id can be associated with single or multiple cells. And how to do it is up to network’s implementation.

For temporal domain case A, apart from PW length it is agreed “Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available)”. In current RRC specification IE “allowedCellsToAddModList” become a whitelist when “useAllowedCellList” in reportConfigNR is set to be true. So basically, these two parameters can be reused for report predicted measurement results of temporal domain case A. Actually, same parameter can be also used for temporal domain case B.

Proposal 21: legacy parameter “allowedCellsToAddModList” can be reused as optional cell list for temporal domain case A and case B

For temporal domain case B there is a new parameter i.e. “Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement)”. Whether current SMTC configuration within MONR (measObjectNR) can be reused or not depends on the implementation of temporal domain case B:

- Case B-1: To purely save UE’s measurement efforts without prolonging SSB periodicity on purpose. Note this implementation can accommodate both legacy UE and new UE.

- Case B-2: To save SSB transmission and hence network energy. In this case the SSB periodicity is prolonged, and UE need measure transmitted SSB. Since UE is capable to predict measurement result in between transmitted SSB, UE can have a shorter measurement period. Note this implementation can cause problem for legacy UE.

Let’s take example 2 of temporal domain case B (Figure 5.2.1.1-3 in TR 38.744) as example.

Figure 4-1

The SMTC period associated with both actual measurement instance (yellow) and skipped measurement instances is called e-SMTC (equivalent SMTC period) in this paper. For case B-1 the e-SMTC is the same as configured SMTC period regardless how does it skip the measurement. For case B-2, the configured SMTC refers to actual measurement instances (yellow) i.e. the skipped measurement instances should be called predicted measurement instances without skipping. Then the e-SMTC is not equal to configured SMTC anymore. Plus, UE with difference capability could have different predicted measurement instances and hence different e-SMTC.

Observation 1: For UE with different capability, the e-SMTC is different for case B-2. And because of this, network and UE have ambiguity about the e-SMTC.

The alignment of e-SMTC matters because it closely related to measurement delay, the core requirement of RRM measurement. There could be 2 options to resolve this problem:

Option 1: Only specify implementation case B-1 in R20 i.e. reuse existing SMTC parameter

Option 2: To figure out solution to deal with misalignment between UE and network

To us option 1 is the right way to go considering implementation case B-2 has backwards compatibility issue and more spec impact. In addition, implementation B-2 also cause complexity in RAN4.

Proposal 22: In R20 only solution for implementation case B-1 will be specified and current SMTC parameter can be reused.

For inter-frequency prediction, the simple way to indicate frequency carrier (measured and predicted) is to reuse measObjectNR.

Proposal 23: legacy measObjectNR is reused to indicate measured and predicted frequency carrier

Inference reporting

For RRM measurement prediction, the agreements are captured as following in current TP:

UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message:

- For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported;

- For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported;

- For inter-frequency prediction, the latest predicted measurement result is reported.

Table 5-1

For temporal domain case A, it is possible that UE can report one or more than one predicted measurement results in PW. RAN2 agreed that PW length is configured by gNB. But gNB has no idea how much instances measurement results within PW could be because sampling period supposes to be UE’s implementation. Nevertheless, gNB does know the potential range of sampling period, which is defined as SMTC period in current spec. For example, if SMTC period is 40ms and PW length is 200ms, then the possible maximum instances is 200/40=5. Furthermore, how much instances are needed is also up to gNB’s implementation. So, what gNB can configure is a maximum number of instances of predicted measurement results UE can report in measurementReport message for one cell.

Proposal 24: For temporal domain case A, gNB can configure maximum number of instances of predicted L3 cell level measurement results UE can report per cell in one measurementReport message.

When UE receive such configuration and get ready to report predicted measurement results, one issue is whether any timing information is needed for predicted measurement results. Our view is that it is not necessary. The inference parameter PW is negotiated between UE and gNB via applicability reporting procedure. When a PW length is applicable it basically means UE will run the model with configured PW. And the reported predicted measurement results should be full of PW, but not just part of the PW. So, from the number of reported measurement results within PW, gNB can easily figure out the timing information.

Proposal 25: For temporal domain case A, apart from predicted measurement results in PW, no timing information is needed per measurement result.

For temporal domain case B, one question is whether an indication is needed to show whether it is actual measurement results or predicted measurement results. For sub-case 1 and sub-case 3 some of the L1 beam level measurements are skipped and L3 filtering is kind of iteration operation, so obviously there is no any actual measurement result is possible. For sub-case 3, the same issue for input L3 cell level measurement result. That’s why what UE can report is always predicted measurement result and hence not any indication is needed to differentiate between actual or predicted measurement result.

For inter-frequency prediction, it is obvious all the reported measurement result is predicted.

Proposal 26: For temporal domain case B, reported measurement result should be always predicted measurement result.

Proposal 27: For inter-frequency prediction and predicted frequency carrier, reported measurement result should be always predicted measurement result.

Conclusion

Here are proposals from this paper:

General aspects:

Proposal 1: SSB is taken as starting point as reference signal type for normative work in RAN2.

Proposal 2: Only RSRP is specified in R20 as measurement quantity i.e. RSRQ and SINR are not specified in R20

Proposal 3: For AI mobility, SA (and CA) and NR-DC are supported network architecture

Proposal 4: For NR-DC, MN and SN can configure inference configuration independently based on legacy coordination on radio capability on measurement. And it is up to UE’s implementation to coordinate the inference configurations from MN and/or SN.

Proposal 5: The network architecture i.e. (NE)EN-DC and NE-DC are not in the scope in R20.

Proposals about applicability procedure:

Proposal 6: The following text in Table 3-2 from TR [2] are agreed:

6.1, UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 of TR [2] in a RRCReconfiguration message

6.2, Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

6.3, If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported.

6.4, Applicability can be updated via UAI.

6.5, No prohibit timer needs to be introduced for prohibiting applicability reporting.

Proposal 7: When applicability status is changed from applicable/inapplicable to inapplicable/applicable, UE shall report the updated applicability status via UAI message only.

Proposal 8: Upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF, UE keep inference configuration (full and/or partial inference configuration) without spec impact.

Proposal 9: Applicability reporting is not supported for RRCRestablishmentComplete message

Proposal 10: Applicability reporting is supported for RRCResumeComplete message

Proposal 11: Full or partial inference configuration is stored and restored in the Inactive AS context

Proposal 12: A new IE in HandoverPreparationInformation message is introduced to convey complete applicability information from source gNB to target gNB.

Proposal 13: During handover, the UE may receive an inference configuration related to the target gNB via RRCReconfiguration message. Then UE reports its applicability status to the target gNB via RRCReconfigurationComplete message.

Proposal 14: If applicability status of an inference configuration is changed immediately before handover procedure without reporting via UAI in time, UE should report the updated status via UAI after handover is completed.

Proposal 15: The agreement i.e. “A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not” is reverted i.e. it is not specified.

Proposal 16: RAN2 is kindly requested to discuss how to effectively check applicability of inference configuration

Proposals about Inference configuration:

Proposal 17: Associated ID can be configured as optional inference parameters for temporal domain case A/B and inter-frequency prediction

Proposal 18: Associated Id is unique within a PLMN

Proposal 19: The length of Associated Id is 24 bits. Once it is assigned to UE, the meaning is not changed i.e. recycle is not necessary

Proposal 20: Associated Id can be associated with single or multiple cells. And how to do it is up to network’s implementation.

Proposal 21: legacy parameter “allowedCellsToAddModList” can be reused as optional cell list for temporal domain case A and case B

Observation 1: For UE with different capability, the e-SMTC is different for case B-2. And because of this, network and UE have ambiguity about the e-SMTC.

Proposal 22: In R20 only solution for implementation case B-1 will be specified and current SMTC parameter can be reused.

Proposal 23: legacy measObjectNR is reused to indicate measured and predicted frequency carrier

Proposals about inference reporting:

Proposal 24: For temporal domain case A, gNB can configure maximum number of instances of predicted L3 cell level measurement results UE can report per cell in one measurementReport message.

Proposal 25: For temporal domain case A, apart from predicted measurement results in PW, no timing information is needed per measurement result.

Proposal 26: For temporal domain case B, reported measurement result should be always predicted measurement result.

Proposal 27: For inter-frequency prediction and predicted frequency carrier, reported measurement result should be always predicted measurement result.

Reference

[1] RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2] TR 38.744 V19.0.0

[3] R2-2507696, Correction on AI for NR Air interface feature in TS38.300

[4] R2-2505778, RRC open issues for AIML for NR air interface