3GPP TSG-RAN WG2#132	R2-2508268

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.1.1

Title:	Functional requirements on 6GR User plane

Document for:	Discussion and decision

Introduction

In RAN2#131bis meeting, for 6GR UP, some agreements were made as follows [1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we would like to provide our views on the required functionalities of 6GR User Plane (UP).

Discussion

General requirements

Most of the services which were supported in 5G NR are expected to also be supported in 6GR to avoid the negative impression from the market that 6GR underperforms 5G NR. This includes e.g. XR, FWA, URLLC, Industrial IoT, LPWA etc. Unified design is expected to be applied for the support of variable existing services/needs which require the same feature. Unified design is also expected to be able to provide better forward compatibility and facilitate the further development of 6G when new services appear particularly in industries in the near future. In the study phase of 6GR, RAN2 should consider and study the possibility to support 6GR requirements of new and existing services by using one design with a good scalability.

Since there are many new and existing services with different requirements, in design of user plane architecture and protocols, it is better to have a compact protocol stack but with flexible features/functions/toolbox adapted by the service needs. In NR, the UP protocol for supporting vertical services, like TSN (Time sensitive network), SDT (small data transmission) or Ethernet header compression (EHC) was not supported in Rel-15, therefore it has implicitly caused market fragmentation.

Therefore, it is important for the day-1 design to avoid constraints when evolving technologies in the future.

Proposal 1:	6GR UP protocol should aim to support flexible functions adapted by service needs.

Taking into account the IMT-2030 technical performance requirements (TPRs) in [2], the theoretical data rates required for 6GR would be more than 50Gbps which is much higher than NR (in NR, 20Gbps for DL and 10Gbps for UL), to achieve the requirements, it obviously requires more efficient data delivery mechanism among UP sublayers for both the NW and the UE sides. In addition to the higher data rates, latency requirement should be kept as in NR (4ms for eMBB and 1ms for URLLC) to support low latency services. Given the situation, RAN2 should study 6GR UP protocol to achieve further less processing time than NR as a design target.

Proposal 2:	6GR UP protocol should aim to enable further shorter processing time than NR UP.

Also to reduce the processing time, 6GR UP protocol should target a lean protocol design. We think it would be a good starting point by reducing and/or merging duplicated functions in the existing NR UP sub-layers.

Proposal 3:	6GR UP protocol should aim to lean design than NR UP.

Baseline functions

In NR, Layer-2 includes MAC, RLC, PDCP and SDAP sublayers. According to TS 38.300 [3], the functions for the L2 protocol includes following:

- MAC

  - Handling of DL assignment and uplink grant;

  - Multiplexing/demultiplexing;

  - Scheduling information reporting, e.g., SR, BSR and PHR;

  - HARQ;

  - Logical channel prioritisation (LCP);

  - Random access;

  - BWP related operations;

  - DRX.

- RLC

  - Transfer of upper layer PDUs;

  - Sequence numbering independent of the one in PDCP (UM and AM);

  - ARQ (AM only);

  - Segmentation (AM and UM)/re-segmentation (AM only) of RLC SDUs and Reassembly of SDU (AM and UM);

  - RLC SDU discard (AM and UM);

- PDCP

  - Transfer of data;

  - Maintenance of PDCP SNs;

  - Header compression and decompression;

  - Ciphering and deciphering, Integrity protection and integrity verification;

  - Duplication;

  - Reordering and in-order delivery;

- SDAP

  - Mapping between a QoS flow and a data radio bearer;

  - Marking QoS flow ID (QFI) in both DL and UL packets.

Many of these NR L2 functions are inherited from LTE L2 functions. In our view, the functions of NR L2 protocols can be taken as the baseline of the required functions for 6GR UP.

Proposal 4: The functions for 6GR UP include the following as a baseline:

  - Handling of DL assignment and uplink grant, multiplexing/demultiplexing, scheduling information reporting e.g., SR, BSR and PHR, HARQ, Logical channel prioritisation (LCP), random access, BWP or carrier related operations and DRX.

  - Data transfer, sequence numbering, ARQ, segmentation and/or concatenation, SDU discard, header compression and decompression, ciphering and deciphering, re-ordering, mapping between a QoS flow and a data radio bearer.

Potential enhancements

The SDAP layer is a newly added layer by NR due to the finer user plane data processing based on QoS flows introduced by 5G core network. The main purpose of the SDAP layer is for QoS flow to DRB mapping. This function is quite simple and is most likely to be kept in 6G. If 6G core network will introduce new QoS framework which needs to be reflected in the AS layer protocol design, RAN2 can revisit the needs for enhancements on the SDAP layer. This will be triggered by the study of other WGs, such as SA2. Further, if it is determined that AI/ML or sensing related data transfer will be carried by user plane, then the SDAP layer may need to be enhanced as well.

Proposal 5:	Keep the current functions in the SDAP layer as baseline. RAN2 may consider enhancements of SDAP if triggered by other WGs (e.g., SA2), or by data transfer of AI and sensing discussions.

The PDCP layer in NR has the following functions: SN maintenance, header compression and decompression, ciphering and deciphering, integrity protection, SDU discard, duplication, split routing if configured, re-ordering, in-order delivery, and so on. The RLC layer in NR supports TM, UM, AM modes with basic functions like data transmission, ARQ, segmentation and reassembly, SDU discard, etc.

If we look into the detailed functions in the PDCP layer and the RLC layer, there are some similar or duplicate functions, such as data transfer, header addition, SN maintenance, retransmission, status report, SDU discard, data volume calculation for BSR and DSR, and so on. These duplicate functions in the two protocol layers increase the processing delay and the complexity of hardware processing, which violates the agreements on the requirements and objective of 6GR UP design. From overhead point of view, it is also inefficient to have too many sub-headers due to too many sub-layers.

Observation 1: NR UP protocol stack is inefficient in terms of header overhead and processing delay.

Observation 2: NR PDCP layer and RLC layer have some similar/duplicate functions which increase the hardware complexity.

There are many inter-layer interactions between PDCP and RLC in NR. For example, RLC provides indication of successful delivery of PDCP PDUs to PDCP, and PDCP indicates SDU discard to RLC. Besides, for enhancements to support XR, there are quite some features involving PDCP notifications to the RLC layer, such as delay-critical data indication, delay-reporting data indication for each DSR reporting threshold, trigger indication for remaining time-based retransmission and remaining time-based polling. These cross-layer signaling introduces processing delay and specification difficulties.

Observation 3: Some features involve quite many PDCP notifications to RLC layers, which increase UE complexity and standard efforts.

Based on the above reasons, we propose to study integrating the PDCP layer and the RLC layer into one layer. After integration, data transmission and retransmission, SN maintenance, status report, SDU discard, PDU header, data volume calculation can be jointly designed, or redundancy can be removed. The cross-layer notification can be avoided as a result. The CA/DC impact on the integrated layer can be studied. During the study, it can also be considered that some functions of the RLC layer are moved to the MAC layer, such as ARQ, which can be jointly designed with HARQ function.

Proposal 6:	RAN2 to study the feasibility and advantages for combining the functions of NR PDCP and NR RLC into one layer in 6GR.

During RAN2#131bis meeting, some discussions on whether 6G user plane enhancements are applicable for user data only or also for AI/ML. There seems to be no common view on this issue in the meeting. In our view, data transfer for AI/ML and sensing may use a new data plane, which will be discussed in agenda item 10.3.3.1. Eventually the new data plane may utilize the 6G user plane protocol stack for carrying the AI/ML and sensing data. However, so far it is unclear whether exactly the same protocol stack will be used for AI/ML/sensing and user data. It is possible a different or leaner protocol stack or function set is designed for data plane. To focus on enhancements for legacy user plane, it is better to restrict discussion on functional requirements for legacy user plane, and leave AL/ML and sensing related discussion under the other agenda item for now.

Proposal 7: Functionalities for 6G user plane focus on legacy data transfer. Potential enhancements to support AI/ML and sensing data transfer will be considered after study on data transfer for AI/ML and sensing data transfer makes progress.

Conclusion

In this paper we presented some views on functional requirements for 6GR UP and have come up with the following proposals:

Proposal 1:	6GR UP protocol should aim to support flexible functions adapted by service needs.

Proposal 2:	6GR UP protocol should aim to enable further shorter processing time than NR UP.

Proposal 3:	6GR UP protocol should aim to lean design than NR UP.

Proposal 4: The functions for 6GR UP include the following as a baseline:

  - Handling of DL assignment and uplink grant, multiplexing/demultiplexing, scheduling information reporting e.g., SR, BSR and PHR, HARQ, Logical channel prioritisation (LCP), random access, BWP or carrier related operations and DRX.

  - Data transfer, sequence numbering, ARQ, segmentation and/or concatenation, SDU discard, header compression and decompression, ciphering and deciphering, re-ordering, mapping between a QoS flow and a data radio bearer.

Proposal 5:	Keep the current functions in the SDAP layer as baseline. RAN2 may consider enhancements of SDAP if triggered by other WGs (e.g., SA2), or by data transfer of AI and sensing discussions.

Proposal 6:	RAN2 to study the feasibility and advantages for combining the functions of NR PDCP and NR RLC into one layer in 6GR.

Proposal 7: Functionalities for 6G user plane focus on legacy data transfer. Potential enhancements to support AI/ML and sensing data transfer will be considered after study on data transfer for AI/ML and sensing data transfer makes progress.

References

- RAN2#131bis RAN2 chair’s note

- TR 38.914, Study on 6G Scenarios and requirements

- TS 38.300 v18.3.0