3GPP TSG-RAN WG2 Meeting #132	R2-2508355

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.3.1

Source:	Nokia

Title:	On Management and Model Transfer for AI/ML, and Data Collection

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses two main topics: AI/ML management and data collection. On AI/ML management, we address the applicability reporting procedure and the ability to evaluate an initially inactive configuration as well as an initially active configuration to aid in determining applicability in advance of configuration. We propose to study fallback in an approach that minimizes the time between the last valid report and the report subsequent to fallback, and we propose to study post-deployment validation of AI/ML configurations.

On data collection, we discuss the unification of configuration of reference signals and measurement quantities and two different types of data collection. The first type of data collection comes in the form of measurement reports or measurements collected by the UE. We propose to study whether it is requried that the 6GR be able to decode all data passing through it or if it can be passed through the user plane and if so, whether the UP supports sufficient QoS mechanisms to properly prioritize the transmission. Then, a new type of data is discussed – UP transmissions as data. In this new type of data, the transmission itself is the data, similar to how an RS or SRS transmission are a type of data affected by the physical channel. We propose to study the new type of data in addition to measurement report data.

# 2	AI/ML Management

## 2.1	Applicability

In 5G-A a control mechanism for AI/ML-enabled configurations which was introduced is called applicability reporting. The purpose of applicability reporting is for the UE to evaluate and report whether it has an ML model to execute on an AI/ML-enabled configuration and that it has the resources to do so. The burden of this applicability reporting rests solely on the UE, which evaluates its own conditions, the NW conditions and the configuration itself, and reports the result to the gNB. If the AI/ML-enabled configuration is applicable, then it can be activated and used for inference.

As an optimization, a second type of configuration consisting of “inference-related parameters”, or a subset of a full AI/ML-enabled configuration, was introduced. The procedure using inference-related parameters introduces an additional set of downlink and uplink messaging before a full AI/ML-enabled configuration is signalled to the UE. However, the optimization is minimal, particularly for the beam prediction use case for which this feature was designed, because the CSI-RS resources and most of the full configuration parameters are still required to be configured.

Observation 1: The applicability reporting framework in 5G-A supported applicability reporting on full inference configurations and on sets of inference related parameters.

Observation 2: The use of different IEs and configuration mechanisms for a full inference configuration and for sets of inference-related parameters in applicability reporting is highly suboptimal for two reasons

- New fields added to the full inference configuration will need to be added to the set of inference related parameters configuration, which is error prone and an overhead. There is significant overlap already.

- The configurations for sets of inference-related parameters are set in OtherConfig, outside of the cell configuration and a set of inference-related parameters cannot be converted into a full inference configuration, requiring further configuration after reporting applicability on a set of inference-related parameters.

The concept of applicability reporting based on sets of inference-related parameters was designed to allow the network to probe the UE for which AI/ML-enabled configurations it can execute, avoiding a configuration failure or an ambiguous state when a UE is configured with an inapplicable configuration.

For 6GR use cases, we think that configurations could be provided for evaluation which could be rapidly activated, or converted into a full configuration, after having been determined to be applicable. Configurations could be deactivated and reverted to use in the applicability reporting procedure and activated to become a full configuration.

Proposal 1: Study applicability reporting whereby the configuration under applicability evaluation is the same as that which could be executed, and wherein the configuration can be initially active (for direct use after applicability evaluation) or inactive (solely for applicability evaluation), which can be activated later.

## 2.2	Fallback

In 5G-A, fallback was left to implementation. That is, to fall back from an AI/ML configuration to a non-AI/ML configuration, the gNB issues an RRCReconfiguration message releasing the AI/ML configuration while configuring a new non-AI/ML configuration. The procedure is functional but it is inefficient, requiring a reconfiguration for each switch between AI/ML and non-AI/ML. In some cases, such as in beam prediction and CSI prediction, the UE could be provided a semi-persistent configuration, enabling a more efficient switching. However, the behaviours and mechanisms for activation and deactivation of an AI/ML configuration were not fully addressed or harmonized across use cases.

Observation 3: AI/ML models can encounter scenarios in which they experience performance loss and questionable validity of the measurement reports they generate.

Therefore, it is necessary to enable efficient and low delay transition from an AI/ML configuration to a non-AI/ML configuration such that the least number of inaccurate reports is sent to the 6GR. To support such a transition, the resources used as input to the AI/ML configuration need to be switched to the resources used as input to the non-AI/ML configuration and the reporting configuration needs to be switched from the AI/ML configuration to the non-AI/ML configuration in a single step.

Observation 4: To implement efficient fallback, switching between the resources used as input to the AI/ML and non-AI/ML and switching between the reporting configuration for AI/ML and non-AI/ML should be supported in one step.

Proposal 2: Study mechanisms to enable NW-controlled single step fallback from an AI/ML configuration to non-AI/ML configuration to ensure low delays and efficient transitions.

## 2.3	Post-deployment validation and testing

The testing and validation of the new 6GR UE AI/ML-based features (use cases) is another important aspect. In RAN4, for 5G-A Rel-19, the post-deployment testing and validation procedures have already been under discussion, even without considering 6G use cases which make use of on-device online training/finetuning (continuous learning) [see]. The 5G-A Rel-19 post-deployment validation solution was agreed to rely on the specified performance monitoring mechanisms.

In 6GR we expect over-the-air post-deployment validation procedures to play a key role as part of the NW-UE collaboration mechanisms, ensuring in-field stable and trusted performance of UE-side AI/ML features [R4-2513072].

When operating in the field, the following aspects of UE AI/ML feature operation may impact performance:

- Data drift / mismatch between the radio conditions encountered by the UE in the field and the training data, which may impact UE ML functionality performance.

- Update or fine tuning or addition/removal of models for a given UE Feature (sub-use case), resulting in an unexpected change of UE ML functionality performance.

- Repeated report from UE on the inapplicability of certain configurations, although these are included in the UE capabilities

Therefore, the main target for the post-deployment validation procedures in RAN2 is to enable the 6GR/NW to improve the ML management (“LCM”) decision-making process on the network side for the AI/ML use cases (offline and/or online training) at the UE-side. To achieve this, we need to enhance the current monitoring procedures and introduce mechanisms to configure and retrieve standardised ‘explainability’ information from the UE, reflecting its ML behaviour and ML-based reports. For example, the standardised ‘explainability’ information can be designed to provide indication on the radio conditions in the measurements used as inputs to the ML feature, potentially complemented by the use cases specific performance monitoring KPI.

Ultimately, based on the received ‘explainability’ information from the UE, the network decision can be more proactive when utilizing knowledge on past ‘explanation’s and RRM actions taken. This in turn, can improve the overall UE radio performance and minimizes unnecessary measurement request and reconfiguration signalling. A clear example is the reduced need for triggering applicability reports from the UE, when certain radio conditions are estimated based on the ‘explainability’ information.

Observation 5: The ML management in the 6GR benefits from more proactive actions or decisions, which can be enabled by the post-deployment validation procedures and the usage of the ‘explainability’ information retrieved from the UE.

The ‘explainability’ information is meant to relate to the UE-side ML functionality or ML-enabled configuration (measurement configuration, report configurations, applicability reporting) similar as specified in 5G-A AI/ML Rel. 19. Therefore, any ML model specific information and management remain out of scope of the intended post-deployment validation specifications.

These new 6GR post-deployment testing and validation procedures imply, at least, enhancements to the performance monitoring configuration and reporting procedures introduced in 5G-A AI/ML Rel. 19-20.  In addition, for any UE-side use case with online learning / finetuning [see R1-2508118], the configuration procedures for the different phases of the online training/finetuning, such as the exploration/tuning, also needs to be designed in conjunction with the post-deployment testing and validation procedures.

Proposal 3: Study the design of in-field post-deployment validation mechanisms, in combination with monitoring mechanisms, including necessary support from the ML management procedures (configuration, reporting) to ensure consistent quality and long-term UE-side ML performance in the network.

# 3	Data Collection

## 3.1	Configuration

In 5G-A, Release 19, all of the use cases defined for NW-side and UE-side AI/ML configurations use a type of reference signal (RS) such as channel state information (CSI) RS, synchronization signal block (SSB) RS, or positioning RS (PRS). Each type of RS is configured differently: CSI-RS are configured in CSI-MeasConfig; SSB-RS are configured in the physical broadcast channel (PBCH) of the SSB (note that these can be configured in a CSI-ResourceSet by SSB Index); and PRS is configured over LTE positioning protocol (LPP).

Observation 6: In 5G-A Release 19, reference signal configurations for NW-side and UE-side data collection are provided in the same way, using the same configuration IEs.

For standardized measurement types for NW-side and UE-side data collection, we see the same potential for simplification of configuration. We can take two standardized types as an example from 5G-A, Release 19: layer 1 (L1) reference signal received power (RSRP) and beam index. Whether collected for NW-side or UE-side data collection, the same measurement quantity corresponding to L1-RSRP and beam index could be used. Because much of UE-side data collection was left proprietary in 5G-A, Release, 19, there was no opportunity to reuse the measurement quantity.

Observation 7: The same standardized measurement quantities are relevant for NW-side and UE-side use cases and their configuration could be provided in the same way, using the same configuration IEs.

To achieve a clean specification without replication of features for NW-side and UE-side data collection, we should study how to take advantage of any commonality.

Proposal 4: Study the feasibility of defining a common configuration of measurement signals such as CSI reference signals and the configuration of measurement quantities such as L1-RSRP for standardized measurements for NW-side and UE-side data collection.

## 3.2	Data Transfer

In the RAN2#131bis contributions, new use cases requiring a data transfer framework were presented. Companies have argued about the drawbacks of the 5G NR user plane in supporting the new requirements imposed by use cases such as sensing or certain AIML functionalities. While comparison with the 5G NR UP is valid, we are at the beginning of a new generation, and an evolved 6GR UP is to be expected. In our view, requirements arising from these new use cases should be taken into account when designing the 6GR UP.

Observation 8: A generic 6GR User Plane design could be designed to meet the diverse data collection requirements of the identified use cases.

Based on past contributions, some initial assumptions may be inferred.

- Assumption 1 – Need for termination of transferred data in the RAN
For new use cases, including AIML and sensing, it must be studied which data types, if any, require the real-time consumption and termination in the 6GR. Once those datatypes have been identified, they should be evaluated against the capabilities of the control plane (CP). In 5G NR, a user plane (UP) DRB is terminated at the RAN, and its contents are subsequently forwarded to the UPF, which terminates the PDU session carried by the DRB. The gNB is not capable of decoding the data. Therefore, depending on the outcome of the study on datatypes, we may need to study the use of the UP for real-time data collection.

- Assumption 2 – QoS Service Enhancements
Existing DRBs can be configured with the required QoS, and it needs to be studied whether the existing QoS framework in the UP can satisfy the data transfer requirements for UE-side data collection. Additionally, the data characteristics of 6G use cases such as AIML and sensing should be analysed to determine whether the existing UP procedures require optimization.

Proposal 5: Study if any AI/ML data transfer requires termination in the 6GR and consider sensing when its study starts.

Proposal 6: Study whether there are any latency requirements for AI/ML data transfer which cannot be realized through prioritization mechanisms in the CP and through QoS framework for the UP and consider sensing when its study starts.

## 3.3 Reference Transmission

The previous section described the transfer of data in the form of reports, e.g., measurement reports and proposed to study whether it would be required that the transfer of that data terminates in the 6GR. The type of data described in this section is more comparable to a physical transmission such as sounding reference signals (SRS) and reference signals (RS), e.g., CSI-RS, transmission. In both physical layer transmission and the UP transmission introduced in this section, signalling/data transmission enables measuring/calculating performance indicators. Here, unlike physical layer transmission, the UP transmission is used to calculate higher-layer metrics.

Consider the ML-based Radio Resource Management (RRM) control use-cases, which operate in a real-time closed-loop manner. These use cases involve ML making decisions (e.g., beam/ MCS selection, handover parameters) that aim to enhance network performance outcome/metrics such as user plane throughput, latency, mobility key performance indicators.

The performance outcomes of these decisions serve as feedback (to obtain the ground truth label), enabling immediate and continuous refinement of future decision-making by performing trial and error on user plane traffic. However, this exploration of decisions may not always lead to immediate performance improvement. Therefore, to obtain such performance feedback without affecting the user experience, generation of a new data type such as UP transmissions designed in particular for supporting the exploration isolated from user data transmission, is needed. We refer to this UP transmission as reference transmission.

Observation 9: UP packets transmitted as reference transmissions must be differentiable from regular UP traffic.

Observation 10: UP packets transmitted as reference transmissions must not negatively impact user experience.

For example, the 6GR decides to change the MCS and configures the UE to transmit some UP packets (reference transmission) to the 6GR. UE generates and transmits the UP packets using the new MCS. The 6GR calculates the performance metrics like the achieved throughput of UP packets, e.g., by using HARQ feedback. The performance metrics are used as feedback for training and finetuning. These metrics cannot be calculated only in physical layer and hence, the new data type should be studied in RAN2.

In such cases, the ML model requires real-time feedback for the trial and error so that it can change its action or decision in a timely manner and observe the feedback again for improving the next decision(s). Hence, low latency data collection is required. In RAN2, we should study the impact of introducing such new data type on the design of 6GR UP since the termination points for the reference transmission are UE and the RAN. Additionally, the UE configuration for reference transmission should be studied in RAN2.

Observation 11: To enable ML-based RRM using real-time performance metrics, the UE and 6GR will need the ability to generate UP packets, terminating in the UE or the 6GR, for the purpose of performing trials based on specific configurations. These UP packets would neither carry measurements nor user data.

Proposal 7: Study the need for reference transmissions between UE and 6GR as new type of data for enabling training/adaptation of ML-based RRM and other use-cases while minimizing the user experience impact.

# 3	Conclusion

This document has made the following observations and proposals

Observation 1: The applicability reporting framework in 5G-A supported applicability reporting on full inference configurations and on sets of inference related parameters.

Observation 2: The use of different IEs and configuration mechanisms for a full inference configuration and for sets of inference-related parameters in applicability reporting is highly suboptimal for two reasons

Proposal 1: Study applicability reporting whereby the configuration under applicability evaluation is the same as that which could be executed, and wherein the configuration can be initially active (for direct use after applicability evaluation) or inactive (solely for applicability evaluation), which can be activated later.

Observation 3: AI/ML models can encounter scenarios in which they experience performance loss and questionable validity of the measurement reports they generate.

Observation 4: To implement efficient fallback, switching between the resources used as input to the AI/ML and non-AI/ML and switching between the reporting configuration for AI/ML and non-AI/ML should be supported in one step.

Proposal 2: Study mechanisms to enable NW-controlled single step fallback from an AI/ML configuration to non-AI/ML configuration to ensure low delays and efficient transitions.

Observation 5: The ML management in the 6GR benefits from more proactive actions or decisions, which can be enabled by the post-deployment validation procedures and the usage of the ‘explainability’ information retrieved from the UE.

Proposal 3: Study the design of in-field post-deployment validation mechanisms, in combination with monitoring mechanisms, including necessary support from the ML management procedures (configuration, reporting) to ensure consistent quality and long-term UE-side ML performance in the network.

Observation 6: In 5G-A Release 19, reference signal configurations for NW-side and UE-side data collection are provided in the same way, using the same configuration IEs.

Observation 7: The same standardized measurement quantities are relevant for NW-side and UE-side use cases and their configuration could be provided in the same way, using the same configuration IEs.

Proposal 4: Study the feasibility of defining a common configuration of measurement signals such as CSI reference signals and the configuration of measurement quantities such as L1-RSRP for standardized measurements for NW-side and UE-side data collection.

Observation 8: A generic 6GR User Plane design could be designed to meet the diverse data collection requirements of the identified use cases.

Proposal 5: Study if any AI/ML data transfer requires termination in the 6GR and consider sensing when its study starts.

Proposal 6: Study whether there are any latency requirements for AI/ML data transfer which cannot be realized through prioritization mechanisms in the CP and through QoS framework for the UP and consider sensing when its study starts.

Observation 9: UP packets transmitted as reference transmissions must be differentiable from regular UP traffic.

Observation 10: UP packets transmitted as reference transmissions must not negatively impact user experience.

Observation 11: To enable ML-based RRM using real-time performance metrics, the UE and 6GR will need the ability to generate UP packets, terminating in the UE or the 6GR, for the purpose of performing trials based on specific configurations. These UP packets would neither carry measurements nor user data.

Proposal 7: Study the need for reference transmissions between UE and 6GR as new type of data for enabling training/adaptation of ML-based RRM and other use-cases while minimizing the user experience impact.