3GPP TSG RAN WG2 Meeting #132                                                                   R2-2508366

# Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	8.12.2

Source:	Ofinno

Title:	Remaining MAC issues in MIMO

Document for:	Discussion and decision

# Introduction

In this contribution, we would like to share our views on the remaining MAC issues.

# Discussion

## TAT expiry of sTAG for mode-B UEIBR

In the current procedure, when the TAT for an SCell expires, all PUCCH resources and configured uplink grants associated with that SCell are released by the RRC or cleared at the MAC layer, respectively. This can also be applied to UE-initiated CSI reporting because the PUCCH/PUSCH resources cannot be used on a SCell when the TAT of the SCell expires. In particular, if a UE-initiated report configuration uses a PUCCH resource linked to the expired TAT on the SCell, that resource is released by RRC. Similarly, if a Type-1 CG for a UE-initiated report configuration is associated with the expired TAT on that SCell, it is cleared by MAC. The benefit of these behaviors is that the network can reassign the released/cleared resources to other UEs for better resource efficiency and system capacity. The above behaviors have been agreed by RAN2#131:

RAN2#131 Agreement - If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact

For mode-B, if the PUCCH resource and the Type 1 CG are configured in the serving cell(s) associated with the same TAG, there is no issue. However, if the PUCCH resource and the Type 1 CG are configured on different serving cells associated with different TAGs, there are two cases, as illustrated in Figure 1, worth highlighting for further discussion.

- Case 1: If the TAT of the TAG (e.g., TAG 2) associated with the Type 1 CG expires while the TAT for the PUCCH-associated TAG (e.g., TAG 1) is still running, following the RAN2#131 agreement, the UE only clears the Type 1 CG. In this case, since the UE cannot use the Type 1 CG, the UE’s behavior requires clarification, specifically addressing whether the UE should transmit the PUCCH notification when the associated Type 1 CG for CSI report is unavailable.

- Case 2: The TAT of the TAG associated with the PUCCH resource (e.g., TAG 1) expires, while the TAT of the TAG associated with the Type 1 CG (e.g., TAG 2) remains running. Following the RAN2#131 agreement, the UE releases the PUCCH resource but retains the Type 1 CG. However, since the UE cannot perform the PUCCH transmission for the UEI report, the retained Type 1 CG cannot be utilized.

Figure 1: TAT expiry cases for different Cells with different TAGs

In RAN2#131bis meeting, the Case 2 (If the TAT for PUCCH is expired while the TAT for Type 1 CG/PUSCH is running) has been addressed by the following agreement, i.e., the UE should clear the Type 1 CG/PUSCH if  the TAT for PUCCH is expired while the TAT for Type 1 CG/PUSCH is running.

RAN2#131bis Agreement - For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, and if the Type1 CG is only associated to this PUCCH for mode-B UEI reporting, the UE clears the Type1 CG PUSCH.

However, the Case 1 (If the TAT for Type 1 CG/PUSCH is expired while the TAT for PUCCH is running) is more controversial. In RAN2#131bis meeting, majority companies support that UE releases the PUCCH resource in such condition, but some companies raised potential concerns, so the chair made the following guidance to let the delegate further check more details with RAN1 experts.

RAN2#131bis Chair’s guidance ?? Baseline, can confirm in the next meeting: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, and if the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI), UE releases the PUCCH.

In the Post131bis MAC open issues email discussion [1], the Case 1 has been discussed, and the following two options are on the table:

- Option 1 (baseline from RAN2#131bis): UE releases the PUCCH if the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI).

- Option 2 (to avoid any impact to other UCI transmission): UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

In our view, we prefer the Option 1. We think the concern in Option 2 is a rare case and could be avoided by the NW implementation. Since the Option 1 was proposed to only release the PUCCH resource specifically for mode-B UEIBR, the other PUCCH resources will not be released. In such condition, the DCI can still schedule the HARQ-ACK to be transmitted on other PUCCH resources, which works well in the legacy before introducing the UEIBR. The problem only occurs when the UE is configured with only one PUCCH resource to be shared by all the different types of UCIs (e.g., HARQ-ACK, P/SP CSI, SR, UEIBR, etc.). However, the maximum number of the PUCCH resources that can be configured is 128 per PUCCH group. Although there is no restriction on the NW implementation, it doesn’t make sense that the NW configures only one PUCCH resource for all the different types of the UCIs, since this will cause lots of collisions. Moreover, based on the current TS 38.214, if the number of event instances reaches the eventInstanceCount, the UE should keep transmitting UEIRI on PUCCH until the counter is reset, so this will result more collisions if the PUCCH resource for UEIRI is configured to be shared with other UCIs. RAN1 has also agreed the dropping rule that LRR > UEIRI > SR, so there will be many dropped UCIs in such configuration.

However, if the proponents of the option 2 has strong concern on releasing the PUCCH resource, we can accept the Option 2. Transmitting the PUCCH when no CG/PUSCH is available for CSI report transmission offers no benefit and leads to unnecessary UE power consumption, so the UE should at least not transmit the UEIRI on the PUCCH when the associated CG type 1 has been cleared even if the PUCCH is not released. This rationale aligns with the agreement reached in the previous meeting regarding the UE behaviors on PUCCH/PUSCH transmission outside the DRX Active Time.

For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running:

- Option 1: UE releases the PUCCH. (Consider the TP in the Appendix 1 as baseline).

- Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Consider the TP in the Appendix 2 as baseline).

## UEIBR during measurement gap

In legacy, during an activated measurement gap, the UE cannot perform the transmission of “SR” and “CSI” (as highlighted below).

TS 38.321 ## 5.14 Handling of measurement gaps During an activated measurement gap that has not been cancelled (as specified in clause 10.6 in TS 38.213 [6]), the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH. NOTE: The MAC entity does not consider there is a measurement gap occasion if it is activated but cancelled.

For mode-A:

- The mode-A UEIRI on PUCCH serves a function analogous to that of “SR”, so the UE should also not transmit mode-A UEIRI during an activated measurement gap.

- The mode-A UE-initiated CSI reporting during an activated measurement gap has been covered by the legacy “CSI”. No further change is needed.

- Note that the legacy “CSI” cannot cover the first step of UEIRI on PUCCH. From PHY perspective, TS 38.213 clearly specifies that the CSI and the UEIRI are different UCI types. From MAC perspective, we did specify two different behaviors for reporting CSI and transmitting PUCCH in many places (e.g., under activated BWP, there are two bullets, one for reporting CSI; and the other for transmitting PUCCH). That’s also why we have defined two different terminologies for the two steps in the spec, i.e., (1) UEIRI on PUCCH and (2) UE-initiated CSI reporting, instead of simply using one term (e.g., UE-initiated CSI) to cover both steps. In this understanding, we believe the first step of UEIRI on PUCCH is not covered by the current spec “CSI” and should be specified clearly.

For mode-B:

- Same as mode-A, during an activated measurement gap, the UE shall also not transmit the mode-B UEIRI and the mode-B UE-initiated CSI report.

- Following the same principle as the DRX active time condition that has been agreed/captured for mode-B (i.e., UE shall not transmit UE Initiated Report Indication on PUCCH and the associated mode-B UE-initiated CSI report on PUSCH in this DRX group if the PUCCH or the PUSCH resource would not be in Active Time)  the UE shall not transmit UE Initiated Report Indication on PUCCH and the associated mode-B UE-initiated CSI report on PUSCH if the PUCCH or the PUSCH resource would be during an activated measurement gap. (The TP is similar to DRX active time condition).

- For mode-B, no matter whether the current spec can cover both steps, it does not cover the case that the PUCCH is not within a measurement gap while the PUSCH is within a measurement gap. Because these two steps are independently considered, if there is no change, the UE will only determine whether to transmit PUCCH based on whether the PUCCH is within a measurement gap without considering whether the following PUSCH is within a measurement gap or not.

During an activated measurement gap:

- UE shall not perform the transmission of UE Initiated Report Indication for mode-A UE-initiated CSI reporting; and

- UE shall not perform the transmission of UE Initiated Report Indication on PUCCH and the associated mode-B UE-initiated CSI report on PUSCH if the PUCCH or the PUSCH resource would be during an activated measurement gap.

- (Consider the TP in the Appendix 3 as baseline)

# Conclusion

Based on the analysis above, we made the following proposals:

Proposal 1	For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running:

- Option 1: UE releases the PUCCH. (Consider the TP in the Appendix 1 as baseline).

- Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Consider the TP in the Appendix 2 as baseline).

Proposal 2	During an activated measurement gap:

- UE shall not perform the transmission of UE Initiated Report Indication for mode-A UE-initiated CSI reporting; and

- UE shall not perform the transmission of UE Initiated Report Indication on PUCCH and the associated mode-B UE-initiated CSI report on PUSCH if the PUCCH or the PUSCH resource would be during an activated measurement gap.

- (Consider the TP in the Appendix 3 as baseline).

# Reference

R2-2508129 Report of Rel-19 MIMO MAC open issues (Samsung)

# Appendix

## Appendix 1

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 5> if there is PUCCH resource configured on a serving cell referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig that provides these configured uplink grants: 6> notify RRC to release the PUCCH resource for the serving cell; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; - 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

## Appendix 2

## 5.2 Maintenance of Uplink Time Alignment … - For mode-B UE-initiated CSI reporting, if the configured grant Type 1 for mode-B UE-initiated CSI reporting is associated with a TAG of an expired timeAlignmentTimer, the UE Initiated Report Indication on the PUCCH is not transmitted.

## Appendix 3

## 5.14 Handling of measurement gaps During an activated measurement gap that has not been cancelled (as specified in clause 10.6 in TS 38.213 [6]), the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, CSI, and UE Initiated Report Indication for mode-A UE-initiated CSI reporting; 1> not perform the transmission of UE Initiated Report Indication on PUCCH and the associated mode-B UE-initiated CSI report on PUSCH if the PUCCH or the PUSCH resource would be during an activated measurement gap; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH. - NOTE: The MAC entity does not consider there is a measurement gap occasion if it is activated but cancelled.