3GPP TSG-RAN WG2 #132	R2-2508442

Dallas, USA, 17 – 21 December 2025

Agenda item:		10.4	Mobility

Title:		Discussion on mobility procedures in 6G

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During last RAN2 meeting, some initial discussion on 6G mobility was made, and the following agreements were made.

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, we provide our further consideration on 6G mobility.

# 2. Discussion

2.1	6G mobility procedure

In the 5G NR, various mobility schemes were specified in addition to the legacy handover procedure, and the most representative ones include CHO, DAPS and LTM. From our perspective, in the first release of 6G, both handover (L3 triggered) and LTM cell switch (L1/L2 triggered) procedure should be supported. Because the handover, which is the baseline mobility procedure of previous generations of wireless, has been proved in the real implementation of its usability. And the LTM cell switch procedure, which is a rising star in the mobility families and has the benefit of lower latency and less signalling overhead, can be supported to meet the high requirement of the 6G system. And supporting both L3 and L1/L2 triggered mobility can cater different measurement capability of the UE in different scenarios.

Proposal 1: Support both L3 triggered (handover-like) and L1/L2 triggered (LTM-like) mobility procedure in 6G.

During last RAN2 meeting, we have agreed to design 6G mobility by taking robustness into consideration. In our understanding, conditional mobility procedure, i.e., UE triggers mobility procedure upon execution condition fulfillment, is a very good way to improve the handover robustness by eliminating the mobility failure caused by unsuccessful transmission of handover command. Therefore, we think this feature should be be supported in 6G for stronger mobility robustness.

Proposal 2: For 6G mobility robustness, support conditional mobility procedure in 6G.

For interruption time minimization during mobility, the extreme way is using DAPS mobility procedure by which we can achieve 0ms interruption time. However, DAPS procedure is much more complex in the user plane design, and so far there is less interest to implement it in the 5G network. Considering that the 6G only requires 0ms for inter-TRP mobility procedure, we suggest to not consider DAPS-like scheme in the first release of 6G. Instead, other schemes which are also helpful for interruption time reduction can be supported, for example RACH-less mobility and early UL/DL synchronization.

Proposal 3: For interruption time minimization, do not consider DAPS-like mobility procedure in the first release of 6G. Instead, other schemes like RACH-less mobility and early DL/UL synchronization can be supported in 6G.

For service continuity, 5G have already supported early CSI acquisition in Rel-19, which can minimize throughput degradation by providing CSI report associated with the PCell to the target node as soon as possible. Additionally, RAN2 will work on dynamic SCell activation/deactivation during mobility procedure in Rel-20 mobility WI. We think at least the early CSI acquisition can be supported in 6G, and whether to support dynamic SCell activation/deactivation during mobility procedure can be discussed after completion of the Rel-20 mobility WI.

Proposal 4. For service continuity in 6G, support early CSI acquisition in 6G. Whether to support SCell dynamic activation/deactivation during mobility can be discussed after completion of the Rel-20 5G mobility WI.

In the 5G NR, the handover procedure and LTM cell switch procedure use different RRC signalling branches and different RRC procedure branches, which increases the signalling overhead and makes it complicated to implement the same enhancement to both of handover and LTM procedure. For example, if the network wants to configure CHO and LTM for the same UE, the network needs to signal the UE two set of signallings corresponding to each of the CHO and LTM separately. And if any enhancement is applied to both handover and LTM, the signalling and procedure part should be designed and specified for handover and LTM separately.

For 6G mobility, it would be good to start with study of a unified mobility framework, including a single RRC signalling branch and a single procedure description. Such that any combination of the mobility features can be supported easily without any duplication and redundant work.

Proposal 5. RAN2 should aim to support 6G mobility procedure under a unified mobility framework with a single RRC signalling branch and single procedure branch.

2.2	RRM measurement

As discussed above, both L3 triggered and L1/L2 triggered mobility procedure should be supported in 6G. therefore, both L1 measurement and L3 measurement need to be supported.

However, in current 5G NR measurement framework, the configuration and reporting of L1 measurement and L3 measurement procedure are completely independent. Specifically, the L1 and L3 measurement are configured by different RRC signalling and the measurement results reported at different levels.

For measurement resources configuration, the L3 measurement results may be derived based on the L1 measurement results, or the L3 measurement resources may be a subset of the L1 measurement resources for the same cell. However, based on the 5G NR measurement configuration, even if the resources (e.g., SSB) for L1 and L3 measurements are the same, it still need to be configured separately by RRC signalling (e.g., SSB RS configurations provided in both the MeasObject for L3 measurement and the LTM-SSB-Config for LTM L1 measurement), which is redundant and increases the signalling overhead. Therefore, a unified measurement resource configuration should be studied for 6G.

For measurement report configuration in 5G, the event LTM2 to event LTM5 can be configured by LTM-CSI-ReportConfig to trigger the L1 MR, while the event A3 to A5 can be configured by MeasConfig to trigger the L3 MR. Those events are similar from the perspective of entering and leaving condition (e.g., event LTM2 and event A2 have the same condition with the different parameters), but use the different signalling. In addition, the event triggered and periodic MR report also can be configured for both L1 and L3 measurement. Then, for 6G measurement report configuration, it would be good to study a unified framework, including at least the unified configuration of measurement event and report type.

Proposal 6: RAN2 study a unified measurement configuration framework of L1 and L3 measurement, including at least the following aspect:

- Unified measurement resource configuration

- Unified measurement report configuration (e.g., event, report type)

In addition, for L3 measurement reporting, the cell-based triggering condition with RRC reporting is used, while for L1 measurement reporting, the beam-based triggering condition with MAC CE is used. In one aspect, the duplicated RRM measurement procedures increase UE complexity and UE processing overhead, especially when L1 and L3 measurements and reporting are configured simultaneously, the UE must concurrently maintain different reporting logical (i.e., RRC and MAC CE reporting). In another aspect, MAC CE based measurement reporting procedure is beneficial in reducing processing and transmission delay, while the RRC based MR procedure is more flexibility compared to MAC CE. Therefore, it is beneficial to first study on whether a unified measurement reporting procedure is needed. Then, if a unified MR procedure is pursued in 6G to support both L1 measurement reporting and L3 measurement reporting, the appropriate approach should be studied to trade-off between MAC CE based approach and RRC approach.

Proposal 7: RAN2 study on whether and how to support a unified measurement reporting procedure for both L1 and L3 measurement reporting.

# 3. Conclusion

In this contribution, we provide our further consideration on 6G mobility, and have the following proposals:

Proposal 1: Support both L3 triggered (handover-like) and L1/L2 triggered (LTM-like) mobility procedure in 6G.

Proposal 2: For 6G mobility robustness, support conditional mobility procedure in 6G.

Proposal 3: For interruption time minimization, do not consider DAPS-like mobility procedure in the first release of 6G. Instead, other schemes like RACH-less mobility and early DL/UL synchronization can be supported in 6G.

Proposal 4. For service continuity in 6G, support early CSI acquisition in 6G. Whether to support SCell dynamic activation/deactivation during mobility can be discussed after completion of the Rel-20 5G mobility WI.

Proposal 5. RAN2 should aim to support 6G mobility procedure under a unified mobility framework with a single RRC signalling branch and single procedure branch.

Proposal 6: RAN2 study a unified measurement configuration framework of L1 and L3 measurement, including at least the following aspect:

- Unified measurement resource configuration

- Unified measurement report configuration (e.g., event, report type)

Proposal 7: RAN2 study on whether and how to support a unified measurement reporting procedure for both L1 and L3 measurement reporting.

# References

[1] Meeting report of RAN2 #131bis