3GPP TSG-RAN WG2 #132	R2-2508493

Dallas, USA, Nov. 17th - 21st

Source:                    	DENSO CORPORATION

Title:  	RRC corrections for Rel-19 NR NTN DL-CE

Document for:        	Discussion and decision

Agenda Item:         	8.8.2		RRC corrections

- Introduction

According to the chair notes for last meeting RAN2 #131bis[1], the following agreements are made for NR NTN RRC corrections:

Agreements:

1.	[S024]: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available

2.	[H252]: We add the “N closest…” in the procedural text

3.	[H250]: Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisted SMTC and measurement gap configuration” (can come back to this after the discussion on inter-frequency support)

4.	[V208/X251] Confirm that bitmap is used to indicate the N closest reference location.

5.	[Q200]: Location-based SMTC enhancement is not supported in moving cell scenario (to be captured in Stage2). We specify that smtc5 with multiple offsets is not broadcast in a Earth-moving cell scenario (to be captured in the smtc5 field description in system information)

6.	[C006]: We change the limitation as “The total number of configurable SMTCs offsets across smtc4list and smtc5list is 6.”

7.	[C009]: RAN2 understands that if the NW wants to reuse the periodicity in smtc1, the NW needs to configure smtc4 (we capture something accordingly in the field description)

8.	[V206] Change the need code for mbs-SessionAreaList-r19 from Need R to Need S. 	Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent

9.	[O710]: Fix the description for warningMessageSegmentType in SIB7 (same applies to IoT NTN)

10. 	[Z256] is Rejected

11. 	[Z258] is Rejected (issue to be discussed in cross-WI discussion)

12.	[V201] is Rejected

Agreements:

1. 	[V204/H250] We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized. We introduce a new (per-UE) capability with signalling for the support in RRC connected of 2 periodicities in the inter-frequency case

2.	[X250/V205] The introduction of a new distance threshold (that is associated with the Rel-19 reference locations) is not pursued

In this paper, we would like to discuss our proposals for some enhancements on current running CR.

- Discussion

In the last meeting, it was agreed that the Rel-19 SMTC enhancements would be extended to cover inter-frequency scenarios. According to the current CR[2], this extension requires the reporting of the closest reference locations for each frequency. Considering support for the scenarios 1 and 3 described in R2-2507763[3], configuring and reporting reference locations per frequency could provide more accurate information in those cases. However, as commented by several companies, the closest reference location report contains information about the UE’s position, which is inherently independent of frequency. Furthermore, for the following reasons, we think it is preferable to define a common reference location across both intra-frequency and inter-frequency cases:

- Even though reference locations are configured per frequency, the closest reference location report may not provide highly accurate location information due to privacy considerations. RAN2 has already agreed not to introduce a distance threshold, and the determination of the "closest" location is highly dependent on the UE implementation. Additionally, the bitmap report does not include any information about the relative order of distances.

- It is not necessarily the case that intra- and inter-frequency cells are arranged in neat, separate hexagonal patterns. Therefore, we are not sure about the effectiveness of configuring six reference locations separately for intra- and inter-frequency.

Therefore, we think it would be simple and straightforward to configure common reference locations across intra- and inter-frequency based on network implementation, and then UE just reports closest reference locations independent of frequency.

Proposal 1: The network configures common reference locations across intra- and inter-frequency, and the UE reports closest reference locations independent of frequency.

In case configuring common reference locations across both intra- and inter-frequency, the maximum number of reference locations could be a concern. However, we think it is not necessary to increase this number. As discussed above, highly accurate location information is not expected, and having too many reference locations would increase UE power consumption due to the calculation of distances for each location.

Proposal 2: RAN2 will not increase the maximum number of reference locations even if reference locations are configured commonly across intra- and inter-frequency.

- Summary and proposal

This paper discussed about enhancements on current running CR for downlink coverage enhancement and proposed the following:

Proposal 1: The network configures common reference locations across intra- and inter-frequency, and the UE reports closest reference locations independent of frequency.

Proposal 2: RAN2 will not increase the maximum number of reference locations even if reference locations are configured commonly across intra- and inter-frequency.

- References

[1] R2-2507703, “Report from Break-out session on NR-NTN and IoT-NTN”

[2] R2-2507785, “Initial corrections to NR NTN Phase 3”

[3] R2-2507763, “Summary of the offline discussion [AT131bis][303][NR NTN] RRC open issues (Ericsson)”