3GPP TSG-RAN WG2 Meeting #132	R2-2508548

Dallas, USA, Nov. 17th – 21st 2025

Agenda item:	10.4 (Mobility)

Source:	Sony

Title:	Discussion on 6GR Rel-20 mobility aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108, the 6G study item was agreed [1], where mobility related part of the study item is captured as follows:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3].

At RAN2#131bis, RAN2 reached the following initial agreements [6]:

Agreements:

Continue to study:

-	Idle mode –

-	Connected mode

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Design goal:

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

Agreements:

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this paper, we discuss mobility related part of the 6G study item: unified measurement framework, HO types based on DL RS, UL mobility based on UL RS for connected mode mobility as well as idle/inactive mode mobility such as cell selection/reselection and UE power saving/energy efficiency (EE) schemes.

# 2 Discussion

## 2.1 Mobility in connected mode

## 2.1.1 Measurements framework

Measurements consume a portion of UE power; hence, the UE power consumption has been an important topic and RAN2 has introduced mechanisms like relaxed measurements for idle/inactive UEs in 5G NR. For connected mode measurements, a UE normally relies on network configuration. For the same reason, the UE power consumption will be an important topic for 6G as well and there should be an effort to further reduce UE power consumption. We think there are at least two solution directions:

- DL RS in connected mode periodicity and its impacts on UE energy efficiency (EE) and performance

- UL RS measurements at the base station (EE) as discussed in the later section of this document.

DL RS periodicity and RS design should be discussed in RAN1 and RAN4 first, taking into account impacts on UE energy efficiency (EE) and performance.

Proposal 1. RAN2 should, together with RAN4 and RAN1, study Energy Efficiency improvements in the measurement framework.

In NR, for inter-cell mobility, currently there are different measurements and reporting modes, for example, for L3 HO and LTM mobility where measurement configuration belongs to the current serving cell and neighboring cells. L3 HO measurements are reported via RRC message, and LTM measurements are conveyed via MACE and UCI on PUCCH/PUSCH.

For intra-cell mobility, there are also different measurements and reporting for beam management where in Rel-15, a UE periodically measures and reports the best beam(s) to then network, and Rel-19, it was added a UE-initiated beam Reporting (UEIBR).

For the scheduling functions in the serving cell, there are various measurements and complex reporting modes for Channel state information acquisition (CSI) from the UE side.

In addition, for all of the above measurements, the configuration of reference signals used to derive the measurement quantities may be different, such as SSB or CSI-RS.

At least in NR, the differences occurred due to the purpose of the measurement, and also some features were added in the later releases which somehow resulted in increased complexity, overhead and power consumption for both UE and network.

Hence, in 6G, it should be studied how to support/configure a unified measurement framework for mobility and scheduling functions which include common reference signals configuration, measurement configuration and reporting configuration.

Proposal 2. RAN2 should study how to support unified measurement framework for mobility (inter-cell and Intra-cell) and scheduling functions which include common reference signals configuration, measurement configuration and reporting configuration.

## 2.1.2 Legacy HO Types based on DL RS measurements

L3 Handover: L3 handover procedure has, since 2G timeframe, been supported where the mobility of the connected mode UE is controlled by the network, as shown in Figure 1. The UE measures the signal quality/strength of the serving cell and neighbouring cells and then reports to the serving cell. Based on the measurements, the serving cell makes decision whether to handover the UE to one of the neighbouring cells termed as target cell. The gNB’s decision of sending handover message to the UE could be based on some predefined conditions, for example if the signal quality of the target cell is better than the signal quality of the serving cell. As soon as UE receives the handover command/RRCReconfiguration message, it synchronises and switches to the new cell by sending RRCReconfiguration complete message.

One of the drawbacks of the L3 handover is the long data interruption time due to poor radio link condition in some scenarios as the UE is mostly at the cell edge, which needs a number of retransmissions and even the radio link may sometimes be lost which results delays and significant overhead. The typical L3 handover latency is about 75ms which has significant impacts on some services like URLLC and XR. Regardless of drawbacks, in our view, L3 HO has been around for several generations, and it works well in the field [3], so it should be supported in 6G as well.

Figure 1. Basic Handover Procedure.

L3 Conditional Handover (CHO): As said earlier, the UE is mostly at the cell edge when handover procedure is triggered which is not always in a favorite radio link condition. CHO has been specified in Rel-16 for 5G NR. Initially a UE provides preliminary measurement reports which identifies the potential target cells whilst still in good radio link condition with the serving cell, and handover preparations/configurations are performed for the UE with the identified potential target cells where some specific execution conditions are included in the configurations. The UE starts evaluating these conditions, and in a later stage if one or more conditions are met, the UE carries out HO execution and completion without instant signaling exchange with the current serving cell. This leads a more successful handover with lower chance of radio link failure.

L1L2 Triggered Mobility (LTM): With similar motivation for L3 CHO, to reduce the overhead and interruption time, LTM has been recently specified in Rel-18 and Rel-19 for 5G NR where L1/L2 signals and messages are employed to speed up the HO procedure, as shown in Figure 2. Initially a UE provides preliminary L3 measurement reports to the gNB where measurement reports identify the potential candidate cells. Then the gNB decides to configure LTM and initiates LTM preparation for multiple candidate cells for the UE. The UE makes DL and UL synchronizations with multiple candidate cells in advance. In a later stage, the UE provides L1 or L3 measurement reports to the gNB where measurement reports identify fewer candidate cells for handover. Based on these latest measurements, the gNB makes decision to handover the UE to a single target cell by issuing a cell switch command (CSC) signaled via a MAC CE. Then the UE applies the target cell’s LTM configuration according to the cell switch command and switches to the target cell. During cell switching, if valid timing advance (TA) value is available, the UE performs RACH-less cell switch, and if no valid TA value is available the UE performs RACH-based cell switch. The UE accesses the new cell by sending sending RRCReconfiguration complete message. In LTM, the interruption time is approximately reduced to 30ms.

Figure 2. LTM Procedures [2]

Conditional LTM (CLTM): CLTM cell has been developed in Rel-19 5G NR to achieve high robustness whereby the CLM procedure is executed without immediate signalling exchange between the UE and its serving cell. CLTM is executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met. CLTM offers shorter interruption time.

Proposal 3. Rel-19 LTM should be the baseline for any HO work in 6G as LTM exhibited short interruption time and simple design. In addition,

- L3 HO has been around for many generations, and it works well in the field [3], in our view, L3 HO should be supported.

In NR, as we discussed above, there are L3 HO, L3 CHO, LTM and LTM CHO mobility types/schemes which are specified in different releases. The consequence of specifying in different releases is that there are duplicated features/functionalities which introduced signaling and implementation complexity as well as overhead into the system. In 6G, it would be desirable to achieve a unified mobility framework to avoid duplication of the same functionalities where unified mobility framework includes all features such as RRC configurations, early DL/UL synchronization, UE configuration processing, triggering handover command, conditional handover and early CSI acquisition.

Proposal 4. RAN2 should study on unified mobility framework to avoid duplication of the same functionalities where unified mobility framework includes all features such as RRC configurations, early DL/UL synchronization, UE configuration processing, triggering handover command, conditional handover and early CSI acquisition.

## 2.1.3 Uplink Mobility based on UL RS measurements

Uplink-based mobility was somewhat introduced and discussed in 5G NR study item, although it was not specified at the end. Traditionally, the handover (HO) is triggered by a UE to transmit DL measurement report to the serving cell and then the serving cell decides to initiate the HO to the target cell. Another alternative is that a UE transmits UL Reference Signal to the target cell(s) and then the target cell sends measurement reports to the serving cell/source cell and source cell decides to initiate the handover procedure. This may mean that the UE transmits one or multiple Reference signals to multiple target cells. In this case, the network compares the measurements from different candidate cells and decides which TRP/cell(s) best serve the UE. Alternatively, the target cell can initiate a context fetch from serving cell and admit the UE in its cell straight away.

The scenario that this UL-based mobility is very beneficial in the dense network deployment which is intended to improve the capacity in dense urban areas as shown on Figure 3. In this scenario, the UL-based mobility would reduce power consumption at the UE due to decrease of DL measurements and reporting. The requirement from the network side is to coordinate and configure the UL Reference Signals to all possible candidate cells.

Figure 3: UL Mobility between different TRPs/cells.

Figure 4 shows a more detailed example of UL-based mobility procedure, where serving cell exchanges configuration of the UL signal configuration with the neighbouring cells in step 1. Then in step 2, the UE receives UL signal configuration of the neighbouring cells from the serving cell. In step 3, the UE transmits UL reference signal to the neighbouring cells based on the configuration. In step 4, based on the measurements, one of the target cells decides to admit the UE and sends HO Request (including HO RRC configuration) to the serving cell where the serving cell transmits RRCReconfiguration message to the UE in step 5. In step 6, if Timing advance (TA) is not available at the UE, the UE carries out Timing Advance (TA) measurement (e.g., UE-based TA) or transmit RACH to target cell to synchronise with the target cell. At the end, the UE switches to the target cell by transmitting RRCReconfiguration complete message (carried over Message A or Configured Grant (CG)), notifying the target cell that it is now completed the handover procedure.

Figure 4: Example of UL Mobility procedure.

In 6G, UL-based mobility should be studied at least for connected mode as the network is able to coordinate the UL Reference Signals between the serving cell and candidate target cells. It should be noted that there are already some UL signals in 5G NR that are coordinated between the serving cell and neighbouring cells, such as LTM RACH preamble coordination for UL synchronisation purpose, NES UL wake-up signals employed to wake-up NES cells and coordination of sounding reference signal (SRS) configuration for positioning purpose. Hence, in 6G, these various uplink signals can be unified probably to one signal that serves to multiple purposes if possible.

Proposal 5. UL-based mobility should be studied at least for connected mode mobility.

## 2.1.4 Distributed TRPs/Super cell mobility

In 5G operation, a Heterogenous Network (HetNet) is deployed, where a dense macro cell consists of multiple Transmission & Reception Points (TRP). An example of HetNet is shown in Figure 5, where the multi-TRPs deployment consists of a macro-gNB labelled as TRP#0, providing a macro coverage and multiple small cells labelled as TRP#1, TRP#2 and TRP#3. The small cells typically transmit in lower power than the macro-gNB, thereby forming smaller coverage area within the macro cell coverage. The small cells are used to alleviate some traffic from the macro-gNB which is beneficial in a cell with dense UEs. Small cells can be deployed at the cell edge to enhance the macro cell edge coverage. The TRPs may be inter-connected, for example the small cells can be connected to the macro-gNB, which enables coordinated scheduling and multi-TRP MIMO.

Figure 5: Heterogenous Network (HetNet)

In addition, increasing the number of directional antennas at the gNB will improve the spectral efficiency as the signal gain improves due to beamforming and with less interference on other directions. But still there will be cell-edge UEs with less data rate and even if the transmit power is increased the interference will equally rise in the same direction.

Furthermore, by increasing the number of small cells (with different cell IDs), in addition to the interference, the frequency of the cell switching will increase due to the reduced cell sizes and user mobility, leading to more frequent HO-related disruptions.

As a result, small cells are no longer preferred deployment scenario in the cell densification since the interference and HO-related disruptions are increased.

A more realistic scenario of deploying large antenna arrays is by distributing subarrays/TRPs connected through optical fibres or wireless backhaul, over a large geographic area as illustrated in Figure 6, to provide a more or less uniform distribution of the received signal power, or in other words, more or less uniform coverage in everywhere (or same area throughput). The achievement is to enable the signals from distributed TRPs to be spatially multiplexed or jointly combined at the receiver (UE). To reduce the complexity, since a UE is likely to have strong signals from only a few near-by TRPs, a subset of TRPs can be selected at the UE. These TRPs belong to the same cell (i.e., same cell ID, Super cell) [3-4], hence, mobility is simplified in that large geographic area. The name of this can be Super cell/Distributed TRPs. This will play a key role in heterogeneous networks to ensure uniform coverage over large geographic area and to serve moving UEs.

Figure 6: Distributed TRPs/Super Cell for HetNet deployment.

Proposal 6. RAN2 should study mobility aspects of distributed TRPs over a large coverage area, for example beam level, TRP level and super-cell level mobility.

## 2.2 Mobility in Idle/Inactive states

Cell selection/reselection has a long history in all generations of cellular generation from NMT, and GSM, to WCDMA, LTE and NR. This will surely continue to be supported in the 6th generation. The procedure involves the UE performing measurement to make sure it is camping on a suitable cell, to be prepared to receive potential paging message. The procedure can be power consuming, and therefore different methods involving DRX and Power save states have been introduced to reduce the UE power consumption.

Nevertheless, are the current solutions that we have up to 5G NR the most optimal or are there any room for further improvements?

But related to the overall Energy Efficiency objectives, potentially further optimization can be discussed for 6G, e.g. reducing the UE effort/power consumption for RRM measurements in idle mode for a certain type of devices like e.g., IOT devices with predictable traffic, involving Low-Power wake up radio (LP-WUR) also for neighbour cell measurements.

Cell selection and cell reselection are performed by UEs in order to camp on a suitable cell in general and to camp on the best available cell when entering connected mode. The measurement on the serving cell is normally performed on every paging occasion, therefore it does not consume very much power. Searches for and measurements on neighbor cells are performed when the serving cell measurements are degrading.

In order to improve the Energy Efficiency (EE) in the UE, it has been proposed that the LP-WUS is used to send the paging indication to the UE, at least when in good coverage. If the LP-WUR is used by the UE for detecting paging, LP-WUR should also be used for doing the serving cell measurement when in good coverage. Otherwise, the normal receiver still needs to synchronize to the normal reference signals from the serving cell and the LP-WUS signalling would not improve the power consumption.

Proposal 7. RAN2 to study the energy efficiency enhancements like reduced RRM measurements in idle/Inactive mode for certain device types, usage of LP-WUR for paging reception and serving/neighbour cell measurements for cell reselection in idle and inactive mode/state.

# 3 Conclusion

In this contribution, we have discussed mobility related part of the 6G study item, covering unified measurement framework, HO types based on DL RS, UL mobility based on UL RS for connected mode mobility as well as idle/inactive mode mobility such as cell selection/reselection and UE power saving/energy efficiency (EE) schemes. We have the following proposals:

Proposal 1. RAN2 should, together with RAN4 and RAN1, study Energy Efficiency improvements in the measurement framework.

Proposal 2. RAN2 should study how to support unified measurement framework for mobility (inter-cell and Intra-cell) and scheduling functions which include common reference signals configuration, measurement configuration and reporting configuration.

Proposal 3. Rel-19 LTM should be the baseline for any HO work in 6G as LTM exhibited short interruption time and simple design. In addition,

- L3 HO has been around for many generations, and it works well in the field [3], in our view, L3 HO should be supported.

Proposal 4. RAN2 should study on unified mobility framework to avoid duplication of the same functionalities where unified mobility framework includes all features such as RRC configurations, early DL/UL synchronization, UE configuration processing, triggering handover command, conditional handover and early CSI acquisition.

Proposal 5. UL-based mobility should be studied at least for connected mode mobility.

Proposal 6. RAN2 should study mobility aspects of distributed TRPs over a large coverage area, for example beam level, TRP level and super-cell level mobility.

Proposal 7. RAN2 to study the energy efficiency enhancements like reduced RRM measurements in idle/Inactive mode for certain device types, usage of LP-WUR for paging reception and serving/neighbour cell measurements for cell reselection in idle and inactive mode/state.

# 4 References

- RP-251881, New WID: Study on 6G Radio, RAN#108.

- TS 38.300 V19.0.0

- R2-2506899, Consideration on the mobility in 6GR, RAN2#131bis, CMCC

- R2-2507169, Consideration on 6G Mobility, RAN2#131bis, ZTE Corporation.

- Minutes of RAN2#131bis meeting, Prague, Czech Republic.