3GPP TSG-RAN WG2 Meeting #132	R2-2508586

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	7.0.2.16

Source:	ZTE Corporation, Sanechips

Title:	XR Remaining Issues

Document for:	Discussion, Agreement

# Introduction

In this contribution, we will discuss the wording for DSR triggering and give our proposal.

# Discussion

According to DSR procedure in TS 38.321 [1], DSR is triggered if the smallest PDCP discard timer value is less than RemainingTimeThreshold among PDCP SDUs that has not been transmitted in any MAC PDU.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

In RAN2#131bis meeting, the following agreements have been made:

- RAN2 understanding is that: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

- We will try to clarify this in MAC. FFS the wording and CB next meeting

Considering that an RLC SDU may be transmitted with or without segmented, using “have not been completely transmitted in any MAC PDU” may be interpreted as that “the RLC SDU have not be transmitted integrally (i.e. without segmented)”, this understanding should be avoid. In fact, the RAN2 intention is to describe that “that at least one PDCP SDU or corresponding RLC SDU segment has not been transmitted in any MAC PDU”, we can use this text to formulate the RAN2 agreement.

Proposal 1. Clarify “that have not been transmitted in any MAC PDU” in the MAC spec to “that at least one PDCP SDU or corresponding RLC SDU segment has not been transmitted in any MAC PDU,” in order to report the delay information of RLC SDU and SDU segment that have not been transmitted.

Based on the proposal, we provide the TP in Annex.

# Conclusion

Based on the discussion we propose the following:

Proposal 1. Clarify “that have not been transmitted in any MAC PDU” in the MAC spec to “that at least one PDCP SDU or corresponding RLC SDU segment has not been transmitted in any MAC PDU,” in order to report the delay information of RLC SDU and SDU segment that have not been transmitted.

# Reference

- 3GPP TS 38.321 V18.7.0 (2025-09), NR; Medium Access Control (MAC) protocol specification(Release 19)

- R2_131bis XR and LTE Broadcast notes (Dawid)_EOM

# Annex

38.321 TP for proposal 1

### 5.4.9 Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that at least one PDCP SDU or corresponding RLC SDU segment has not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel. If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72. 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE 1: The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5. A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.