3GPP RAN WG2 Meeting #132                                                                                     R2-2508598

Dallas, US, Nov. 17th – 21st, 2025

Agenda Item:	10.3.3.1

Source:	HONOR

Title:	Consideration on radio protocols for 6G system data

Document for:	Discussion and Decision

- Introduction

In RAN#108 meeting, the industry has approved the 6G Study Item (RP-251881[1]), marking the first concrete blueprint for the next generation of wireless system. As a potential key technology of 6G, RAN2 group is responsible for discussing and promoting the study of radio interface protocol architecture and procedures for 6GR.

In RAN2#131-bis, RAN2 has agreed to study the standardized data collection framework with some requirements as a baseline and proposed to study the common requirements for various types of data [2]:

Agreements

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

In this contribution, we try to analyze the requirements and functions required for 6G system data and list the initial considerations on radio protocols.

- Discussion

  - Overview

In the TR 22.870 [3], SA WG1 has defined the 6G system data which is generated and controlled by the 6G system. 6G system data, e.g., AI/ML data, sensing data, energy efficiency information and NF context, is different from user traffic data which is application-level data transmitted transparently between the UE and DN, thus presenting new requirements for 6G data collection framework.

Observation 1: SA1 defines 6G system data which is generated and controlled by the 6G system, including AI/ML data, sensing data, energy efficiency information and NF context.

RAN2 has agreed to further study requirements of AI/ML service and sensing is also in the scope of 6GR. Therefore, it is suggested to re-use the terminology/description captured in SA1 to refer the data related to AI/ML and sensing.

Proposal 1: Discuss to use similar SA1 terminology to describe various types of data: 6G system data which is generated and controlled by the 6G system, e.g. AI/ML data, sensing data.

Data collection is a crucial part of the data handling procedure for AI/ML. In 5G NR, the corresponding procedures are captured in the framework of AI/ML [4], where data collection is defined as a process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference. In addition to data collection, data delivery is another crucial part of the data handling procedure to support AI/ML training. The data could be AI/ML model for inference or datasets for model training, and it could be delivered from one entity to another entity in any manner.

For sensing in NR, sensing entity will perform the sensing service requested by sensing function (SnF). The SnF may request the sensing entity to send the raw data or generate sensing result with initial processing. The procedure will happen between UE and SnF when UE is selected as the sensing entity. In the other case, the sensing entity could be RAN node and the SnF could collect data from RAN. Therefore, the data could be collected from the network nodes, or UE for the purpose of sensing. Then, the sensing entity collects the sensing data and delivers the sensing data or results to the SnF.

Observation 2: Data collection and data delivery are important functionalities of AI/ML and sensing, and NR has been discussing data collection and data delivery mechanisms for specific use cases.

In our view, 6G Day-1 RAN design should take the existing and potential use cases for data services into account to design the signaling and procedures for data collection and data delivery with addressing both universal and scenario-specific requirements. From the RAN node’s perspective, the corresponding functionalities could be summarized as:

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

As more use cases evolve, the RAN node may need to accumulate large amount of data for various purposes, potentially leading to the duplicated collection of same or similar data, as well as the wastage of energy and resources. In addition, NR is studying how to leverage AI to optimize RAN’s performance and management within the network. It would be beneficial if the RAN node could be aware of the data content and adapt accordingly for performance improvement. To facilitate the issue, data management should be studied with potential framework enhancement.

Proposal 2: RAN2 to study how to support the Data Collection, Data Delivery, and/or Data Management to fulfil the data service requirements.

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

–	Data Management: The RAN node could be aware of the data and utilize data for various purposes, e.g., performing AI-enabled data analysis and adapt accordingly for performance improvement.

  - 6G system data handling

2.1.1 Requirements of Data Collection

In 5G NR, several use cases have been studied on leveraging AI/ML to improve network performance, such as CSI feedback enhancement, beam management and positioning enhancement. In each scenario, data collection process has been thoroughly discussed according to service types (e.g inference, training), possible end point pairs, minimize UE impacts.

As 6G networks progressively integrate advanced capabilities like AI/ML and multi-dimensional sensing, they will need to collect and manage massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Therefore, 6G networks shall consider these dimensions and support efficient data collection and control mechanism to meet both universal and scenario-specific requirements across diverse use cases, such as AI, sensing, XR services, and digital twin networks.

From the RAN side’s perspective, facing these new data characteristics, the 6G network capability of data collection needs to be extended based on the 5G network, including enhancements in controllability, UE selection, and visibility.

- Controllability

Data collection is crucial for enabling various AI/ML-assisted use cases. In 5G NR AI/ML air interface discussions, it is agreed that data collection initiation and configuration for data collection are under network control, a principle that should be carried over to 6GR. Specifically, the RAN node can determine if and when to start or stop data collection and configure whether the UE is allowed to request data collection. For some use cases, such as beam management, the RAN provides the data collection configuration for training a UE-side model. In other cases, such as positioning, the configuration may come from outside the RAN (e.g., LMF). Even in these latter cases, some level of RAN awareness and control is necessary to ensure that UE performance is not adversely affected by data collection measurements.

- UE selection

In the Rel-18 study on AI/ML air interface, for network (NW)-side model training [4], a set of general data collection principles involving UE capability demands is expected to be considered, including:

-	UE to support data logging,

-	UE to report the collected data periodically, event-based, and on-demand,

-	The UE memory, processing power, energy consumption, signaling overhead should be considered.

Therefore, when the network-side training entity requests data from RAN nodes, the RAN may need to select appropriate UEs based on data requirements. Moreover, in the current SA2 discussions [5], the UE-side training entity can instruct specific UEs, groups of UEs, or any UE to collect and report data, which implicitly involves the UE selection process. As mentioned before, the network sided data collection involves buffer management: when the UE reaches its buffer limitation, it stops the logging measurements. Our understanding is that the same principle applies to the UE selection during UE-side data collection.

Proposal 3: For 6GR, RAN2 to study the possible functionalities of Data Collection, which include: controllability, UE selection, and visibility.

2.1.2 Requirements of Data Transfer

Based on prior work in 5G and analyses of AI/ML and sensing in our other contribution [6], transfer requirement of 6G system data could be described via Data size, transmission latency, reliability, transfer frequency, multiple end point pairs and privacy & security protection.

According to the above transfer requirements, the data volume of the AI/ML data may be very large. And for the online training/AI model transmission, the reliability of data transmission needs to be guaranteed. Meanwhile, online training for AI needs to be transmitted within certain latency requirement. Therefore, in order to be applicable to different types of data transmission, it is necessary to support some basic functions for data transfer. For example, the four types of data mentioned in [6] may require different bearers for transmission, and different bearers employ different scheduling/transmission strategies.

Current specification provides serval functions for data transmission, such as QoS mapping, security policy, compression, (re)transmission, segmentation, in-order/out of order delivery, etc. All of these functions can be used to support the above requirements.

In some scenario, the model is UE-specific and the related dataset/model should be delivery to unique UE and it is necessary to deliver the data via unicast mode. However, the generalization ability of a model can not only improve its performance, but also enhance resource efficiency and model robustness. Therefore, it is desirable for the dataset/model parameters to be applicable across multiple UEs. Simultaneous delivery of these dataset/parameters to a group of UEs is an energy-efficient approach for the RAN node. Thus, the existing MBS multicast/MBS broadcast mechanisms can be further considered for model/dataset delivery. RAN2 to take unicast as the baseline in the initial discussion stage; and further study whether and how to support multicast or broadcast scheme for data delivery, e.g, AI/ML dataset/model parameters transferring.

Proposal 4: For 6GR, RAN2 to study the possible functionalities of Data Transfer, which can include: QoS mapping, security policy, compression, (re)transmission, segmentation, in-order/out of order delivery, unicast/multicast mode etc.

2.1.3 Requirements of Data Management

It has been agreed to take the standardized data collection framework with controllability and visibility of standardized data as the baseline. It makes it possible that the RAN node could be aware of the data content, especially when RAN node is the termination point.

Observation 3: It is possible that RAN could be aware of the data content if standardized data is introduced.

In this case, RAN could not perform the data collection procedure if the requested data has been collected before, e.g., stored in the RAN side. And then RAN could adopt the data for different purpose. For example, measurement data reported by UEs could be accessible to multiple data consumers within 3GPP system to support various services, e.g., analyzed and utilized by native AI for flexible resource allocation and optimized network performance.

Observation 4: It is efficient if RAN could adapt the collected data accordingly for different performance improvement purpose.

Proposal 5: RAN should study how to support the Data Management, including duplication detection, data storage and data distribution, to support efficient data utilization.

  - Consideration on Radio Protocol

In 5G, AI/ML process is discussed case by case and data could be transferred via multiple path, e.g. UCI/DCI/RRC to meet requirements of different scenarios. Therefore, there will be repetitive work when new scenarios appear. 6G Day-1 RAN design should take the existing and potential use cases for data services into account to design the signaling and procedures for data collection, data transfer and data management with addressing both universal and scenario-specific requirements.

For 6G Day-1, 6GR could consider introducing the data plane which is different with control plane and user plane to support handling of 6G system data, including Data Collection, Data Delivery and Data Management.

In the other hand, RAN2 has evaluated how to support data collection and delivery of AI/ML via enhancing of control plane and user plane in NR. However, it is hard to define an efficient mechanism constrained by backward compatibility. For 6GR, it is possible to natively support a scalable and forward-compatible data handling with capture data handling functionalities in Control Plane and User Plane.

Observation 5: For 6GR, there are two options for unified data handling:

Option 1: introduce the data plane for 6G system data;

Option 2: enhance the user plane or control plan for 6G system data.

Considering system-level solution, the data handling should also include data sharing and consistency within and across RAN, CN and OAM domains, aiming to reduce duplication and simplify integration—requirements not typically seen in traditional services. Meanwhile, the 6G system shall support mechanisms for data handling that minimize the impact to traditional services.

In the other hand, the 6G data framework is also studied in SA2, with a scope that may include identifying the data type and requirements, designing the end-to-end system architecture, defining the procedures and supporting policy control/QoS for 6G data framework. This would introduce new requirements for RAN, and RAN2 should keep coordination with SA2 to be aware of the requirements to provide an efficient data handling solution.

Observation 6: SA2 is studying how to identify the data type and requirements, designing the end-to-end system architecture, defining the procedures and supporting policy control/QoS for 6G data framework.

Proposal 6: 6GR should study whether and how to introduce the data plane or how to enhance the user plane or control plan to support the functionalities of data handling.  And coordination with SA2 is necessary.

The 4G/5G protocol stack design applies a well-defined protocol layered structure, where each layer provides its own services. If a new plane is introduced, the protocol layered structure should be also used. Given that the new plane is expected to handle data with different QoS requirements, the possible protocols and their corresponding functions can be as following:

- A protocol supporting QoS mapping, segmentation, etc.

- PDCP protocol supporting security handling, compression, in-order delivery/out-of-order delivery, etc.

- RLC protocol supporting TM/UM/AM, transmission/retransmission, segmentation, etc.

- MAC protocol supporting multiplexing/demultiplexing, scheduling, etc.

Proposal 7: RAN2 to study the possible protocol stack and functionalities of each sublayer if the new Data plane is introduced.

- Conclusions

In this contribution, we analyze the possible requirements and functionalities of 6G system data handling. Based on the analysis, we give the initial consideration on radio protocol and make the following observations and proposals:

Observations

Observation 1: SA1 defines 6G system data which is generated and controlled by the 6G system, including AI/ML data, sensing data, energy efficiency information and NF context.

Observation 2: Data collection and data delivery are important functionalities of AI/ML and sensing, and NR has been discussing data collection and data delivery mechanisms for specific use cases.

Observation 3: It is possible that RAN could be aware of the data content if standardized data is introduced.

Observation 4: It is efficient if RAN could adapt the collected data accordingly for different performance improvement purpose.

Observation 5: For 6GR, there are two options for unified data handling:

Option 1: introduce the data plane for 6G system data;

Option 2: enhance the user plane or control plan for 6G system data.

Observation 6: SA2 is studying how to identify the data type and requirements, designing the end-to-end system architecture, defining the procedures and supporting policy control/QoS for 6G data framework.

Proposals

Proposal 1: Discuss to use similar SA1 terminology to describe various types of data: 6G system data which is generated and controlled by the 6G system, e.g. AI/ML data, sensing data.

Proposal 2: RAN2 to study how to support the Data Collection, Data Delivery, and/or Data Management to fulfil the data service requirements.

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

–	Data Management: The RAN node could be aware of the data and utilize data for various purposes, e.g., performing AI-enabled data analysis and adapt accordingly for performance improvement.

Proposal 3: For 6GR, RAN2 to study the possible functionalities of Data Collection, which include: controllability, UE selection, and visibility.

Proposal 4: For 6GR, RAN2 to study the possible functionalities of Data Transfer, which can include: QoS mapping, security policy, compression, (re)transmission, segmentation, in-order/out of order delivery, unicast/multicast mode etc.

Proposal 5: RAN should study how to support the Data Management, including duplication detection, data storage and data distribution, to support efficient data utilization.

Proposal 6: 6GR should study whether and how to introduce the data plane or how to enhance the user plane or control plan to support the functionalities of data handling.  And coordination with SA2 is necessary.

Proposal 7: RAN2 to study the possible protocol stack and functionalities of each sublayer if the new Data plane is introduced.

- References

- RP-251881

- RAN2#131-bis Chairman notes

- TR 22.870, “Study on 6G Use Cases and Service Requirements”, 3GPP

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP

- TR 23700-04, “Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML); Phase 2”, 3GPP

- R2-2508589