3GPP TSG-RAN WG2 Meeting #132	R2-2508602

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	8.7.2

Source:	HONOR

Title: 	Discussion on the issue of RLC-H02

Document for:	Discussion and Decision

1	Introduction

In the summary of RLC open issue for R19 XR [1] before RAN2#131bis, the following issue is discussed:

| RLC-H02 | In section 5.3.4, the content when STATUS PDU is triggered by other triggering conditions are reused. However, when t-RxDiscard expires, RX_Next is updated to RX_Next_Discard_Trigger and after the update, RX_Next might be larger than RX_Highest_Status. Hence, the following condition does not hold anymore. - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: Hence, when the status report is triggered by t-RxDiscard, a straightforward solution would be just to set the ACK_SN to the updated RX_Next. Then, the SDUs before RX_Next are indicated as ACK The TP can be follows: When constructing a STATUS PDU not triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== When constructing a STATUS PDU triggered by the expity of t-RxDiscard, the AM RLC entity shall: - set the ACK_SN as the updated RX_Next [Rapp]: As you mentioned, when RX_Next is larger than RX_Highest_Status, the condition of “RX_Next <= SN < RX_Highest_Status” is not satisfied. Therefore, the current specification means only ACK_SN will be included in the STATUS PDU, i.e. the current specification works well and we think no change is needed. |
| --- | --- |

In the RAN2#131bis [2], RAN2 discussed offline RLC-H02 and confirmed that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

In this contribution, we discuss the potential solutions related to this issue.

2	Discussion

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall, at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running. This single STATUS PDU should include the SN of all RLC SDUs that have been positively acknowledged, negatively acknowledged and discarded.

Observation 1: The STATUS PDU should include the SN of all RLC SDUs that have been positively acknowledged, negatively acknowledged and discarded.

When t-RxDiscard expires, RX_Next is updated to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received. And after the update, RX_Next might be smaller than RX_Highest_Status as shown in Fig.1. If the AM RLC entity just sets the ACK_SN as the updated RX_Next (SN=10 in Fig.1) when t-RxDiscard expires, it may result in the SN of RLC SDUs (SN=10~12) that have not been completely received not being included in STATUS PDU.

Fig.1 When t-RxDiscard expires, RX_Next is updated and smaller than RX_Highest_Status.

Observation 2: When t-RxDiscard expires, RX_Next may be smaller than RX_Highest_Status, and in this case STATUS PDU should include SNs that RX_Next <= SN < RX_Highest_Status.

Therefore, the condition for constructing a STATUS PDU with only ACK_SN (positively acknowledged and discarded) should not be “triggered by the expiry of t-RxDiscard”.

We suggest to use “if RX_Next < RX_Highest_Status”. The TP can be follows:

When constructing a STATUS PDU, the AM RLC entity shall: - if RX_Next < RX_Highest_Status: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== - else: - set the ACK_SN as the updated RX_Next.

Proposal 1: (RLC-H02) If RX_Next >= RX_Highest_Status, the AM RLC entity shall	set the ACK_SN as the updated RX_Next when constructing a STATUS PDU.

3	Conclusions

In this contribution, we made the following proposal:

Observation 1: The STATUS PDU should include the SN of all RLC SDUs that have been positively acknowledged, negatively acknowledged and discarded.

Observation 2:  When t-RxDiscard expires, RX_Next may be smaller than RX_Highest_Status, and in this case STATUS PDU should include SNs that RX_Next <= SN < RX_Highest_Status.

Proposal 1: (RLC-H02) If RX_Next >= RX_Highest_Status, the AM RLC entity shall	set the ACK_SN as the updated RX_Next when constructing a STATUS PDU.

4	References

- [POST131][507][XR] Final 38.322 CR (vivo)——R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp.docx

- R2_131bis XR and LTE Broadcast notes (Dawid)_2025-10-15 1615