3GPP TSG-RAN WG2 Meeting #132	R2-2508650

Dallas, US, 17th–21st  November, 2025

Agenda item	: 10.3.1.3

Source	: Tejas Networks

Title	: Discussion on UL Scheduling in 6G

Document for	: Discussion

# Introduction

At RANP#108, the 6G study item was agreed [1], where Radio interface architecture related part of the study item is captured as follows:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

At RAN2#131bis, RAN2 reached the following initial User Place (UP) agreements [3]:

Agreements on UP

1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2. UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This paper discusses issues with current UL scheduling in 5G and how these issues can be addressed using contention- based UL grant in 6G.

# Discussion

When uplink (UL) data arrives at the User Equipment (UE) and if no UL resources are currently available for transmission, in this situation, which recurs repeatedly especially for DL-heavy traffic or UL sporadic traffic, the NR UE transmits a scheduling request (SR) on PUCCH to the gNB using pre-configured dedicated SR resources. SR is used to notify the gNB of the UE’s need for UL resources. After receiving the SR, the gNB is only aware that the UE has data pending for transmission, but it lacks information regarding the volume of data in the UE’s UL buffers.

To avoid inefficient utilization of radio resources, the gNB typically responds by allocating a small UL grant, sufficient for the UE to transmit a Buffer Status Report (BSR). The BSR provides the gNB with an indication of the amount of UL data awaiting transmission in the UE’s buffers. Upon reception of the BSR, the gNB can then allocate an appropriately sized UL grant, enabling the UE to transmit its buffered data as shown in Figure 1. The latency introduced by the initial uplink (UL) grant procedure triggered via SR and BSR is a significant contributor to overall transmission delay. The SR is just a single bit indication which the PUCCH encodes efficiently. But since dedicated SR PUCCH resources must be assigned to all UEs, they recur typically only every 5-20 ms, introducing additional latency.

In 5G, configured grant (CG) was introduced to avoid this issue by pre-configuring UL grant to UEs. CG is very useful for services with periodic and deterministic size traffic like Voice or URLLC, but it is not useful for UL data which arrives at irregular intervals. Therefore, a more robust scheduling mechanism is needed in 6G to address UL scheduling latency. Contention-based (CB) uplink grant is one such feature that can potentially solve this issue.

Figure 1 UL Data Transmission using SR/BSR

Using Contention Based (CB) shared resources, uplink synchronized UEs can transmit uplink data without sending Scheduling Request in advance. That would reduce both the latency and the signaling overhead. gNB configures UEs with CB configuration such as CB-RNTIs, CB grant etc before providing the UL CB grant. The process for UL data transmission using UL CB grant is shown in Figure 2.

Proposal 1: RAN2 shall study UL scheduling enhancements like contention-based uplink resources to enable lower latency.

Figure 2 UL data transmission using CB grant

In step2 of Figure 2, UL CB grant is shared by multiple UEs, if several UEs s attempt a contention-based uplink transmission upon receiving the same contention-based uplink grant i.e., on the same set of uplink resources allocated by a contention-based uplink grant, a transmission collision results and it is likely that none of the transmissions will be successful. To use CB grants efficiently, further study is needed to reduce collision probability among users contending for CB grant.

Proposal 2: RAN2 shall study mechanisms to reduce collision probability in contention-based UL scheduling.

# Conclusion

Proposal 1: RAN2 shall study UL scheduling enhancements like contention-based uplink resources to enable lower latency.

Proposal 2: RAN2 shall study mechanisms to reduce collision probability in contention-based UL scheduling

# References

- RP-251881, Study on 6G Radio.

- RP-252912, Revised SID: Study on 6G Radio

- 3GPP RAN2 #131bis Chair Notes (2025.10).