3GPP TSG-RAN WG2 #132	R2-2508768

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.2.1

Source:	InterDigital, Inc.

Title:	Design of 6GR UE capabilities

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion

# Introduction

RANP #108 approved a study item (SI) Description (SID) for 6G Radio (6GR) [1]. This contribution discusses various capability signalling aspects, and explores some potential improvements compared to NR.

# UE capability framework

6GR must define a clear framework for managing UE capabilities. Effective capability definition and management are central to achieving interoperability, scalability, energy efficiency, and signalling optimization.

## Minimum / Mandatory Set of UE Capabilities

The framework should define the smallest set of mandatory features common to all UEs. This baseline will be derived from network operation, scalability, deployment scenarios, and energy efficiency considerations.

Several companies propose defining device categories or types (e.g., eMBB, RedCap, XR, Satellite/NTN, IoT), each with a clearly specified baseline capability set. Each baseline should include the minimal functions necessary for basic attach, paging, and data transfer. This approach ensures that new device families can be introduced without significant signalling changes.

6GR UE capability design should therefore include:

- A minimum baseline capability set supported by all UEs.

- Device-type–specific mandatory capability sets extending the baseline.

Proposal 1: 6GR UE capability design should include a minimum baseline capability set supported by all UEs, and device-type specific mandatory capability sets.

## Other UE Capabilities

Beyond the baseline, the framework should manage capabilities through function- or feature-based groupings with minimal interdependency.
A “function” or “feature” should distinguish one device type from another — for example, mobility, fixed wireless access, wearables, or IoT.
Grouping such features under common signalling bits (e.g., bits indicating mobility support with default, minimum, or extended levels) simplifies exchanges while allowing UEs to signal support beyond the baseline.

The framework should support both mandatory and optional features, enabling flexible grouping to form device types or feature sets.

Proposal 2: Optional feature indicators can be used to signal differences (“delta”) relative to the baseline device-type capability set.

## Device-Type Based Capability Sets

Each device type can represent a predefined set of UE capabilities that characterize the expected functionality for a given category/type of device (e.g., eMBB, XR, IoT, RedCap, Satellite/NTN).

During registration or UE capability exchange, the UE reports the list of supported features and/or device types to the network.

The network could then assign device-type identifiers (DT-IDs) and provide the associated configuration profiles for each type. Each configuration profile represents the baseline capabilities for that device type—including L1/L2/L3—with optional extensions that can be configured dynamically, for example based on optional capabilities reported on top of the device type baseline.

In essence, this allows the network to assign the same capability profile to UEs of multiple types. For example a UE supporting feature x, y, z can be configured as the same UE type as a UE supporting only features x, y (but not z). UE type X would therefore be a device supporting feature x, y. A single ID can be used to identify this capability set.

Another way would be to simply define device types in the specification, and their associated minimum feature set.

Proposal 3: Support network configurable as well as pre-defined (in the standard) capability sets.

## Two-Stage Capability Definition

One aspect that has always been problematic, is the lack of commercial availability at the time of completing work items/releases. In NR, to allow maximum implementation flexibility, all new features are defined as optional, with the necessary signalling defined upon freezing the release. This results in a massive number of options, and a huge number of different devices supporting different feature combinations. In many cases, the capability signalling is redundant, because in practise that most features contain a core set of features which all devices and vendors need to support for the feature to work correctly, and a set of optional enhancements that most do not implement.

A staged approach could instead be adopted for defining device-type capabilities:

Stage 1 – Reserve bits

- Upon closing a release, 3GPP specifies the complete list of optional features for a release. Instead of also defining all of the capability signalling, instead reserve signalling bits or containers (i.e. define configurable capability signalling) for future capability extensions/definitions.

Stage 2 – Define device type minimum capability set, and optional capability bits:

- Vendors or product implementers define practical device types as subsets of those standardized features, grouping them into deployable capability profiles (for example, mission-critical, IoT, XR, or NTN devices). For example, once more than one chipset and network vendor have completed testing, then the definition of what the minimum feature set is for that feature (e.g. the minimum supported by all vendors) and the optional features (supported by only some vendors) can be made. This two-stage design allows flexibility during specification and implementation, while minimizing capability signaling overhead by basing the final definitions on practical deployment.

Proposal 4: Consider the possibility to define configurable capability signalling at the end of a release (e.g. using reserved bits), then later define those capabilities once practical deployments are known.

## Modular Capability Definition

To manage the increasing diversity of UE capabilities in 6GR, a modular capability framework is proposed. This approach extends the traditional RRC configuration model to handle capability signalling and configuration in a structured, scalable manner.

Capabilities may be organized into modules reflecting functional granularity and feature grouping:

- The Base Capability Module defines the mandatory set of capabilities common to all 6GR UEs.
It includes fundamental L1/L2/L3 functions, baseline mobility, RRC connectivity procedures, and essential QoS and security features required for registration and data operation.

- Derived Capability Modules define additional capabilities that are optional or device-type–specific, such as:

  - High-throughput and extended bandwidth support (eMBB).

  - Low-latency or low-power modes (IoT, RedCap).

  - AI/ML-assisted operations and sensing-related capabilities.

  - Multi-carrier or NTN-related capabilities.

Each derived module can be independently activated, modified, or deactivated based on the network configuration, service requirements, or UE operating mode. The UE and network exchange compact identifiers (module IDs) or delta profiles rather than re-signaling the full capability set. A UE would always be configured with the Base Capability Module and may be provisioned with Derived Modules according to its supported device type(s). Capability modules can be defined in ASN.1 as separate, self-contained structures, enabling independent evolution across releases. The network may request, activate, or deactivate specific capability modules using lightweight RRC signalling or dedicated MAC control elements. Optional features within each module can be signalled via delta updates relative to the baseline configuration.

This modular design allows the 6GR system to support a wide range of UE implementations—from minimal IoT devices to advanced AI-driven systems—without redefining the entire capability framework for each new feature introduction.

Proposal 5: Support module-based capability reporting and configuration procedures using delta and reference profiles.

## Dynamic Capability Management

Dynamic capability management allows a UE to adapt its active capability set or device type during operation with minimal signalling overhead. This feature may be necessary to support multi-mode devices (e.g. devices that can act as an IoT device or an eMBB device), devices with changeable form factor (e.g. foldable phones), and devices with temporary hardware constraints such as overheating. With network control and under certain conditions it may be not impact NW complexity in managing cell-wide capabilities and reconfigurations.

A UE could be provisioned with multiple capability configurations or device-type profiles (e.g., eMBB, XR, IoT, NTN). Each configuration is associated with a unique identifier (e.g., Device-Type ID or Capability-Set ID) and would include the relevant base and derived modules for that profile.

Dynamic switching enables the UE to change its active configuration quickly based on conditions such as:

- Service or application triggers: e.g., switching from an IoT profile to an XR/eMBB profile when initiating a high-bandwidth service.

- QoS or latency requirements: e.g., activating enhanced reliability or throughput modules when a specific QoS flow is initiated.

- Power or thermal constraints: e.g., switching to a low-power configuration when entering an energy-saving state.

- Network triggers: e.g., upon reception of a specific RRC message or MAC CE indicating the required capability profile or device-type ID, potentially after UE request.

The network can command or acknowledge these changes using a short message (e.g. RRC message or MAC message with limited content) containing the device-type ID or module ID.

In summary, the network maintains a list of configured device types or capability sets per UE, each identified by a unique ID. The UE stores the configurations corresponding to these IDs locally and can activate them upon trigger conditions or network indication. Switching between profiles is achieved through signalling that references only the ID or a delta configuration.

Proposal 6: Support event triggered (UE or NW-initiated) capability change and reconfiguration procedure under network control.

## ASN.1 Optimisation

To ensure efficient and scalable capability signaling in 6GR, ASN.1 structures should be simplified and modularized to reduce message size and processing complexity.

The following principles are proposed:

- Flattened hierarchy: Reduce deep nesting of IEs to minimize encoder/decoder complexity and avoid redundant optional layers.

- Modular containers: Define separate ASN.1 containers for Base and Derived capability modules, aligned with the modular capability framework.

- Reference and delta signalling: Support capability reporting by referencing predefined profiles (e.g., device-type IDs or module IDs), with optional delta fields to indicate deviations from the baseline.

- Explicit extension points: Include clear extension markers and version control to enable future capability additions without redefinition of existing structures.

- Compact representation: Apply defaults and bounded OPTIONAL fields to limit message size and maintain deterministic parsing.

Proposal 7: Adopt ASN.1 structural simplification and encoding methods that support Modular capability containers (Base/Derived modules), reference-based and delta capability reporting, and forward-compatible extension and compression mechanisms.

# Conclusion

This document has made the following observations and proposals:

Proposal 1: 6GR UE capability design should include a minimum baseline capability set supported by all UEs, and device-type specific mandatory capability sets.

Proposal 2: Optional feature indicators can be used to signal differences (“delta”) relative to the baseline device-type capability set.

Proposal 3: Support network configurable as well as pre-defined (in the standard) capability sets.

Proposal 4: Consider the possibility to define configurable capability signalling at the end of a release (e.g. using reserved bits), then later define those capabilities once practical deployments are known.

Proposal 5: Support module-based capability reporting and configuration procedures using delta and reference profiles.

Proposal 6: Support event triggered (UE or NW-initiated) capability change and reconfiguration procedure under network control.

Proposal 7: Adopt ASN.1 structural simplification and encoding methods that support Modular capability containers (Base/Derived modules), reference-based and delta capability reporting, and forward-compatible extension and compression mechanisms.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, CR, June 9-13, 2025.