3GPP TSG-RAN WG2 Meeting #132 	R2-2508782

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	9.2.2

Source:	Panasonic

Title:	Considerations for Deployment Scenario 2 with Topology 2

Document for:	Discussion

# Introduction

In the previous meeting, RAN2 started the Rel-20 work item on A-IoT phase 2. One important task led by RAN2 is to specify support for Deployment Scenario 2 with Topology 2 (D2T2, see RT 38.848) with intermediate UE as Reader in Rel-20. The following agreements were achieved in the previous meeting.

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this paper, we discuss some further considerations for supporting D2T2, including device addressing over Uu interface, resource time domain property and transaction ID

# Discussion

## Device addressing over Uu

In Rel-19 (topology 1), AS ID is used for addressing the device in A-IoT interface (i.e. between gNB reader and device) at least for inventory + command case.

It has been specified in Rel-19 that AS ID is assigned by reader. It is up to reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. For CBRA, this procedure is done by Msg2. For CFRA, this is done by first R2D command message transmission.

For the command addressing between gNB reader and CN (AIOTF), NGAP ID is used. It is the gNB reader who generates the NGAP ID and manages the mapping between NGAP ID and AS ID. After generating the NGAP ID, gNB reader reports it to AIOTF in inventory report. Then for the follow-up command request, AIOTF use NGAP ID for addressing, and gNB reader translates it back to corresponding AS ID which is used in the A-IoT air interface.

However, in topology 2, reader and gNB are separate. The following questions need to be discussed for topology 2: Which node should generate AS ID; which node should generate NGAP ID; whether there is a need for addressing A-IoT device over Uu interface and if so, what ID should be used.

Which node should assign AS ID

In our view, it is reasonable to let UE reader generate AS ID. If AS ID is generated by gNB in topology 2, some interaction between gNB and UE reader is required. For example, in CBRA case, after reader receives Msg1, it needs to inform gNB that a RN16 is received by Msg1, and gNB then assigns a AS ID and informs it to the UE reader. This not only introduces additional signalling overhead via Uu interface, but also contributes to prolonging the completion of random access. Therefore, AS ID should be assigned by UE reader itself. To avoid the situation that different UE readers choose the same AS ID, gNB may configure non-overlapping ID range to UE readers.

Proposal 1. For topology 2, AS ID is assigned by UE reader using the same message as Rel-19.

Which node should assign NGAP ID

In the last RAN plenary, it has been agreed to select RRC-based solution to support topology 2. In this case, gNB should have enhanced NGAP protocol layer to support A-IoT functions. After gNB receives the service request from CN, gNB will trigger UE reader to perform A-IoT operation via NR Uu RRC. As the NGAP interface is terminated at gNB, gNB should be responsible for assigning NGAP ID for A-IoT device and uses it in communication with CN (AIOTF). Since this may be decided by RAN3, RAN2 can make such assumption to discuss other related aspects, such as addressing over Uu.

Proposal 2. RAN2 assumes for topology 2, NGAP ID is assigned by gNB.

Whether there is a need for addressing A-IoT device over Uu

For inventory-only service, the procedure ends when CN obtains all the inventory reports from gNB. The inventory report would contain a list of A-IoT NAS PDU with each item encapsulate one A-IoT device (permanent) ID. Neither UE reader nor gNB need to decode it. There is no further DL data transmission to UE reader related to this A-IoT inventory service. Therefore, no addressing is needed in Uu interface. In this case, NGAP ID is also not required to be generated.

The following figure in TR 38.769 illustrates the message flow for inventory only service for topology 2.

On the other hand, if there is a follow-up command service, both NGAP ID and addressing in Uu are needed, in order to reach the target device.

Observation 1. For inventory-only service, addressing device via Uu interface is not needed. But for command service, device needs to be identified over Uu interface.

What ID for addressing A-IoT device over Uu

One solution is to use AS ID. In this case, UE reader needs to report AS ID to gNB (e.g. in the above step 4a/4b. RRC inventory report). Then gNB generates NGAP ID and manages mapping between AS ID and NGAP ID, like in Rel-19.

However, this may introduce additional signaling overhead over Uu.

Second solution is to use local index associated with each item in the RRC inventory report (in step 4a/4b). For example, one UE reader reports the following list to gNB:

Index 1: A-IoT device 1 NAS PDU

Index 2: A-IoT device 2 NAS PDU

…

In this case, UE reader needs to maintain a mapping table between the AS ID and index. See below for example:

AS ID #1: index 1

AS ID #2: index 2

…

On the other hand, gNB generates NGAP ID and maintains the mapping table between index and NGAP ID. See below example:

Index 1: NGAP ID#1

Index 2: NGAP ID#2

…

The advantage of second solution is to reduce the signaling overhead caused by transmitting AS ID from UE reader to gNB. But the price is that UE reader needs to keep a mapping table between index and AS ID.

Proposal 3. RAN2 to decide on what ID should be used for addressing A-IoT device over Uu:

- AS ID, or

- Entry index associated with RRC inventory report

## Resource allocation to UE reader

In the previous meeting, the discussion started with a question whether a timer is needed to determine the validity of the A-IoT radio resources allocated by the gNB to UE reader. It turns out this is actually related to what the resource looks like in time domain. From our perspective, the following cases can be considered:

Case 1: continuous and “infinite” in time

- The resource allocation only configures the resource in frequency domain. It is infinitely valid in time domain, until a release message (or a new resource allocation message) from gNB is received by the reader.

Case 2: continuous and “finite” in time

- The resource allocation configures both frequency and time. In time domain, the resource is continuous and valid for a time duration. After time duration, it becomes invalid without receiving additional release message.

Case 3: discontinuous and “infinite” in time

- The resource allocation configures both frequency and time. In time domain, it has periodicity (within one periodicity, resources are valid for some slots and invalid for rest of slots).

Case 4: discontinuous and “finite” in time

- The resource allocation configures both frequency and time. In time domain, it has both periodicity and validity duration. The periodicity is repeated until validity duration is reached

As seen from the above 4 cases, there are two aspects related to time domain to be discussed: finite vs infinite, and continuous vs. discontinuous. For the first aspect, the choose may depend on how much error protection in temporary out of connection situation. For example, when UE reader is in RLF, if the allocated resource is considered valid. The validity should not be forever. Therefore, to configure it “finite” in time e.g. using a timer is justified. On the other hand, if the allocated resource is considered invalid during and after RLF, to configure it “infinite” in time without timer may work. However, for robustness and to avoid different understanding between gNB and UE about the RLF timing, it is better to restrict the resource configuration to be finite in time.

Proposal 4. The A-IoT resources configured to UE reader should be finite in time, i.e. it becomes invalid after some time duration without releasing message.

The second aspect is continuous vs discontinuous. In our view, it may depend on RAN1 design of how/whether periodic/quasi-periodic system information is sent, and how much flexibility is allowed on when R2D or D2R is sent related to timing/frequency synchronization signal. One way of design is that gNB sends resource configuration message to UE only once, which configures a pattern consisting of one time duration for A-IoT operation and one time duration for Uu communication. Alternative way would be gNB sends multiple resource allocation messages (based on demand for A-IoT services from CN), each message only provides UE a time duration for A-IoT resources. The first method would be discontinuous and second method would be continuous resource. The debating point is that method 1 gives more flexible for UE reader, however, it may waste resources because there could be no demand for A-IoT service but resources are reserved periodically. Method 2 is demand based, but more signalling overhead for providing the resources. In general, if RAN1 decides on short periodicity of timing/frequency synchronization signals, discontinuous resources can be allocated. Otherwise, on-demand assignment for continuous resources would be sufficient. RAN2 can wait for RAN1 progress.

Proposal 5. RAN2 wait for RAN1 progress on periodic/quasi-periodic system information discussion before deciding on whether the A-IoT resources should be configured continuous or discontinuous in time.

## Reader ID and transaction ID

Since Rel-19 deployment considers only D1T1 where reader is fixed base station, it is feasible to place BS reader well separated in order to ensure no overlapping multi-reader scenario. However, Rel-20 deployment supports D2T2 where UE is a reader. Since UE reader can be moving around, it is impractical to assume that the coverage of different readers is always non-overlapping. In other words, it could happen that one A-IoT device receives paging messages to perform A-IoT operation from different readers.

For inventory service, it is sufficient for device to respond to one (“best”) reader and ignore the paging from other readers to save the energy and signaling overhead. In Rel-19, transaction ID is introduced to avoid duplicated responses. More specifically, for CBRA, if device sees in the paging message the same transaction ID as the one that device has successfully responded before, device will not respond in order to avoid duplicated responses. In Rel-19, how to generate transaction ID is up to reader implementation (i.e. not specified).

In multi-reader scenario, if transaction ID is generated by reader implementation as Rel-19, it is likely different transaction IDs will be used by different readers even if the paging is associated with the same CN request. If the device cannot determine whether the different transaction IDs are resulted from different services or different readers (but same service request), it is effectively requested that the device always needs to respond to multiple readers even for the same service request.

On the other hand, the length of transaction ID is 6-bit per Rel-19 design. This gives some room to incorporate reader identity. We suggest to use part of transaction ID, e.g. 2 bits, for reader identification purpose. It should be clearly specified which part of transaction ID is related to service identification and which part for reader identification. With this separation, device can skip the redundant responses if the transaction ID is related to the same service request.

Proposal 6. RAN2 to agree to specify the composition of transaction ID: which part is for reader identification and which part is for service identification.

Furthermore, since A-IoT operation does not have a “serving and non-serving reader” concept per Rel-19 design, some rule needs to be defined for device to know the device-reader association for multi-reader scenario in Rel-20. With such association, device will respond only to the corresponding reader until such association is changed. For example, the association should be regulated by a timer to avoid ping-pong. Before timer expires, the device only responds to the paging from the same associated reader, and ignore the paging from other readers regardless of the same or different service request.

Proposal 7. RAN2 to discuss the need to introduce device-reader association, and whether/how device-reader association is kept for a certain time.

# Conclusion

In this paper, we have discussed some considerations for supporting D2T2. The proposals are as follows.

Device addressing for topology 2

Proposal 1. For topology 2, AS ID is assigned by UE reader using the same message as Rel-19.

Proposal 2. RAN2 assumes for topology 2, NGAP ID is assigned by gNB.

Observation 1. For inventory-only service, addressing device via Uu interface is not needed. But for command service, device needs to be identified over Uu interface.

Proposal 3. RAN2 to decide on what ID should be used for addressing A-IoT device over Uu:

- AS ID, or

- Entry index associated with RRC inventory report

Resource allocation

Proposal 4. The A-IoT resources configured to UE reader should be finite in time, i.e. it becomes invalid after some time duration without releasing message.

Proposal 5. RAN2 wait for RAN1 progress on periodic/quasi-periodic system information discussion before deciding on whether the A-IoT resources should be configured continuous or discontinuous in time.

Reader ID and transaction ID

Proposal 6. RAN2 to agree to specify the composition of transaction ID: which part is for reader identification and which part is for service identification.

Proposal 7. RAN2 to discuss the need to introduce device-reader association, and whether/how device-reader association is kept for a certain time.