3GPP TSG-RAN2 Meeting #132		         	  	      R2-2508845

Dallas, USA,  Nov. 17th - 21st, 2025

Agenda item:	10.3.2.1

Title:	Discussion on 6G RRC states

Source:	CMCC

Document for:	Discussion

Introduction

In RAN2#131bis meeting reached the following conclusions[1]:

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we analyze RRC_INACTIVE State and discuss the introduction of a new Energy Efficient Sub-state, focusing on its design path and exploring how it fulfills the functionalities outlined in the previous meeting regarding UE mobility, context management, energy efficiency, and fast state transitions.

Discussion

Analysis of RRC_INACTIVE State

5G standalone has been widely deployed in our network. But RRC_INACTIVE state has not been enabled during 5G era. In last meeting, we have shared the reasons from our perspective:

- Network planning complexity for RNA. A key obstacle lies in architectural inconsistency. Tracking area is widely used in 4G and 5G. In the 5G architecture, RRC_IDLE mobility is managed by the AMF using a TA list, a mature and stable mechanism inherited from LTE, and its planning area is generally unchanged from 4G to 5G. However, RRC_INACTIVE introduces a parallel RAN-managed RNA, whose area requires replanning, resulting in architectural inconsistencies and configuration complexity. Furthermore, when the UE executes an RNAU, it may trigger UE context retrieval procedure, which require Xn interface available within RNA. Such requirement makes the RNA planning even more complex.

- No power saving gain comparing to IDLE state. From an energy-saving perspective, RRC_IDLE can already provide significant long-term power savings for a wide range of latency-insensitive services. For example, IoT applications like smart metering can report data over several hours or even days. In these scenarios, the deep sleep provided by the RRC_IDLE state maximizes battery life. Therefore, many scenarios don't require the rapid resuming provided by RRC_INACTIVE. The RRC_IDLE state not only fully meets these performance requirements but also offers superior energy efficiency over its entire life-cycle.

- Lack of 5G use case requiring low state transition delay. From a latency perspective, the low "state transition latency" is one of the advantage for RRC_INACTIVE state. However, in 5G, we haven’t recognized a use case that requires low state transition delay. Even URLLC service is more sensitive on per packet delay, rather than state transition delay. For example, URLLC services, to ensure extreme end-to-end latency per packet and high reliability, cannot tolerate the control plane latency introduced by waking up and resuming the connection from RRC_INACTIVE. Therefore, terminals carrying URLLC services actually remain in the RRC_CONNECTED state for extended periods, using mechanisms like C-DRX to balance power consumption and responsiveness. In short, truly latency-critical service scenarios do not transition to the RRC_INACTIVE state.

Observation 1: RRC_INACTIVE has not been widely enabled in 5G networks due to the following reasons: 1) Network planning complexity for RAN Notification Area (RNA); 2) Limited power saving gain compared to RRC_IDLE; 3) Lack of use cases requiring low state transition delay.

To address the above-mentioned issue(s) and enable commercial deployment and use cases for RRC_INACTIVE, the following two solutions/directions are considered:

First, further optimize context fetch to reduce control plane latency. In RRC_INACTIVE, the UE AS context is stored only in the last serving gNB. When the UE moves within the RNA, reselects to a target gNB, and initiates the RRC Resume procedure, this target gNB must initiate the "Retrieve UE Context" procedure via the Xn interface towards the old gNB to obtain the context (e.g., security keys, RRC configuration, and bearer information). This signalling interaction introduces non-negligible control plane latency, which largely offsets the low air interface latency advantage of the RRC Resume procedure itself. The SDT (Small Data Transmission) introduced in Rel-17 also has this issue. When the UE attempts to perform RA-SDT on the target gNB, the target gNB still needs to complete the context fetch first before it can decrypt and process the data transmitted by the UE via the SDT mechanism.

Therefore, for context fetch optimization, further study SDT by a more lightweight context fetch mechanism, where the target gNB retrieves only the minimal context set (e.g., security keys) required to perform SDT, rather than the complete UE context, to reduce signalling overhead.

Second, further optimize RNA planning. Regarding the reduction of the RNA area, the only potential advantage is the reduction of RAN paging load, but it does not address the network deployment complexity issue mentioned previously. On the contrary, narrowing the RNA range leads to frequent RNAU, increasing UE power consumption and signaling overhead, and making network management more complex. Therefore, this direction is not recommended.

Observation 2: Further narrowing the RNA range leads to higher network overhead and deployment complexity.

Another alternative is to enlarge the RNA range (e.g., merging it with the TA). The advantages of this solution are evident: it can significantly reduce RNAU signalling overhead and simplify network deployment complexity. This is a direction worth considering. However, this solution must address the more severe context fetch issue. If it is assumed that the RNA can be equal to the TA, and context fetch can be performed via either the 6G-AMF through the NG interface or the Xn interface, whether this introduces additional complexity and can meet the CP latency requirement remains to be studied. Another issue is that the enlarged RNA may make it more difficult to address the context fetch latency.

Proposal 1: For the RRC_INACTIVE state evolution, the following two directions are considered:

- Context fetch optimization: further study SDT by a more lightweight context fetch mechanism.

- Enlarge the RNA range (e.g., merging it with the TA): it needs to be studied whether context fetch via the 6G-AMF through the NG interface introduces additional complexity and CP latency.

Study of a New Energy Efficient Sub-state

Core Concept

Unlike the RRC_INACTIVE state, which has no air interface connection, this Energy Efficient Sub-state aims to maintain an extremely low-overhead air interface connection, thereby avoiding the need to go through the full random access and RRC resume procedures for each small data transmission. Furthermore, taking 5G SA network as an example, a single cell can support up to 2400 connections. However, according to live network data, the number of actual maintained air interface connections is far below this upper limit, indicating that a lightweight connected Energy Efficient Sub-state is feasible.

Therefore, we propose to introduce an Energy Efficient Sub-state, which is specifically designed for scenarios where the UE is in RRC_CONNECTED but there is no active data transmission.

When the network transitions the UE to Energy Efficient Sub-state, it configures the UE with a cell list and pre-configurations for the cells within that list. This pre-configuration information can include dedicated resources within these cells: for example, contention-free uplink resources for obtaining (early) TA or notifying the network of arrival, PDCCH/PDSCH configuration, Configured grant, etc. To maximize UE power consumption savings, the UE can be configured with corresponding activation conditions instead of performing continuous measurements in this Energy Efficient Sub-state. The network can also configure UL/DL WUS configuration, and both the NW and the UE can achieve power saving via the WUS wake-up mechanism.

So when the UE is within the cell-list coverage area, its mobility can be handled by UE-based mobility (e.g., cell reselection or conditional reselection similar to C-LTM). gNBs within the area can pre-fetch and store the UE context. A baseline solution is for all gNBs to store the context, while other network-friendly solutions can be further studied. Meanwhile, UL/DL WUS can be used within the area to wake up the gNB or the UE, reducing power consumption for both the NW and the UE. Due to the pre-configuration, very low UE state transition latency can also be achieved. The characteristics of this Energy Efficient Sub-state are:

- Latency-free context: The UE always maintains an RRC connection, and its context (AS Security, Bearer configuration, etc.) always resides in the current serving gNB. This fundamentally eliminates the context acquisition latency of RRC_INACTIVE.

- Low state transition overhead: The transition from Energy Efficient Sub-state to RRC_CONNECTED does not involve RRC signaling (such as RRCSetup/RRCResume), but is triggered by L1/L2 signaling, resulting in extremely low latency.

- Energy saving: Both network and UE can use UL/DL WUS for on-demand wake-up. Via DL WUS, the UE can reduce monitoring of paging or scheduling. For the network, longer sleep duration can be achieved by using UL WUS combined with DTX mechanism.

Proposal 2: We propose an Energy Efficient Sub-state, applicable for scenarios with no data transmission in RRC_CONNECTED, which is characterized by:

- A pre-configured cell-list and pre-configuration for the cells therein.

- UE based mobility (e.g., cell reselection or conditional reselection similar to C-LTM).

- UL/DL WUS for energy saving for both NW and UE.

- Low state transition latency.

Required Functionalities

- UE based mobility:

The proposed Energy Efficient Sub-state mobility scheme adopts UE-based mobility, implemented via cell reselection or conditional reselection similar to C-LTM. From the UE perspective, cell reselection re-uses the existing 5G mechanism. The other new candidate solution, i.e., conditional reselection similar to C-LTM, is introduced as follows.

C-LTM technology defined in 5G-A Rel-19, constructing a "network-assisted, UE-executed" framework. Under this framework, When the network configuration execution conditions are met (e.g., the target cell quality meets specific standards), the UE will autonomously move to the target cell within the pre-configured area. Furthermore, if the UE notifies the network of its arrival at the target gNB via pre-configured CFRA resources, the network can know the precise UE location. This may save paging, achieving CP latency and network overhead reduction.

To ensure the robustness of this scheme, a clear boundary handling mechanism must also be defined, i.e., how mobility outside the pre-configured area is handled. We propose research in the following directions:

- Network-controlled handover can be accompanied by configuration updates.

- UE falls back to IDLE and performs connection reestablishment.

- UE falls back to IDLE and performs connection setup.

Proposal 3: For UE-based mobility in the Energy Efficient Sub-state, the following two solutions can be considered:

- Cell reselection.

- Conditional reselection similar to C-LTM.

- UE context and identification:

In Energy Efficient Sub-state, the UE remains in the RRC_CONNECTED state. This means that the UE's complete AS context (including AS Security, Bearer configuration, C-RNTI, etc.) is always active and live in the current serving gNB. The UE's identity is its C-RNTI, not its I-RNTI. This is a fundamental difference from the RRC_INACTIVE state, which requires the new gNB to reactively retrieve the UE context from the previous gNB via the Xn interface when the UE recovers on a new gNB. The architecture fundamentally eliminates this context retrieval latency because the context is always located on the current serving node.

Regarding how to ensure the currently serving gNB obtains the UE AS context in a timely manner, A baseline solution is:

- When the network sends pre-configuration and the UE enters Energy Efficient Sub-state, the network distributes the UE's AS context to all candidate nodes within the area. It should be noted that this method incurs significant overhead for the network.

Other network-friendly solutions can be further studied.

Proposal 4: Proactive context fetch is supported for Energy Efficient Sub-state to ensure the target gNB can acquire the AS Context in a timely manner. Meanwhile, mechanisms that are friendly to network overhead should be studied.

- Energy efficient for both UE and NW:

UE Energy Saving: In Energy Efficient Sub-state, the UE is in deep sleep and does not perform periodic measurements or reporting by default. Furthermore, existing 5G energy-saving technologies (e.g. DCP) can be combined to further reduce PDCCH monitoring overhead, achieving power consumption close to RRC_IDLE.

Network Energy Saving: As mentioned previously, introducing UL WUS combined with DTX configuration enables the network to achieve longer sleep duration for power saving.

- Fast transitions, data transfer:

This relies on the pre-configuration framework and context acquisition methods we discussed above, as well as the enhancements proposed in 2.2.3

Proposal 5: In Energy Efficient Sub-state, further energy efficiency gains for both UE and network can be pursued by reducing measurement and reporting, and via UL/DL WUS-based wake-up mechanisms.

Fast State Transition

The goal is to minimize control plane latency and data rate ramp-up latency. This relies on the pre-configuration framework and context acquisition methods discussed earlier, as well as enhancements to the following key technologies:

Minimize control plane latency

- RACH-less functionality:

Within the mobility framework outlined in 2.2.2, this RACH-less functionality should not only apply to PCells but also extend to SCells required to support 6G broadband services. Specifically, before transitioning to connected state and starting data transmission, the TA of potentially active SCells under the PCell should be obtained in advance.

- Pre-Configured Uplink Resources:

To enable the UE to initiate transmission immediately when uplink data is available without waiting for PDCCH scheduling on the network side, it is possible to study pre-configuring uplink resources for the UE in Energy Efficient Sub-state, i.e., RACH-less transmission based on configured grant (CG).

- Proactive Context Fetch:

As described in 2.2.2.

Minimize data rate ramp-up latency

- Early CSI Acquisition:

For PCell, we propose supporting an early CSI acquisition mechanism similar to that introduced in 5G-A Rel-19, allowing the UE to directly utilize uplink resources to feed back a CSI report before transitioning to connected mode for data transmission.

In addition, to support high data rate services, we propose to study early CSI acquisition for SCell. In fact, when the 5G-A Rel-20 NR_Mob_Ph5 project was initiated, the possibility of studying early CSI acquisition for SCells was discussed. However, at the time, the industry (especially UE manufacturers) was not enthusiastic about this, mainly due to unacceptable UE overhead. These objections focused on the fact that this feature would force UEs to bear huge, real costs for uncertain benefits. Specifically, this includes significant power consumption overhead, as the UE must wake up the receiver, tune the RF, and measure CSI-RS on multiple candidate SCells. Furthermore, there is the accompanying computational overhead, as CSI calculation (especially under Massive MIMO) is one of the most power-intensive operations for the UE. Finally, it would also introduce significant uplink overhead, as the UE needs to report CSI from multiple SCells, consuming valuable PUSCH/PUCCH resources and increasing signaling complexity. Under the R20 framework, this huge, uncertain UE overhead is considered unacceptable.

Although the discussions at the 5G-A R20 stage failed to reach a consensus on early CSI for SCells due to the aforementioned UE overhead issues, we believe this feature is crucial for the quality of experience of 6G services such as XR. Supporting early CSI acquisition for SCells is necessary to achieve low-latency transitions from Energy Efficient Sub-state to broadband services, and it can significantly improve UP performance. Therefore, we propose that support for early CSI acquisition for SCells should be studied in 6G.

Observation 3: Due to high UE power consumption and computational overhead, early CSI acquisition for SCell is not supported in 5G-A Release 20 LTM.

Proposal 6: It is proposed to study the support for RACH-less functionality, CG configuration and early CSI acquisition for both PCell and SCell during the transition from Energy Efficient Sub-state to RRC_CONNECTED state.

Summary

In summary, we summarize the characteristics of the proposed Energy Efficient Sub-state and compare them with RRC_INACTIVE state.

Proposal 7: Take table 1 as baseline and capture it in TR:

Table 1: Comparison of RRC_INACTIVE/Energy Efficient Sub-state Characteristics

| Feature | RRC_INACTIVE | Energy Efficient Sub-state |
| --- | --- | --- |
| UE Context | Stored in last serving gNB | Conservative: Stored in all gNBs within pre-configured area (Other overhead-friendly solutions to be studied) |
| UE ID | I-RNTI | WUS |
| CP Latency | Medium | Low |
| UE Power Consumption | Low | Low |
| Paging | RNA based paging message | Cell-list based WUS |
| Mobility Handling | cell reselection | UE-based mobility(cell reselection or conditional reselection similar to C-LTM) |

Conclusions

Observation 1: RRC_INACTIVE has not been widely enabled in 5G networks due to the following reasons: 1) Network planning complexity for RAN Notification Area (RNA); 2) Limited power saving gain compared to RRC_IDLE; 3) Lack of use cases requiring low state transition delay.

Observation 2: Further narrowing the RNA range leads to higher network overhead and deployment complexity.

Proposal 1: For the RRC_INACTIVE state evolution, the following two directions are considered:

- Context fetch optimization: further study SDT by a more lightweight context fetch mechanism.

- Enlarge the RNA range (e.g., merging it with the TA): it needs to be studied whether context fetch via the 6G-AMF through the NG interface introduces additional complexity and CP latency.

Proposal 2: We propose an Energy Efficient Sub-state, applicable for scenarios with no data transmission in RRC_CONNECTED, which is characterized by:

- A pre-configured cell-list and pre-configuration for the cells therein.

- UE based mobility (e.g., cell reselection or conditional reselection similar to C-LTM).

- UL/DL WUS for energy saving for both NW and UE.

- Low state transition latency.

Proposal 3: For UE-based mobility in the Energy Efficient Sub-state, the following two solutions can be considered:

- Cell reselection.

- Conditional reselection similar to C-LTM.

Proposal 4: Proactive context fetch is supported for Energy Efficient Sub-state to ensure the target gNB can acquire the AS Context in a timely manner. Meanwhile, mechanisms that are friendly to network overhead should be studied.

Proposal 5: In Energy Efficient Sub-state, further energy efficiency gains for both UE and network can be pursued by reducing measurement and reporting, and via UL/DL WUS-based wake-up mechanisms.

Observation 3: Due to high UE power consumption and computational overhead, early CSI acquisition for SCell is not supported in 5G-A Release 20 LTM.

Proposal 6: It is proposed to study the support for RACH-less functionality, CG configuration and early CSI acquisition for both PCell and SCell during the transition from Energy Efficient Sub-state to RRC_CONNECTED state.

Proposal 7: Take table 1 as baseline and capture it in TR:

Table 1: Comparison of RRC_INACTIVE/Energy Efficient Sub-state Characteristics

| Feature | RRC_INACTIVE | Energy Efficient Sub-state |
| --- | --- | --- |
| UE Context | Stored in last serving gNB | Conservative: Stored in all gNBs within pre-configured area (Other overhead-friendly solutions to be studied) |
| UE ID | I-RNTI | WUS |
| CP Latency | Medium | Low |
| UE Power Consumption | Low | Low |
| Paging | RNA based paging message | Cell-list based WUS |
| Mobility Handling | cell reselection | UE-based mobility(cell reselection or conditional reselection similar to C-LTM) |

References

- Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic.