3GPP TSG-RAN2 Meeting #132			R2-2508858

Location, Country, Date

Agenda item:	10.3.1.3

Title:	Considerations on scheduling and retransmission for 6GR UP

Source:	CMCC

Document for:	Discussion

Introduction

RAN#109 approved the revised 6G SID [1], where one of the primary responsibilities of RAN2 is the study of 6G user plane architecture and protocol design.

In the last meeting [2], RAN2 has conducted preliminary discussions on 6GR UP aspects and reached some consensus. This contribution will further discuss 6GR RAN-level QoS/QoE framework and Service-awareness RAN.

Discussion

Reducing 6G uplink latency is essential to support delay-sensitive applications and enable real-time interactions, such as immersive XR and industrial control.

  - UL scheduling information

In NR, uplink scheduling information serves as the prerequisite for triggering uplink data transmission, typically realized through SR in combination with DSR or BSR. However, the SR mechanism is known to suffer from several inherent limitations, e.g., latency introduced by the static SR periodicity and the occurrence of false alarms.

Still, at least two types of scheduling information are necessary for 6GR, i.e., the scheduling request information for UE without grant and the scheduling information for UE with grant.

There won’t necessarily to be exactly two dedicated information, but the functionality should be supported in 6GR.

Proposal 1: RAN2 agree that the UL scheduling information(s) should accommodate UL data transmission needs for UE without or with a UL grant, i.e., the functionality of SR and BSR/DSR.

These issues largely stem from the limited PUCCH resources available for SR transmission, and any enhancement to SR signaling inevitably involves considerations from the RAN1 perspective.

If the SR could carry richer information, e.g., a lightweight UE identifier, buffer/delay status, it would enable the RAN to make more informed and immediate scheduling decisions. In this way, the delay of “waiting SR transmission – waiting for grant – BSR reporting” could be partially or entirely bypassed, thereby reducing uplink latency and avoiding unnecessary SR false alarms.

Figure 2.1-1: Example of UL SR-BSR/DSR delay

Such an enhancement would be beneficial for most service types, especially those requiring low latency or timely uplink responsiveness.

Observation 1: Most service benefits from extending SR to capture more information, e.g., UE identifier to avoid SR false alarm and buffer/delay status for latency reduction.

Therefore, it is natural to consider enhancing the SR by allowing it to carry more information. However, it is evident that the PUCCH in NR cannot support such a large payload due to its inherent design limitations. For the PUCCH in 6GR, on the other hand, this may become feasible, but up to RAN1.

Alternatively, If PUCCH is not feasible, the UE could transmit an SR-like message directly over the PUSCH, embedding richer information including UE RNTI, delay and buffer status.

However, any of the above approaches would inevitably have impact on RAN1, particularly in terms of uplink control channel, configuration, and the coordination between PUCCH and PUSCH resources.

Proposal 2: RAN2 agrees that the UL scheduling information without grant should capture more information, e.g., UE identifier, buffer or delay status.

If the PUCCH in 6G still cannot accommodate additional information, the UL scheduling information would have to be transmitted over the PUSCH. However, in this case, when the UE does not have an effective uplink grant, it would typically need to wait for the next SR occasion, which reintroduces the previously discussed latency and detection issues.

To address this, it is worth discussing the feasibility of allowing the UE to transmit uplink scheduling information in a contention-based manner, i.e., the UE could opportunistically access limited PUSCH resources to convey essential scheduling-related information without waiting for a dedicated grant.

This approach could significantly shorten the scheduling loop, enabling faster uplink initiation and more adaptive scheduling decisions, especially in scenarios with bursty or latency-sensitive traffic.

Proposal 3: For UL scheduling information with grant, RAN2 should discuss whether it should include both buffer and delay status simultaneously and whether it can be contention-based.

Another issue of uplink scheduling information that can be improved lies in the BSR and DSR. These two MAC signaling reports share many similarities in terms of functionality, triggering conditions, and reporting procedures. In R-19, a separate DSR was introduced mainly to avoid impacting the legacy BSR design and behavior.

However, in 6GR, delay-sensitive services are expected to become more prevalent and even dominant. Given this trend, it would be reasonable to consider merging BSR and DSR into a unified MAC status report that flexibly accommodates both conventional and low-latency data needs. Furthermore, since 6GR does not have to maintain backward compatibility with NR, the convergence of BSR and DSR could simplify the uplink reporting framework and reduce UE implementation complexity. Therefore, it is suggested that RAN2 evaluate the feasibility of such an integration as part of the broader effort to streamline uplink scheduling signaling in 6G.

  - UL radio resource allocation

Legacy uplink resource allocation in NR is primarily realized through two schemes: DG which relies on PDCCH signaling for resource indication, and CG which is based on RRC configuration and activated either by RRC signaling or PDCCH.

DG is suitable for the majority of traffic types, while CG was initially introduced to reduce PDCCH overhead for services such as voice, where packet generation follows a relatively quasi-periodicity and size pattern. Later, Type-1 RRC-based CG gained broader use, as its quick activation capability allowed faster uplink transmission initiation, making it suitable for latency-sensitive applications as well.

In 6G, service types such as industrial control, UAV operation, and autonomous driving will impose even stricter latency requirements. Therefore, further reducing radio resource allocation latency will be essential to meet these service demands.

The approaches to reducing latency differ between CG and DG. For DG, most of the delay originates from the SR–BSR/DSR-based scheduling procedure discussed earlier. Hence, any effective reduction in DG latency will depend on how those mechanisms evolve. Further discussion on DG delay optimization can therefore be grounded upon the improvements proposed for uplink scheduling information in the preceding sections.

Observation 2: The allocation latency of DG is mostly related to UL scheduling information, i.e., SR and BSR/DSR, RAN2 may wait for any progress on UL scheduling information before studying low-latency DG.

For CG, uplink transmissions triggered by Type-1 CG generally exhibit lower latency compared to DG and Type-2 CG. However, the main limitation of Type-1 CG lies in its lack of transmission flexibility. Any modification of transmission parameter including periodicity, N of PRBs/RBGs or MCS requires deactivating and reactivating CG through RRC/MAC signaling, which incurs significant overhead and consumes considerable time.

Therefore, it is recommended to study enhancements to Type-1 CG that enable faster and more flexible adjustment of allocated resources. Such improvements would allow the CG configuration to adapt dynamically when the provisioned resources no longer match the traffic demand, ensuring that low-latency and high-efficiency uplink transmission can be maintained even under varying traffic conditions.

Proposal 4: RAN2 should study radio resource configuration and adjustment enhancement for CG to adopt service with bursty traffic.

  - Retransmission

In NR, retransmissions are primarily handled by two sublayers, the fast HARQ at MAC and the reliable ARQ at RLC. PDCP also provides retransmission functionality, though it is mainly used to ensure data continuity during handovers and in dual connectivity scenarios rather than for regular error recovery.

HARQ is generally efficient and its main limitation is reliability which arises from the predefined maximum number of retransmissions. Limited maximum retransmission times can lead to residual errors, which is not too rare. Beyond this, HARQ has no significant drawbacks in terms of reliability or latency.

However, a notable limitation lies in the granularity of retransmission. HARQ operates at the CBG level to avoid retransmitting correct data, whereas the granularity of data submitted from the physical layer to the MAC entity is the TB or MAC PDU. As a result, when an error occurs in one or few CBGs, the affected TB cannot be fully delivered to the MAC immediately, leading to an intrinsic delay in data propagation even when only a portion of the TB is in error.

Figure 2.3-1: Example of delay of correct CBG.

This granularity mismatch between HARQ and PHY-MAC data delivery can impact latency-sensitive applications and may warrant reconsideration in the context of 6G, where even sub-millisecond delays could be critical.

Observation 3: The granularity mismatch between HARQ on CBG level and PHY-MAC data delivery at TB level leads to additional delay for correct CBGs.

Proposal 5: RAN2 should support data exchange on sub-TB granularity between MAC and PHY, e.g., CB or CBG.

Another potential improvement lies in the ACK in PUCCH. The introduction of CBGs was originally intended to avoid retransmitting correctly received data unnecessarily. For 6GR, MAC PDUs are expected to become even larger, which makes CBGs as the minimum retransmission unit less optimal. At the same time, increasing the number of ACK bits to indicate finer-grained reception status would impose additional overhead on the PUCCH, which is already a limited resource.

Figure 2.3-2: The reduction of retransmitted data size with ACK for CB.

Therefore, it is recommended that RAN2 study enhancements to the ACK that support more flexible or finer-grained feedback without excessively increasing control channel overhead.

Proposal 6: RAN2 should discuss whether to study ACK enhancement with regarding to the exceeding size of MAC PDU and limited PUCCH resource in 6GR.

For ARQ, Although RLC ARQ provides high reliability, its polling-based mechanism in NR introduces additional time delays, as illustrated in the figure below. Moreover, the status reporting itself incurs signaling overhead, which can further impact latency-sensitive services.

Figure 2.3-3: The UL retransmission delay comparison

Another challenge for ARQ arises from the data exchange interacts between RLC and MAC entity. During retransmission, RLC must retrieve the data from its ARQ buffer and forward it to the MAC for retransmission, which results in repeated memory copying and read/write operations. These redundant memory operations not only increase processing load but also contribute to additional latency.

By consolidating the retransmission handling within a single entity, such overhead could be reduced, streamlining the data flow and improving overall efficiency. This approach would help minimize both memory access delays and processing complexity, which becomes increasingly important as packet sizes grow and services demand lower end-to-end latency in future 6G networks.

Proposal 7: To avoid unnecessary memory operation, ARQ and HARQ should be consolidated and performed on the same entity.

Conclusion

Based on above, from the perspective of CMCC, the following is observed and proposed:

UL scheduling information aspect

Proposal 1: RAN2 agree that the UL scheduling information(s) should accommodate UL data transmission needs for UE without or with a UL grant, i.e., similarity to the functionality of SR and BSR/DSR.

Observation 1: Most service benefits from extending SR to capture more information, e.g., UE identifier to avoid SR false alarm and buffer/delay status for latency reduction.

Proposal 2: RAN2 agrees that the UL scheduling information without grant should capture more information, e.g., UE identifier, buffer or delay status.

Proposal 3: For UL scheduling information with grant, RAN2 should discuss whether it should include both buffer and delay status simultaneously and whether it can be contention-based.

UL radio resource allocation aspect

Observation 2: The allocation latency of DG is mostly related to UL scheduling information, i.e., SR and BSR/DSR, RAN2 may wait for any progress on UL scheduling information before studying low-latency DG.

Proposal 4: RAN2 should study radio resource configuration and adjustment enhancement for CG to adopt service with bursty traffic.

Retransmission aspect

Observation 3: The granularity mismatch between HARQ on CBG level and PHY-MAC data delivery at TB level leads to additional delay for correct CBGs.

Proposal 5: RAN2 should support data exchange on sub-TB granularity between MAC and PHY, e.g., CB or CBG.

Proposal 6: RAN2 should discuss whether to study ACK enhancement with regarding to the exceeding size of MAC PDU and limited PUCCH resource in 6GR.

Proposal 7: To avoid unnecessary memory operation, ARQ and HARQ should be consolidated and performed on the same entity.

References

[1]	RP-252476, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2]	R2_131bis_ChairNotes_25-10-17_17-00_final