3GPP TSG RAN WG2 #132	R2-2508915

Dallas, USA, Nov 17th-21st, 2025

Agenda Item:	8.17

Source: 	Xiaomi

Title:  	Discussion on the remaining issues for the IoT NTN TDD

Document for:	Discussion and Decision

Introduction

In this contribution, we discussed the UL SPS collision, cell selection and OCC, and provide our suggestions accordingly.

Discussion

UL SPS collision

Proposal 1: Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

In the last meeting, some companies raised concerns regarding potential UL SPS collisions between different UEs when a UE postpones its UL SPS resource to the next valid Ul subframe. RAN2 agreed it can be discussed in the Nov. meeting [1].

We agree that such collisions may occur, mainly due to the limited availability of valid UL subframes, which constrains uplink resource allocation. While the possibility of UL SPS collisions exists under resource-limited conditions, we think that addressing this issue in the specification is not necessary. This is because solutions cannot completely resolve the root cause, i.e. the limited uplink resource availability. Instead, we consider that the network has sufficient flexibility to manage such scenarios through implementation specific mechanisms, such as proper scheduling or resource allocation strategies.

Proposal 1: The handling of potential UL SPS collisions should be left to network implementation, given that such collisions are caused by the limited availability of valid uplink resources.

Cell selection

In the last meeting, it was discussed that the IoT TDD cell may need to be deprioritized during UE neighbor cell measurements. The intention is to allow the UE to prioritize IoT NTN cells during cell selection or reselection. RAN2 made the following agreement [1]:

RAN2 understands if UE supports both IoT NTN band and IoT TDD band, it is up to UE implementation whether to select an IoT NTN cell or an IoT TDD cell. FFS on spec impact.

In our understanding, for cell selection, it is entirely up to UE implementation to select a suitable cell. For cell reselection, the network can configure appropriate parameters to prioritize either the IoT NTN cell or IoT NTN TDD cell. Therefore, no additional specification impact is expected to support UE selection between IoT NTN and IoT NTN TDD cells.

Proposal 2: No specification impact is identified for supporting UE selection between IoT NTN and IoT NTN TDD cells.

OCC

In RAN2#131, RAN2 briefly discussed OCC for IoT-NTN TDD mode and assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode. RAN2 has sent LS to RAN1 for confirmation. In the reply LS [2] (in R1-2508110), it is indicated that RAN1 cannot confirm RAN2’s assumption “that OCC defined for NB-IoT NTN Rel-19 can be supported for NB-IoT NTN TDD mode without any RAN1 spec impact”. Given this, we think RAN2 can conclude that OCC is not supported for IoT NTN TDD mode and this should be reflected in the RRC spec, e.g. in the field description of npusch-OCC-Enabled. Following TP can be used.

npusch-OCC-Enabled Used to enable OCC for NPUSCH format 1 single tone. E-UTRAN does not configure this field in IoT NTN TDD.

Proposal 3: RAN2 to conclude not supporting OCC for IoT NTN TDD and adopt the above TP in RRC spec.

Conclusions

In this contribution, we discussed the UL SPS collision, cell measurement and OCC, and provide the proposals as follows：

UL SPS collision

Proposal 1: The handling of potential UL SPS collisions should be left to network implementation, given that such collisions are caused by the limited availability of valid uplink resources.

Cell selection

Proposal 2: No specification impact is identified for supporting UE selection between IoT NTN and IoT NTN TDD cells.

OCC

Proposal 3: RAN2 to conclude not supporting OCC for IoT NTN TDD and adopt the above TP in RRC spec.

References

[1] R2-2507703, Report from session on NES, NR NTN and IoT NTN

[2] R1-2508110, LS reply on OCC for IoT-NTN TDD mode