3GPP TSG-RAN WG2 Meeting #132	R2-2508929

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	9.2.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT topology 2

Document for:	Discussion and Decision

1 Introduction

It was agreed in WID [1] to support scenario 2 with topology 2 for A-IoT.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and HO, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we will discuss the radio resource allocation and UE reader selection in A-IoT topology 2.

2 Discussion

## 2.1 General

The SI phase studied whether it is feasible for the UE reader to be in RRC_IDLE/INACTIVE/CONNECTED state when selecting the UE reader or performing A-IoT services, as well as the protocol impact. According to the R20 WID, only the scenario where the UE reader is in RRC_CONNECTED will be standardized. Additionally, out-of-connection cases such as RLF and handover will be addressed.

Some clarifications are needed for WID as the requirement for a UE in RRC connected state can be interpreted in two ways: either only RRC_CONNECTED UEs should be selectable as UE readers, or the UE reader needs to be in RRC_CONNECTED state only during the A-IoT service.

Proposal 1: Clarify in the WID that “RRC CONNECTED UE reader” means:

- UE reader should be in RRC CONNECTED state during the operation of A-IoT services; or

- only UE in RRC CONNECTED state can be selected as UE reader.

## 2.2 Radio resource allocation for A-IoT interface

In the last meeting, RAN2 achieved the following agreements about radio resource allocation [2]：

RAN2 #131bis Agreement: 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

During the SI phase, RAN3 discussed the signalling and procedures for solutions based on RRC, NAS and user plane. And SA2 has down selected the RRC-based solution, the detailed procedure for the RRC-based solution is outlined below. According to the last meeting agreements, the CN request triggered resource allocation is the baseline procedure. The gNB can directly configure a set of radio resources and validity criteria (if applicable) based on a service request from the CN.

Figure 1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution [3]

To allocate appropriate resources for the A-IoT interface, the AIoTF can provide additional assistance information except the existing to the gNB. This information may include service-related details to assist the gNB allocate resources effectively, such as, the estimated service complete time, the estimated service number. The signalling to transmit the assistance information can be the same with the R19 assistance information.

Proposal 2: The AIoTF should provide assistance information to the gNB to facilitate the resource allocation for the A-IoT interface.

As Topology 2 focuses solely on RRC_CONNECTED UE reader, the UE receives resource configuration via dedicated signalling. Regarding resources validity, RAN2 has discussed two validity mechanism, option 1 where resources remain valid until the network explicitly releases them, and option 2 where resources are allocated along with a time period. For option 1, this explicit release method can be utilized for resource efficiently by re-allocation to other UE readers when this UE reader has completed the A-IoT service procedure earlier than expected by the BS. For option 2, validity timer mechanism is beneficial for the UE reader’s service continuity and is complementary to above cases. E.g., when the serving cell change doesn’t occur to the UE reader, it can continue to perform A-IoT operations until timer is expired.

Proposal 3: The UE reader receives resource configuration in dedicated signalling along with a time period indicating the valid duration of the allocated resource.

Proposal 4： The UE reader determines the A-IoT radio resource invalidity when the UE reader receiving an explicit release indicator from the serving cell or the validity timer is expired.

For temporary out of connection scenarios, e.g., RLF, HO, upon RLF/HO is detected, before UE reader (re)select or handover to other cell, the configured resource is still valid. It means that the resource is valid after the resource configured and becomes invalid when the UE reader reselects or switches to a new cell. So the A-IoT services can continue to operate even during temporary out of connection.

Proposal 5: The resource configured in the latest cell is still valid during temporary out of connection.

## 2.3 Assistance information of UE reader selection

It was agreed during the SI phase that UE reader selection would be determined by RAN3 and SA2. However, for the RRC based architecture, the gNB may also be involved in the selection process, which could have RAN2 impacts.

RAN2 #128 Agreements: From RAN2 perspective, at least for RRC based architecture, there may be radio related reasons for gNB to be involved in reader selection. Whether the gNB or CN selects the reader, and/or what information is shared between gNB and CN is up to RAN3/SA2

In the SI phase, RAN3 has studied UE reader selection for RRC based architecture, and the corresponding conclusions are as follows [4]:

UE reader selection for RRC based architecture: Option 1: CN only Option 2: RAN only Option 3: CN may provide a candidate list of UE reader, RAN further select target UE reader RAN3 #125bis Agreements: In T2, all options are still open, different alternatives are possible as per call flows.

For option 1, if CN (e.g. the AIOTF) selects the UE reader, it requires assistance information of UE reader such as Uu link quality between UE reader and gNB, RRC status of UE reader, location of UE reader, and mobility state of UE reader to assist CN in selecting a more suitable UE reader. The assistance information can sourced from the AMF or the RAN.

A stable Uu link ensure that the UE reader can reliably complete the A-IoT service without interruption due to Uu RLF. As R20 specifies that only UEs in the RRC connected state support A-IoT operations, selecting the RRC connected UE as the reader can reduce the latency caused by the state transition. Moreover, selecting a UE reader that is closer to the device location and has lower mobility is also more conducive to the completion of A-IoT services.

Proposal 6: For option 1 of UE reader selection in RAN3 agreements, the assistance information for UEs capable of acting as a reader (e.g. Uu link quality, RRC state, position, mobility state) from the AMF or RAN should be considered for UE reader selection.

In both option 2 and 3, the gNB selects UE reader. The key difference is that in option 3, the CN may provide a candidate UE reader list to the gNB, whereas in option 2, the gNB can select from all reader capable UEs in the cell. When making the selection, the gNB may consider factors such as Uu link quality, RRC status, of UE reader, and mobility state of UE reader.

Consequently, a UE capable of acting as a reader would need to report assistance information (e.g., Uu link quality, position, mobility state) to the gNB. Since the gNB already knows the RRC state of the UE, it can utilize all this information to select the most suitable target UE reader for the A-IoT service.

Proposal 7: For option 2 and option 3 of UE reader selection in RAN3 agreements, a UE capable of acting as a reader should report assistance information (e.g. Uu link quality, position, mobility state) to gNB.

## 2.4 Traffic differentiate in Uu link

In topology 2, the traffic handled by the intermediate UE primarily comprises DL/UL Uu traffic and the A-IoT traffic. When the UE reader receives the inventory or command request from the AIoTF, it triggers A-IoT related procedure, such as paging, random access, and data communication. Upon completion of these RAN procedures, the UE reader forwards the A-IoT report (e.g., device ID and command report) to the gNB. As specified in TS 23.369 for R19, the NG-RAN may aggregate multiple Inventory Report message based on assistance information before reporting the response to the AIoTF.

According to the updated RAN2 WID scope, only RRC-based data transmission solution is supported, meaning A-IoT traffic is transmitted as RRC signalling. In cases where the UE reader does not aggregate inventory report message, if these reports are transmitted over the existing SRB like normal RRC signalling, it can incur frequent BSR or SR report to obtain UL grants.

To address this issue, the UE reader needs to distinguish A-IoT traffic from its regular RRC signaling. RAN2 should introduce a new Signaling Radio Bearer (SRB) dedicated for A-IoT traffic to differentiate the existing RRC signalling and A-IoT traffic.

Proposal 8: A new SRB should be introduced for UE reader for A-IoT traffic over the Uu link.

3 Conclusion

In this contribution we discussed A-IoT topology 2 with the following proposals:

Proposal 1: Clarify in the WID that “RRC CONNECTED UE reader” means:

- UE reader should be in RRC CONNECTED state during the operation of A-IoT services; or

- only UE in RRC CONNECTED state can be selected as UE reader.

Proposal 2: The AIoTF should provide assistance information to the gNB to facilitate the resource allocation for the A-IoT interface.

Proposal 3: The UE reader receives resource configuration in dedicated signalling along with a time period indicating the valid duration of the allocated resource.

Proposal 4： The UE reader determines the A-IoT radio resource invalidity when the UE reader receiving an explicit release indicator from the serving cell or the validity timer is expired.

Proposal 5: The resource configured in the latest cell is still valid during temporary out of connection.

Proposal 6: For option 1 of UE reader selection in RAN3 agreements, the assistance information for UEs capable of acting as a reader (e.g. Uu link quality, RRC state, position, mobility state) from the AMF or RAN should be considered for UE reader selection.

Proposal 7: For option 2 and option 3 of UE reader selection in RAN3 agreements, a UE capable of acting as a reader should report assistance information (e.g. Uu link quality, position, mobility state) to gNB.

Proposal 8: A new SRB should be introduced for UE reader for A-IoT traffic over the Uu link.

- References

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, Huawei, Beijing, China, September 15-18, 2025.

- R2-2500002, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA18 - 22 November, 2024.

- TR 38.769， Study on solutions for Ambient IoT (Internet of Things) in NR，v19.0.0.

- RAN3 #125-bis Meeting Report, ETSI MCC, Hefei, China, 14 - 18 Oct 2024.