3GPP TSG-RAN WG2 Meeting #132		R2-2508941

Dallas, USA,  Nov. 17th - 21st

Source:	Qualcomm Incorporated

Title:		Discussion on 6G RRC states

Agenda Item:		10.3.2.1

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses RRC state related issues in 6G.

# Discussion

## 2.0	General

In last RAN2 meeting, RRC state was briefly discussed and achieved following general agreements. This contribution continues to discuss the characteristics of RRC state.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

## 2.1   IDLE state and CONNECTED state

In NR, RRC IDLE state has the following characteristics (not an exhaustive list) :

- PLMN selection;

- Broadcast of system information;

- Cell re-selection mobility;

- Paging for mobile terminated data is initiated by 5GC;

- DRX for CN paging configured by NAS.

NR CONNECTED state has the following characteristics (not an exhaustive list) :

- NG-RAN connection is established for UE;

- The UE AS context is stored in NG-RAN and the UE;

- NG-RAN knows the cell which the UE belongs to;

- Transfer of unicast data to/from the UE;

- Network controlled mobility including measurements.

It is assumed the NR IDLE state and CONNECTED state basic characteristics will be inhereted to 6G.

- RAN2 confirms following NR RRC_IDLE state characteristics will be inhereted to 6G Idle state.

- PLMN selection;

- Broadcast of system information;

- Cell re-selection mobility;

- Paging for mobile terminated data is initiated by CN;

- DRX for paging configured by NAS.

- RAN2 confirms following NR RRC_CONNECTED state characteristics will be inhereted to 6G Connected state.

- RRC connection with RAN node is established for the UE;

- The UE AS context is stored in RAN node and the UE;

- RAN node knows the cell which the UE belongs to;

- Transfer of unicast data to/from the UE;

- Network controlled mobility including measurements.

## 2.3	  Inactive mode

#### 2.3.1	Paging

For NR Inactive state, RAN paging was introduced, and RNA was configured to the UE. When DL data arrives at the gNB, the gNB pages the UE within the RNA. If the UE moves out of RNA, UE needs to perform RNA update.

On top of IDLE state paging, the UE in NR inactive state needs additionally to implement RAN paging and perform RAN paging area management (periodically or area change triggered). UE monitors paging in IDLE and Inactive state using different paging parameters, e.g. (extended) DRX cycle, paging record ID. In IDLE state, UE needs to monitor CN paging using DRX parameters negotiated by UE and network (CN/RAN) and S-TMSI; in Inactive state, UE needs to monitor RAN paging using DRX parameters configured by RAN and I-RNTI.

Such different location area management procedures and paging monitoring procedures in IDLE and Inactive state require duplicate standards and implementation efforts in both the UE and the network.

Besides, in NR Inactive state, RAN paging delivery also depends on Xn interface deployment, and it is observed that Xn interface has not been implemented successfully today, so Inactive state has not been deployed widely.

In NR, different location area management and paging monitoring procedures for Idle and Inactive require duplicate standards and implementation efforts in both the UE and the network. RAN paging is also dependent on Xn interface connectivity which is not successfully deployed.

In order to simplify the standard efforts and UE implementation complexity on Inactive mode, it is proposed to introduce a common paging procedure for IDLE and Inactive mode.

- Paging in Inactive mode should be same as Idle state, i.e.

- Only CN-initiated paging is applicable

- Same paging DRX parameters and same paging ID

- Same paging area management

#### 2.3.2	UE context storage

In NR, UE AS context is stored in anchor RAN node, and the UE-associated NG connection with the serving AMF and UPF is kept.

If the UE accesses a gNB other than anchor gNB, the new gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB. After path swtiching, the new gNB triggers release of the UE context at the last serving gNB by means of the XnAP UE Context Release procedure. In addition, during RNA update procedure when the UE moves out of the configured RNA, the new gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the anchor gNB.

It can be seen from the existing UE context retrieval procedure, Xn interface has to be implemented. However, it is observed that Xn interface has not been implemented successfully today, so it may be still challenged whether it could be successful in 6G.

In NR, UE context storage and retrieval relies on Xn interface deployment, which is main roadblock to wide deployment of Inactive state.

Another possible way is to store UE AS context in CN as a container, which is similar with UE radio access capabilities storage in CN today. When UE resumes in a RAN node, the RAN node can retrieve UE AS context from CN.

Alternatively a new logical storage node can be defined. This storage node could cover a large area scope (e.g. equivalent with CN) than a single RAN node. This will save UE context relocation procedure along with UE mobility. This will also impact RAN3 and SA2 specification. Then RAN2 should study in coordination with RAN3/SA2 which network node the UE AS context should be stored.

Along with the introduction of service-based RAN arthitecture, it is feasible to deploy a new storage function to store UE context.

- In 6G Inactive state, the UE AS context is stored in both UE and a network node. RAN2 should study in coordination with RAN3/SA2 which network node stores UE AS context:

- Store UE AS context in RAN node

- Store UE AS context in CN as a container

- Store UE AS context in a new logical storage node

#### 2.3.3	Mobility in Inactive mode

For intra-6G mobility, similar to NR, Inactive mode mobility should follow IDLE state behavior, e.g. UE should perform cell (re)selection.

For inter-RAT mobility, since Inactive state is already supported in NR. It also makes sense to consider keep UE AS context during inter-RAT cell reselection instead of returning to IDLE

- UE should perform cell (re)selection in Inactive mode.

- For intra-6G mobility, Inactive mode mobility should follow IDLE state behavior.

- For inter-RAT mobility, RAN2 considers to keep UE AS context during inter-RAT cell reselection with NR.

#### 2.3.4	UE context modularization and partial UE context resume

In 5G, UE AS context may include several contents: Bearer (SRB/DRB)/channel related context; AS security context; Compression context; Measurement related context. In addition, a 5G UE might be provided with multiple configurations of the same “type” depending on different criteria e.g., different configs for TN vs. NTN, LP-WUR vs. MR, Redcap vs. non-Redcap, different DRB/PDU configurations depending on service (e.g., different PDU sessions for voice vs. data vs. emergency), different configurations depending on slice used (e.g., eMBB vs. URLLC).

When UE resumes to CONNECTED state due to one type of traffic, all of the UE context will be resumed and the serving gNB will retrieve all UE AS context from the last serving gNB. This introduces unnecessary signalling overhead.

In NR, when UE resumes to CONNECTED state due to one type of traffic, all of the UE context needs to be resumed and the serving gNB retrieves all UE AS context from the last serving gNB. This introduces unnecessary signalling overhead and increase the latency for UE context retrieval.

One possible enhancement could be to modularize AS context and support multiple contexts for the same UE. That means UE context can be divided into several sub-contexts. The context division could be based on use cases, or device types or network types. The following two figures provide an example, in which UE context is divided into four sub-contexts: context 1 is for XR service in TN network, context 2 is for eMBB in TN network; context 3 and 4 are for XR and eMBB respectively in NTN network.

When the UE resumes into CONNECTED state, UE can indicate which UE context(s) need to be resumed, and RAN node can indicate to the UE the accepted context(s) to be resumed. The general procedure could be shown as following figure. With this, RAN node only needs to retrievel requested UE context.

- RAN2 should study how to support UE AS context modularization based on use cases (e.g. XR, eMBB), UE types (e.g.,. Redcap UE, normal UE), network types (TN vs. NTN).

- RAN2 should study how to support partial UE AS context resume using modularized AS context during resume procedure, e.g, UE can request to be resumed AS context module(s).

# Conclusion

IDLE state and CONNECTED state

- RAN2 confirms following NR RRC_IDLE state characteristics will be inhereted to 6G Idle state.

- PLMN selection;

- Broadcast of system information;

- Cell re-selection mobility;

- Paging for mobile terminated data is initiated by CN;

- DRX for paging configured by NAS.

- RAN2 confirms following NR RRC_CONNECTED state characteristics will be inhereted to 6G Connected state.

- RRC connection with RAN node is established for the UE;

- The UE AS context is stored in RAN node and the UE;

- RAN node knows the cell which the UE belongs to;

- Transfer of unicast data to/from the UE;

- Network controlled mobility including measurements.

Inactive mode-paging

- In NR, different location area management and paging monitoring procedures for Idle and Inactive require duplicate standards and implementation efforts in both the UE and the network.

- Paging in Inactive mode should be same as IDLE state, i.e.

- Only CN-initiated paging is applicable

- Same paging DRX parameters and same paging ID

- Same paging area management

Inactive mode-context storage

In NR, UE context storage and retrieval relies on Xn interface deployment, which is main roadblock to wide deployment of Inactive state.

Along with the introduction of service-based RAN arthitecture, it is feasible to deploy a new storage function to store UE context.

- In Inactive mode, the UE AS context is stored in both UE and a network node. RAN2 should study in coordination with RAN3/SA2 which network node stores UE AS context for Inactive mode:

- Store UE AS context in a RAN node

- Store UE AS context in CN as a container

- Store UE AS context in a new logical storage node

Inactive mode-Mobility

- UE should perform cell (re)selection in Inactive mode.

- For intra-6G mobility, Inactive mode mobility should follow IDLE state behavior.

- For inter-RAT mobility, RAN2 considers to keep UE AS context during inter-RAT cell reselection with NR.

Inactive mode-UE context modularization and partial UE context resume

When UE resumes to CONNECTED state due to one type of traffic, all of the UE context will be resumed and the serving gNB will retrieve all UE AS context from the last serving gNB. This introduces unnecessary signalling overhead and increase the latency for UE context retrieval.

- RAN2 should study how to support UE AS context modularization based on use cases (e.g. XR, eMBB), UE types (e.g.,. Redcap UE, normal UE), network types (TN vs. NTN).

- RAN2 should study how to support partial UE AS context resume using modularized AS context during resume procedure, e.g, UE can request to be resumed AS context module(s).

# Reference

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109, Sep 2025

[2] R2-2508002	RAN2#131bis Meeting Report