3GPP TSG-RAN2 Meeting #132			                                                          R2-2508950

Dallas, USA, 17th-21th  November

Agenda item:	10.3.3.3

Title:	Security Requirements in 6GR

Source:	CMCC

Document for:	Discussion

# 1	Introduction

In RAN Plenary #108 agreed the RAN Working Group SID “Study on 6G Radio” in [1]RP-251881, including an objective related to RAN security aspects:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

At RAN2#131bis, RAN2 has discussed the access security of L2, MAC CE security, and reached a preliminary concensus to send a LS to SA3 as below:

Send LS to SA3 to indicate the existing 5G MAC CE information and that some of these control information may be carried over in 6G L2. Ask them what information would require security. Explain RAN2 concerns of overhead (size and mobility security context exchange) and processing. Please identify only critical information that needs to be secure and what type of security (i.e. integrity, ciphering). Indicate that if there are information that critical to be protected RAN2 and SA3 should work jointly to develop a solution. Nice to get a response as soon as possible.

Based on the post-meeting email discussion, we have sent the LS containing RAN2 views with 6G L2 security[2] to SA3. This LS is clearly significant, as our observation on SA3's progress[3] in the previous SA3#124 meeting indicates they have only noted RAN-side security as part of 6G security SID research. However, whether low-layer security will be discussed remains FFS. Therefore, this LS is expected to help them define the scope of RAN-side security research. However, in our understanding, the purpose of this LS is to collaborate with SA3 on access network security issues as early as possible. It does not mean that this LS contains all the access network security issues that RAN2 can currently observe. Therefore, this paper is to provide our observation and view on other security requirements in 6G.

# 2	Discussion

2.1	Security Protection of System Information

Lessons from 5G/5G-A：

System information broadcast by RAN nodes is not protected in 5G&5G-A. A cell periodically broadcasts synchronization signals and SI. These broadcasted messages are intended for all UEs which are camping on a cell. In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services.

The TR33.809[4] in R18 have addressed key-issues and solutions that will potentially enhance 5G system's resistance to false base stations even further. It also includes security protection for system information, key-issue #2, related to RAN2.

5.2 Key Issue #2: Security protection of system information 5.2.1 Key issue details Broadcasting system information (SI) is one of the functions of the RRC protocol, defined in TS 38.331 [2]. A cell periodically broadcasts synchronization signals and SI. These broadcasted messages are intended for all UEs which are camping on a cell. In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services. The system information includes information, among others, like cell (re-)selection parameters, neighbouring cell information, frequency priority, blocklisted cell, common channel configuration information, NAS common information, and public warning system (PWS) messages. In general, the system information is applicable for UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED. UE in IDLE mode performs PLMN selection, monitor paging, performs cell selection, cell re-selection, and applies access control before making an access attempt. In future releases, other services such as MBMS, proximity services, etc. are also likely to be supported by UEs in IDLE mode. This key issue is about investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). Since SI messages are broadcast messages meant for all UEs, it is not apparent that an integrity and replay protection is strictly necessary. Nevertheless, in general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages. SA3 has studied the PWS security in TR 33.969 [3], in which the security mechanisms used to protect the Warning notifications message from Public Warning System against false base station can be reference for the study in the present document. There are some distinct challenges when studying the broadcast messages protection as below: a) Key management. It is because of heterogenous trust-boundaries, and diverse regulations (or requirements) per countries (or regions); b) Time synchronization. It is because of difficulty to achieve fairly acceptable time synchronization between one gNB and other gNBs, and between UEs and gNBs; c) Signaling complexity. It is because of restrictive signaling expected from UEs in RRC_IDLE. Nevertheless, it is only prudent if the 5G system could be enabled (i.e., support) to achieve protection of SI messages in general. NOTE 1: This key issue is concerned with the "over-the-air" interface. Therefore, integrity protection of SI "within-the-network" is not in the scope of this key issue. 5.2.2 Security Threats Lack of protection of SI could potentially have following impacts in some cases: - DoS attack on UE - Rogue services 5.2.3 Potential Requirements 5G system should provide a means to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a cell.

Among them, the security of system information in 5G that concluded by SA3 mainly involves the following threats:

#1 DoS attack on UE: attempts to hinder the UEs' access to the network.

#2 Rogue services: attempts to deliver unauthorized or unsolicited services (e.g., SMS and calls) to the UEs.

To address these security threats, SA3 has studied up to 10 potential solutions in TR 33.809[4].

Solutions

Solution #7: Verification of authenticity of the cell

Solution #9: Using symmetric algorithm with assistance of USIM and home network

Solution #11: Certificate based solution against false base station

Solution #12: ID based solution against false base station

Solution #14: Shared key based MIB/SIBs protection

Solution #19: AS security based MIB/SIBs integrity information provided by gNB

Solution #20: Digital Signing Network Function (DSnF)

Solution #21: Certificate based solution against false base station for Non-Public Networks

Solution #26: KI#2 with PKC-based and without tight time synchronization

Solution #27: Short-lived asymmetric key-based solution for protecting system information

Despite the abundance of solutions, TR 33.809[4] reached the following conclusion due to the lack of consensus among companies on various solutions and the absence of clear assessment of the severity of threats to 5G system information protection:

7.2 Conclusions on Key Issue #2 This key issue is not concluded.

Observation 1: Lessons from 5G: Although SA3 studied the security design of system information during the SI stage of NR, the final NR specification does not capture the security protection of system information. The main reason may be that companies have not clearly defined the severity of threats to 5G system information protection. This point may be an issue we need to highlight to SA3 from the outset of collaboration, requiring them to consider it.

Although RAN2 has not yet reached a complete and conclusive design for 6G system information, the broadcast feature of system information in 6G should still exist at least. Therefore, the security risk of system information is most likely to still come from the false base station as shown in Figure 1.

Figure 1 False Base Station Threats of System Information

Look forward to 6G, system information still has broadcast characteristics, so the risk of replay attacks may still exist. This requirement has already been captured in SA1 SID “Study on 6G Use Cases and Service Requirements” , see TR 22.870 V0.3.1 [5]

5.5.3 Use case on UE selecting a base station after assessing its legitimacy 5.5.3.1 Description The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS is a major security flaw in the existing communication system. There is an important opportunity to address this concern in the next generation wireless systems. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system. By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, False Base Station attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure.

Beyond the previously discussed DoS and Rogue service attack, the RAN2 may also consider two other types of system information threats with unique damaging implications.

First, there are threats targeting public safety systems (e.g., ETWS/CMAS). An attacker could forge or tamper with SIBs carrying emergency alerts (e.g., for earthquakes), broadcasting false information to a massive number of users. This transcends mere service disruption, constituting a severe attack that may directly trigger social panic and public safety incidents.

Second, there are attacks aimed at specific services (e.g., MBMS, NTN). For instance, an attacker could illicitly use the broadcast/multicast service to push spam, exhausting user equipment resources, or tamper with critical parameters (e.g., ephemeris data) in Non-Terrestrial Networks, causing UEs to fail satellite access or experience severe positioning errors, directly undermining service integrity and reliability.

Threats besides DoS and Rogue service attack：

—Public safety threats in ETWS and CMAS related SIBs.

—Service attack of MBMS, NTN and any other service-related SIBs.

Proposal 1: In addition to DoS and Rogue service attacks detected by SA3 in 5G, potential threats from RAN2 prespective may include: 
1) Public safety threats involving ETWS and CMAS; 
2) Service attacks targeting MBMS, NTN, and other service-related SIBs.

Similar to the challenges posed by the L2 security mechanism, we must also address SA3's potential impact on RAN2. The following analysis examines the possibles lesson of RAN2 effects observed in 5G research regarding SA3's proposed solutions.

Conserns：

—SIB Overhead: Signature/integrity protection/hash fields need to be transmited along with system information, and the corresponding overhead may have a significant impact on the design of system information

—System information that is protected and does not need to be protected: Whether all SI information or some part need to be protected and how to indicate partially protected system information.

Mechanism after recognized false system information:

When a UE detects that system information is falsed, the potential response mechanisms to be studied may include: 
1) Barring the false base station, where the UE adds the rogue base station to a local blacklist and can optionally report it to the network;

- Ignoring the false system information, which applies to a UE already camped on a legitimate cell, allowing it to maintain the current connection and report the anomaly to the network;

- Performing cell reselection, which serves as the most critical safeguard, requiring the UE to immediately trigger reselection to escape the current false base station.

Proposal 2: Possible RAN2 concerns posed by the System Information security mechanism can be informed to SA3: 
1) SIB Overhead; 
2) System Information that is protected and does not need to be protected.

Proposal 3: Mechanism after recognized false system information may need RAN2 to study:
1) Barring the False Base Station;
2) Ignore the False SI;
3) Perform cell reselection;

SA3 is in the process of  6G SID: Way forward for 6G Study Item on Security for the 6G System. From our prespective, the key issue is about whether the system information in 6G network face the above security threats of or or what new potential problems exist. SA3 needs to first determine whether there is an agreement on this issue to avoid ineffective solution discussions. Then, SA3 can investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is or other threats. We can wait for their progress and then consider whether it is necessary to enhance the protection of system information and further impacts of 6G system information design.

Proposal 4: A key question to SA3 regarding system information security is whether the system information in 6G network is genuinely faces security threats or what are new potential risks. RAN2 can wait the output of SA3's threat analysis before considering specific technical solutions. However, RAN2 can provide inputs including P1 and P2 about RAN2’s key question, extra consideration of threats and conserns to SA3.

In the just-ended RAN2 meeting, an LS is sent to SA3 to reflect RAN2’s security consideration to SA3 as following [2], We also think that if we can reach some consensus on the potential needs of system information security protection at this meeting, it is necessary to collaborate with SA3 on this issue and inform the above considerations from the RAN2 perspective. Based on the above discussion, we propose that in addition to MAC layer security, RAN2 should also include system information security protection as a necessary requirement for subsequent alignment between SA3 and RAN2 as shown in Annex.

Proposal 5: RAN2 is proposed to incorporate the security protection for System Information as a requirement for subsequent alignment between SA3 and RAN2, and send LS to SA3 to clarify RAN2’s consideration and concerns. The detail solution should be determined by SA3.

# 3	Conclusion

In this contribution, the following observations and proposal is provided.

Observations:

Observation 1: Lessons from 5G: Although SA3 studied the security design of system information during the SI stage of NR, the final NR specification does not capture the security protection of system information. The main reason may be that companies have not clearly defined the severity of threats to 5G system information protection. This point may be an issue we need to highlight to SA3 from the outset of collaboration, requiring them to consider it.

Proposals:

Proposal 1: In addition to DoS and Rogue service attacks detected by SA3, potential threats from RAN2 prespective may include: 
1) Public safety threats involving ETWS and CMAS; 
2) Service attacks targeting MBMS, NTN, and other service-related SIBs.

Proposal 2: Possible RAN2 effects posed by the SI security mechanism: 
1) SIB Overhead; 
2) SI that is protected and does not need to be protected.

Proposal 3: Mechanism after recognized false system information may need RAN2 to study:
1) Barring the False Base Station;
2) Ignore the False SI;
3) Perform cell reselection;

Proposal 4: A key question to SA3 regarding system information security is whether the system information in 6G network is genuinely faces security threats or what are new potential risks. RAN2 can wait the output of SA3's threat analysis before considering specific technical solutions. However, RAN2 can provide inputs including P1 and P2 about RAN2’s key question, extra consideration of threats and conserns to SA3.

Proposal 5: RAN2 is proposed to incorporate the security protection for System Information as a requirement for subsequent alignment between SA3 and RAN2, and send LS to SA3 to clarify RAN2’s consideration and concerns. The detail solution should be determined by SA3.

# 4	References

[1] RP-251881, “New SID: Study on 6G Radio;

[2] R2-2507933   LS on Early Alignment on Access Stratum security aspects .

[3] Meeting Report SA3_124.

[4] TR 33.809 V18.1.0;

[5] TR 22.870 V0.2.1;

# Annex: Draft LS

3GPP TSG-RAN2 Meeting #132			                                                      RP-25xxxxxx

Dallas, USA, 17th-21th  November

Title:	LS on security requirements in 6G

Response to:

Release:	Rel-20

Work Item:	FS_6G-REQ

Source:	CMCC[To be RAN2]

To:	SA3

Cc:

#### Contact Person:	Li Chai

#### E-mail Address:	chaili@chinamobile.com

1. Overall Description:

As stated in the previous LS, RAN2 has also started the study on control and user plane aspects for 6GR and new progress has been made in other security consideration besides L2. To prevent misalignment between SA3 and RAN2 on security requirements related to AS protocol during SA3 study on the 6G security issues, this LS provides more potential security requirements from RAN2’s perspective on other threats for SA3’s consideration:

- In 5G Rel-18, SA3 has studied TR33.809 about the potential schemes to protect system information against false base stations attacks. However, those solutions has not been specified in 5G. Looking ahead to 6G, system information will still be in broadcast manner, so the risk of replay attacks may still persist. Therefore, in 6G, SA3 and RAN2 may need to collaborate to identify system information threats caused by pseudo base stations and potential solutions.

- During the second round RAN2 discussions on AS security, security for system information was raised as a potential topic for study. In addition to DoS and Rogue service attacks detected by SA3 in Rel-18, potential threats from RAN2 prespective may also include public safety threats involving ETWS and CMAS and service attacks targeting MBMS, NTN, and other service-related SIBs.

During the second round RAN2 discussions on AS security, concerns were expressed about the impacts, such as SIB Overhead (e.g., raised by added signature field), SI that is protected and does not need to be protected.(e.g., essential SI).

2. Actions

To SA3 group

ACTION: 	RAN2 respectfully asks SA3 to take the above RAN2 consideration and concerns into account. To progress the 6G study,RAN2 expects to receive the feedback from SA3 until RAN2#133-bis(13 - 17 April, 2026).

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #133	09 - 13 February     2026	            Stor-Göteborg, SE

TSG-RAN2 Meeting #133-bis	13 - 17 April      2026