3GPP TSG-RAN WG2 Meeting #132	R2-2508989

Dallas, USA, Nov. 17th-21st, 2025

# Agenda item:	10.3.1.3

# Source:	ETRI

# Title:	Discussion on retransmission and uplink scheduling

# Document for:	Discussion, Decision

- Introduction

In RAN2#131bis meeting, agreements on user plane have been made as the followings:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This contribution discusses the ARQ and HARQ mechanisms and associated retransmission limitations for achieving the requisite reliability of data delivery. The current operational constraints and their resulting impact on performance are thoroughly discussed. Furthermore, potential enhancements to data retransmission and scheduling mechanisms are proposed to optimize overall system reliability and efficiency.

- Discussion

Limitations of RLC ARQ

The RLC protocol of 5G NR enhances reliability by recovering data transmission errors over the radio link through the ARQ function. However, the conventional ARQ scheme of 5G RLC poses several fundamental challenges in ultra-low latency and hyper-reliability service scenarios envisioned for 6G:

- Increased Latency: ARQ involves multiple steps to detect and request retransmissions when packet loss occurs. RLC employs a timer to identify sequence errors or missing packets, and the waiting time until this timer expires constitutes a major source of delay. Another source of latency is the feedback process. the receiver must generate and transmit a Negative Acknowledgment (NACK), which the transmitter must then process before initiating retransmission. This round-trip procedure introduces delays ranging from several to tens of milliseconds, which are intolerable for HRLLC services.

- Rigid Operational Parameters and Complexity: The operational parameters of RLC ARQ (e.g., maximum retransmission attempts, timer values) are statically configured, typically based on the assigned QoS class. As a result, the protocol lacks the flexibility to adapt to rapidly changing channel conditions or variations in data urgency within the same QoS class. Moreover, RLC ARQ requires complex operations such as Sequence Number (SN) management and Status Report processing, which consume CPU resources and complicate the flexible functional split between the Centralized Unit (CU) and Distributed Unit (DU).

- Inefficient Duplication with Lower-Layer HARQ: 5G NR also employs HARQ in the MAC/PHY layers, which provides a faster and more efficient retransmission mechanism. HARQ operates within a short Transmission Time Interval (TTI) and leverages physical-layer coding techniques to recover errors without discarding the packet. Since RLC ARQ is triggered only upon HARQ failure, a highly reliable HARQ design often makes RLC ARQ redundant, adding unnecessary complexity and delay overhead.

Observation 1. 5G NR ARQ is inefficient in handling traffic that requires both low latency and high data rates, due to excessive retransmission delays, the complexity of RLC window maintenance, and redundant functionalities.

Proposal 1. Study a retransmission mechanism with either ARQ-less or simple ARQ for error recovery.

Re-transmission and reliability

Data retransmission is implemented through a two-level hierarchical scheme: the fast, lower layer HARQ and the upper layer, high-reliability ARQ. HARQ leverages rapid ACK/NACK feedback for quick data recovery, while ARQ, operating over a slower but more reliable feedback loop, ensures end-to-end data delivery. The performance of HARQ is feedback-constrained, fundamentally limited by the reliability of its feedback channel. A typical and critical failure is the NACK-to-ACK feedback error, often occurring with a probability of approximately 0.1%, which subsequently sets the HARQ's intrinsic data transmission error floor at the same level. Since many mission-critical and ultra-reliable applications demand reliability levels significantly exceeding this 0.1% limit, the higher-layer ARQ retransmission becomes essential to meet these stringent service requirements. Error detection capability, however, is ultimately CRC-constrained, governed by the performance of the CRC; for instance, a 24-bit CRC provides an undetected error probability of 6x10-8.

The retransmission in ARQ, triggered when HARQ fails due to residual errors (such as those caused by NACK-to-ACK feedback errors), significantly increases the overall data reliability. However, this reliability gain comes at the expense of increased latency. The ARQ process introduces substantial delay due to the time required for error detection at the receiver and the feedback associated with transmitting higher-layer, robust feedback back to the transmitter. HARQ offers intrinsic advantages. It enables fast retransmission and performance enhancement. This enhancement comes from combining received signals across attempts. However, HARQ cannot reliably meet extreme reliability targets. Therefore, using ARQ becomes mandatory. This necessity compromises the fast transmission goal of many services. The current architecture thus presents an inherent trade-off between the low-latency characteristics of HARQ and the ultra-high reliability provided by ARQ.

Let’s study highly reliable and low-latency feedback mechanism for HARQ enhancements for 6G. HARQ is a foundational technique that significantly improves data performance by enabling rapid retransmissions and leveraging data combining at the receiver to utilize all received signal instances. A critical consideration for 6G is ensuring the robust transmission of HARQ feedback data (ACK/NACK) to the sender. If this feedback is unreliable, the sender may incorrectly assume the transmission succeeded (missing a NACK) or failed (missing an ACK), leading to transmission failure or unnecessary retransmissions. By guaranteeing the reliability of HARQ feedback data transmission, we can more effectively achieve the service performance requirements during subsequent HARQ retransmissions. This study focuses on methods to strengthen HARQ feedback data reliability, with the goal of improving overall data transmission performance and substantially reducing transmission latency.

Observation 2. The retransmission latency for data in HARQ is shorter than that in ARQ, enabling faster data transmission.

Observation 3. The reliability of data delivery via HARQ is fundamentally limited by the reliability of the HARQ feedback transmission. Consequently, improving the reliability of the HARQ feedback data directly enhances the reliability of the data delivered through HARQ.

Proposal 2. Study a mechanism to enhance the reliable delivery of HARQ feedback data.

Limitations of Logical Channel Prioritization (LCP)

Logical Channel Prioritization (LCP) mechanism is a critical component in 5G NR for radio resource allocation and QoS assurance for traffic. However, it exhibits several limitations, when faced with the advanced QoS requirements of 6G:

- Starvation of Low Priority Logical Channels: LCP relies on Priority and Prioritized Bit Rate (PBR) parameters. If high-priority channels continuously receive data, low-priority channels may have limited transmission opportunities, leading to a 'starvation' effect and increased overall transmission latency.

- Difficulty in Meeting HRLLC Requirements: The sequential nature of the LCP procedure (which involves data moving from RLC to MAC and constructing the MAC PDU) introduces a time cost. This acts as a bottleneck for achieving low latency targets required by HRLLC. In addition, in 5G NR, after the LCP procedure is performed for all logical channels, a single transport block (TB) is generated and transmitted, meaning that new, higher-priority data cannot be transmitted when the TB is retransmitted. The Preemption Indication (PI) scheme, introduced in 5G NR to enable immediate URLLC transmission, degrades the QoE of eMBB traffic and generates signaling overhead. Accordingly, novel mechanisms for transmitting HRLLC data in coexistence with IC traffic should be investigated.

- Static Priority Parameters: The priority values used in LCP are statically configured via RRC signaling. While adequate at service initiation, they fail to reflect the real-time user experience or the urgency of control information.

Observation 4. The 5G NR Logical Channel Prioritization (LCP) mechanism is inadequate to meet the 6G requirements. Therefore, a new priority handling and enhanced LCP mechanism that enables more fine-grained and dynamic configuration is required.

Proposal 3. Study a priority handling mechanism and enhanced LCP in the MAC.

Uplink scheduling

The UE initiates data transfer by requesting UL resources and subsequently receiving a resource grant. A significant source of initial latency is introduced by the required SR and BSR procedures, which are mandatory when the UE lacks available UL resources. While existing grants permit the UE to send a BSR to request further allocation, the absence of granted resources necessitates the prior transmission of an SR. This SR is typically sent periodically over a dedicated PUCCH, and the periodic nature of the SR transmission directly increases the initial delay, as the accumulated latency is proportional to the configured SR periodicity. This cumulative delay—comprising the SR transmission time followed by the BSR transmission time—is particularly critical for the first packet arriving into an empty UE buffer, creating a fundamental data transmission latency that severely impacts end-to-end performance across both the RAN and the application layer. Furthermore, this delay negatively affects time-critical data transmission and is especially harmful to TCP-layer data transfer; specifically, a delay in receiving the ACK message for a transmitted TCP segment can prematurely trigger an aggressive TCP flow control mechanism, consequently leading to a substantial reduction in the effective transmission rate.

Time-critical data is inherently subject to strict latency constraints and must be transmitted within an allowed delay bound, beyond which the data is discarded by the communication node. To facilitate efficient scheduling, the UE transmits a DSR to the gNB, conveying the distribution of the allowed transmission delay as crucial scheduling information. A fundamental challenge arises from the common practice where data stored in the UE buffer is transmitted sequentially, meaning that any preceding non-time-critical data must be fully transmitted before the time-critical data can be sent. This sequential processing introduces an undesirable transmission delay for the time-critical data. Therefore, there is a clear need for scheduling algorithms that incorporate the time-critical data's priority, utilizing information from the DSR report and the data transmission process itself, to ensure time-critical data is prioritized over non-time-critical data.

Observation 5. The necessary SR and BSR procedures, required for the transmission of the first-arrived data when the buffer is empty, introduce additional latency to the overall data transmission time.

Observation 6. The sequential transmission inherent in the buffer causes time-critical data to incur additional latency as it waits for the preceding non-time-critical data to be processed and transmitted.

Proposal 4: Study a mechanism to reduce latency for optimizing the scheduling request process and enabling the delivery of time-critical data with higher priority over to non-time-critical data

- Conclusions

In this paper, we discuss the retransmission for achieving fast data delivery and uplink scheduling. Followings are observations and proposals made in this paper:

Observation 1. 5G NR ARQ is inefficient in handling traffic that requires both low latency and high data rates, due to excessive retransmission delays, the complexity of RLC window maintenance, and redundant functionalities.

Observation 2. The retransmission latency for data in HARQ is shorter than that in ARQ, enabling faster data transmission.

Observation 3. The reliability of data delivery via HARQ is fundamentally limited by the reliability of the HARQ feedback transmission. Consequently, improving the reliability of the HARQ feedback data directly enhances the reliability of the data delivered through HARQ.

Observation 4. The 5G NR Logical Channel Prioritization (LCP) mechanism is inadequate to meet the 6G requirements. Therefore, a new priority handling and enhanced LCP mechanism that enables more fine-grained and dynamic configuration is required.

Observation 5. The necessary SR and BSR procedures, required for the transmission of the first-arrived data when the buffer is empty, introduce additional latency to the overall data transmission time.

Observation 6. The sequential transmission inherent in the buffer causes time-critical data to incur additional latency as it waits for the preceding non-time-critical data to be processed and transmitted.

Proposal 1. Study a retransmission mechanism with either ARQ-less or simple ARQ for error recovery.

Proposal 2. Study a mechanism to enhance the reliable delivery of HARQ feedback data.

Proposal 3. Study a priority handling mechanism and enhanced LCP in the MAC.

Proposal 4: Study a mechanism to reduce latency for optimizing the scheduling request process and enabling the delivery of time-critical data with higher priority over to non-time-critical data

- References

- RP-251395, Revised SID: Study on 6G Scenarios and requirements.

- RP-251881, New SID: Study on 6G Radio.

- TR 38.914, Study on 6G Scenarios and requirements.

- R2-2507241, Discussion on User Plane Design for 6GR