3GPP TSG RAN WG2 #132	R2-2509054

Dallas, USA, Nov 17-21, 2025

Agenda item:	10.3.3.1

Source:	MediaTek Inc

Title:	AI/ML Framework and Data Transfer Design

Document for:	Discussion and Decision

# Introduction

This document discusses the following SID objectives [1]:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

.. Sub-objecives a) – d) are not addressed here ..

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- .. This sub-objective is not addressed here

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

# Data Transfer Design

## 2.1			 Cases of Uu Data transfers

Legacy Type Real-time data Reporting (UE->Network)

Typically, in 5G/4G et, the UE reports measurements to the Node B or to a positioning node etc, for real-time operation. RAN2 usually discriminates between Real-time data that is critical to maintain UE connection with the network, e.g. radio measurement reports for mobility which are very urgent (SRB1), and other Real-time data such as positioning measurements (SRB2). For 5G AIML it is expected that UE may report data for the purpose of AI algorithm monitoring, details FFS.

Legacy Type Non Real-time data (UE->Network)

For the purpose of SON, the UE can report problem event reports (SRB2) but are not urgent. For MDT, network side ML data collection, and QoE measurement collection, the UE can report collected logs (SRB2/4). It is expected that also logs for the purpose of UE side ML offline training need to be collected, with significant data sizes.

New cases / aspects in 6G: Real-time data (UE->Network)

Sensing measurements UE->Network could be real-time, e.g soft real time for e.g. intruder detection, or UE->Network e.g. hard real-time using sensing for maintain Radio Connection. However, it is FFS which sensing use cases shall be addressed by 6GR.

New cases / aspects in 6G: Non Real-time data (UE->Network)

Sensing measurements UE->Network for offline ML training. However it is FFS which sensing use cases shall be addressed by 6GR). Data volumes could be large.

In general increased use of AIML (for 6GR and RAN) is expected to result in higher data volumes collected for offline AIML training.

In general the trend of increased automation and autonomy in network operation is expected to require more data collection, in the network and also from UEs.

DL Cases (Network -> UE)

FFS: real-time: no use case has yet been agreed in 5G or seems to be on the radar for 6G that requires significant DL Data transfer for inference or training.

Non-real time: The main case of DL data transfer seems to be AI model download. As it is possible to do pre-load it is expected that in most cases this is not critically urgent. Cases where AI need to be disabled when applicable UE side model is not available in time may occur, but the consequence can be limited as 6GR shall be able to operate also without AIML.

## 2.2			 Discussion

Real-time cases: Similar real-time cases as previous G’s are expected (i.e. with limited data volumes), and it seems reasonable to assume such data (measurement reports etc) could be handled in similar ways as for previous Gs. We assume that whether there are sensing hard real time cases would need explicit 6G scope decision, e.g. whether to support using sensing measurements for direct RRM decisions that impact Radio connection (instead of using regular radio measurement reports).

Observation 1: No specific discussion is required for real-time data. FFS if sensing would bring such case.

Non-Real-time cases: MDT, SON, QoE visibility, AIML for 6GR and RAN all have the purpose to assist / support / enhance operation of 6G RAN and 6GR. The overhead and cost of such support functionality need to be kept limited, and it should not compete with or cause deterioration of commercial service provided to end users, e.g. it should/shall be possible that a end user doesn’t pay for such data transfers. It is noted that Data volumes could be significant.

Detailed aspects of Data transfer solutions, modelling of such solutions, e.g. by QoS parameters by RB type etc, may be dependent on decisions by other groups, e.g. network architecture decisions where to house intermediate Data storage and processing functions by RAN3, SA2, SA5.

Proposal 1 (objective of data transfer design): Non-RT data transfer without charging the end user shall be enabled as a regular case. It shall be possible to avoid load impacts to charged traffic due to non-RT data transfer, e.g. by specific network control. L2 enhancements for Uu efficiency of non-RT file transfer can be considered.

# Data Transfer and Data Collection Cases

Following the discussion in subclause 2, the following table assembled.

| UE involved Data Transfer and Data Collection cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Real-time Data (hard) | UE => xNB | Direct Radio Resource Mgmt and Control, by L3 or L1 measurement report from the UE. Data Volumes: Small | xRRC (SRB1), xUCI on xPUCCH or xMAC CE. Always direct from UE to xNB due to real-time requirements and importance to maintain RL. |
| 2 | Interactive Data (soft RT) | UE => Positioning Server | UE assisted network based pos, or UE based Pos, e.g. by positioning measurement report. Data Volumes: Small | FFS mechanism |
| 3 | Non-RT Data | UE => xNB UE => OAM UE => CN | Data for 6GR and 6G Network operation, i.e. including SON, MDT, QoE, Training of network side AIML algorithms, by event repots (RLF, RACH fail), by UE logging and reporting of logs. Data Volumes: Medium / Large | FFS mechanism |
| 4 | Non-RT Data | UE => vendor server (FFS: xNB => Vendor server) | Data for 6GR. Data Collection for Training of UE side side AIML algorithms, by UE logging and reporting of logs Data Volumes: Large | FFS |
| 5 | Non-RT Data | UE => UE (FFS: xNB => UE) | Data for 6GR. Data Collection for on-device training of UE side side AIML algorithms, by UE logging and reporting of logs Data Volumes: FFS | FFS |
| 6 | Non-RT Data | Vendor server => UE | UE side AI model download/deployment Data Volumes: Medium / Large | FFS |
| 7 | Interactive Data (soft RT) | UE => Sensing Server | UE assisted network based sensing or UE based sensing, (e.g. for Intruder detection), e.g. by Sensing result report. Data Volumes: FFS | FFS mechanism |

Table 1 UE involved Data Transfer and Data Collection cases

Proposal 2: Capture the Data collection and Data transfer cases including termination points as in Table 1.

# Data Collection and Data Management

## General

For the Data Collection and Management objective, a model with two types of interfaces is assumed, Data Provider interfaces, and Data Consumer interfaces, supported by MNO owned network infrastructure for control and storage, the box in the middle, see the example figure. One intention is that different WGs can focus on different interfaces, e.g. RAN2 RAN3 focus mainly on Data Provider side, and SA2 focus on Data consumer/client side.

Data Provider (s)

Data Consumer

MNO

Data Management and Storage

Data Collection Control

Data Request
Data Collection Request

Data Transfer

Data Transfer

Data Provider

Interfaces

Data Consumer

Interfaces

Figure 1: Data Framework(s)

A Data provider(s)/producer is a data producer in a live network: Typically a 3GPP entity such as a UE, or a gNB. Could also be a subsystem of 3GPP entities (e.g. RAN, LMF+UEs),

A Data Consumer could be either of:

- A 3rd party vendor server (e.g. a server by a UE side modem vendor like MediaTek) for training of AIML algorithms/functions for deployment in MNO network (e.g. for UE side models.) For support of offline training of device algorithms, for acquisition of vendor-related performance statistics (when applicable).

- A UE physical device, e.g. for support of on-device training, or UE smart desicion making

- Could also be An MNO network internal entity, e.g. a gNB, that e.g. trains AIML algorithms/functions in MNO network, or makes other network automation decisions,

Proposal 3: RAN2 to capture of the Figure 1 Data framework(s), as an assumption, paving the way for further discussion.

## Other aspects essential for device vendor

Requirements on collecting data from devices for network and Radio operation are expected to increase. In many cases such data is pure overhead. There are limited business drivers for UE vendors to implement data collection for other cases than UE vendor algorithms. For REAL feasibility, it is thus very important to make data collection simple and light weight for UEs and UE vendors.

Proposal 4: A single mechanism for Data collection of non-RT data from UEs shall be supported, regardless purpose of the collected data: SON, MDT, AIML UE side, AIML network side. (FFS sensing data). RAN2 may need to ensure the convergence required to make this a single mechanism (FFS).

An essential/unconditional requirement: For data collection from the UE for UE side purpose it need to be possible to collect vendor-specific data, in order to have reasonable time to market and to allow competition between UE vendors, e.g. to allow using additional complementary loosely correlated data, which is a natural benefit/characteristic of AI based / data driven functions. Note that any vendor specific data could still use log files of standardized format, and with semantic explanations such data can be fully visible to MNO.

Proposal 5: For data collection from the UE for UE-side purpose including model training and vendor performance monitoring, collection of vendor-specific data shall be fully supported.

# Realistic Life Cycle Management

Model Training + Test (e.g. by test data set)

Data Acquisition

non-RT

Data Eng

Analysis

Device Testing

Model Deployment

Operation
Data Acq. for Inference, Inference, Control (act-deact, Monitoring, UE autonomous switching etc)

Performance feedback, to vendor

MNO / Live System

Vendor

Figure 2: Life Cycle, Vendor Managed Model

The following Life Cycle Types for UE side AIML should be considered:

Type1: Vendor Managed Model Off-line training (baseline case)
See Figure 2, Vendor managed model, UE side (or UE part of 2-side), Offline (re-)training, OTA model deployment need to be supported. Either A: fully tested device HWSW functionality also for updates, Alternatively B: light/no device level testing for updates.

For Type 1 life cycle, pure Cell/Site specific models may be associated with scalability issues.

Type 2: On-Device Training
On-Device Automated Training (no figure). Cell-specific localized adaptation is feasible from LCM perspective, including adaptation towards the typical path/trajectory of a user. 3GPP LCM support impact FFS (e.g. data sharing among devices for some use cases). Beside LCM there may be impact on e.g. configuration of functionality. Could e.g. be applicable to AI native stack (L1L2L3), FFS. Assume finetuning or similar approach, and that impact of such finetuning may need to be limited etc, especially for cases impacting performance with requirements / test cases (device need to remain test compliant). When combined with Compute Offload for the actual training Type 2 life cycle converges to be similar to an automated version of Type 1 Life cycle. HOWEVER we think that for discussion claity it is good to discriminate Type 1 and Type 2.

Type 3: Non-vendor managed UE model (Need is FFS)
NON-vendor managed UE side (or UE part of 2-side) models. E.g. an Open, Network-trained or MNO-trained model is installed on the device. Whether Open, Network- or MNO-trained models is in-practice feasible for any agreed 3GPP use case(s) is not clear. A major difficult issue is who takes responsibility, for performance and consequences.

Life Cycle Type would typically be specific for use case and detailed vendor solution. A certain solution for a use case could use a mix of approaches, e.g. allow deployment and updates according to Type 1 but also do Type 2 on-device training (fine-tuning) for localized optimization e.g. for frequently visited cells.

Proposal 6: General guidance on LCM
- Product-Realistic Life-Cycle assumptions need to be taken in 6G. 
- Life-cycle assumptions can be both general and use case specific.
- RAN2 shall be responsible for assembling assumptions and making relevant decisions on lcm support. 
- Type 1, 2, 3 Life cycles, and figure 2, as described above can be a starting point.

Proposal 7: On what to standardize: LCM activities performed in the live network / MNO domain are the principal candidates for standardization, i.e. Model deployment, Operation (consistency handling, capabilities, configuration, monitoring, activation, deactivation), performance feedback to vendor, data collection. Other activities such as model training shall not be standardized, instead it shall be possible to use state-of-the-art tools.

# 4	Proposals

~~~~~ Data Transfer Design ~~~~~

Proposal 1 (objective of data transfer design): Non-RT data transfer without charging the end user shall be enabled as a regular case. It shall be possible to avoid load impacts to charged traffic due to non-RT data transfer, e.g. by specific network control. L2 enhancements for Uu efficiency of non-RT file transfer can be considered.

~~~~~ Data Transfer and Data Collection Cases ~~~~~

Proposal 2: Capture the Data collection and Data transfer cases including termination points as in Table 1.

~~~~~ Data Collection and Data Management ~~~~~

Proposal 3: RAN2 to capture of the Figure 1 Data framework(s), as an assumption, paving the way for further discussion.

Proposal 4: A single mechanism for Data collection of non-RT data from UEs shall be supported, regardless purpose of the collected data: SON, MDT, AIML UE side, AIML network side. (FFS sensing data). RAN2 may need to ensure the convergence required to make this a single mechanism (FFS).

Proposal 5: For data collection from the UE for UE-side purpose including model training and vendor performance monitoring, collection of vendor-specific data shall be fully supported.

~~~~~ LCM support ~~~~

Proposal 6: General guidance on LCM
- Product-Realistic Life-Cycle assumptions need to be taken in 6G. 
- Life-cycle assumptions can be both general and use case specific.
- RAN2 shall be responsible for assembling assumptions and making relevant decisions on lcm support. 
- Type 1, 2, 3 Life cycles, and figure 2, as described above can be a starting point.

Proposal 7: On what to standardize: LCM activities performed in the live network / MNO domain are the principal candidates for standardization, i.e. Model deployment, Operation (consistency handling, capabilities, configuration, monitoring, activation, deactivation), performance feedback to vendor, data collection. Other activities such as model training shall not be standardized, instead it shall be possible to use state-of-the-art tools.

# 5	Reference

[1]	RP-252912, 3GPP TSG RAN “Revised SID: Study on 6G Radio”;	NTT DOCOMO, CMCC, AT&T, Vodafone