3GPP TSG-RAN2#132	R2-2509167

# Dallas, USA, November 17-21, 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Discussion on remaining RILs for NR NTN Rel-19

Document for:	Discussion/Decision

# Introduction

At RAN2#131, RAN2 has completed the work related to DL coverage enhancements and broadcast services for NTN (see WID [2]).

According to the rapporteur’s assessment of the collected RILs, however, certain open issues exist. The following RILs are marked as “ToDo”:

A200, C003, H251, H255, E010, E011, (S058), N085, V230 and Z255. We also revisit the decision taken for V206.

In what follows, we address those RILs one by one.

# Discussion

## 2.1	DL coverage enhancements RILs

### A200

[Description]: The current assumption from CR rapporteur is SMTC in SIB2 is associated with reference location in SIB19, and it is used for serving cell measurement. We think it is not always the case. Note that when NW provides smtc in SIB2, it should associate with the largest SSB periodicity of serving and neighbor cell. For example, if serving cell has a SSB periodicity of 160ms, and neighbor cell has a SSB periodicity of 20ms, the smtc in SIB2 has to be with 160ms periodicity. That is, it is actually for neighbor cell measurement. And smtc5list can provide another periodicity of 20ms for serving cell measurement.

Therefore, we would like to clarify that there should be an explicit reference location associated with legacy smtc configured in SIB2.

[Proposed Change]: Allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

[Comments]:

[Rapp] In our understanding, smtc is used for the serving cell. However, this needs to be clarified in RAN2 first.

We agree with the observations made by the company that has raised the RIL. Namely, if the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the network may not be able to configure the SMTC periodicity (for that legacy SMTC) equal to the SSB periodicity of the serving cell. This is true if the SSB periodicity of at least one intra-frequency neighbor cell is larger than the SSB periodicity of the serving cell, as clarified by RAN4. However, it does not break RAN4 requirements to configure an SMTC periodicity that is larger than the SSB periodicity of an associated cell. Nevertheless, one may argue the downside is that the UE may not benefit from the denser SSB transmissions in the serving cell, provided that the UE measures on the serving cell using SMTC.

- [A200] If the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the UE may not benefit from denser SSB transmissions in the serving cell (provided that the UE measures on the serving cell using SMTC).

In our understanding, the legacy SMTC always applies to the serving cell and should therefore always be used, i.e., the legacy SMTC is not applicable to location-based selection. This becomes clearer reading RAN4 specification TS 38.133 section 4.2.2.2, where SMTC is used to define the requirements for measurement and evaluation of the S criterion in the serving cell for cell selection. If RAN2 were to change this tenet of cell selection, it would need a strong technical justification and consider the potential impact in other working groups.

Reject RIL A200 and clarify that legacy SMTC (smtc in SIB2) applies to the serving cell and is not used for location-based SMTC selection.

### H251

[Description]: It was agreed to have 7 SMTCs altogether on a single frequency. Serving cell does not require a reference location, and in this case the legacy smtc is used for the serving cell measurement. However, this understanding is a bit different from legacy releases because smtc is now changed to a cell-specific SMTC rather than a frequency-specific SMTC. Also, the field description of smtc related to SMTC adjustment based on PDD needs to revised so that UE does not need to consider neighbour cell propagation delay.

[Proposed Change]: Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell and smtc5list is not configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. If the field is broadcast by an NTN cell and smtc5list is configured, smtc is for serving cell measurements and the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference equals to 0 ms.

[Comments]:

[Rapp] In our understanding, even if smtc5list is configured, there could be neighbours to be detected within legacy smtc. Thus, legacy text still applies.

Consistently with previous releases, we consider it is essential that the UE continues to adjust the gNB-UE propagation delay autonomously for NTN neighbours following the actual propagation delay difference. We understand the objective is to clearly mandate the behaviour when the propagation delay difference is effectively 0 ms, such as when the SMTC corresponds to the serving satellite. However, regardless smtc could be considered cell specific, this is nothing new, legacy may already include scenarios where multiple neighbour cells are served by the same satellite as the serving cell, resulting in the same situation, i.e., propagation delay difference equal to 0ms. Thus, we believe a clarification is not needed since the present text is more general and well suited to the new changes.

Reject H251.

### H255

[Description]: It still needs to be made clear in the spec how many SMTCs can be configured in total, and whether some configuration (e.g., the following examples made in 	R2-2506866) is allowed and how many SMTCs are counted in such configuration:

|  | smtc4list (periodicity1) | smtc5list | Explanation of smtc5list |
| --- | --- | --- | --- |
| Entry 1 | offset 1, PCI_list1 | -, -, - | offset 1, PCI_list1, periodicity1 |
| Entry 2 | offset 2, PCI_list2 | -, -, periodicity2 | offset 2, PCI_list2, periodicity2 |
| Entry 3 | offset 3, PCI_list3 | -, -, periodicity2 | offset 3, PCI_list3, periodicity2 |

We think the above configurations are sensible. Take the second configuration (highlighted in yellow) as an example, the NW may configure offset 2 + PCI_list2 in SMTC4 for legacy UEs (and they will use the periodicity in smtc) and configure periodicity2 (which could be smaller than the periodicity in smtc, so that R19 UEs can reduce the measurement delay for neighbour cells with smaller SSB periodicity). In this case, this configuration should be considered as a single SMTC, because they target at the same offset and PCI list. If offsets are different or PCI lists are different (i.e., the SMTC in smtc5list configures not only periodicity), they should be considered as separate SMTCs.

Since RAN4 requirements are based on SMTC, we think it is necessary to clarify the total number of SMTCs and in which case the SMTC in smtc5list with delta configuration is considered as a separate SMTC.

In our understanding, if two configured SMTCs share the same offset and PCI list, they are considered as a single SMTC and R19 UEs shall only use the SMTC with smaller periodicity.

[Proposed Change]:

Clarify that: The total number of SMTCs across smtc4list and smtc5list does not exceed 6. The SMTCs having the same pci-List and offset are considered as a single SMTC regardless of whether periodicity is configured or not (i.e., the SMTC in smtc5list which is not configured with pci-List and offset is considered as the same SMTC with that at the same position in smtc4list). For the configured SMTCs having the same pci-List and offset but different periodicities, R19 UEs will only use the SMTC with the smaller periodicity (i.e., periodicity in smtc5list).

Suggested TP:

smtc4list, smtc5list

Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The network only includes SMTC offsets in smtc5list for (quasi-)Earth fixed cells. The total number of configurable SMTC offsets across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc and smtc5list is 2. The total number of configured SMTCs across smtc4list and smtc5list does not exceed 6. SMTCs with the same offset and pci-List are regarded as the same SMTC, and UE will use the SMTC with the smallest periodicity if multiple periodicities are configured for the SMTC. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

In line with previous discussions, we agree with the RIL proponent that the total number of SMTCs across smtc4list and smtc5list should not exceed 6. This has already been captured in the latest rapporteur’s CR since RIL H257 was marked as PropAgree.

However, the proponent argues that the SMTCs having the same pci-List and offset must be considered as a single SMTC. However, we believe this scenario shall not happen. The purpose behind introducing delta signalling was doublefold, support legacy UEs and avoid signalling overhead. Therefore, for an existing SMTC in smtc4list, the network will not repeat pci-List and offset in smtc5list. Provided this does not happen, it becomes clear that if an entry in smtc5list includes pci-List, offset, and periodicity, it constitutes a (new) SMTC and this clarification is not needed. Regarding the second change, we agree that the Rel-19 UEs will only use the SMTC with the smaller periodicity (i.e., periodicity in smtc5list), if provided, this is the reason why this new field was introduced. Therefore, the behaviour is evident, and a clarification is not needed in the specification.

Reject H255. The current specification text is clear enough to define delta signaling between smtc4list and smtc5list and the associated UE behavior.

### E010

[Description]: The current solution for UE-assisted SMTC selection in RRC_CONNECTED mode requires two RRC reconfigurations of the UE, for each UE upon each transition to RRC_CONNECTED mode. This may cause significant signaling overhead and pose a scalability issue for NTN, where cells are large and radio resources are extremely scarce. The additional, second RRC reconfiguration is needed, because at the time when the usual, first RRC reconfiguration is performed, the network does not yet know which are the relevant SMTCs to be configured for the UE.

[Proposed Change]: The above problem can be avoided for UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode by allowing the UEs to report the N closest reference locations, i.e., by allowing them to add referenceLocationReport, in the RRCResumeComplete message based on prior UE configuration. For UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode, AS security is enabled after reception of the RRCResumeRequest message by the network. Hence, the RRCResume and RRCResumeComplete message are subject to the same protection (i.e., cyphering and integrity protection) as the RRCReconfiguration and RRCReconfigurationComplete message.

This change enables the network to efficiently resume RRC connections of UEs (without RRC reconfiguration).

[Comments]:

[Qualcomm] This probably can be discussed to do it properly as the network may need to add new reference location list in RRCResume. Otherwise after being inactive so long, just reporting outdated reference location is not useful. Please change to ToDO.

Regarding the comment, it is true that if the UE has moved to a different cell (e.g., due to UE or satellite movement), the reference location set stored by the UE may be outdated. Nonetheless, we note that in a typical NTN quasi-Earth-fixed cell deployment, it is highly unlikely that the reference location set will change due to satellite movement since the set will be configured by the operator (e.g., OAM) during the deployment phase based on satellite footprint characteristics. In any case, similar to other UAI reports, the UE releases assisted-SSB-MTC-Config upon initiating RRCResumeRequest see section 5.3.13.2. Therefore, if the network would like to obtain this report, it would need to include it in the RRCResume message. The network can optionally include a new set of reference locations in the configuration or refer to the list in system information. This prevents the UE from using a potentially outdated configuration.

Accept E010. Take the TP in Annex as a baseline.

### N085

[Description]: A UE may not only support less SMTCs than signalled in SMTC4 and SMTC5, but also only a single periodicity.

[Proposed Change]: Use “or” instead of and for SMTC4 and SMTC5 to be supported

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports only a single periodicity or less SMTCs than what is included in smtc4list or smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

[Comments]:

[Rapp] A UE that only supports one periodicity can safely ignore smtc5list in SIB2. In the MO, provided network has UE capabilities, this will not be configured. We agree with the intention that this should be somewhat captured.

We share the concerns of the company that has raised the RIL but prefer to address it differently. Location-based SMTC selection and the additional SMTC periodicity are two separate features, so they should be kept separate as much as possible.

First, a UE that does not support the additional SMTC periodicity must apply the legacy SMTC periodicity, which is given by smtc, or smtc1 respectively. Moreover, as discussed in [3], the legacy SMTC periodicity must be equal to (or larger than) the largest SSB periodicity among serving cell and intra-frequency neighbor cells, and it is not an issue to monitor SSB transmissions at a certain period using an SMTC window of a longer period, i.e., measuring less frequently. Consequently, the UE can set up SMTC windows based on the legacy SMTC periodicity and any of the SMTC offsets given in smtc5list. The offset of an entry in smtc5list can be considered for location-based selection even if the entry is associated with an additional periodicity.

- [N085] Location-based SMTC selection should be applicable independent of the additional SMTC periodicity. A UE that does not support the additional SMTC periodicity may select any entry of smtc5list and apply the legacy SMTC periodicity as provided in smtc.

The proposed change may be understood as: A UE may ignore an entry in smtc5list if the entry is associated with the additional SMTC periodicity and if the UE does not support the feature. To avoid that misunderstanding, we propose adopting the following changes instead:

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and instead applies the periodicity provided in smtc. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

For RIL N085, introduce the following clarification: “If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and applies the periodicity provided in smtc.”

## 2.2	MBS-related RILs

### C003

[Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope.

[Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIBXX.

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

[Comments]:

[Rapp] Even thought it is unlikely that both ways of delivering the information may be configured simultaneously, RAN2 should further discuss which one is prioritized (if needed).

[Qualcomm] Which ever area UE wants to use should work pointing to (may be not exactly same) almost same region. Otherwise this should be error case.

Firstly, it is highly unlikely that a network operator would select both ways to deliver the same information to the UE: ISA and Service Announcement. The Service Announcement is a mandatory feature of MBS which contains essential information for the UE, we understand this should have prevalence. In any case, the evaluation of whether the UE is inside or outside the ISA (or TSA) is up to implementation.

Secondly, there already exist other NTN features such as discontinuous coverage where this situation happens. Similar information can be provided to the UE via AS and NAS. Therefore, even if were to happen, this can be simply solved by implementation and there is no need to mandate specific behaviours in the specification.

Reject C003. It is up to UE implementation how to use ISA (or TSA) if both ISA and TSA are delivered to the UE.

### E011

[Description]: Following RAN2 agreements, a UE may initiate the establishment or release when it enters/leaves the ISA of the MBS service in question. However, the text captured in 5.9.3.2 is not sufficient to ensure these limitations. For instance, there are some clauses such as “upon start of the MBS session” that would allow a UE to acquire establish the MRBs even outside the ISA..

[Proposed Change]: It is simpler to include the geofencing limitation in the general configuration of broadcast MRBs (section 5.9.3.1) so that it applies both to initial establishment/release but also to updates. Here an example:

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH and are located within the Intended Service Area associated with the MBS service, if any.

[Comments]:

[Qualcomm]We suggest this to change to ToDO. This should not be applicable to legacy UEs. It should be applicable to only UEs capable of supporting ISA and this is not clear to us.

Following previous RAN2 agreements, it should be captured how the ISA applies to the configuration. There are some clauses such as “upon start of the MBS session” that would allow a UE to acquire establish the MRBs even outside the ISA. Therefore, a simple geofencing limitation in the general configuration of broadcast MRBs (section 5.9.3.1) so that it applies both to initial establishment/release but also to updates

Although we understand Qualcomm’s concern, this should not be an issue since the procedural text does not affect legacy, nor applies to non-ISA capable UEs (e.g., terrestrial UE). These devices will not understand the ISA signaling and will not execute this specific procedural option. In RRC, there are some other examples where ISA is referred while there is no explicit mention to ISA NTN capable UE, for instance, in section 5.9.3.2 “upon leaving the ISA(s) associated with an MBS broadcast service the UE is interested in.”

Agree E011.

### Z255

[Description]: When a ISA is the same as serving cell, NW can only include intendedServiceAreaId without including the areaCoordinates, this help saving unnecessary signalling overhead. Also it is more flexible for NW to signal ISA which is a combination of ISA the same as serving cell and other ISAs.

[Proposed Change]: Make areaCoordinates as optional, and clarify UE understanding in field description when it is absent.

SIBXX information element

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    SEQUENCE {

center-r19                      ReferenceLocation-r17,

radius-r19                      INTEGER(0..65535)

}                         OPTIONAL

}

}

SIBXX field descriptions

areaCoordinates Indicates the ISA as shape of circle or polygon. Absence of this field indicates the ISA has the same coverage as serving cell.

This matter was thoroughly discussed and addressed at the previous meeting, resulting in the agreement on RIL V206. Accordingly, the specification now clearly states that if an ISA is absent from mbs-SessionAreaList-r19, the MBS service is applicable to the entire cell as in legacy. Therefore, no further action is required, and this RIL should be rejected.

Reject Z255. The absence case has already been clarified in V206 in mbs-SessionAreaList.

### V206

[Description]: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area. So Need S is supposed to be used for mbs-SessionAreaList-r19

[Proposed Change]: Change Need R to Need S for mbs-SessionAreaList-r19

[Comments]:

[Rapp] In our understanding, based on RAN2 previous agreements, if the ISA (ID) is not provided for a certain service, then legacy mechanisms apply (it is intended for the whole area). Usually, legacy behaviour does not need an explanation, it is only the new functionality that needs to be clarified in new parameters. This should be discussed in the next meeting.

During the previous meeting, consensus was reached on V206 with the following agreements:

- Change the need code for mbs-SessionAreaList-r19 from Need R to Need S

- Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent.

These have been incorporated into the latest version of the endorsed RRC CR for NR NTN.

mbs-SessionAreaList

Indicates the list of intended service areas associated with an MBS broadcast session in an NTN cell. If this field is absent, the MBS broadcast session can be received within the entire cell area.

Upon reviewing Z255, it has become evident that the current agreement may introduce ambiguity. Since the ISA can also be provisioned to the UE via Service Announcement, the network might omit an ISA for a specific MBS service in the MCCH, while simultaneously associating the same MBS service with a Target Service Area in the Service Announcement. This inconsistency could lead to confusion for the UE, as one parameter suggests geofencing, whereas the other implies cell-wide applicability. To resolve this, we propose revising the formulation of V206. Specifically, if the ISA is absent in MCCH, the legacy behavior should apply: the MBS service is available throughout the MBS Service Area. The definition of the MBS Service Area is provided in Stage 2.

For V206, change the field description of mbs-SessionAreaList to “if this field is absent, the MBS broadcast service can be received within the MBS service area”.

## 2.3	TEI-19 RILs

### S058

[Description]:

As discussed in LTE TN to NR NTN for LTE RIL X500, how to handle the SMTC when redirecting to NR NTN is not clear. First of all, since it is not considered to be supported to measure an NR NTN frequency when the UE is connected to NR TN, then there is a question of how to handle the smtc, if the smtc is not configured in the CarrerInfo-v1900. In anticipation of potential future support of measuring NTN in TN, we believe that for this release, the field can be mandatory to signal.

Furthermore, the offset should be clarified to be based on the actual propagation delay.

[Proposed Change]: To follow the proposed change from X500 (or otherwise follow the approach from LTE TN to NR NTN):

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. In this release the field is mandatory present in CarrierFreqInfoNR-r19 and the corresponding offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE’s propagation delay difference between serving cell and neighbour cells equals to 0 ms, and the UE adjust the offset based on the actual propagation delay.

At the previous meeting, the optional inclusion of smtc parameter in the IE CarrierInfoNR-r19 was discussed and it was agreed to leave this decision to network implementation. In our view, mandating specific behaviour for all potential scenarios is unnecessary and could lead to over-specification. Consequently, this RIL, which mirrors the intent of X500, should also be rejected for NR NTN.

As agreed for LTE TN to NR NTN mobility WI, whether to include smtc in CarrierInfoNR-r19 is up to implementation. Reject S058.

## 2.4	LTE to NR NTN RILs

### V230

[Description]: When smtc-19 is configured, the offset adjustment behavior should be added in the FD of smtc included in RRC Release.

[Proposed Change]: The offset adjustment based on the actual propagation delay difference should be added in the FD of smtc included in RRC Release. E.g.,

smtc

The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If E-UTRAN includes smtc-r19, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the difference between the eNB-UE propagation delay for serving cell and gNB-UE propagation delay for neighbour cells is equal to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

During the previous meeting, the responsibility for compensating smtc propagation delay in measurements from LTE TN to NR NTN was discussed. For NR NTN, a UE-based solution to compensate propagation delay was introduced established in Rel-17 (RAN2#116bis), due to the need to account for varying propagation delays of SSBs from multiple satellites. However, in TN to NTN mobility scenarios, the circumstances differ. When a UE connected to TN is redirected to NTN, the network would typically configure a single (target) satellite within a specific frequency, provided, the coverage area of an NTN cell generally encompasses that of a TN cell which is smaller. The network is aware of the satellite’s position, and it is capable of compensating the propagation delay. In this way, the UE is not required to obtain its GNSS position upon receiving the message since activating GNSS in TN operation would result in significant energy consumption. Otherwise, the network would be unable to configure release with redirection unless the UE’s GNSS position is reliably available.

For V208 and S058, the network compensates the propagation delay between the gNB and the satellite for smtc when included in RRCConnectionRelease. Reject V208.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	[A200] If the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the UE may not benefit from denser SSB transmissions in the serving cell (provided that the UE measures on the serving cell using SMTC).

Observation 2	[N085] Location-based SMTC selection should be applicable independent of the additional SMTC periodicity. A UE that does not support the additional SMTC periodicity may select any entry of smtc5list and apply the legacy SMTC periodicity as provided in smtc.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reject RIL A200 and clarify that legacy SMTC (smtc in SIB2) applies to the serving cell and is not used for location-based SMTC selection.

Proposal 2	Reject H251.

Proposal 3	Reject H255. The current specification text is clear enough to define delta signaling between smtc4list and smtc5list and the associated UE behavior.

Proposal 4	Accept E010. Take the TP in Annex as a baseline.

Proposal 5	For RIL N085, introduce the following clarification: “If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and applies the periodicity provided in smtc.”

Proposal 6	Reject C003. It is up to UE implementation how to use ISA (or TSA) if both ISA and TSA are delivered to the UE.

Proposal 7	Agree E011.

Proposal 8	Reject Z255. The absence case has already been clarified in V206 in mbs-SessionAreaList.

Proposal 9	For V206, change the field description of mbs-SessionAreaList to “if this field is absent, the MBS broadcast service can be received within the MBS service area”.

Proposal 10	As agreed for LTE TN to NR NTN mobility WI, whether to include smtc in CarrierInfoNR-r19 is up to implementation. Reject S058.

Proposal 11	For V208 and S058, the network compensates the propagation delay between the gNB and the satellite for smtc when included in RRCConnectionRelease. Reject V208.

# 4	References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

R2-2506154, Discussion on Reply LS on SMTC enhancements, Ericsson, RAN2#131, Bengaluru, India, August 25–29, 2025.

# 5	Annex

## 5.1	Relevant agreements for DL CE

During RAN2#130, the following agreements related to DL coverage enhancements were captured:

WA: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode

RAN2 to agree that the maximum configured SMTCs per frequency for idle/inactive UEs is 6.

To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

During RAN2#131, the following agreements related to SMTC enhancements were captured:

The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs).

We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

In connected mode the NW will only configure up to 4 STMC.

UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

The procedure for UE assisted SMTC configuration in connected mode is follows:

- - For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

- - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

- - The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

- - The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

- - The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

## 5.2	TS 38.331 TP for E010

#### 5.3.13.4	Reception of the RRCResume by the UE

[…]

1>	if the RRCResume message includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state;

[…]

2>	if the UE is configured in this RRCResume message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;

[...]

#### –	RRCResume

RRCResume-v1900-IEs ::=             SEQUENCE {

assisted-SSB-MTC-Config-r19             SetupRelease {Assisted-SSB-MTC-Config-r19}                      OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE {}                                                     OPTIONAL

}

#### –	RRCResumeComplete

RRCResumeComplete-v1900-IEs ::=         SEQUENCE {

applicabilityReportList-r19             ApplicabilityReportList-r19                                             OPTIONAL,

referenceLocationReport-r19             BIT STRING (SIZE (6))                                                   OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                             OPTIONAL

}