3GPP TSG RAN WG2#132	R2-2509421

Dallas, USA, 17th - 21st, November 2025	was R3-258712

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1070 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sending GW release indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (CATT, Nokia, ZTE, Huawei, Samsung, Qualcomm) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - When a UE Xn handover to the target NG-RAN node from the NR Femto node is successful, the NR Femto node should send a UE CONTEXT RELEASE with the GW Context Release indication to the serving GW to allow the GW releasing the UE conext when needed. However, in 4.10.2.1, the functionality of NR Femto node for sending the NR Femto GW the UE CONTEXT RELEASE with the GW Context Release indication is missing. - In 4.10.4, the “IE” suffix should not be used in the note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 4.10.2.1, add the functionality of sending the NR Femto GW the UE CONTEXT RELEASE with the GW Context Release indication at successful Xn handover, to allow the NR Femto GW to release the UE context. - In 4.10.4, remove “IE” subffix for “cellReservedForOtherUse”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The functionality of sending the NR Femto GW the UE CONTEXT RELEASE with the GW Context Release indication at Xn handover for NR Femto node is missing. - Break the principle that “IE” suffix is not used in note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.2.1, 4.10.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - This CR was endorsed by RAN3 in R3-258712. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 4.10.2	Functional Split

#### 4.10.2.1	NR Femto node

An NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE 1:	The NR Femto node may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF;

NOTE 2:	NR Femto nodes may be deployed without network planning. An NR Femto node may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-		The TAC and PLMN ID(s) used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message;

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

-	Sending the UE CONTEXT RELEASE REQUEST message including a GW Context Release Indication to the NR Femto GW after the successful Xn handover initiated by the NR Femto node, in order to indicate that the NR Femto GW may release the UE context.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 4.10.4	Access Control

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE:		The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto physical node may activate a physical cell shared by both PLMN and PNI-NPN, as specified in clause 4.6, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>