3GPP TSG-RAN WG2 Meeting #90 	R2-152836

# Fukuoka, Japan, 25 - 29 May 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 1820 | rev | 3 | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on extended RSRQ range support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  |  | Date: |  | 2015-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CR 1696 introduced two features: extended RSRQ value range and new RSRQ definition. The parallel requirements on RSRQ measurement implied some dependencies between the UE capabilities: extendedRSRQ-LowerRange-r12 and rsrqOnAllSymbols-r12. Both of the capabilities indicate UE support for measurement configuration and reporting. Ideally, in practice the UE will support both of the features jointly. While reporting RSRQ according to the new definition on all OFDM symbols by the UE can be controlled by the network by measRSRQ-OnAllSymbols-r12 means, the reporting of RSRQ value from extended range remains unclear due to the split between upper range and lower range support. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RSRQ-Range Information Element description is updated by: - Adding clarification on that RSRQ value from any extended range is reported by the UE indicating support of extendedRSRQ-LowerRange-r12 or rsrqOnAllSymbols-r12. Along with Rev.3 of the CR: - Correction that requirements on receiver actions are not imposed on eNB implementations - Editorial correction of the sentence referring to mapping table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the change is not approved there is no UE behaviour on extended RSRQ range reporting specified. Impact analysis Impacted functionality: Extended RSRQ range Inter-operability: - If the network is implemented according to the CR and the UE is not the network will not benefit from the extended RSRQ range - If the UE is implemented according to the CR and the network is not there are no inter-operability issues foreseen |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 36.133 [16]. A given field using RSRQ-Range-v1250 shall only be signalled if the corresponding original field (using RSRQ-Range i.e. without suffix) is set to value 0 or 34. Only a UE indicating support of extendedRSRQ-LowerRange-r12 or rsrqOnAllSymbols-r12 may report RSRQ-Range-v1250, and this may be done without explicit configuration from the E_UTRAN. The UE shall use the value indicated by the RSRQ-Range-v1250, if received, and ignore the value signalled by RSRQ-Range (without the suffix).

# RSRQ-Range information element

-- ASN1START

RSRQ-Range ::=						INTEGER(0..34)

RSRQ-Range-v1250 ::=				INTEGER(-30..46)

-- ASN1STOP