3GPP TSG-RAN WG2 Meeting #93bis	R2-162242

Dubrovnik, Croatia, Malta, April 11th - 15th, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.302 | CR |  | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on sidelink related description in TS36.302 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-3-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - In Rel-13 eD2D, out of coverage discovery is introduced for public safety use case. It means for the UE supporting out of coverage discovery should support PSBCH. But currently, as stated in TS36.302, it is only mandatory for UE supporing sidelink communication. - The current description in section 8.3 does not consider the priority handling between Uu and sidelink during the Sidelink Discovery Gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PSBCH should also be mandatory for UE supporting out of coverage public safety related sidelink discovery. - The priority handling during the Sidelink Discovery Gap for Transmission or Reception is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions in TS 36.302 remain incorrect. Impact analysis Impacted functionality: No functionality is impacted. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is inter-operability problem. 2. If the UE is implemented according to the CR and the network is not, there is inter-operability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

8.3	Sidelink

The table 8.3-1 describes the possible combinations of physical channels that can be sent in parallel from UE perspective in the sidelink within the same subframe. Table 8.3-2 describes the possible combinations of physical channels that can be received in parallel from UE perspective in the sidelink within the same subframe.

Table 8.3-1: Sidelink transmission

|  | Physical Channel Combination | Transport Channel Combination | Mandatory dependent on UE radio access capabilities | Comment |
| --- | --- | --- | --- | --- |
| 1 | PSDCH | SL-DCH | Mandatory for UE supporting sidelink discovery | The UE supporting sidelink discovery transmits sidelink discovery messages on the camped cell (idle) or PCell (connected). |
| 2 | PSBCH | SL-BCH | Mandatory for UE supporting sidelink communication and for UE supporting public safety related sidelink discovery | The UE supporting sidelink communication transmits MasterInformationBlock-SL messages in PSBCH on one preconfigured frequency. |
| 3 | PSSCH | SL-SCH | Mandatory for UE supporting sidelink communication | The UE supporting sidelink communication transmits sidelink data in PSSCH on one preconfigured frequency. |
| 4 | PSCCH | N/A | Mandatory for UE supporting sidelink communication | The UE supporting sidelink communication transmits sidelink control information in PSCCH on one preconfigured frequency. |
| NOTE: Depending on the UE capability, the UE may be able to perform simultaneous Uplink and Sidelink transmissions. If the UE is unable to perform simultaneous Uplink and Sidelink transmissions, the UE prioritises the Uplink transmissions if there is no Sidelink Discovery Gap for Transmission. NOTE: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication transmissions (PSBCH or PSSCH or PSCCH) and sidelink discovery transmission (PSDCH). If the UE is unable to perform simultaneous transmission of sidelink communication and discovery, the UE prioritises sidelink communication transmissions if there is no Sidelink Discovery Gap for Transmission. |  |  |  |  |

Table 8.3-2: Sidelink reception

|  | Physical Channel Combination | Transport Channel Combination | Mandatory dependent on UE radio access capabilities | Comment |
| --- | --- | --- | --- | --- |
| 1 | PSDCH | SL-DCH | Mandatory for UE supporting sidelink discovery |  |
| 2 | PSBCH | SL-BCH | Mandatory for UE supporting sidelink communication and for UE supporting public safety related sidelink discovery |  |
| 3 | PSSCH | SL-SCH | Mandatory for UE supporting sidelink communication |  |
| 4 | PSCCH | N/A | Mandatory for UE supporting sidelink communication |  |
| NOTE: For sidelink communication, the UE shall be able to perform simultaneous Downlink and sidelink communication reception. For sidelink discovery, depending on the UE capability, the UE may be able to perform simultaneous Downlink and sidelink discovery receptions. If the UE is unable to perform simultaneous Downlink and sidelink discovery receptions, the UE prioritises the Downlink receptions if there is no Sidelink Discovery Gap for Reception. NOTE: If the configured resources for reception of sidelink communication and sidelink discovery are overlapped, the UE prioritises sidelink communication reception if there is no Sidelink Discovery Gap for Reception. |  |  |  |  |

Change end