3GPP TSG-RAN WG2 Meeting #94                              	R2-163419

Nanjing, China, May 23rd – 27th, 2016

Source:	CATT

Title:	New SDU Type for PC5-based V2X

Agenda Item:	8.2.4

Document for:	Discussion and Decision

Introduction

In SA2#114 meeting, a new PDCP SDU type “V2X-non-IP” was defined [1]. This contribution proposes to introduce this new SDU type into PDCP specification.

Discussion

In current PDCP specification, there are only three PDCP SDU types: IP, ARP and PC5 Signalling.

In SA2#114 meeting, a new PDCP SDU type “V2X-non-IP” was defined, and the description in TR23.785 is shown below [2]:

#### 6.2.1.3 User plane stacks Figure 6.2.1.3-1: User Plane for PC5 interface supporting V2X In this release, "V2X-non-IP" is only supported over PC5 link Legend: - PC5-U: The PDCP/RLC/MAC/PHY functionality is specified in TS 36.300 [XX]. - For PDCP SDU type "V2X-non-IP", stage 3 will define the next layer header in order to support multiple V2X application formats. NOTE: Stage 3 can decide if the "V2X-non-IP" SDU type is to be a new type, or to be merged with any existing SDU types, e.g. PC5-SP, also whether it should be generic for all "non-IP" data over PC5.

This new PDCP SDU type "V2X-non-IP" is used to represent the V2X non-IP data over PC5. Since this new SDU type is very different from thethree SDU types currently used, it’s better to introduce a new SDU type into PDCP specification. And from PDCP layer point of view, there is no need to differentiate the types of V2X “non-IP” data, therefore, the new PDCP SDU type "V2X-non-IP" can be generic for all V2X "non-IP" data over PC5.

Proposal: Introduce a new PDCP SDU type “V2X-non-IP” into PDCP specification (other than IP, ARP and PC5 Signalling), which is generic for all V2X "non-IP" data over PC5.

Conclusion

It is proposed to discuss and capture the following proposals at RAN2:

Proposal: Introduce a new PDCP SDU type “V2X-non-IP” into PDCP specification (other than IP, ARP and PC5 Signalling), which is generic for all V2X "non-IP" data over PC5.

References

[1] SA2#114 Chairman Notes.

[2] S2-162038, Open issues and resolutions for PC5 based V2X, Qualcomm Incorporated, LG Electronics.