3GPP TSG-RAN WG2 #94	Tdoc R2-164030

Nanjing, China, 23 – 27 May 2016

Agenda Item:	9.4.1.1

Source:	Ericsson

Title:	Targeted Protocol Latency

Document for:	Discussion, Decision

# Introduction

The discussions at the RAN2 meeting in April resulted in broad consensus that focus needs to be put on the user plane and control plane latency in order to meet the requirements agreed upon in [2] (Control plane latency of 10 ms; user plane latency of 4 ms for MBB; user plane latency of 0.5ms for UL, and 0.5ms for DL for URLLC services).

As discussed in [4], changes such as numerologies with shorter symbol duration, fewer symbols per subframe and reduced processing delays are key technology components for realizing lower end-to-end latency. In this contribution we provide more detailed explanations and examples of the targeted user plane latency.

# Discussion

## L1 & L2 Latency

In this section we show some example frame structures and time lines to get a better understanding of the anticipated and achievable latency.

Figure 1 depicts an frame structure with the LTE numerology (15 kHz subcarrier spacing; ~72 µs symbol duration; 7 symbols per subframe (reduction by factor 2 compared to LTE)). Such a configuration would enable large cells where a relatively long cyclic prefix and large timing advance are needed. Besides that, it allows sending the UCI feedback across several symbols (similar to PUCCH in LTE) and thereby to achieve a robust transmission.

Figure 1: DL HARQ decoding and uplink feedback (LTE numerology)

We assume that the data symbols within a subframe are filled “frequency first” and that the UE starts decoding the subframe as soon as it has received the first (few) OFDM symbol(s). In this example the UE sends an uplink reference symbol after the guard period and the uplink HARQ feedback in the subsequent symbol. In this example the eNB is able to schedule the HARQ retransmission or a new transmission in the same HARQ process after a gap of one subframe after the end of the UCI. With this numerology and frame structure an HARQ RTT of (21 x 72 µs =) ~1.5 ms could be achieved which would be a significant improvement compared to LTE.

Figure 2 depicts another frame structure with 7 symbols per subframe (reduction by factor 2 compared to LTE) and a symbol duration of ~18µs (reduction by factor 4 compared to LTE). Such a short symbol duration is primarily suitable for relatively small cells where the cyclic prefix can be reduced compared to LTE (preferably by the same factor as the symbol duration). In addition to the shorter symbol duration we assume that all UEs in the cell apply a relatively large timing advance. The intention is however not to compensate for long propagation delay but rather to shift the uplink subframe “into” the downlink subframe. Like in Figure 1, the UCI appears at the end of the UL subframe but due to the large timing advance this is in practice closely after the end of the preceding downlink subframe. As a consequence, there are another 4 symbol durations until the start of the subsequent downlink subframe which may be sufficient for the eNB to schedule the next (re-)transmission. With such a configuration, the HARQ RTT could shrink to 2 subframes which is equivalent to ~250µs in this numerology.

Figure 2: DL HARQ decoding and Uplink Feedback (60 kHz SC spacing)

Which of these examples can or should be applied depends on the use cases, on the deployment scenarios (e.g. cell size, coverage situation, latency requirements …) and on the feasible processing in UE and network. Even though RAN1 will discuss the frame structure and the frame structure above should only be considered as examples, it is important that RAN2 investigates the impact on the latency in and above L2 and develops user plane protocols that can utilize and support what the physical layer offers.

The layer 2 user plane protocol stack should support various numerologies and frame structures.

It may be argued that very low L1/L2 processing delays are challenging to implement for a UE. While we acknowledge this concern, we would also like to highlight that the accelerated processing allows dimensioning smaller L1/2 memory: As has been discussed several times in RAN2, UE and network need to offer sufficient memory to fully utilize the underlying link while waiting for (H)ARQ feedback. This “bandwidth-delay-product” increases with increasing data rates but decreases with decreasing L1/2 latency.

The required L1 soft-buffer and the L2 memory is determined by the bandwidth-delay-product. A reduction of the L1- and L2 RTT reduces therefore the required memory on UE and network side.

## Uplink Access Scheme

The UL scheduling was found to have significant impact on the latency and on the achievable (TCP) downlink throughput in LTE. The study item on latency reductions in LTE therefore suggested investigating means to enhance Semi-Persistent Scheduling so that the UE utilizes the pre-scheduled grant only if it has data to send. Besides that, it has been discussed to make use of a contention based uplink scheme where the eNB grants the same time-/frequency resources to multiple UEs and resolves the contention afterwards. This would allow larger time/frequency allocations for individual UE which could possibly enable higher peak data rates but which also bears the risk of collisions and hence delay spikes.

RAN1 and RAN2 should therefore also aim to enhance the scheduled uplink MAC by reducing the latency compared to what was achieved in LTE.

If the eNB can allocate dedicated resources with less delay, there will be less need for pre-scheduling- and contention based schemes.

Figure 3 shows a frame structure similar to the one in Figure 2. Here, the UE sends a dedicated scheduling request in the last 3 symbols of a subframe and obtains the corresponding DL grant in the subsequent subframe. That uplink grant becomes valid in the following uplink subframe. As can be seen, the delay for requesting the uplink resources is in the order of 10 OFDM symbols. Since we assume here an increase of the subcarrier spacing by a factor of 4 and a corresponding reduction of the symbol duration, the time from sending a scheduling request until transmitting the uplink data could be in the order of 180µs. This would be a significant improvement compared to the 8 ms currently enabled by LTE and may be sufficient for most applications. Similarly as above, one should note that this numerology and processing requirement will not be applicable in all deployments. However, it shows the technology potential and reveals how important it is to develop medium access schemes jointly between RAN1 and RAN2.

Figure 3: Dynamically scheduled uplink with reduced processing times

Reducing the latency for requesting uplink transmission resources (D-SR  UL Grant) improves significantly the TCP file download performance and maintains the  benefits from accurate scheduling and link adaptation.

For many use cases, dynamic scheduling with significantly reduced latency is a preferable over pre-scheduling and contention based uplink access schemes.

## Unlicensed Spectrum

As discussed in the context of LAA, low processing delay is vital for operation in unlicensed spectrum: If the UE needs too much time to process a downlink transport block, it may have to perform another LBT with exponential back-off before sending the uplink feedback even though the eNB had done such an LBT before sending the data and even though one would typically consider the feedback to be part of the data transmission. However, if the gap becomes too long, other devices/systems may consider the channel to be available and use it for their own transmissions. Therefore, significantly reduced processing and feedback delays will ease the operation in unlicensed spectrum as shown in Figure 4.

In unlicensed spectrum, a low delay between downlink data reception and uplink feedback transmission allows sending the feedback back-to-back in the same TXOP.

Figure 4: Downlink data and Uplink feedback in same TXOP with short processing delay

Besides the processing of downlink data, also the short uplink scheduling request-, grant- and transmission delays discussed in section 2.2 are beneficial in unlicensed spectrum. If the delay between reception of a scheduling request and the transmission of the uplink grant is sufficiently short, the eNB needs to perform only a short LBT to discover hidden-node problems. And subsequently, the UE may use the UL grant after a short LBT.

In unlicensed spectrum, a low processing delay of UL grants allows sending the corresponding UL data in the same TXOP without exponential LBT.

# Conclusion

Based on the discussion in section 2 we propose aiming for the following when designing the user plane protocol stack for NR.

Aim for a HARQ feedback delay in the order of the guard period

Aim for scheduling request  grant delay in the order of 1 subframe

Aim for Scheduling Grant  UL Data TX delay in the order of 1 OFDM symbol.

While the processing delays are of course also determined by the physical layer design (determined by RAN1), RAN2 should discuss whether enhancements to the L2 user plane stack are necessary and possible to achieve the anticipated low processing delays.

In order to achieve this low processing delay changes both to physical layer as well as to the user plane protocol stack may be needed.

Discuss whether it is necessary and possibly to enhance the L2 user plane protocol stack to enable the anticipated low latency.

# Text Proposal

Based on the discussion in section 2 we propose adding the following text proposal to the TR.

In order to fulfil the tight latency requirements captured in 3GPP TR 38.913, a careful trade-off between latency and other KPIs is vital for the design of the physical layer and the L2 user plane stack. From user plane protocol perspective NR should aim for 1) a HARQ feedback delay in the order of the guard period 2) a scheduling request to grant delay in the order of 1 subframe 3) a Scheduling Grant to UL Data TX delay in the order of 1 OFDM symbol..

# References

3GPP TR 36.881, “Study on latency reduction techniques for LTE”, v0.6.0, 2016-03-07, http://www.3gpp.org/DynaReport/36881.htm

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, v0.3.0, http://www.3gpp.org/DynaReport/38913.htm

R1-163226, Frame Structure for New Radio interface, Ericsson, RAN1-84bis, 2016-04-11

R2-162752, Latency in NR, Ericsson, RAN WG2 #93bis; Dubrovnik, Croatia, 11 – 15 April 2016

Time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE)

Time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point via the radio interface in both uplink and downlink directions, where neither device nor Base Station reception is restricted by DRX

Beginning of the first transmission attempt until the beginning of the earliest possible retransmission attempt